DOUBLE BAZOOKA/COAXIAL DIPOLE ANTENNA

-------------------------------------

by Morris Lundberg, K4KEF

Eighty meters is 500 KHz wide, from 3.5 MHz to 4.0 MHz.  Your

typical, standard dipole antenna will show a bandwidth of about

50 to 100 KHz.  That means that you'll have to resonate it at the

high or low end of this band, to have a 2:1 or less SWR over the

frequencies you wish to operate.  So you tune it to the CW or

Voice portion of the band.  Wouldn't it be nice to have an

antenna that was resonant in the center of the band and had an

SWR of 2:1 or less across the entire 80 meter band?

Enter the Coaxial Dipole Antenna (alias "Double Bazooka").

This antenna is broadband; it will cover the entire 80 meter

amateur band with an SWR of 2:1 or less.  The "Bazooka" antenna

was developed by the staff of M.I.T. for radar use.  The original

"Bazooka" used coaxial cable for the entire radiating elements.

The adaptation used by most amateur operators uses coax only for

the broadbanding portion of the antenna, while the remaining

portion of the elements are constructed of twinlead or ladder

line (see attached sketch).  Ladder line is preferable for its

inherent strength.

This is a single band antenna.  It will not radiate harmonics

of your operating frequency.  In addition, there is very little

feedline radiation, which is great for those who have problems

with TVI.  Its broadband characteristic makes it ideal for 80

meters and 10 meters.  On the other hand, a seperate antenna is

required for each band.  The Bazooka antenna consists of a half-

wavelength of coaxial line with the outer conductor opened at the

center and the feedline connected to the open ends.  The outside

of the coax and the ladder line operate as a half-wave dipole.

The inside of the coax elements, which do not radiate, are

quarter-wave shorted stubs which present a high resistive

impedance to the feed point at resonance.  Off resonance, the

stub reactances change in such a way as to cancel the antenna

reactance, thus increasing the bandwidth of the antenna.

In the attached sketch, the SWR curves are shown for two double

bazooka antennas; one for the 80 meter version and one for 40

meters.  Note that the SWR at resonance on 80 meters (3.75 MHz)

is 1.2:1 and that the SWR at the band edges is about 2:1.  The

curves were drawn from readings taken with the DAIWA CN-620B

power meter.  The antennas were installed at a height of about 25

feet (unfortunately, I have no trees for supports) and the axis

of the antenna elements were at 90 degrees with respect to each

other.  The 40 meter antenna used TV twin-lead for the ends of

the elements.  Both antennas used Tandy RG-58/U coax for the

broadbanding elements and the feedline.  Apparently the Tandy

coax I used had a velocity factor of 0.64, since the resonant

length turned out to be 84 feet on 80 meters (as compared to the

calculated 87 feet).  More expensive coax may have a more

consistent velocity factor.

At the very center of the coax used in the elements, very

carefully cut away about one inch of the outer vinyl jacket.

Then cut the exposed shield all the way around at the center of

the exposed area.  Be careful that you do not cut the dielectric

material or the center conductor in the process.  Twist the two

pieces of exposed shield into small pig-tails.  These are the

feed-point terminals for the antenna.  The center conductor of

the feedline is soldered to one and the shield of the feedline to

the other.  Now solder the center conductor and shield together

at each end of the antenna element.  Solder the two ladder line

wires to the end of the antenna element.  At the other end of the

ladder line, solder the two wires together.  The ladder line now

appears to the antenna to be a very thick extension of the

radiating element, contributing to broadbanding the antenna.

I've solved the mechanical strength problem by using a square

piece of plastic at the antenna center, drilling a small hole on

each side of the coax, wrapping a small wire around the coax and

through the holes and twisting the wire together on the other

side.  After this, a small amount of quick setting epoxy secures

the coax to the plastic support and prevents the wire from

untwisting.  Once the antenna elements and the feedline are

secured to the plastic square in this way, the plastic square

takes the strain, protecting the delicate radiating element

feed-point.  A thorough coating of silicone rubber or epoxy seals

and protects the feed-point from the weather.  A similar

technique at the point where the ladder line is soldered to the

shorted end of the coax provides strength and a weather-tight

seal.

I've constructed several of these antennas and they have all

performed as expected.  The quality of the coax used seems to

have little effect on the antenna's performance.  I've found that

Tandy coax will work well in this application.  RG-58/U was the

coax of choice for small size and light weight.  There is no

reason, however, that RG-8 coax wouldn't work as well or better.

It's larger surface area would probably provide better low

signal level reception.  Its large size would make it more

obvious to the neighbors and its larger weight might be a problem

in your installation.  Its larger diameter conductors would,

certainly, take more stress and strain than RG-58.  Eventually,

the outer black vinyl coating on cheaper coax will migrate into

the inner dielectric material, contaminating it.  As is true with

any cheaper coax, this changes the properties of the coax, i.e.,

impedance, loss and velocity factor.  Over the long term, the

exposure to the sun's untraviolet rays cause this contamination

of coax which hasn't been constructed to prevent it.  For that

reason, you may wish to use non-contaminating coax, such as

RG-141 or RG-213.  After several years of exposure to all kinds

of weather, however, my Double Bazooka's show little degradation

in performance, using Tandy RG-58/U coax (Radio Shack).

K4KEF.