SB-058

TR-2400 Tone Encoder RFI               12-14-79

In some cases, users may report RF interference, or motor-boating, when using a tone encoder in their TR-2400.  A secondary complaint of excessive tone level may also be encountered.

1.	For RFI, bypass the B+ terminal to ground with a 100 uf cap.

2.	For excessive tone input to the TR-2400, install a 100 K ohm resistor between the encoder and the TR-2400 Tone input terminal.

JEB/yn

(c) 121479TKC