SB-817

TS-180S Use with a Linear Amplifier                        3-20-80

1.   Fast Protection with a Linear:

For complaints of fast protection, such as when operating CW semi-

break-in with a linear, check C73 at the base of Q1 on the LPF unit X51-

1180-00.  If it is a 1uf, change to 3.3 uf, 25v or greater.  This will

compensate for slow linear relay closure.

2.   Premature Protection

The radio should protect at 3:1 SWR, or at 24W open (no) load.  To

test the protection circuit, load the radio at 24W into a know 500 ohm

dummy load through a Bird 43 wattmeter.  Disconnect the load from the

wattmeter.  The protection light should just lite.  Adjust the protection

pot VR1 on the LPF unit for protectio at 24W with no load.

3.   For complaints of VOX CYCLING, MEMORY JUMP, READOUT JUMP, or TONE

"BEEP" at key up or down with a linear amp, suspect linear induced relay

transients entering the radio via the ASSY relay wiring.

For example, DRAKE L4B linears before Serial Number 5500 do not have a

quench diode across the DC operated relay.  Install a 1N4005 or equivalent

across the relay in the linear to eliminate the back EMF induced by the

linear into the TS-180S.  Similarly, Henry linears also don't have a diode

across the relay and may give these symptoms when operated with a TS-180S.

JEB/yn

(c) 32080TKC