SB-941

SERVICE MANUAL RX SECTION LEVEL DIAGRAMS  10/28/88

INTRODUCTION

This bulletin will explain the development and use of

Kenwood Service Manual receiver section level diagrams.

Since they list typical RF and audio levels, the diagrams

provide the most efficient means of locating a faulty RX

circuit in a transceiver.

In the past, Kenwood has recorded RF levels in volts (V),

decibels (dB), and more recently dBu and dBm. Conversion

tables and an equation are provided at the end of this text

so you can change the RF levels to the form of your choice.

Using the level diagrams require basic test equipment and a

couple of test fixtures. One fixture is an 8 ohm non

inductive load that replaces the internal speaker. The other

is an injection probe. It is used to inject RF from your

signal generator into specific sections of the receiver.

Both fixtures are discussed at the end of this text.

HOW THE RX SECTION LEVEL DIAGRAM IS DEVELOPED

This section will describe how the level diagrams are made

for CW and SSB transceivers. It will then discuss the

different methods used for FM transceivers.

A working transceiver is tuned to its center frequency. For

HF rigs, this would be 14.200MHz. A Standard Signal

Generator (SSG) set to the same frequency is connected to

the antenna terminal of the transceiver. The SSG is set to a

level that will not allow the AGC circuits to function (no

S-METER reading). For HF rigs, this would typically be 0dBu

(0.5uV).

An 8 ohm load is inserted into the external speaker jack and

the AF gain control is adjusted to obtain 0.68V across the

load (measured with an AF VTVM). The AF gain setting is not

changed from this point on. The SSG is replaced with a 50

ohm load to terminate the antenna input.

An injection probe is connected to the SSG. The ground clip

is attached to the chassis of the transceiver. The probe is

placed at the first point listed on the level diagram just

past the antenna input. The SSG level is adjusted until the

audio output is again 0.68V. The SSG level is then listed on

the diagram as a typical reference for that point. This

process is repeated until all measuring points are assigned

a reference level.

NOTES

After each mixer stage, the SSG must be adjusted to the

respective IF frequency. As an example, after the first

mixer on the TS-940S, the frequency would be set to

45.05MHz. After the second mixer, the frequency would be set

to 8.83MHz.

The levels in the audio stage of the receiver are measured

with an AF VTVM. These measurements are made when the SSG is

connected to antenna terminal (just after the AF gain is

set).

FM RX SECTION LEVEL DIAGRAMS

FM transceiver level diagrams are made in the same fashion

as CW/SSB diagrams. The transceiver is tuned to its center

frequency (or close to it). A low level signal is applied to

the antenna terminal. The audio output is held constant

while the injection method is used. The differences  are as

follows:

On some FM transceivers, the injected signal is modulated

(typically a 1KHz tone at +/- 3.0KHz deviation is used).

Other transceivers are tested without modulation.

On some transceivers, the constant output is measured at

12dB SINAD. On others it is measured at 20dB of noise

quieting (NQ).

Each Service Manual level diagram contains footnotes that

describe the method used. Due to the variations and

available test equipment, it may be necessary to change the

initial set up and then write your own level diagram.

USING THE LEVEL DIAGRAM TO TROUBLESHOOT A FAULTY RECEIVER

It is not practical to set the audio output to a specified

level by injecting a signal into the antenna terminal of a

transceiver that has low sensitivity. In many cases, the

input signal would be so strong that it would attack the AGC

circuits. This would cause all measurements in the receiver

to vary from the typical levels in the diagram. The best

approach to this situation is to start at the end of the

receiver circuit and work towards the antenna.

Connect a 50 ohm load to the antenna terminal and an 8 ohm

load to the external speaker jack. Inject a signal at the

end of the IF stage. The IF frequency and required level are

listed on the Service Manual level diagram. Adjust the AF

gain control to obtain the specified audio level at the 8

ohm load. Do not change the AF gain setting from this point

on. Inject the next signal in the middle of the receiver

chain (again refer to the level diagram for the frequency

and level). If the audio comes up to the specified level,

the faulty component is somewhere between the antenna

terminal and the middle of the receiver. If the audio does

not come up, the bad component is between the two injection

points.

The next injection point would again cut the chain in half

in order to work towards the faulty component. For instance,

if the problem is between the antenna terminal and the

middle of the receiver, inject the next signal in the middle

of those two points. Continue in this manner until the

problem can be isolated to a small portion of the circuit.

NOTE

Do not forget that D.C. measurements around an isolated

circuit are necessary. For instance, an amplifier will not

have any gain if an open resistor cuts off the supply

voltage.

INJECTION PROBE

An injection probe is a coax that connects to your SSG, has

a 0.01uF capacitor on the center conductor of the coax, and

a tip connected to the other end of the capacitor. The

shield of the coax connects to a ground clip. Many

electronic supply shops sale injection probes. If you decide

to purchase one, make sure it contains a 0.01uF capacitor

between the tip and center conductor of the coax (used to

block D.C.). If you decide to fabricate your own cable, it

can be done with a few simple parts as shown in Figure 1.

8 OHM LOAD

An 8 ohm non inductive load is used to substitute the

internal speaker on the transceiver. The load is connected

to the external speaker jack for convenience. For the

purpose of troubleshooting a faulty receiver, it is not

necessary to purchase a precision 8 ohm resistor (also,

precision resistors are inductive). Six carbon composite 47

ohm, 1/2 watt resistors connected in parallel make an

adequate load. Figure 2 shows a typical 8 ohm load

fabrication.

DECIBELS

As mentioned in the introduction, Kenwood Service Manuals

have listed levels in V, dB, dBu, and dBm. Unless otherwise

specified in the footnotes on the level diagram, dB and dBu

levels are equal.

Aside from the conversion tables on page 6, a conversion

equation and examples are provided.

V(out)

EQUATION:   dB = 20 log --------

V(in)

In the equation, solving for dB would yield a value that is

the difference between the two voltages. In other words,

suppose you already know that 2uV = 12dBu and 15.8uV =

30dBu. You can see that the difference is 18dBu. Plugging

these uV values into the equation would yield the same

result. With this in mind, one can plug in a known voltage

from a conversion table, plug in a voltage not listed in the

table, and find the dB difference. To make the answer

simplistic, use 0.5uV for dB/dBu and 224mV for dBm. Since

they equal 0dB/0dBu/0dBm respectively, the difference is the

actual value of the unknown voltage.

Japanese "SSG"					American "SSG"

-6dB ------------------------------ 0.25uV

0dB ------------------------------ 0.5uV

6dB ------------------------------ 1uV

12dB ------------------------------ 2uV

24dB ------------------------------ 8uV

30dB ------------------------------ 15.8uV

40dB ------------------------------ 50uV

50dB ------------------------------ 158uV

60dB ------------------------------ 500uV

70dB ------------------------------ 1.58mV

80dB ------------------------------ 5mV

90dB ------------------------------ 15.8mV

100dB ------------------------------ 50mV

120dB ------------------------------ 0.5V

TABLE 1 (dB or dBu to V conversion)

Japanese "SSG"					 American "SSG"

-119dBm ------------------------------ 0.25uV

-113dBm ------------------------------ 0.5uV

-107dBm ------------------------------ 1uV

-101dBm ------------------------------ 2uV

-88.9dBm ----------------------------- 8uV

-83dBm ------------------------------- 15.8uV

-73dBm ------------------------------- 50uV

-63dBm ------------------------------- 158uV

-53dBm ------------------------------- 500uV

-43dBm ------------------------------- 1.58mV

-33dBm ------------------------------- 5mV

-23dBm ------------------------------- 15.8mV

-13dBm ------------------------------- 50mV

0dBm ------------------------------- 224mV

7dBm ------------------------------- .5V

TABLE 2 (dBm to V conversion)