SB-819

TR-2400 Volume Control Replacement                        3-20-80

When replacing the volume control in a TR-2400, certain precautions should

be observed to insure against inducing new problems to either the control

or the radio.

When installing a replacement volume control R05-2409-05, soolder the leads

to the potentiometer sparingly, to prevent flux or solder from running down

the tabs and into the joint between the tab and the deposited resistance

material.  If this precaution is not observed, future failure of the new

control is possible.

Secondly, when reassembling the radio, tape insulate the PCB ground pathe

directly adjacent to the power switch tabs on the volume control.  This

will prevent accidental shorting of the battery to ground, and loss of the

B+ RF choke, L101.

Note:     Reference page 2, Service Bulletin #808, issued 2/12/80 for a

picture of the PCB/Vol control insulation.

JEB/yn

(c) 32080TKC