SB-022

Service notes on TR-2200A                             10-31-77

TRANSMITTER

1.   No TX RF output:  Bad 2SC1169 Q9 on X56-1210-11.

2.   No TX:  Shield plate touching to circuit board under Q8 driver 2SC741.

3.   No TX:  Open D2 MI301 on X5501130-01.

4.   No TX:  Repair broken RCT line from crystal deck to TX oscillator.

5.   No TX:  Bad Q13 2SA562Y.  No T13 voltage on TX X56-1210-00.

6.   No modulation:  Bad D4 1s1658 on X56-1210-11.

7.   No TX:  Capacitor C21 lead touching to ground at Q4 on X56-1210-11.

8.   No TX:  No collector voltage to Q7 Q8.  Bad Q15 2SC735 Hi/Low control

for Q14 on X56-1210-11.

9.   TX distortion:  Check D3 Zener 9 volt supply regulator for TX low

level stages on X56-1210-11.

GENERAL Note On TR-2200A

1.   Battery plate overheats:  If the red and yellow leads of the external

DC cord are connected together, the battery plate will overheat.

RECEIVER

1.   Receive Sensitivity poor:  Bad 10.245 MHz xtal on X55-1130-11.

2.   Receive Sensitivity poor:  Bad MI301 D4 on X56-1080-00.

3.   Receive Sensitivity poor:  Bad Q1 3SK41L on X55-1130-11.

4.   Bo battery operation:  Miscontact of internal switch in power jack.

5.   No C9 B+ to receiver:  Bad Q12 2SC496 on X56-1210-11.

6.   Batteries will not charge:  Check for a shorted battery plate.

7.   Low Rx sensitivity:  Bad D3 1S2588.

8.   No squelch:  Check Q15 2SC458 on X55-1130-11.

9.   Poor squelch when cold:  Replace D14-16 1N60.

10.  No audio:  Bad qudio IC NPC575 Q16 on X56-1210-11.

(c) 102177TKC