SB-852

MC-46 DTMF Mic. With older equipment                       1-25-82

The MC-46 DTMF (Dual Tone Multi Frequency, or Touchtone (TM)) microphone

may be adapted for operation  with any VHF or UHF radio by changing the

supplied six pin mic connector to a five pin, four pin, or other connector,

as required.  Radios not originally supplied with regulated, filtered B+ at

the mic connector need also to be modified to power the microphone.

The MC-46 standard six pin mic connector is part no E07-0651-05, and is

wired:

Pin 1 White    Mic Output

2 Red      PTT

3 Blue     Down

4 Green    Up

5 Brown    B+ (8.5 VDC +/- 10%)

6 Shield

The Four pin connector part no is E07-0403-05.  Radios built with a four

pin jack do not supply B+ to operate the mic, and must be modified to

deliver a constant source of DC to operate the mic and DTMF keyboard.  Any

Kenwood product of this vintage will have a B+ line designated "C9" (Common

9V DC), wich is regulated, filtered, and noise-free.  Rewire the mic as

described below:

Pin 1 White    Mic Output

2 Red      PTT

3 Shield   Common Ground

4 Brown    C9

Rewire the radio mic jack: pin 3 to common ground, pin 4 to C9.

Rewire the stock mic for pin  3 common ground.  Otherwise, the mic element

will be returned to C9, instead of ground.  Note this change on the radio

schematic.

The Five pin connector part no. is E07--551-05.  Radios built with a 5 pin

jack supply correct B+ for a DTMF mic (originally designated MC-45, and no

longer available).  Wire the mic as described below:

Pin 1 White    Mic Output

2 Red      PTT

3 Brown    B+

4,5 Shield Common Ground

Mating chassis connectors are:

Four Pin       E06-0403-05

Five Pin       E06-0552-05

Six Pin        E06-651-05

JEB/sh

(c) 82481TKC