SB-046

TS-520/520S CW Frequency Shift                8-6-79

Reports of intermittent frequency shift during CW operation, on the order

of a few hundred cycles, may be traced directly to the Carrier Unit.

On the Carrier unit X50-0009-01, install a ground lead of #18 tinned wire

or similar, between the cases of T1 transformer and X3, the LSB XTAL.

Recheck frequencies after installing this ground lead.

Please annotate your service manual.

JEB/yn

(c) 8679TKC