SB-887

TS-430S Noise Blanker Improvements                            10-17-84

Some operators have indicated a desire to have their TS-430S noise blanker

operate similar to the noise blankers in the TS-120/130S series.  By

performing the following modifications the operator will be able to

increase the effectiveness of the TS-430S Noise Blanker Circuit.

Procedure:

On the RF UNIT (X44-1510-XX) change the following components.

R80 change to a 100 K ohm 1/4 watt resistor

R81 change to a 56 K ohm 1/4 watt resistor

C125 change to a 560 pF 25v disc ceramic capacitor

Note this is an optional change and may not be performed under warranty.

No realignment is required.

CLM

(c) 121589TKC