SB-047

TS-180S Increase of RX Audio Output                7-30-79

RX audio output from the TS-180S may be increased a total of 4.7 dB.  This

is an optional change which you can make at the owners request.

On the IF unit X48-1240-00 change components:

R206  2.2K change to 1 K ohn

R207  2.2K change to 4.7 K ohm

On the AF Gain Control, between pin 1 and 3;

C4 .047uF change to .027uF mylar

Please annotate your service manual.

JEB/yn

(c) 8679TKC