SB-901

TR-2600A PLL Unlock Troubleshooting                 10-18-85

The owners manual for the TR-2600A lists the lithium battery as a possible

cause of the display showing UL.  You may wish to check the battery, but

realistically on a new radio this is probably not the reason for the unlock

condition.

I.   If the radio showl UL when first turned on you should perform the

following test. This will help you to isolate the symptom:

1.   With the radio turned OFF, press and hold the 5 key, and then

turn on the radio.  Release the 5 key.  If the display shows a frequency

indication you may have a pad X2 on the TX unit.

2.   If there is no frequency display other than UL then proceed with

step II.

3.   You may also wish to check crystal X1 the Reference oscillator

(10.240 MHz at pin 16 or 17 of IC1.)

II.  There are several other points to check.

1.   Check trimmer capacitors TC$, TC5 and TC6.  If you press on the

trimmer and the UL dissappears replace the trimmer.

2.   Check pin number 18 of IC1, it should be 5.4 - 6 vdc.

a. If the voltage is over 6.5vdc replace Q17, Q18, and IC1.

b. If the voltage is less than 5vdc replace Q17 and Q18.

3.   Check the CLK signal at the flex pc board connector, and pin 10

of IC1.

a. The CLK signal should be aprox 4 - 4.4 vdc.

Pin 10 should be aprx. 0 - .2 vdc.

b. If pin 10 is high check Q16, and for cold soldering on the

flexible pc board between the keyboard and the RX unit.

4.   Check the alignment of the PLL section, as given in the Service

manual.

5.   Press the RESET pushbutton on the back of the radio.

Time required for this modification is 1/2 hour or less.

CLM

(c) 121589TKC