SB-874

TS-930S RF Feedback                            7-15-83

For reports of  RF feedback from the external speaker, especially when

using a vertical antenna, or a linear amplifier, check for correct

installation of capacitor C16 on the EXT SPKR jack.

C16 should go from ground to the white wire on the opposite side of the

jack.  Some units have the capacitor soldered to the middle terminal in

error!

Note:  This bulletin is applicable to radios with serial numbers prior to

3070420.

Time required for this change is 1/2 hour.

CLM

(c) 121589TKC