SB-942

SERVICE MANUAL TX LEVEL DIAGRAMS			  10/30/88

INTRODUCTION

Kenwood service manual transmit level diagrams provide

typical RF levels found in working transceivers. This

information provides a fast method of locating a faulty

circuit in a transmitter chain. To use the level diagrams,

an AF VTVM, RF VTVM, and 50 ohm resistive load are required.

In addition, since many units are tested with the drive

output terminated into the load, a coax cable must be

fabricated. A sample cable is shown below.

HOW THE TX LEVEL DIAGRAM IS DEVELOPED

The transmit level diagrams are developed in a similar

manner as the receive diagrams. Basically, a working

transceiver is tuned for a specified output power at center

frequency. The mic audio circuit is measured with an AF VTVM

and the RF circuit is measured with an RF VTVM. The results

are listed on the diagram as typical levels for a working

transceiver. The exact setup used to make a diagram varies

with the model. The setup details are listed in footnotes on

the diagram. No matter what procedure is used, the end

result is a list of typical levels for a working

transceiver.

For the most part, variations in setup include what mode is

used, if audio is applied to the mic input, and what

reference is used for power output. Additionally, many units

have the drive signal connected directly to a 50 ohm load.

This requires a fabricated coax cable that can be inserted

into the drive socket and attached to a load. A coax cable

with a terminal that fits the drive socket is available in

the Kenwood Parts Department. SB-905 lists the part numbers

of coax cables that are available in several lengths and

style. Once a cable is obtained, a connector that matches

your 50 ohm load can be soldered to the cable.

USING THE TX LEVEL DIAGRAMS

As mentioned above, certain units are terminated at the

drive output. This procedure can be used in troubleshooting

to determine if normal drive power is available. Simply

connect the fabricated coax cable to the drive terminal and

the 50 ohm load. If the load receives full drive power as

indicated on the level diagram, the trouble spot is in the

Final or LPF unit. In this case, reconnect the Final unit

and continue checking towards the antenna terminal. If low

drive is measured, continue measuring towards the mic input

until a faulty circuit is found.

NOTE

Once a faulty circuit is isolated, DC measurements must be

made to determine if normal supply voltage is present.