SB-812

TS-520, TS-820 CW filter select control                        2-14-80

In the TS-520SE, the heater switch has been replaced with a CW WIDE/NARROW

select switch.  For those operators who desire this type feture for their

transceiver, the following information may be considered as a guideline.

TS-520S series transceivers

1.   A minature SPDT switch may be installed at the operators preference,

on the rear side, or bottom panel.  Use this to switch the CW or SSB

filters by wiring the BROWN lead, which normally is transferred to the CW

terminal on the IF unit, between the SSB filter, for WIDE , and CW for

NARROW>

2.   A Traditional solution has been to use the FIX CH switch, position 3

and 4 for filter switching.  This may be donw by completely rewireing the

switch, or by leaving the switch wiring and the filter wiring as for normal

operation, and bridging the following terminals:

A.   FIX CH C terminal to IF SSB Terminal

B.   FIX CH 4 terminal to IF CW terminal

Placing the FIX CH switch to 4 will operate the SSB filter in CW

mode.

TS-820S Series Transceivers

1.   Similar to the TS-520 series, a miniature SPDT toggle switch may be

installed at the operators preference a the rear or on the obttom cover.

2.   The FIX CH switch, or Digital Hold switch may be rewired to control

the CW wide/narrow function

On the MODE switch, S6-1, the SSB filter control is GREEN, and the CW

filter control line is GREY.  Break the GREY line and connect it from the

MODE switch to the SPDT common terminal.   Connect the GREY line to the IF

unit ot one terminal, and bridge tht GREEN SSB line to the other Switch

terminal.

JEB/yn

(c) 21480TKC