SB-867

TS-930S SSB TX Tone Quality                                  3-2983

The transmitted audio tonal quality of the TS-930S may be improved by the

following procedure:

In the TS-930S Service Manual page 64, make the following change:

5. 100 W Final Bias VR2 adjust for 70 ma

VR1 adjust for 1.3A

Note:  This change is applicable to units with serial numbers prior to S/N

3080001.

Installation time for the procedure is 1/2 hour or less.

JEB/sh

(c) 121589TKC