SB-895

TS-430S PLL Unlock at High Temperatures (Revision)                 9-14-85

This service bulletin supercedes bulletin 891, which concerned low input

level to IC-16 as the radio warmed up.  This bulletin incorporates

information contained in Bulletin 891 plus additional information.

PROCEDURE:

On the Control Unit (X53-1290-XX)

Check transistor Q8.  If it is not a 2SC1815Y change it to this type

(Serial numbers 408XXXX-509XXXX).

For serial numbers before 408XXXX change R29 to 820 ohms and C41 to 100 pF.

In addition to the changes listed above check for the following signals:

(You must check these at a dial frequency of exactly 14.000.000 MHz)

TP-1 6.0v (L18 adjusts)

TP-2 70 MHz

TP-3 45.3 MHz

TP-4 6.35v (L10 adjusts)

Time required for this modification is 1 hour or less.

CLM

(c) 121589TKC