SB-870

TR-7950/30 RX Interference                            4-8-83

Some users have reported FM Broadcast or TV video carrier interference to

the receiver.  Normally the TR-7950/30 has excellent two signal

intermodulation characteristics.  For a reported case of Broadcast or TV

IMD the following modification may reduce or eliminate the interference.

Parts Required

Qty  Description

1    MI-308 Diode

Procedure

1.   REmove the top and bottom covers.

2.   Remove the 4 screws that secure the heat sink assembly to the rear of

the set.  Gently pull the heat sink assembly to the rear, to allow access

to the final unit X54-1270-1-,11.

3.   Remove the 5 screws securing the PC board to the heat sink.  Remove

the 2 screws securing the final module (M57733/M57726) and remove the screw

that secures Q2 to the heat sink.  Carefully desolder L9 from the antenna

connector. (Dont change the spacing of the coils!!!)

4.   Gently fold the assembly over to allow access to the foils side of the

final assembly and remove D2, and replace it with the MI-308 diode.

5.   Reverse steps 1-3 to reassemble.  Reconnect L9 but don't change the

spacing of the coils.  Use heat sink compound when replacing the final

module.

This is an optional change and may not be performed in warranty.

(c) 121589TKC