SB-871

TR-9130 Low Drive Level                            7-05-83

For reported cases of low transmit power output check the level of the TX

driver transistor Q7 of the Drive unit (X$7-1090-00).  If the TX drive

level is low make the following changes.

Old Part            New Part

Q7   3SK97               3SK122(L)

R36  100 ohms            68 ohms

Time required for this change is 1/2 hour.

CLM

(c) 121589TKC