SB-909

TS-940S AVR Unit Capacitor Change                 4-8-86

In those areas of the country that are subject to large poer line voltage

fluctuations, excessive voltage may be applied to capacitor C12 on the AVR

Unit (X43-1500-00).  These voltage fluctuations might exceed the capacitor

breakdown voltage.  Replacing capacitor C12 with a capacitor having a

higher breakdown voltage will help prevent this potential problem.

Procedure:

On the AVR Unit change C12 from a 6800 uF capacitor to a 5600 uF 25V

capacitor (C90-2037-05).

Time required for this modification is 1 hour or less.

(c) 121589TKC