SB-804

TR-7625 Intermittent TX                           2-7-80

When servicing a TR-7625 for mechanically intermittent TX, check the heavy

WHITE DRB lead at the push-on connector directly in front of relay RL1 on

the RX TX unit.  This post may either become seperated from the land, or

the land may crack from vibration.  Resolder the post, plus reinforce the

connection with a small piece of solid wire soldered on the foil side of

the board.

JEB/yn

(c) 2780TKC