SB-872

TS-930S CW Pitch Tone Shift                            7-06-83

Some users of the TS-930S have reported a slight change in the CW pitch

tone when the MONI switch is turned ON and OFF.

This may be corrected by changing the following components:

On the Signal unit (X57-1000-XX) change R826 from a 1 K ohm resistor to a

10 K ohm resistor and remove C601, a .022uF capacitor from the foil side of

the circuit board.

Notes:  This change is applicable to units before serial number 3070221

only.

Time required for this change is 1/2 hour.

CLM

(c) 121589TKC