SB-888

TS-430S Intermittent PLL Unlock in FM/AM                          10-17-84

Some owners of the TS-430S have reported intermittent loss of transmit and

Receive when operating in the FM or AM modes, along with a loss of the

digital display.

The cause of this may be due to a loss of the FM/AM Heterodyne Oscillator

signal.  Replacing the components listed below will correct most instances

of this failure.

Procedure:

ON the Control Unit (X53-1290-XX) change R131 to a 22 K ohm 1/4 watt carbon

resistor, and change C144 to a 3 pF 25v Disc ceramic capacitor.

Time required for this modification is 30 minutes.

No realignment is required.

CLM

(c) 121589TKC