SB-900

TS-940S PLL Unlock                 10-18-85

Some users of the TS-940S have reported a blanking of the display

accompanied by a loss of transmit and receive.  Re-adjustment of the PLL

unit will correct this tendency.

PROCEDURE:

On the PLL unit

1.   Set the dial frequency for approximately 1.8 MHz (inside the band)

2.   Using an RF probe at TP5 adjust L22, L23, and L24 for a maximum

reading on the meter.  You should see approximately 250mv.

3.   Adjustment of L24 will produce the greatest change, which may be up to

a 90 degree adjustment from its present position.

Time required for this modification is 1/2 hour or less.

CLM

(c) 121589TKC