SB-816

TS-120S Band PAss Filter Change                        2-7-80

For complaints of out of band interference, especially BCI, the BPF

(Bandpass filter) terminating resistors may be changed for an improvement

of approximately 10dB.

On the RF unit X$$-1260-01 change R3,5,7,9,11 from 1.5K ohm to 470 ohm.  No

adjustments are required.

JEB/yn

(c) 2780TKC