SB-878

TS-530S IF Unit Q2, Q7 Protection                            8-25-83

High voltage transients at the receiver antenna can be felt in the ALC

circuitry and cause a failure of IF unit transistors Q2 and Q7.  Symptoms

encountered bay be a loss of RX sensitivity, and or loss of S-Meter

deflection.

Add the surge suppressor to the AF unit (X49-1150-XX) between terminal FG

and ground, to prevent a reoccurance of this symptom.

Parts required:

Surge Suppressor ERZD03DK331

Time required for this modification is 1/2 hour or less.

CLM

(c) 121589TKC