SB-896

TS-940S LCD Clock Display Erratic Operation                 9-14-85

We have received a few reports concerning erratic operation of teh LCD

clock display when the Main dial is rotated, or the VBT or SLOPE TUNE

controls are operated.  This appears as if you had pressed the SET, SCROLL,

or GRAPH switches.

This erratic operation may be caused by improper timing of the input pulses

applied to the auxiliary display microprocessor, IC1.

1.   This symptom is easily corrected by replacing IC2 MBM2764-30T1 with

MBM2764-30T2, on Digital Unit A.  Early models of the improved IC will

carry the old nomenclature but will be distinguished by a white dot on the

top of the chip.

2.   After the IC has been changed reset the main microprocessor by

pressing and holding the A=B key, and then turning the power switch OFF and

then back ON.  Then release the A=B key.

3.   After this change is made the sub-display will not immediately track

the main display.  A slight delay has been introduced int he new IC to

prevent any timing errors.  It may take approximately 0.5 seconds for the

LCD to track the main display.

4.   Please return the old IC2 to the Service Department along with the WSR

for Credit.

Perform this modification only if the symptom occurs, as a repair.  This

change is not required otherwise.

Time required for this modification is 1 hour or less.

CLM

(c) 121589TKC