The Sony ICF-2010

------------------

by Sheldon T. Hall

Before I tell you about the setup I have for my Sony ICF-2010, let

me tell you that my experience really does not support my making

suggestions or in any way taking on the role of "guru" in this

matter.  I've been in and out of SWLing for about 30 years, ever

since I built a HeathKit AR-3 receiver I got as a Christmas

present, but I've never had much of a radio, or really done much

listening, until recently.

When I bought the 2010 I justified the expenditure to my

Danish-born wife on the basis that she would be able to hear the

occasional broadcast from Denmark, and I was quite chagrinned to

discover that the 2010 would barely pick up Radio Danmark's

pipsqueak transmitter even with the wire antenna Sony supplies.

Thus, I justified the purchase and construction of a "real" antenna.

My antenna is now the $8.95 Radio Shack SWL antenna kit, erected

full length (about 75') between two trees about 65' up.  The

antenna runs ENE-WSW, which is a little askew (my major interest is

Europe) but the trees are too big to move.  Worse still, we are on

the southwestern slope of a small ridge, so even the 65' altitude

of the antenna does not give me a clear shot to the North and East.

We can pick up Radio Danmark with it, though.

The antenna is suspended by high-strength nylon twine, the twine

running through small sailing-dinghy blocks (pulleys) at both ends,

suspended weights being used to tension the antenna yet allow the

trees to sway without putting undue stress on the rigging.  The

blocks are themselves attached to some more twine thrown over the

trees by the simple expedient of tying it to a spare auto-battery

cable clamp, spinning same at the end of a short length of the

twine, David-style, and pretending the trees were Goliath.  This

was remarkably effective, as the weight cleared the tops of the

trees by quite a few feet.

This arrangement has stayed aloft during several good breezes, and

has allowed me easily to lower the antenna for modifications and to

remove a small branch lodged in it during a storm.

Since the antenna's insulators and the blocks are just within the

foliage crowns of the trees, the antenna is quite inconspicuous; it

would be more so if I dyed the twine so its whiteness didn't show

against the largely grey-green colors of the pines and oaks that

march up the ridge.

Although the antenna itself has been the same since I put it up,

I've connected it to the radio several different ways, and I think

the way I have it now is the best I've tried.  I'm quite willing,

however, to admit that my knowledge of electronics is fairly

spotty, and my understanding of RF in general and antennas in

particular almost nil.  To round out my ignorance, I don't have a

piece of electronic test equipment to my name, so any claim I make

as to "best" must be understood to be entirely subjective.

VERSION 1.0

My listening room is on the opposite side of the house from the

electrical service attachment, but only about 20' from the house's

main fuse box, and 10' from the washing machine, the outdoor half

of our heat-pump/air-conditioner, the cable-TV connection, and the

telephone terminal box.  I also have three computers, two modems,

and a printer in the listening room.

So, brainwashed by all the impedence-matching cautions in the

various antenna handbooks, and hoping to subdue any electrical

noise loose in the vicinity, I originally used RG-58U coax as a

lead-in, connecting the center conductor to the antenna (at the ENE

end) and leaving the shield unconnected.  The connection was

carefully wrapped to prevent the shield from absorbing water.  The

coax dropped vertically toward the listening room, clearing the

gutter and assuming a catenary curve to the window, where it came

in through a small hole drilled in the storm window frame.

I've always been told that the antenna is only half of the signal

path (the ground being the other), so I have a galvanized iron

strap, about 1.5" x .125" and 5' long, driven into the

perpetually-damp red Georgia clay just under the listening room

window, and I led a solid wire clamped to the strap up to the

aluminum storm window frame and attached it.  The shield of the

coax was also attached to the window frame, and thus to ground.

The coax was attached to a mini-phone-plug in the conventional way

and plugged right into the 2010.

The result was a tremendous overload on the 2010, until I put the

Antenna Attenuator in the 'local' position, when I got pretty good

results, logging numerous out-of-the-way countries and any major

broadcaster who happened to be on when I listened in.

Then someone pointed out that this rig had no lightning protection,

and that the 2010 is very sensitive to static build-up in outside

antenna; too much and the FETs in the front-end go 'poof.'

Another kill-joy told me that antenna impedence really doesn't make

much difference for receiving, and that a long-wire is high

impedence anyway.

A third well-meaning meddler mentioned that signals arriving here

in Atlanta, regardless of their original polarization, were not

really polarized any more, the various skips and bounces having

thoroughly scrambled the polarization along the way.  He suggested

that an unshielded leadin might be considered a vertical antenna.

My wife suggested that I invent some arrangement that might let me

close the window all the way.

All of which led to ...

VERSION 2.0

I replaced the coax with the unshielded leadin wire from the Radio

Shack kit, suitably lengthened with some hook-up wire of about the

same diameter.  It follows the same path as the coax, and comes in

through the coax-sized hole in the storm-window frame; I used a

rubber grommet to protect the wire from the sharp edges of the

hole.

The leadin is terminated at a barrier strip (also from Radio

Shack), as is the ground wire, and a 2.2 megOhm resistor connects

the leadin to ground to drain off static electricity.

Since the barrier strip is between the storm window and the sash,

and since this is all (reputedly) high-impedence stuff, I used

three feet of 300 Ohm TV twin lead from the barrier-strip antenna

and ground connections to the radio.  The twin-lead is thin enough

that I can close the sash.

The twin-lead ends at the high-impedence side of a 300 Ohm to 75

Ohm television balun coil, whose 75 Ohm side is a mini-phone-plug

that plugs right into the 2010.  The balun is marked Sony, so I

suppose it came with one of their small TV sets.  They probably

have these at Radio Shack, too.

I'd like to think that the balun provides both impedence matching

and static-electricity isolation services.  It doesn't seem to

decrease the signal level.

Compared to version 1.0, this setup seems provide slightly more

signal with no more noise than before, allowing me to run the

2010's RF gain control at half-mast most of the time, lopping off

much of the noise and some of the QRM, yet letting the broadcast

signals through.

I'm sure this whole setup flies right in the teeth of all that is

sacred in Antennadom, but I'm quite pleased with it.  I think the

signal strength is more than enough, the noise level is low on most

bands, and I can just sit back and listen.  I've logged stations

on all continents except Antartica, including domestic services in

the USSR and Africa, and some real low-wattage peashooters in South

America.

As is so often the case, I think I'm the limit, not the hardware.

-Shel Hall [76701,103]

SysOp, CompuServe Gulf Crisis forum.

Here are the specs on the Sony, adapted from a file I found on a local

BBS:

Sony ICF-2010 Specifications

----------------------------

Circuit system: FM:      Superheterodyne

AIR/AM:  Dual conversion superhet

Quartz controlled PLL frequency synthesis.

Frequency range: AIR     116-136 MHz

FM      76-108 MHz

AM      150.0-29999.9 kHz

Aerials:  AIR/FM/SW      External Telescopic

MW/LW          Built-in ferrite/coil

External terminal for AIR/FM

External terminal for AM

Audio: 380 mW at 10% harmonic distortion.

Output: Built-in speaker

Disconnected when earphone plugged in

Recording output jack (minijack)

Level:       0.775 mV (-60 dB)

Impedance:   1 kohm

Earphone output (minijack):

Impedence:   8 ohms.

Stereo output on FM

Mono output on other bands

"Walkman"-type headphone compatible

Power Requirements: Radio 4.5V (Three D size batteries)

Computer/clock 3V (2 AA size)

Supplied ac power adapter for radio (4.5V) operation.

Features:

*  AM has 5 modes: Wide, Narrow, Sync, USB, LSB/CW. Sync is the synchronised

operation which has been mentioned before. There are two advantages using

this:

(1) It can generate a carrier for weak signals subject to fading,

this carrier is synchronized to the original carrier and improves

reception for weak signals (really does work!).

(2) It can eliminate interference from adjacent stations. This can be

achieved by tuning to either the USB or LSB of the station being

received, whichever is not being subject to the interference.

Wide/Narrow I think uses IF filters to change the bandwidth of the

tuned signal. Useful in crowded bands for weak signals.

* Tuning achieved through rotary manual tuning knob, direct digital entry,

memory recall and scanning. AM has a 0.1 and 1 kHz resolution for manual

tuning. FM has a 0.05 MHz resolution and AIR has a 0.025 MHz resolution.

In addition for scanning, other scanning intervals are defined. There is

also a 9 kHz / 10 kHz switchable scanning interval for the MW band.

The manual tuning knob can also be electronically locked.

* Scanning has too many features to list like band skipping, station skipping

defined frequencies scanning, two scan types, memory scanning functions

and so on.

* 32 memory presets possible which stores all characteristics of the station

including both the mode and frequency.

* Timer: Sleep timer as well 4 programmable prioritised timers which can

be programmed to 0, 15, 30, 60 minutes of operation (0 min disables the

timer). The station must be stored in any of the 32 memory locations.

* Other features:

DX/Local switch and RF gain control.

12/24 hour clock

Battery check

Signal strength indicator

Sync indicator (USB, LSB)

Tone selector (High, Low, News)

Volume control, of course!

Display light

* Accesories:

Supplied:

AC power adapter

Single earphone

Shoulder strap

SW long-wire aerial

2 External aerial connectors

Wave handbook

Memo sheet and info sheet

Optional:

Car battery cord DCC-127A

Connecting cord RK-69A

VHF aerial AN-3

LW/MW/SW wide range aerial AN-1

Price: approx $290 - $310 (mail order) - $395 list.