MSYS

(C) Copyright 1992 By HUB COMPUTERS, INC.

INTRODUCTION

MSYS  is a copyrighted program that I am making available to  the

amateur  radio  community  free of charge.  Source  code  is  not

generally  available (if you want it you will have to pay  dearly

for it!).

If you happen to see my system on the air don't get excited  when

you  see that I am running the next numbered version of MSYS.  It

doesn't mean its ready for distribution! I go to the next  number

on my system as soon as I release a version.

KA-NODe is a trademark of Kantronics Inc. The K NODe component of

MSYS is compatible with the Kantronics KA-NODe but is  completely

different code from that used in the Kantronics tncs.

NET/ROM  is  a  trademark  of SOFTWARE  2000.  The  network  NODe

component  of MSYS is compatible with NET/ROMs but is  completely

different  code  from  that sold by SOFTWARE  2000.

The   network  NODe component  of MSYS is  completely   different

from that in the net.exe program and  G8BPQ NODe software.

IBM is a trademark of International Business Machines Corp.

MSYS  is available from the Cleveland Hamnet phone  BBS.  Numbers

are 216-942-6382 and 216-942-7516. Both are 300 to 2400 baud,  at

least!  This is a free bbs (you pay for the call). MSYS may  also

appear on other phone bbs systems, but this is the only one I put

it on directly.

You  may also obtain MSYS from me directly on disk (this  may

be  the  cheapest way really and the fastest when a  new  version

comes  out).  For US addresses the cost is $5 per  version  which

includes  the  disk, mailer and postage. You  may  order  several

versions at a time,  I will hold your request until the  versions

are  released. This  way you get to be the first kid on the block

with   the   new  toy!  When  ordering from  me,  please  specify

which  version  you already have or which one you want. Distribu-

tion  is on 5.25  inch disks.  I keep records with the  following

information  for  people who order msys, so you may want to  give

me  this info. Also please indicate  if  you want me to call  you

(collect) if  it  would  be helpful to answer questions you  send

me about MSYS.

CALL:                    Your callsign. This is the key I record

all the information under

NAME:                    Where to send the disk to!

ADDR1:

ADDR2:

CITY:

STATE:

ZIP:

COUNTRY:

TELEPHONE:           If you want me to call you back (collect)

VERSION:                 The version you have now

OWED:                    How many you have prepaid

COMPUTER:                Speed, memory,disk space, cpu chip etc.

TNC:                     Model & rom versions

DOS:                     Which version?

BBS:                     BBS callsign to send you msgs at

IP:                      Your IP address, if you have one

Mail requests for msys on disk to:

Michael Pechura

10809 Beechwood Drive

Chesterland, OH 44026

You may call me with questions/comments at 216-256-1588.

Canadian  stations  may want to contact VE3WZL  for  distribution

info in Canada.

ADDR:

J. B. Weazle McCreath

Aerial Acres

R.R. No. 4,

Goderich, Ontario, Canada

N7A3Y1

Ordered from me the cost is $7.50.

Other  foreign  stations:  Price is $10 (US)  per  version  which

includes Air Mail postage.

A  new stripped down version of MSYS that supports only  the  BBS

functions is now available. There is no network node, no dx node,

no amtor, no tcp/ip in this version. You can still forward  using

external network nodes and the K node is still there. All of  the

bbs  functions remain, along with modem support. Expanded  memory

is  also  still supported. This version is called  MSYSB  and  is

available  on a separate disk or on Cleveland Hamnet. If want  to

get this disk from me, it is the same

price  as  the others: $5 to US addresses, $7.50  to  Canada  and

Mexico,  and $10 to other countries.

A Multi-User, Multi-Port, Multi-Function

Packet Radio BBS

By Mike Pechura, WA8BXN

(C) Copyright 1992 By HUB COMPUTERS, INC.

All rights reserved.

Version 1.13 February 1992

MSYS Documentation Table of Contents

SECTION - 1

MSYS Installation Instructions                            1

MSYS.OPT                                                  3

DRSI CARDS                                                9

SECTION - 2

MSYS Command Summary                                     10

MSYS SYSOP Commands                                      17

SECTION - 3

MSYS BBS Commands                                        43

REQxxx SERVERS                                           51

DATABASE HELP                                            53

SECTION - 4

Forward File Description                                 55

Hierarchical Forwarding and MSYS                         66

SECTION - 5

MSYS Network NODe Introduction                           69

MSYS DX Node                                             78

MSYS General information file                            83

SECTION - 6

MSYS Utility Support Programs (MUTIL.EXE)                95

SECTION - 7

EDITVIEW (ED Command)                                    126

EDITFILE (EF Command)                                    127

SECTION - 8

HOUSECLEANING (HC command)                               129

MODEM Support                                            131

Sysop commands related to TCP/IP                         133

Samples of MSYS files                                    137

BIT PATTERN CHART                                        140

MSYS AMTOR Interface                                     141

Frequent problems and notes from the author              143

INDEX                                                    147

MSYS Installation Instructions #

(c) Copyright 1992 by HUB COMPUTERS, INC.

Installation  of MSYS is not particularly difficult but  you

should follow the following steps in order.

1.  Make a backup copy of the distribution disk. You can copy

the entire disk with Diskcopy.

2.  Make a default directory on your hard disk. I call mine

MSYS. Use the command:  md c:\msys

3.  Go to the default directory: cd c:\msys

4.  Make required subdirectories: md c:\msys\mail

md c:\msys\files

md c:\msys\help

md c:\msys\mail.bak

5.  Un-arc the distribution disk.

6.   To  move  the help files to the  help  sub-directory.   Type

MOVEFILE

NOTE:  REQUIRED FILE:  HELP\MSYSMSGS.DAT

This file contains a number of the seldom used messages produced by

MSYS.

8.   Change  Directory  to SAMPLES

Edit  the file  c:\msys\samples\msys.opt with  your  favorite

program   editor. See documentation on setup for details.   After

editing move msys.opt file to the default directory.

9.  RUN SETUP (MUTIL.EXE Function 9) to create c:\msys\msys.def

10. Make sure you have at least

files=20  in your config.sys file in the root directory  of  your

boot disk.

11. Using any ASCII editor, create MSYSFWD.DEF. If you don't know

what to put it in now, put the following (to be replaced later):

F0 WA8BXN

WA8BXN

--------

12. Type MSYS and it should begin to run!

13. Type MERge. You can ignore any messages about MSYSBBSB.DAT at

this time.

*** Special Note

Determining  the  type of monitor you have: If you have  a  color

monitor, the default is ok. If you have a monochrome monitor that

uses  and  RCA  phono pin plug then you also are  using  a  Color

1-1

Graphics Adapter interface card and the default should be ok.

If you can do the command MODE MONO (this is a DOS command) and

not  get any error messages and the computer continues  to  work,

you  probably  have a monochrome (TTL) interface card.  You  will

need to specify WINDOW DISPLAY MONOCHROME in your MSYS.OPT file.

MSYS command (typed at DOS prompt)

NOHEARD    -  disables JK,JM,JD,JG,JB and JT recording.

Saves 26,880 bytes

NONODE     -  disables network node, saves 28,104 bytes

NORAMHDRS  -  stores msg hdrs on disk only, saves 136 bytes per

possible msg. This can be quite slow if you have

lots of msgs and no disk cache. Could be useful on

286 systems with cache in EXTENDED memory.

Example: MSYS NOHEARD NONODE NORAMHDRS

IF MSYS DOESN'T SEEM TO BE WORKING PROPERLY, REMove ALL  (EVERY

LAST  ONE!)  RESIDENT PROGRAMS (INCLUDING YOUR ENTIRE  CONFIG.SYS

AND AUTOEXEC.BAT FILES AND SEE IF IT WORKS THAT WAY. IF IT  DOES,

PUT THINGS BACK IN ONE AT A TIME TO SEE WHERE THE PROBLEM IS!

1-2

MSYS.OPT

(c) Copyright 1992 by HUB COMPUTERS, INC.

Before MSYS can be used a configuration file (MSYS.OPT) must

be  created and the program SETUP (MUTIL.EXE Function 9) must  be

run to  create  the file MSYS.DEF. The file MSYS.OPT is a  stand-

ard  ASCII file and may be created with any program  editor.  The

basic format of the  file is:

PORT number AT number INT number SPEED number [NAME description]

PORT number SUBPORT number OF [PORT] number [NAME description]

DIGI callsign FROM [PORT] number TO [PORT] number [AS callsign]

NUMCHANS number

BBS       CALL      callsign

BBS       ALIAS     callsign

KANODE    CALL      callsign

ANSWER    CALL      callsign

FORWARD   CALL      callsign

ID        CALL      callsign

ID        EVERY     number

AX25      FRACK     number

AX25      MAXFRAMES number

AX25      RETRIES   number

AX25      CHECK     number

MONITOR   IFRAMES   on/off

MONITOR   UFRAMES   on/off

MONITOR   SFRAMES   on/off

MONITOR   BFRAMES   on/off

MONITOR   OUTGOING  on/off

MONITOR   PORTS     mask

WINDOW    number    SIZE number number number number

WINDOW    number    BACKGROUND number

WINDOW    number    FOREGROUND number

WINDOW    number    PAGE number

WINDOW    UNUSED    BACKGROUND number

WINDOW    UNUSED    FOREGROUND number

WINDOW    DISPLAY   color/monochrome

PROCESS   processname [WINDOW] number

STACK     processname number

STACK     FORWARD 8000

STACK     DECODE 1500

STACK     CALL 4000

SYSOP     NAME      name

SYSOP     QTH       location

SYSOP     ZIP       zip code

MAKE

Notes:

1. Items enclosed in square brackets ([...]) are optional

2. Numbers assumed in decimal unless prefixed by $ then hex

is assumed

3. on/off means the word ON or OFF

4.  mask is the numeric representation of a bit string  with

the rightmost bit corresponding to port 0. 1=to be

monitored.

1-3

5. Where more than 1 space is shown, 1 or more space are ok

6. Callsign can include SSID (-number)

7. Ordering of the statements is generally not important

8. If there is an * at the beginning of a line it is ignored

9. The bbs alias, Kanode, Answer and  Forward calls must  be

different calls or SSID's.

Output  from the execution of SETUP is the file MSYS.DEF  (a

binary file) and a listing to the display console (which  may  be

routed   to  the printer via redirection:  SETUP   >LPT1:).   The

listing  consists  of two parts: the  original  input  statements

followed  by a complete list of all options generated  (including

defaults).

If  any errors are detected, the MSYS.DEF file will  not  be

created. A sample of a MSYS.OPT file is found in MSYS.OPT on  the

distribution disk.

Explanation of each statement type

-----------------------------------

PORT number at number INT NUMBER SPEED number [NAME description]

PORT number SUBPORT number OF [PORT] number [NAME description]

ThePORT statement has the above two variations. The   number

following the word PORT is the port number. It must be between  0

and  5.  The  version containing the word AT  defines  an  actual

serial  interface  while  the SUBPORT version  defines  a  second

logical port for multiport TNCs such as the KPC-4. There must  be

one PORT statement for each port to be used. Assign port  numbers

beginning with 0 in increments of 1.

Following the word AT is the base I/O address of the 8250 in

the  serial  port, such as $3F8. Following the word  INT  is  the

interrupt number for the 8250, such as 4. After SPEED is the baud

rate to be used, such as 1200. Optionally the word NAME  followed

by some description of the port can be given. A good  description

would be the corresponding radio frequency.

For  the  other  form  of the  PORT  statement,  the  number

following  the  word  SUBPORT  is  the  logical  channel  number,

typically  1  for  the  KPC-4. Next comes  the  word  OF  in  the

statement  optionally followed by the word PORT. Next  comes  the

port  number  of a port statement defining an  actual  port  (ie,

contains  the word AT in its definition). The optional  NAME  and

description is the same as above.

PORT description lines can also have two additional parameters:

RADIOSPEED bitrate# TNCTYPE #

RADIOSPEED  specifies  the bitrate used by the TNC to  the  radio

(300 or 1200 probably). If 300 bps is selected, shortened prompts

are used in the BBS and perhaps elsewhere.

TNCTYPE  is  a number from 0 to 999. Tnc type 0 is a  KPC-x.  Any

other  number causes a file MSYSTNC.# to be read for  initializa-

tion statements to be sent to the TNC. These  parameters would be

used for a tnc other than a KPC-X

1-4

NOTE: You must specify RADIOSPEED if you use TNCTYPE.

For example:

PORT  2  AT  $2F8 INT3 SPEED 4800  NAME  145.01  RADIOSPEED  1200

TNCTYPE 1 (Should be all on one line)

AND IN THE MSYSTNC.1 FILE YOU WOULD HAVE THE COMMANDS THAT  WOULD

PUT YOUR TNC INTO KISS MODE.(see note in general info section)

DIGI callsign FROM [PORT] number TO [PORT] number [AS callsign]

The  DIGI  statement defines a digipeater  callsign.  Repeat

this statement type up to 25 times as desired. The first callsign

is  the callsign that would be used in the VIA list by a  station

trying  to  use the digipeater. The port  number  following  FROM

indicates  the  port that will be using this call.  The  TO  port

number indicated the port on which packets having this the  first

callsign  as  digipeater  should  be  output  on.  For  a  normal

digipeater  the two port numbers will be the same; for a  gateway

they  will  be different. The optional AS  callsign  specifies  a

replacement  for the first callsign in the digipeater  list  when

the packet is sent out the TO port. If omitted the first callsign

is assumed (ie, no change). Consider the following examples:

DIGI MYDIGI FROM 0 TO 0

DIGI MYDIGI FROM 1 TO 1

DIGI W1XX-3 FROM 0 to 0

This  would allow the callsign MYDIGI to be used in the via  list

on  either port 0 or 1. The packet is retransmitted on  the  same

port was heard on. The call W1XX-3 can also be used to digipeat

on port 0.

DIGI MYGATE FROM 0 TO 1

DIGI MYGATE FROM 1 TO 0

This  pair would define a gateway between ports 0 and 1 with  the

callsign MYGATE used in both directions.

DIGI TOF1 FROM 0 TO 1 AS TOF2

DIGI TOF2 FROM 1 TO 0 AS TOF1

This  interesting pair allows TOF1 to be used as  a  gateway

digipeater call on port 0 (no effect if used on port 1). When the

packet is repeated it will have TOF2 in the digipeater list where

TOF1 originally was. The opposite is true on port 1: When TOF2 is

used  as a digipeater call the packet is repeated on port 0  with

TOF1 being substituted for TOF2. The substitution is  transparent

to the users.

NUMCHANS number

NUMCHANS  is used to specify the number of logical  channels

that  will be allocated. Each connection (while  connected)  uses

one  logical  channel. When the KA NODe is being  used  a  second

logical  channel  is  used for outgoing  connects.  Each  logical

channel requires about 7K of memory. They are available for reuse

when a station disconnects.

1-5

BBS       CALL      callsign

This  statement gives the callsign that can be connected  to

for  the  "big"  BBS.  The callsign can  contain  an  SSID.  This

statement is also optional; if omitted stations will not be  able

to connect to the BBS unless a bbs alias is given.

BBS       ALIAS     callsign

This  statement gives the callsign that can be connected  to

for  the  "big"  BBS.  The callsign can  contain  an  SSID.  This

statement is also optional; if omitted stations will not be  able

to connect to the BBS unless a BBS CALL is given.

KANODE    CALL      callsign

This  statement gives the callsign of the KA NODe  which  is

similar  to  the  KA NODe provided in the  Kantronics  tncs.  The

callsign  can  (and  probably  should)  contain  an  SSID.   This

statement  is optional; if omitted stations will not be  able  to

connect to the PBBS.

ANSWER    CALL      callsign

This  statement gives the callsign that can be connected  to

by stations wanting to talk to the SYSOP. It is also used as  the

callsign when the Connect command is used. The callsign can  (and

maybe  should)  contain  an SSID. This statement  should  not  be

omitted.

FORWARD   CALL      callsign

This  statement  gives  the callsign used when  the  BBS  is

forwarding.  It can (and probably should) contain and SSID.  This

statement should not be omitted if the BBS is to be used.

ID        CALL      callsign

This  statement  gives the callsign that will  be  used  for

identification purposes. It must be your actual amateur  callsign

and  should  NOT have an SSID specified. This statement  MUST  be

used.

ID        EVERY     number

This statement specifies the number of minutes between  IDs.

The  list of stations for which there is mail is  generated  with

every ID.

AX25      FRACK     number

Number   is   the   number  of  seconds   to   wait   before

retransmitting  a  packet. All of the AX25  statement  forms  are

optional.

AX25      MAXFRAMES number

Number! specifies the maximum number of frames that  can  be

outstanding before waiting for an ACK. Must be between 1 and 7.

AX25      RETRIES   number

This  is  the  number of retries that will  be  made  before

abandoning a connection.

1-6

AX25      CHECK     number

If   there  is  no activity on a logical  channel  for  this

number of seconds there will be an automatic disconnect.

MONITOR   IFRAMES   on/off

Turns  on or off the displaying of Information  frames.  All

forms of the MONITOR statement are optional.

MONITOR   UFRAMES   on/off

Turns on or off the displaying of Un-numbered frames (except

beacons, see below).

MONITOR   SFRAMES   on/off

Turns on or off the displaying of System frames.

MONITOR   BFRAMES   on/off

Turns on or off the displaying of Beacon frames (Un-numbered

information).

MONITOR   OUTGOING  on/off

Turns on or off the displaying of all outgoing packets.

MONITOR   PORTS     mask

Determines which ports will be monitored. The rightmost  bit

of  this number corresponds to port 0, the next bit to  the  left

port  1 and so on. Ports with 1 bits are displayed  (as  selected

above). A value of 13 (decimal, $D in hex) would enable ports  0,

2 and 3, for example.

WINDOW    number    SIZE number number number number

This  form of the window statement gives the bounds  of  the

window  whose  number is given after the word  WINDOW.  The  four

numbers after the word SIZE are the upper left column (1 to  80),

the  upper  left row (1 to 25), the lower right column,  and  the

lower  right row respectively. All of the WINDOW statement  forms

are optional and should be used with great caution.

WINDOW    number    BACKGROUND number

WINDOW    number    FOREGROUND number

These two forms of the WINDOW statement give the  background

and  foreground  colors of the window specified  after  the  word

WINDOW.  The rightmost number is a color number from 0 to 15  for

foregrounds and 0 to 7 for backgrounds. The corresponding  colors

are:

Black    0     Blue    1    Green    2       Cyan    3  Red     4

Magenta  5     Brown   6    Lt Gray  7       Gray    8  Lt Blue 9

Lt Green 10    Lt Cyan 11   Lt Red   12      Lt Mag  13 Yellow 14

White    15

WINDOW    number    PAGE number

This statement assigns a window to the specified page (0-3).

WINDOW    UNUSED    BACKGROUND number

WINDOW    UNUSED    FOREGROUND number

1-7

These  two statements give background and foreground  colors

to the places on the screen not assigned to any window.

WINDOW    DISPLAY   color/monochrome

This statement selects the kind of display adapter you have,

COLOR or MONOCHROME. If you have a CGA card, select COLOR.

PROCESS   processname [WINDOW] number

This  statement specifies the window that is to be used  for

output from the allowable process names. Be extremely careful  if

you use this statement.

STACK     processname number

This statement specifies the number of bytes to be allocated

for  the  stack  for the allowable process  names.  You  probably

shouldn't mess with the defaults!

SYSOP     NAME      name

You must use this statement to specify your first name.

SYSOP     QTH       location

You must use this statement to specify your location.

SYSOP ZIP zip code

You must use this statement to specify your zip code.

MAKE

Use  this statement if you want to produce the new  MSYS.DEF

file if there are no errors.

Here is a sample MSYS.OPT file for you to figure out!

port 1 at $3e8 int 4 speed 1200 name  223.70

port 0 at $2f8 int 3 speed 1200 name  145.01

digi bxng from 0 to 0 as bxng

digi bxng from 1 to 1 as bxng

digi bxn501 from 0 to 0 as bxn501

digi bxn501 from 1 to 0 as bxn370

digi bxn370 from 0 to 1 as bxn501

digi bxn370 from port 1 to port 1 as bxn370

digi test-4 from port 0 to port 1 as test-5

digi wa8bxn-3 from 0 to 0

bbs call wa8bxn-13

kanode call wa8bxn-1

answer   call wa8bxn-14

forward call wa8bxn-12

sysop name Mike

sysop qth Kirtland, OH

sysop zip 00000

id call wa8bxn

window display color

make

1-8

DRSI CARDS

(c) Copyright 1992 by HUB COMPUTERS, INC.

Minimum things that must be done to use DRSI cards:

Put HAPNDRSI.SYS in root (C:\) directory

Put HAPNKISS.COM where it can be found to be executed.

Put DEVICE=HAPNDRSI.SYS IRQ=7 (or whatever interrupt you have the

board set up for) in CONFIG.SYS. Reboot system.

Put:

PORT 1 AT $0 INT 7 SPEED 1200 NAME *DRSI-0

PORT 2 AT $1 INT 7 SPEED 1200 NAME *DRSI-1

in  MSYS.OPT  and run MUTIL function 9. Notes:  Change  the  port

numbers  to what is appropriate to your system. The ports on  the

drsi card are referenced by the numbers after the AT $. The INT 7

is taken as comments only (but INT something is required).  SPEED

of the DRSI ports is really set on the DEVICE=HAPNDRSI.SYS state-

ment in CONFIG.SYS. SPEED value  given on port statement is  more

less only comments. NAME must begin with *, can be something like

*145.01  if  you  prefer.  Before you start MSYS,  you  must  run

HAPNKISS.

That  should  get you started. For other configurations  and  op-

tions, read the appropriate documentation.

1-9

MSYS Command Summary #

(c) Copyright 1992 by HUB COMPUTERS, INC.

The following is a summary of the commands available at  the

local  keyboard for MSYS. The commands may be abbreviated to  the

capital letters.

*comment    Enables comment line from user to SYSOP

#msgs/use    Displays total number of active msgs and known  user

on BBS

@home       On/Off

ABortfwd    Aborts forwarding as  soon  as possible

ACceptmid   On/Off

ADDUser     Allows sysop to add a user to bbs

APpend      Used to append a file to a message and  other  general

uses

ARChive     Puts given @BBS in specified subdirectory

ARP         Lists the known callsign/IP address

ARP add     Adds digipeaters to TCP/IP stations

ARP DEL     Removes TCP/IP stations from arp list

ATtended    ON/OFF Turn HF forwarding on and off

AUTODest    ON/OFF            Add new DESts for known neighbors

AUTOHold    On/Off   Hold messages that contain call in R: lines

AUTOKill    Kill all messages after forwarding

AUTONode    ON/OFF            Add new neighbors heard

AX25Backoff [Portmask] The rightmost bit is for port 0.

AX25L2V2    [port#] [On/Off]

BACkuptim   Specifies time to make backup

BADuser     ON/OFF

BBs         Enter the BBS as a local user

BBSNode     [ON or OFF]  If on and the node is enabled,  the  bbs

will appear as a node to the network

BBSTimeout# Number of seconds of inactivity on bbs to  disconnect

user

BDigi       Specifies digipeaters for beacons

BElloff     Turns  bells on or off (on turns bell off   off  turns

bell on)

BIDall      Manipulates bids in the BIDLIST.DAT file.

BINterval   seconds          Nodes Broadcast interval

BMaxcon     [port#] Max-BBS-connects

BOOT        Remote sysop only

BPorts      Used to indicate BBS ports only

BText       Specifies one line of beacon text

BUDCalls    Set  or  display the list of calls  with  SSID  to  be

monitored

BUDList     Turn on/off selective call monitoring

CHAnstat    Displays status of a logical channel whose number  is

given

CHEck       Number  of  1  second intervals  of  no  activity  to

disconnect

CL B #      Change border color

CLrscrn     Clear  window  number given or  *  for  clear  all

2-10

windows

CMdwindow   Specifies the window number to be used for commands

(CM 4)

COpy        sourcepathname DEStpathname (include drive: if needed)

Cscript     New form of the Connect command:

CTS         Allows specification for each port if hardware hand

shaking should be used

CRetries    Sets number of retries for forward connect attempt

C#          Attempt to connect to station via digi list  given

on port #

DEBug       Used to display or modify memory

DELEfile    Specifies file name to delete

DELUDays    Specifies the  number of days after which a user  that

has not  connected  will automatically be deleted from

the  system.

DELUSer     Deletes a user from the user list

DESt        ADD, REP, DEL

DFree       Displays amount of disk space free

DIGipeats   Displays digipeater/gateway list

DIRectory    Displays   directory  for path   specified   Example

DIR mail\*.*

DISconnec   Forces a disconnect on logical channel specified

DOcmdfile   To execute a command file

DPorts      Allows sysop to selectively disable digipeater

DRoute      Callsign port#  [digi1...digi8]

DTimedate   Displays current date and time

DUmpbbs     ON/OFF disconnect any bbs that has connected and uses

a command other than S or F.

DXCall      the call, with ssid, that will be used by the the DX

Node.

DXDiddle #  where # is the number of seconds of inactivity before

a 'diddle' packet is sent.

DXEnable    call-list Enables the given callsigns (with ssids) to

connect to the dx node as dx nodes.

DXNconnect  scriptfilename

DXPorts     portmask Rightmost bit is Port 0.

DXRestore   Reads the last recorded DX spotting and WWV

information.

DXSAve      Records the current DX Spotting and WWV information

DXSTart     #nodes #users #dxspots #wwv

EDit        Screen editor for files

EFile       Edits the specified file

EMsghdr     Edits the header of the specified msg number in BBS

EUser       Edits specified user

F2 time     Number  of seconds to  continue  displaying  moni

tored packets after F2 (function key) was last

pressed.

FBbstimes   Allows specification of hours bbs is full function

FCall       Finds the logical channel(s) in use by given callsign

FFile       (find file) FF *.DAT C:\

FIle        Converts specified msg number into file name given.

2-11

FMsg        pattern-string Searches all messages in MAIL.BAK  for

given pattern string.

FOrward     Initiates forwarding immediately

FRAck       Displays  or sets time in seconds  to  retransmit  a

packet

FREE        Deallocates the logical channel specified

FSize       Displays the file size for the pathname specified

FTIme       Forward time

FTPgm       File Transfer Protocol

FUlldup     Used on the specified ports if set to ON.

GEtmsgotd   Reads and displays Message of the day file

GRep        Will search the given file for the specified pattern

HApnstat

HCAll       Allows you to properly  specify  your  bbs hierarchi-

cal callsign.

HCLean      This command automatically deletes specified messages.

HEard  #    The J heard list recording is turned off if #  >

Dispatches

Help        Lists the names of the commands

HOLdlevel   Parameter is a number which if the sum of word

weights matched in message exceeds the value the

message is held.

HOStname    Hosts id name TCP/IP

HReplace    Selects adding to end of existing hierarchical

@BBS field or replace.

ID *        Forces transmission of an ID immediately (also mail

for list)

IGnore      Calls in budcalls list will not be monitored

IPNCheck #  Inactivity timeout in seconds, default 600.

IPNFrack #  Retry timer in seconds, default 30.

IPNRetry #  Retries, default 5.

IPTtl #     TCP/IP Time to live

INitpr      Allows you to specify if printer should be

initialized before each file is printed.

JBbs        Lists (P)BBSs heard and their paths

JDigipeat   Lists Digipeaters heard and their paths

JGateways   Lists Gateways heard and their paths

JHeard      Lists stations heard and those connected to BBS

JKanodes    Lists KA Nodes heard and their paths

JMsys       List other MSYS systems heard

JNetrom     Lists NET/ROM nodes heard and their paths

JTcp/ip     Lists TCP/IP stations and their addresses

KEyboard    Assign keyboard to channel number specified

KILLproc    Abort specified process number (Can be hazardous!)

KPorts      Allows sysop to selectively disable k NODe

LCount      Counts number of lines, characters,and words in file.

(LC Filename)

LOCKkbd     Locks the keyboard

LOGclose    This command closes the log file, renames it to form

of yymmdd.LOG and then opens a new log file.

2-12

LUsers      List known user calls that have connected to BBS

MAKEPriva  On/off {sets/resets making messages private by de

fault}

MAStersrv   Specifies   the   master  WP   server

MAXBullag   command may be used to discard bulletins older than a

specified amount

MAxFrames   Display or change number of frames sent before ACK rcvd

MAXRead     Specifies maximum size message that can be read or file

that can be downloaded

MCForward   Portmask     Ports where forwarding is counted as bbs

connects. Default is all 1's.

MCOn        Number of connects allowed to sysop keyboard

MDir        Creates given directory

MEMory      Display amount of memory free

MERge       Reads MSYSFWD.DEF and copies it to MSYS.FWD

MEXpert     [On/Off]If on, display the message of the day

MHclear     Clears various J heard lists

MIDchar     Message ID indicator (max 4 chars)

MINmem      Allows specification of the minimum free memory

MISsing     Reports number of times forwarding aborted due to

missing text file

MONBframe   Enable/disable monitoring of Beacon (UI) frames

MONIFrame   Enable/disable monitoring of Information frames

MONITor     Enable/disable monitoring of incoming frames

MONOutgoi   Enable/disable display of outgoing frames

MONPorts    Select which ports to monitor (bit string,LSB=port 0)

MONSFrame   Enable/disable monitoring of Supervisory frames

MONSTats    Enables/disables recording of channel usage stats

MONUFrame   Enable/disable monitoring of Un-numbered frames

MONxx       Enable/disable  monitoring  of  PIDs  on   screen   2

monitoring

MSgtraile   On/Off

MYAlias     Display or change bbs alias callsign

MYBbs       Display or change bbs primary callsign

MYCall      Display or change Call/Answer callsign

MYIpaddr    Sets IP address that the system will respond to.

MYKnode     Display or change K NODe callsign

MYTcpcall   Callsign used when sending arp and TCP/IP frames

NDwindow    #                   Net default window size (frames)

NETBbs      ON/OFF              Enable BBS cmd in net NODe

NETCon      ON/OFF              Enable connect cmd in net NODe

NETFbbstime Allows specification of hours NET is full function

NETNode     ON/OFF              Enable the Net Node

NETOnly     ON/OFF If on, only BBSs can connect to bbs via

network

NETRead     Specifies maximum size message that can be read or

file that can be downloaded through the Netnode

NETTalk     ON/OFF Enable Talk cmd in net NODe

NEWUser     # allows you to set the initial  user flags

NInfo       Gives net NODe status

NNode       Takes you into the NODe as if you were connected to it.

NOBEacons  [port mask] If a given port is set to 1 then there

will be no ID, mail for, or BText beacons.

2-13

NOBUlls     On/Off  When on, no bulletins will be forwarded

during normal forwarding.

NODe        SEnd, SAve, REStore, ADD, REP, DEL

NODETimeout #  Number of seconds of inactivity on network node

NPorts      portmask           Ports to which nodes broadcast

NRCall      callsign           Sets net NODe callsign

NRId        mnemonic           Sets net NODe mnemonic identifier

NTtl        #                    Network time to live (max hops)

OCInit      #                    Initial obsolescence count

OCMmin      #                    Minimum obsolescence to broadcast

PAClen      Specifies  maximum number of bytes that are put  in a

packet

PATh        Display information about given user callsign

PForward    Display information about given BBS callsign

PIng        Destination IP address or a mnemonic

PMessage    Message# prints specified message

POrts       Display information about ports

PPersist    [port#] value  (255 to 0)

PQuality    [port#] #            Quality for nodes hear on port

PRInt       Prints a file (on printer)

PROcesses   Display status of processes

PRTmsgs     Enables/disables printing of messages

PWindow     Sets window to be used by a process

QUIT        Terminates execution of MSYS gracefully more or less

RDir        Removes the given directory

Readbakup # Allows you to read a message it the MAIL.BAK dir

REMove      DESt callsign     Deletes DESt through all neighbors

REName      Oldpathname newpathname (must both be on same drive)

REPlaces    Replaces incoming @BBS calls

RESpons #   Number  of  seconds  to  wait  before  sending  out  a

response

RETries     Displays or sets number of retries before disconnecting

REQDIR      ON/OFF enables Request directory server

REQFIL      ON/OFF enables Request File server

REQMaxfil   used to set a limit on the size of files that may be

requested from the REQFIL server

REQQTH      ON/OFF enables Request QTH server

RUn         Allows  you a  DOS program with many limitations.

SCreensave # zero is the default

SEnd         Sends a line of text to specified channel: SE 1  The

msg

SHow/       see DX Node for various commands

SLottime    [port#] value  (0 to 255)

SMTP        Simple Mail Transfer Protocol

SNow        Enables/disables changing screen only during retrace

SOrt        Will sort up to 200 eighty byte records

STatus      Displays some status information

SUspendtr   Number  of  free bytes in input  buffers  below  which

monitoring is turned off.

TABs        ON/OFF Default is on

TACk        Transport ACK delay (responsetime)

2-14

TBdelay     Transport busy delay

TElnet      Used to establish keyboard to keyboard connection

TFrames     Send test frames

TImezone    Allows entry of your time zone

TNatime     Transport no activity timeout time

TOdo        Causes the MSYSTODO.DAT file to be scanned

immediately

TPorts      Enables/disables ports for TCP/IP

Trace25     Enables/disables trace of AX.25 state changes

TraceIP     Enables/disables trace of TCP/IP state changes

TREtries    Transport retries

TRUncate    filename Adds CR/LF to files as needed

TS-HHMMSS   TimeSet command - sets dos time to given value

(must be exactly 6 digits)

TTimeout    Transport timeout (Net FRACK)

TXDelay     [port#] value  (0 to 255)

TXTail      [port#] value  (0 to 255)

TYpe        Allows you to type a file

Users       Writes out all users to file

VErifysysop [ON or OFF] Sysop verification when connected to BBS

VIdeo       Displays or changes video monitor in use

WAtch       Toggles watching of specified channel number

WBackgrnd   Sets background color for specified window

WColors     Gives list of colors and their numbers

WForegrnd   Sets foreground color for specified window

WHO         Displays current users/messages

WHY msg#   wordfile    Searches for the words in wordfile

Window      Displays current window definitions

WPAge       Sets page number to display specified window

WPEvery     Specifies  the  number of days between  sending  out  WP

information.

WQupdate    Worst quality DESt route to record

WRecord #  (window record) which sends all output to a disk file

WSize       Specifies coordinates for specified window

W?          Displays window number in upper left of each window

<Esc>       Cancel partially typed command

Function key usage:

F1 through F4 select page to display (0 to 3 respectively)

F6  causes  the  following characters to the next  return  to  be

treated as a command even if keyboard currently connected to some

other process (such as local bbs, etc.).

F8  key can be used to blank the screen.

Alt-F1 Alt-F2 Alt-F3 keys work like DOS F1 F2 and F3 commands  to

retrieve/edit last sysop command typed.

Ctrl/F2  suspends activity on the monitor screen (F2).

Ctrl/F4 terminates MSYS as if QUIT command was used

Ctrl/F5 is the same as the PRocesses command

Ctrl/F6 assigns keyboard to command processor

2-15

Most  commands  give information on their usage if you  type  the

command name followed by a question mark. Example WS ?

2-16

MSYS SYSOP Commands  #

(c) Copyright 1992 by HUB COMPUTERS, INC.

*comment    Enables comment line from user to SYSOP

#msgs/use     Displays  total  number of active  msgs  and  known

users  on  BBS. The number of messages that can be on the bbs  is

set   by the mailsize program and the maximum number of users  is

500.

@home    On/Off   If  On, causes @bbs  field   of  messages   for

registered users of the system to be replaced  with the  home bbs

they  gave with the NH command (the way it has  been done in  the

past). If Off, the @BBS field is not modified.

ABortfwd   Aborts forwarding as  soon  as possible  (not   always

immediately). You may want  to  DISC  the channel  forwarding  is

using (particularly  if  its  trying  to connect) to speed things

up.

ACceptmid   On\Off   If ON message ID string found on  S  command

will  be used if present.  If OFF or MID is missing, one will  be

generated if BIDall is on.

ADDUser  Format ADDUser callsign

This will allow the sysop to add a user callsign to the user file

without  that station having to connect to the bbs. This  can  be

useful  in conjunction with the automatic @BBS generation.  After

the new user has been added you automatically will EUsers for the

callsign at which time you can fill in the desired fields.

APpend   Sourcefile  DEStfile

This  command  causes  the  source file to  be  appended  to  the

DEStination file. This command may be used to append a file to  a

message file, among other more general uses.

ARChive  @BBS filename

This command will concatenate all current msgs with @BBS given to

FILES/filename.   No   compression   is   done.   Example:    ARC

RLIBBS RLINOTES/JUNK  (This assumes you have a RLINOTES subdirec-

tory  in the FILES directory)

ARP      Used  without an argument lists  the  known  callsign/IP

address correspondence determined by TElnet connects. An optional

argument, CLEAR, may be used to erase all of the ARP entries.

ARP add allows you to manually add (or put it in MSYS.DO) a  sta-

tion (and up to eight digipeaters) to the arp list. Syntax:   arp

add callsign  port# ip-address [digi1 [digi2]]

Examples: arp add WA8BXN 0 44 70 4 8 K8OZ

arp add N8HSP 2 44.70.4.10 WB8CQR-5 WB8APD

You  can put a / and a number after the IP address. For  example,

you can say:

2-17

arp add W3AAA 1 44 80 1 1/24 K8AAA

This says that any ip frames going to an IP address with 44 80  1

in the left 24 bits should go to W3AAA (through K8AAA as a digi).

Thus the number after the / is the number of significant bits  to

keep in comparisons.

ARP delete allows you to remove a single entry from the table.

Syntax:   arp del callsign

Example:  arp del N8HSP

ATtended  When set to ON all forwarding is allowed. If off, ports

with radio speed of 300 will forward only those messages in which

the from call is the idcall (messages that you originate) or  the

to  station  in  the message is the station  you  would  directly

connect to for forwarding. HF reverse forwarding is also inhibit-

ed if ATtended is OFF. Note that the  regulations appear to allow

you  to receive messages unattended. So if you don't have  a  STA

for HF turn AT OFF when you aren't there and remember to turn  it

ON when you are. To be on the safe side, put ATtended OFF in your

MSYS.DO file should the system reboot when you aren't present.

AUTODESt  ON/OFF   Default OFF  Add new DESts for known neighbors

AUTOHold   On/Off This command  specifies what  should  be   done

with messages that contain in them R: lines with your  Hierarchi-

cal  bbs callsign  (see  HCA command). This  normally   indicates

that  the message  has  passed  your system before and   probably

is   in  a routing loop. Currently such messages are held.  AUTO-

Hold  ON means to do this. If AUTOHold is OFF the  messages  will

not be held.

AUTOKill     Kill all messages after forwarding.

AUTONode  ON/OFF   Default OFF  Add new neighbors heard

AX25Backoff  [Portmask]  The rightmost bit is for port 0. A 1 bit

in  a given port position causes the FRACK value for the port  to

be  multiplied by the retry number to determine how long to  wait

before retransmitting packets that haven't been acked.

AX25L2v2 [port#] [On/Off]

AX.25 version 2 protocol:

Version  2 can be enabled for all ports or specified ports.  Ver-

sion 1, the default, may be better for HF opertion.

BACkuptim   Specifies the time to make backup of the mail file.

BADuser      Takes an operand of ON or OFF. When ON,the calls  in

the  BUDCall  list will be totally ignored by  the  system.  Such

stations  will  not  be  able to connect to  the  system  in  any

way(AX.25 or TCPIP) and will not be digipeated. Do NOT use  SSIDs

in  this list so that the listed calls can be used to  match  any

incoming  SSID  the  station may use. Try  to  avoid  using  this

facility unless really necessary as it does add some overhead  in

the processing of every packet received. Hopefully stations  that

2-18

misbehave  will  realize  that they can be  turned  off  and  not

persist in their activity.

BBs  Enter the BBS as a local user this command gets you into the

BBs  as  if  you  had  connected to the BBS,  with  a  few  minor

differences. Of course while you are using the BBS other stations

can  still  connect as usual. When  in the BBS you  use  the  BBS

command  set (B,C,D,H,I,J,K,L,M,N,P,R,S,U,V,W,X,?,*). If you  use

the  L command you will see all messages including   the  private

ones.  Likewise you can Read and Kill private  messages.  Message

texts  can  be gotten from files by going through  the  steps  of

sending a message: use the S command to give DEStination  station

and @BBS, then the title, and then simply ^Z for the text.   Then

do  a  LL 1 command to see the number assigned  to  the  message.

Exit  the  BBS using the B command then use the COPY  command  to

copy a file to replace the empty file in the MAIL directory.  For

example  if  the message was assigned a number of 123,  then  you

might  use the command CO tempfile mail/msg123.dat. See the  COpy

command below for more details of its use.

In the bbs from the local computer keyboard you can include a

file  in the text of a message you are sending. After  the  Enter

Text line, before you do the ^Z, a line of the form \+\  filename

will include that file at that point in the message. Example:

SB ALL @ MSYS

My MSYS.DO file

Here is my msys.do file as an example:

\+\ msys.do

Do not use this file directly! 73 - Mike

^Z

You can specify a window number as an argument to the BB com-

mand.  BB 0 will use the top half of the F1 screen for example.

If were in the BBS locally and leave it for some reason with

it still active and use the BB command you will return to your

previous session rather than starting a new one.

BBSNode [ON or OFF]  If on and the node is enabled, the bbs  will

appear as a node to the network (using the alias as the identifi-

er  and the bbs call as the call). This way connects may be  made

to the bbs directly from the network.

BBSTimeout #

Number  of seconds of inactivity on bbs to disconnect  user,  de-

fault 300 seconds.

Note that when version 2 of the ax.25 protocol is enabled that no

activity for CHEck time no longer causes a disconnect.

BDigi    -  Lets  you specify up to 8  digipeaters  for   beacons

(id,mail, btext) Can be different for each port.

Examples:

bd 4 digi1 digi2        sets digi1 and digi2 as digipeaters for

port 4 only

bd digia digib          sets digia and digib for all ports

bd 5                    clears digipeaters for port 5

2-19

BElloff  on/off       on makes it quiet, off is default

If  BElloff is negative, keyboard connects and normal bell  still

work.  If BElloff is positive, no bells will sound. If BElloff is

zero, all bells work.

BIDall     Manipulates bids in the BIDLIST.DAT file. The  follow-

ing operands may be specified:

+ bidstring             Adds given bidstring to bid file

- bidstring             Deletes given bidstring

= bidstring             Tells if given bidstring is present

#                       Tells number of active bids/mids

ON                      Enables bids for non-bulletins

OFF                     Disables bids for non-bulletins

BINterval seconds - Default 3600 -  Nodes Broadcast interval

BMaxcon  This command limits the maximum number of connects on  a

port.   BBS  connects are exempted from the limit but  count  for

other  users.  Normal users get a BUSY response when over  limit.

BM 1 allows 1 user per port.  BM [port#] max BBS connects.

If   a   port  is  marked BBS ONLY (see   BPports)   the   BMax

value specified  limits  the number of bbs connects. For  non-bbs

only  ports,  the BMax value does not effect number of  bbs  con-

nects.

BOOT     Boots computer (Remote sysop only) The correct reply  to

the  question  asked  by  the  boot  command  is  Yes  (case   is

important).

BPorts     This command is used to indicate which ports are   BBS

ONLY connects. A one in the corresponding  bit  position makes  a

port  BBS ONLY. For example,   BP 5  would only allow   bbs  con-

nects on ports 0 and 2. The default is no BBS only ports. If  you

do use this command to make BBS only ports, you better be sure to

mark all the bbs stations with user flag of 2!

BText   - Lets you specify 1 line of up to 80 characters of  your

own beacon text.  Can be different for each port.

BUDCalls   Set  or  display the list of calls  with  SSID  to  be

monitored  This command is used in conjunction with  the  BUDLIST

command  below.  As operands you give a list of  callsigns,  with

SSIDs,  that are to be monitored.  The calls should be  separated

by  blanks.  To list the current setting of  BUDCalls,  type  the

command with no operands. Examples:

budc w1xx w2xx-1 w2xx

budcalls K9XXX

budc

Note: their can be up to 10 calls.

BUDList   To enable only those packets with calls in the BUDCalls

list to  be monitored use the command BUDL on. To monitor packets

for  all  calls, use the command BUDL off. This  is  the  default

setting.

2-20

CHAnstat   Displays status of a logical channel whose number   is

given.  This  command  requires  an  active  channel  number   as

a  parameter.  Information  about the channel will be   displayed

in  response  to  this  command. The information   includes   the

Port  number  used, the call of the station connected to  and   a

status byte in hex.

CHEck      Number  of  1  second intervals  of  no  activity  to

disconnect.   This    command  specifies  the  number  of  second

intervals  after  which  if  there  has been no  activity  on   a

channel   a  disconnect will automatically be generated.  To  see

what   value   is  currently  set,  give  the  command  with   no

parameters.

CL  B  #     Allows  you to set the border  color  (used  on  all

screens). Use the WC command to see the color list. Default color

is  black  for the border. Any of the 16 color #  (0-15)  may  be

used.

CLrscrn        Clear  window  number given or  *  for  clear  all

windows.  This  command clears the window number given. If  *  is

given  for  the  parameter,  all  windows  are  cleared.  If   no

parameters   are  given,  then the command  processor  window  is

cleared.

CMdwindow   This  command is used to change which window is  used

by  the command  processor. The default command window is  number

0   which by  default is the upper half of display page  0.  This

command  is useful when you want a larger window (to type a  file

for example) to   be  used  by  some  command.  The  new   window

is   cleared automatically by this command. You will then get the

cmd: prompt. (page 0 =F1 page 1 = F2 etc.)

COpy   Sourcepathname DEStpathname (include drive:  if needed)

The copy command will return an error message if it isn't able

to copy all the records. Directories and wildcards can be used.

CRetries - Sets number of retries that there will be for forward

connect attempt

Cscript is a New form of the Connect command:

In  addition  to the usual C# form (like C0 ...) there is  now  a

connect  command in which you do not specify a port  number  with

the C, nor do you give the call of the station to which you  want

to  connect, but rather give the name of a file that  contains  a

connect  script to reach that station. The files must be  in  the

FWD  directory and can have any name. The connect script  is  the

same  format used in the forward file except that the first  line

begins with C rather the other letters used in the forward  file.

You can thus use the command

C  HOME in which case in the FWD directory you would have a  file

called HOME. An example of the contents of this file is

c0 wa8bxn

@olvgn

2-21

#120

+to

#180

+to

#300

+to

This example can be interpreted as follows. In the first line the

C  means this is a connect script, port 0 is to be used  for  the

initial  connect,   and the ultimate destination is  WA8BXN.  The

second line says to make a  connect to OLVGN (which is a  network

node). The following . lines are commands sent to the node, the #

lines are times to wait for responses, and the +to lines indicate

the responses to look for that indicate success. See the descrip-

tions  for forwarding scripts for more information on these  var-

ious option.

Another simple example of what you could put in a connect  script

file is

C0 WA8BXN

@WA8BXN V TOMSYS

That's the entire file; it is a simple way to put the  digipeater

(TOMSYS) needed to reach the destination station (WA8BXN).

CTS   Allows specification for each port if hardware  handshaking

should  be  used with the TNC. If on, MSYS will  suspend  sending

data  to  the tnc when Clear to Send is not true.  This  will  be

quite  useful with AEA tncs which have problems when  MSYS  sends

packets too rapidly to them. Make sure you have the proper  wires

in  the  cable  between the computer and tnc, and that  you  have

your tnc configured properly if you try to use this option.

C#    This  command is used to connect to another  station.   The

letter  C must be followed by a digit indicating the port  number

to be used for the connection. This is followed by a space.  Next

comes an optional window number to be used for the  conversation.

Next  comes the callsign of the station you want to  connect  to,

with  SSID if needed. Following this can come Via and a  list  of

digipeaters to use, separated by commas.

Examples:

c1 w1xxx

c0 w2xxx v k2xxx

c2 k3xxx-2 v n4xxx-3,a5xxx

When   you  issue  this command the screen to  be  used  for  the

conversation  will  be  cleared and you will see  a  line  "Enter

converse for channel # port #". When connection is made you  will

get  a message "*** Connection established to callsign".  If  the

attempt  retries out (determined by the RETries value), you  will

get  the  message  "Quit  converse  for  channel  #  because   no

connect!". In this case the default CALL process window may  show

"***  callsign busy" if this is why the connection could  not  be

made. Pressing any key before getting one of these messages  will

abort  the connect attempt and return you to the current  command

processor  window. When you are connected to a station, you  type

2-22

lines to be sent to the other station and will see lines typed by

the  other station. Once you begin typing a line it will  not  be

interrupted  by  lines  from the other station  until  you  press

return.   To  terminate  the connection, type ^C,  the  letter  D

(^CD) and  press return. If you want to save what you are getting

on  disk, press the \   key  three  times and then press  return.

You   will  get  the message "*** Capture file opened  ***".  All

lines you receive will be   displayed  as  usual as well as being

written  to  the  file CAPTURE.DAT.

When  you  turn capture on with the \\\ at the beginning  of  a

line, you can also specify a filename to be used in place of the

default  CAPTURE.DAT.  For example you can type \\\  junk.abc  to

start  capturing your conversation into the file  JUNK.ABC.  When

you  have  already turned capture on during a  connection,if  you

type something like \\\morejunk.xyz then the current capture file

will  be  closed and now your conversation will be saved  in  the

file  MOREJUNK.XYZ.  Three \followed by return will   alternately

suspend  and  resume the capturing of lines to  disk.   When  you

disconnect (or get disconnected) the capture file will be closed.

Note:  That the default  capture file name is always CAPTURE.DAT.

These  files can be viewed by using the TYpe command  or  edited

with  the  EFile command You could for example  connect  to  some

other  BBS, read a message found there after turning  on  capture

and  then use the capture file to replace a message text for  one

of  your own messages.This procedure is explained in more  detail

for the BBs command above .

\+\ filename

That file will be sent to the station you are connected to as if

you typed it. Example:

\+\ files\stuff

would send the file called stuff from the files subdirectory.

DEBug Used to display or modify memory

Options for debug are: E segadr:disp allows you to modify  memory

at the given address.  D segadr:disp displays memory at the given

address.  All numbers are in hex. Q gets you out of  debug.   Any

non hex input to E gets you out of E.

DELEfile    This  command is used to delete a file. You must give

as   a  parameter  a  single  file  path  name with  or   without

wildcards.    The parameter  can  contain a drive   specification

and/or  directory names for the given file.

DELUDays Specifies the number of days after which a user that has

not  connected  will automatically be deleted  from  the  system.

Default  is 100 days. Locked users (see bit list in  EUser  cmd),

BBS call signs and bad users are not automatically deleted.

DELUSer    This  command removes a user call from  the  BBS  user

list.  The callsign  given as a parameter must match the  a  call

seen  on   the  list  produced  by   LUsers  exactly   (including

case).   Use   this  command to REMove users  that  haven't  been

active lately when  you are getting near the capacity of the list

(currently  500  users). The  last  time  a  given  station  con-

nected  to   the  BBS  may  be obtained by using  the  P  command

2-23

while in the BBS.

DEST    ADD [id:]destcall neighborcall neighborport obs# quality

Adds new DEStination

DEST    REP [id:] destcall neighborcall neighborport obs# quality

Changes obs# & quality

DEST    DEL [id:] destcall neighborcall neighborport Deletes DESt

through given neighbor

DFree      This  command displays the number of bytes free  on  a

disk. If no operands are given the space is for the default  disk

(normally C). If a letter is given as a parameter, the number  of

free  bytes will  be given for that drive. Be sure the  drive  is

ready  before using this command. If its not and you get the  DOS

error message, type I for ignore.

DIGipeats   This command displays the list of digipeater  defini-

tions.

DIRectory  Displays  directory  for path  specified  Example  DIR

mail\*.* This  command displays a sorted directory listing.  Only

the names of the files (or directories) are given, not the sizes.

To  get  the  size of a file, use the  FSize  command   described

below. Names of directories are enclosed in <>. If no argument is

given,  you  will  get   a  directory  for  the  current  default

directory  (normally C:MSYS). To list some other directory,  give

the  path  to that directory and a file name of *.*.  Other  file

specifications can  be  given for a selective directory list.  If

you  include  a drive  specification in the pathname, be sure the

drive is  ready before using this command. Examples:

dir mail\*.*

dir a:*.*

dir files\*.c

DISconnec     This command can be used to force a  disconnect  on

the specified active channel. It will rarely be needed.

DOcmdfile       This command is used to execute a command file.

DPorts mask  (see MONPorts mask)

DRoute callsign port# [digi1 ... digi8]

DRoute DELete

The  first  form of this command allows you to  specify  the

TCP/IP station to which IP frames are routed by default  (Default

Route).  You specify as parameters the callsign of  that  station

(with  SSID)  and  the port number used to  reach  that  station.

Optionally  up to 8 digipeaters can also be specified to be  used

to reach that station.

The  second  form  of this command allows you  to  turn  off

default routing of IP frames.

If you do have a default route specified, then all  incoming

IP  frames that are not otherwise handled by existing  ARP  table

entries  will be sent to the default station. This  station  will

2-24

also  be used for any server functions you initiate  (telnet  for

example)  to  IP  addresses that are not currently  in  your  ARP

table.  Care  must  be  taken in  deciding  if  this  command  is

appropriate to use in your particular situation.

DTimedate     This command displays what the computer thinks  the

current  data  and time is. To change it you  unfortunately  must

exit MSYS and use the appropriate DOS commands. Time is(hh:mm:ss)

DUmpbbs  ON/OFF disconnect any bbs that has connected and uses  a

command  other than S or F.  A bbs is defined as a  station  that

sends  a SID ([...]) line. This is useful in cases where the  two

bbs's  get out of sync and message titles/text are  being  inter-

preted as commands.

DXCall callsign

where  callsign is the call, with ssid, that will be used by  the

DXNconnect  command to connect to a PacketCluster station.  If  a

connect is made to this callsign, it will take the user  directly

into the DX Node.

DXDiddle #

where # is the number of seconds of inactivity before a  'diddle'

packet  is  sent to connected dx node to keep  connection  estab-

lished.

DXEnable call-list

Enables  the  given callsigns (with ssids) to connect to  the  dx

node as dx nodes. This allows MSYS systems to link their dx nodes

together. Example:

DXEnable K8EIW-7 NO8M-2

DXNconnect   scriptfilename

where  scriptfilename is a connect script file found in  the  FWD

directory  using  the same format as the  keyboard  C  scriptfile

command.  Note  that connection to only  a  single  PacketCluster

station is allowed. Your DXCallsign must be enabled as a  Packet-

Cluster  node by the sysop of the node to which you will be  con-

necting.  The PacketCluster command that is used for this is  the

SET/NODE command (not supported in MSYS DX Node).

DXPorts portmask

One bits enable use of the DX node on that port. Rightmost bit is

Port 0.

DXRestore

Reads  the last recorded DX spotting and WWV information. If  the

current maximum number of DX spots or WWV reports (as set by  the

DXSTart command) differs from the values in effect when the  info

was  last stored this command will have no effect.  This  command

should  probably be put in MSYS.DO after the DXSTart  command  if

the DXNode is to be enabled.

DXSAve

Records  the  current DX Spotting and WWV information in  a  file

2-25

called   DXNODE.DAT. This function is done automatically  when  a

QUIT command is issued if the DX Node has been started.

DXSTart #nodes #users #dxspots #wwv

where:

#nodes is max nodes in DX network (default 25)

#users is max users in DX network (default 100)

#dxspots is number of DX spots to be recorded (default 20)

#wwv is number of WWV reports to be recorded (default 5)

If the first parameter is a letter, most of the dx functions are

disabled, causing it to act like a the conference function  found

in the dx node. Example:

DXST Conference only

EDit  filename - screen editor for files. Escape gets to  command

mode, do help for command list.

EFile      This  command is used to edit an ASCII file  with  re-

cords   <= 80 bytes in length. At least one operand  is  required

which is the pathname  to  the  file you want to edit. A   second

operand   may optionally  be  given  that gives the size  of  the

work   area  in  lines to be allocated (default is 200). See  the

separate  documentation for EDITFILE for more information on  the

editor.

EMsghdr      This  command  allows you to edit the  fields  of  a

message  header.   One operand is required which is  the  message

number to edit. This command also automatically sets the  message

length  to the  actual size of the associated message file.  This

is  useful when you have replaced a message file with some  other

file  as described above. This command is also useful  to  change

the  status  of messages to ALL (or others) that  have  not  been

marked as being read  (but actually have been) so that they don't

appear  in  the MAIL FOR list sent out with each ID. If you press

Esc  when editing you will skip the current and remaining  fields

and  see the  header  again.  This allows you to  bypass   having

to  press return for all the fields you don't want to change.  If

you place a string of characters that begins with a period  after

the  message  number, it will be appended to  the  existing  @BBS

field  of  the  given message. Thus if the @BBS of  msg  123  was

WA8BXN  then EM 123 .NTSOH would cause the @BBS field of msg  123

to become WA8BXN.NTSOH. If you type a blank for the BID field the

Bid  is cleared entirely (you still need to get rid of the  $  in

the type).

EUser      Edits specified user

Display the meanings of the possible flags bits.

F2  Time   #  is the number of  seconds  to  continue  displaying

monitored packets after F2 (function key) was last pressed.   The

default  is 600 seconds (10 minutes).  Thus 10 minutes after  you

last  press F2 monitoring will be turned off until you  press  F2

again. If you set F2 to 0 this function will be turned off.

FBbstimes  [Port#-]  Allows specification of hours  bbs  is  full

2-26

function. Times for full bbs use allowed to all ports is required

not optional.

FCall     This command finds the channel associated with a  call-

sign. The single required operand is a callsign without SSID. All

channels  in use by that callsign (with  any  SSID) will be  dis-

played  giving  the Port, process number and  name  and  complete

callsign  (with  SSID). This can be useful when you were  in  the

middle of a conversation with some station (and still  connected)

but  switched the keyboard to some other process (maybe the  com-

mand  processor) and now want to get back to  that  conversation.

Use  the KEyboard command with the appropriate process number  to

do so.

FFile  (find file)  When you give  it  a file  name  pattern  and

optionally  starting path location  and  it will  list all  files

that match the pattern. The following  would find all files  that

have the extension .DAT on your C: disk

FF *.DAT C:/

The  output  from  this command gives the date,  time,  size  and

complete  path  names  for  all  files  that  match  the   search

specification.

FIle     -  converts specified msg number into file  name  given.

(msg# filename.)

This command removes the forwarding lines at the beginning of the

file  it  creates.  If  you really want  to  keep  them,  use  an

appropriate rename command instead.

FMsg  pattern-string Searches all messages in MAIL.BAK  directory

for  given  pattern string. For example, to  find  messages  that

refer to hurricanes you might type  FM hurricane

FOrward   This command causes forwarding to begin immediately  if

it is not currently going on. You might want to use this  command

when you have just entered a new message with an @BBS  specifica-

tion  that you want sent out now. FO [callsign] of a single  sta-

tion  to immediately forward to.  This must  match  the  callsign

(including SSID) found on the F line  in the forward file.

FRAck     This  commands allows you to set or display the  number

of  seconds to wait for an acknowledgment to arrive for a  packet

that  is  sent  out. If it doesn't arrive within  the  number  of

seconds  specified  the packet will be retransmitted.  The  retry

count  for  the  channel will also be  incremented.  Making  this

parameter to small will congest the channel and cause unwarranted

disconnects. The default is 6 seconds. If in doubt, error on  the

high side! (port number optional)

FREE       This  command  allows you to  immediately  release  an

allocated  channel.  The  process  using  the channel  will   see

this   as  a disconnect.   This  command  will  not   be   needed

very   often. Disconnecting  which frees the channel will   auto-

matically   occur  after the CHEck interval has expired  with  no

activity.

2-27

FSize     This  command  can be used to determine the size of   a

disk file.  One operand is required which is the pathname to  the

file you  want the size of. It may include a drive  specification

and directory paths in addition to the file name. Wild cards  are

not  allowed.   Be sure the drive containing the file  is   ready

before using this command.

FTIme      This command sets the time in minutes after  the  hour

that forwarding will start. If time is negative number such as -5

then  you will forward every 5 minutes

FTPgm       Takes a host-id as an argument (it can be either   an

IP address  in  the square brackets or a symbolic  IP   address).

It establishes  a  connection  to the FTP server  of  the  speci-

fied system. FTP is the File Transfer Protocol used in TCP/IP.

FUllduplex    Used on the specified ports if set to ON. This  may

be  useful for satellite operation or with fullduplex  repeaters.

Examples:

FU ON           turns full duplex on for all ports

FU 2 OFF        turns full duplex off for port 2

GEtmsgotd    This   command causes the Message of  the  day  file

(MSYS.OTD)  to  be read and displayed. The Message of the day  is

sent  to   all  stations connecting to the BBS.  You  can  create

and/or  change this file using the EFile command. Keep  the  con-

tents short!

GRep  (filename pattern)  will search the given file for the

specified  pattern, displaying lines that match. Case is  ignored

in the search.

HApnstat

HCAll   This  command  allows you to properly  specify  your  bbs

hierarchical  callsign. This is the callsign put in the R:  lines

of   messages  as they pass through your system. If   you   don't

give this  a  value, your ID callsign will be used. Hint: if  you

run  more   than   one copy of msys for some  reason   and   pass

messages  between   the  two systems, make  your  HCAll  slightly

different  on each.   I  might  use  for  example   WA8BXN.OH.USA

on   one   and WA8BXN.OH.USA.NA  on  the other. This  will   pre-

vent   unnecessary holding  (or  deleting, see OLdbids)  of  mes-

sages  validly  passed from one system to the other.

HCLean    This command allows you to automatically delete  speci-

fied messages every day at specified time. (see MSYS.HCL)   Oper-

ands are:

HCL  -  <no operand> shows time in  minutes  after  00:00

(midnight) that housecleaning happens.

HCL? - Gives a short description

HCL#  -Sets the time for housecleaning in  minutes  after

00:00 (midnight)

HCL NOW - Manually initiates housecleaning

2-28

HEard  #  The J heard list recording is turned off if  the number

of  dispatches  per second in the last minute is  less  than  the

number  given. Default value is 3100 which j heard  recording.  J

Heard  list recording takes quite a bit of computation for  every

packet heard. This command was added to speed things up on slower

computers. (you might want to use a value of 10)

Help      This command lists the names of the available  commands

with the required part of the command name in capital letters.

HOLdlevel  Parameter is a number which if the sum of word weights

matched in message exceeds the value the message is held. Because

of this new command, the HOStname command now requires the  first

3 letters be used.

HOStname    TCP/IP HOSTNAME

HReplace    This command selects  adding  to  end  of    existing

hierarchical  @BBS field info from BBSTONTS.DAT file (if HRep  is

off)  or  replacing  anything  past  the  first  part  with   the

information from the BBSTONTS.DAT file (if HREP is ON). Note that

if some msys knows how to forward from some existing part of  the

@BBS field, no changes are made to it.

ID  *        This command will send out an ID on  all  the  ports

immediately.  You might use it to see if all your radios go  into

transmit as they should. It also will send out the MAIL FOR list.

The  text of the ID is not displayed at this time, but  is  shown

once during initialization. Nothing is displayed by the automatic

IDs sent out (on the F1 screen). To see what mail is  outstanding

for local users, type ID. To really force an ID to be sent,  type

ID *

IGNORE    on/off  Off is default

When on calls in the budcalls list will not be monitored

INitpr  command allows you to specify if printer should  be  ini-

tialized  before each file is printed. Set it to ON if  you  need

the initialization. If you need to press buttons (to select draft

or  letter quality mode for example) that you don't want  cleared

before each file, set it to OFF.

IPNCheck #  Inactivity timeout in seconds, default 600.

IPNFrack #  Retry timer in seconds, default 30.

IPNRetry #  Retries, default 5.

IPTtl #   Where number is the time to live. Default is 16.

JBbs      Lists PBBS'S heard and their paths

JDigipeat Lists Digipeaters heard and their paths

JD command screens out funny callsigns.

JD command also lists callsigns with /R in id.

2-29

JGateways Lists Gateways heard and their paths

JHeard    Lists stations heard and those connected to BBS or PBBS

This command gives a list of the last 10 stations that  were

heard on the ports or connected to the BBS or PBBS.

JKanodes  Lists KA Nodes heard and their paths

JMsys     Lists other MSYS systems heard.

JNetrom   Lists NET/ROM nodes heard and their paths

These  commands give the PBBS systems, KA Nodes  and  others

that  have been heard on the ports. A BBS is a system that  sends

out  a beacon that contains /B after what looks something like  a

callsign. A KA NODe is identified by /N in its beacon to ID.  For

both  of  these commands the date, time, callsign  and  path  are

given. This pair of commands is useful to identify the PBBS  that

goes  with a given KA NODe. The Net nodes are also included.  The

real  call for such nodes are given in []. Real Netroms  get  the

real call in {}.

JTcp/ip  List TCP/IP stations and their addresses

KEyboard  The  KEyboard  command allows you to manually  reassign

the  keyboard.  As  a  parameter you give the channel number   of

the process   you   want  to have the keyboard.  You   can    get

the channel number from the lower right part of the F1 screen. To

get  the   keyboard back  to  the  command processor so  you  can

issue  this   command  either  press ^F6 to assign the   keyboard

to   the command   processor  or  press  F6 to temporarily   send

keyboard  input  to   the   command process up to and   including

the  next return key.

When   you press a key, it is placed in the input   buffer   of

the  process  to  which  the  keyboard  is  logically  connected.

Initially  this  is the command process. When  you  give  certain

commands (such as BB, PB, C) the keyboard will be assigned to the

new  process  created  for  the  command.  When  the  process  is

terminated,  the keyboard is automatically assigned back  to  the

command scheduler.

KILLproc      Abort specified process number (Can be  hazardous!)

Never  use this command to kill a permanent process.  Better yet,

just don't use this command!

KPorts mask                 (see MONPorts)

These  commands  allow you to selectively disable the K  NODe  on

any ports or use of the ID call as a digipeater  (DPorts)  on any

ports.  The  mask  works just like the  port  mask  for  the

MONPorts  cmd. A 1 bit enables the corresponding port,  with  the

rightmost  bit  corresponding to port 0. These commands  will  be

most useful for HF operations if you don't want the K NODe to  be

accessible to or from the HF port, for example.

LCount   This command counts the number of lines, characters  and

2-30

words in a given file.  (LC filename with path)

LOCKkbd  This  command  "locks" the keyboard  so  that  any  keys

pressed  are  ignored. I put it in for two reasons. First,  I  am

getting  RF  into  my keyboard from HF radios  and  it  generates

random  characters. Second, my 3 year old son sometimes likes  to

press the keys. To "unlock" the keyboard, simply type lock  again

(no  CR  needed this time). When unlocking the  keyboard,  "lock"

must be all LOWER case.

LOGclose  This command closes the log file, renames it to form of

yymmdd.LOG and then opens a new log file.

LUsers   Use   this  command  to  see  who  has   connected   and

more importantly how many users are in the user file since it can

hold only 500 users currently.  LU takes  an  optional   argument

to  list  users  flagged   as BAD, EXPert, LIMited, LOCked, XFAst

and BBS system.  Example: LU EXP

MAKEPriva On/off {sets/resets making messages private by default}

MAStersrv     Specifies   the  master  WP  server   (default   is

AD8I.OH.USA.NA and probably shouldn't be changed).

If MAS is set to NONE, no WP messages will be sent.

MAXBullag  command may be used to discard bulletins older than  a

specified  amount  as soon as they are received. Default  age  is

32000.   A  reasonable value might be 21 days.  Bulletin  age  is

determined by the date on the last R: line found in the bulletin.

If  MAXBullage is set to a negative number, bulletins older  than

the  number given (in days) will be held rather than discarded.

MAXFrames Display or change number of frames sent before ACK rcvd

MAXRead - specifies maximum size message that can be read or file

that can be downloaded.

MCForward  [port mask] Allows you to control Multiple Connections

during  Forwarding.  The operand for this command is a  port  bit

mask.  Default is all 1's in the mask which allows forwarding  to

go  on as in previous versions. If a particular port has its  bit

set  to 0, then forwarding on that port is counted as a bbs  con-

nect  on  that port. Thus if you have BMaxcon set to 1  for  that

port and you are forwarding on that port, a bbs connect will  not

be allowed (since you are already using the port). Also, if there

are already BMaxcon bbs connections for that port forwarding will

be  bypassed for that port. Careful use of these commands  should

reduce  the congestion on HF forwarding as you now have  complete

control  over the level of multiconnection for both bbs and  for-

warding activities combined.

In the portmask argument you put 1's in the ports on which  you

want forwarding to be counted as a BBS connect.  Using MCF  along

with BM (and BP if you have marked any stations as BBSs),  allows

you to limit a given port to a single bbs connect or forward at a

time. If you want this on port 0, use the following:

2-31

MCF $FE

BM 1 1

BP $01

MCOn  specifies number of connects allowed to the sysop  keyboard

at  one time if MCon 1, only allows an incoming call to the  key

board to be answered if there are no other keyboard  connections.

MCOn 0 prevents all incoming keyboard requests.

MDir  Makes specified directory

MEMory   Display amount of memory free

MERGE     (no parameters, except for ? for info).

This  command  reads  MSYSFWD.DEF  and  copies  it  to  MSYS.FWD,

expanding  any  include  files ($filename). A new  file  is  also

produced,MSYS.BBS  that  is a sorted list of all  possible  BBS's

that can be forwarded to (including things like ARRL, 44*,  etc).

This  command was added for 2 reasons: previously, if there  were

any  include  files in the MSYS.FWD files they did not  get  read

when  a PF command was used and secondly, the new file is  needed

for   hierarchical   forwarding.  The  leftmost   part   of   the

hierarchical  route  only  is  used  for  forwarding   and    the

contents of the new file  were  needed  for efficient processing.

So,  the  bottom line is rename your existing  MSYS.FWD  file  to

MSYSFWD.DEF and then use the MERGE command.  Any future   changes

you  make  to your forward file must  be   done   to  MSYSFWD.DEF

followed   by use of the MERGE  command.  Its  called  MERGE  be-

cause it merges in the include files by the way.

MEXpert  [On/Off]If on, the message of the day will be  displayed

to  all  non-bbs marked  stations. If off, it will only  be  dis-

played to non-expert, non-bbs stations.

MHclear [letter [port or B]] clears the various J heard lists

MH clears everything

MH D clears all digi's heard

MH D 1 clears digi's heard on port 1

letters  are HBKDN or G (Heard, Bbs's, K  Nodes,  Digi's,

Netroms)

MIDchar  Message ID indicator (max 4 chars)

MINmem    Allows specification of the minimum free memory as  re-

ported by the MEM for which a new connect request will be accept-

ed.  Default  is 32000. Values less than 16000 probably  will  be

disastrous.

MISsing     Reports number of times forwarding aborted due to

missing text file

MONBframe Enable/disable monitoring of Beacon (UI) frames

MONIFrame Enable/disable monitoring of Information frames

2-32

MONITor    This  is  the master command  for  monitoring  packets

heard.   If this is set to OFF then there will be no  monitoring.

If set to ON then the types of packets selected will be monitored

for calls as selected by BUDCalls on the ports set by MONPorts.

MONOutgoi Enable/disable display of outgoing frames

MONPorts  Select which ports to monitor (bit string, LSB=port 0)

This   command if given no argument displays which ports  are  to

be  monitored  (all defined ones by default). If an   number   is

given  for  an argument, its bit pattern determines  which  ports

will be monitored. The rightmost bit corresponds to port 0. A one

bit enables a channel for monitoring.

MONSframe Enable/disable monitoring of Supervisory frames

MONSTats   enables/disables recording of channel stats.  Defaults

to OFF.

MONUframe This group of commands determines which kind of  frames

will  be  monitored.  If no operand is given,  then  the  current

setting of the  variable will be displayed. If a single parameter

is   given  (either ON or OFF), then the variable will  be  given

that value.

MONxx    -  Replaces MONCC, MONCF etc - now can specify  any  hex

value for PIDs to ignore on screen 2 monitoring

MSgtrailer  If on when a message is read on the bbs (except  with

RN or RE) a line is sent at the end of the message indicating its

number and who it was from.

MYAlias - display or change bbs alias callsign

MYBbs   - display or change bbs primary callsign

MYCall     This command displays your keyboard callsign if  there

is   no  argument,   or  sets the  keyboard  callsign  (used   to

connect   and respond to connects) to the call given as an  argu-

ment  (including SSID).

MYIpaddr  -  Takes four integers in the range 0 to 255.  It  sets

the  IP address that the system will respond to. Example: MYI  44

70   4  6   If  MYIpaddress  is not specified,  or  set  to   all

zeros,  the TCP/IP support will be inactive.

MYKnode  - display or change K NODe callsign

MYTcpcall      -  Display or change a callsign (with  SSID)  that

will  be used when sending arp and TCP/IP frames. It can  be  the

same as the ID call in most cases.

NDwindow  #  Default - 4  Net default window size (frames)

NETBbs    ON/OFF  Default  ON  Enable BBS cmd in net NODe

2-33

NETCon    ON/OFF  Default  ON  Enable connect cmd in net NODe

NETFbbstime  [Port#-] Allows specification of hours bbs  is  full

function. Times for full bbs use allowed to all ports is required

not optional.

NETNode   ON/OFF  Enables the Net Node

NETOnly   This command is used to indicate if BBSs only or any

stations can connect to the BBS via the network. If ON, only BBSs

are allowed to connect via the network. If OFF, then any  station

can connect to BBS via network.

NETRead - specifies maximum size message that can be read or file

that can be downloaded.

NETTalk   ON/OFF  Default  ON  Enable Talk cmd in net NODe

NEWUser    #  allows you to set the initial  user flags  for  all

new  users (see EUuser command for the bit   position  meanings).

Thus  you  can choose to make new users limited  (or   I  suppose

for   a closed board, "bad users")  until  they   identify  them-

selves somehow.

NInfo                          Gives net NODe status

NNode       This  command takes you into the NODe as if  you  had

connected  to it.  When in the NODe you can issue sysop  commands

if  you need to by putting \ in front of them as you can  in  the

BBS.   Particularly useful are the NODe, DEStination  and  REMove

commands  when in the NODe to modify neighbor nodes and  DEStina-

tion nodes.

NODETimeout #  Number of seconds of inactivity on network node or

K  node  at  command prompt to disconnect user.  Default  is  300

seconds (5 minutes).

NOBEacons [port mask] If a given port is set to 1 then there will

be no ID, mail for, or BText beacons.

NOBUlls    On/Off    When  on, no bulletins  will   be  forwarded

during  normal  forwarding . Useful in clearing backlog  of  mes-

sages, particularly during earthquakes and hurricanes!

NODe      SEnd                 Force known DESt transmission

"        SAve                 Write net NODe Database to disk

"        REStore              Read net NODe Database from disk

"        ADD call port qual [digis]    Adds neighbor NODe

"        REP call port qual [digis]   Changes quality of

neighbor NODe

"        DEL call port      Deletes neighbor nodes & its DESts

NPorts    portmask (see monitor ports)  Defaults 0  Ports to

which nodes broadcast

2-34

NRCall    callsign             Sets net NODe callsign

NRId      mnemonic             Sets net NODe mnemonic identifier

NTtl      #     Default 64  Network time to live (max hops)

OCInit    #     Default  6  Initial obsolescence count

OCMmin    #     Default 10  Minimum obsolescence to broadcast

OCM is settable for each port

PAClen   -  specifies maximum number of bytes that are put  in  a

packet before it is sent. Return also sends current packet.

PATh      - displays information about given user callsign  (same

as P on BBS)

PForward   - displays information about given bbs callsign  (same

as PF on BBS)

PIng (destination) Where destination is either an IP address like

[44.70.4.6.]  or  a mnemonic defined in MSYSHOST.NET.  The  reply

comes  in  window 2 (where the connect  and  disconnect  messages

appear)and gives the round trip time in seconds.

PMessage  message#  - This will print the given message.  If  you

have  MAIL.BAK as a directory then the  file  MAIL.BAK/MSG###.DAT

will  be  printed.  This  will look  nice.  If  you  don't,  then

MAIL/MSG###.DAT  will be printed. This will be only  the  message

text.  This command just makes it easier to print a message  than

using the PRInt command.

POrts  -    Various  information  is  given  by   this   command.

Interesting to look at but may not be very useful.  POrt can have

operand  on/off.  When  off a port will not  respond  to  the  id

callsign  (with  any  SSID) and will no  longer  digipeat  or  be

usable from the K NODe.  BBS will not id unless you use it.   Id

will be very short if it has to id (no text for the UI  frame).If

you   call  a station with the port off this  will   still   work

because  incoming packets from that station don't include a  con-

nect packet.

The POrts command will take a single parameter (ON or OFF) in

addition  to POrt port# ON/OFF. With just the  single  parameter,

all ports are turned on or off. Useful when you want to shut down

the  system.  PO  OFF allows the current users  to  finish  while

preventing further connects.

PPersist    [port#] value  (255 to 0)

PQuality  [port#] #  Default 192  Quality for nodes hear on port

PRInt     Prints a file (on printer)

The argument is the pathname to the file to print. Make sure  the

drive   is   ready before using this command.   Make   sure   the

printer is ready (and exists) before using this command. Printing

is done to LPT1: via BIOS calls. This command will queue up to 25

2-35

print  requests.  Print with no operands lists the files  in  the

queue.

PROcesses    This  command displays for each active process   its

number,  program running for that process, an  integer  parameter

passed   to the process (usually a port or channel  number),  the

least   amount of memory left in the stack for the  process  (la-

beled  SPmin),   and  the  window  used   for   output   by   the

process.  Of  particular importance  is SPmin if the system  does

bizarre things.  If  this number  is  less than say 100 the stack

is too small.  If  its  a negative number, you have problems  for

sure!

PRTmsgs   on/off  Enables printing of incoming messages  as  they

are received.

PWindow   If  given one parameter this command displays the  name

and  window used by the given process number. If two numbers  are

given  as  parameters, the first is the process  and  the  second

number  is the window number to assign to that process.

QUIT       After you use QUIT command (or CTRL/F4) the screen  is

cleared.  JK  etc  information is saved when you do  a  QUIT  and

reloaded when bbs is started again.

RDir  REMove specified directory

Readbackup  #    Command  allows you to read  a  message  in  the

MAIL.BAK directory (if it exists). Example:

cmd: R 12345

REMove    DESt callsign        Deletes DESt through all neighbors

REName    Oldpathname newpathname (must be on same drive)

REPlaces   Reads bbs call/@bbs replace file

RESpons  #    Where  # is the number of seconds  to  wait  before

sending  out a response. This is the T2 timer value in the  ax.25

spec.  If  you  set RES to 0 it will work  the  old  way,  always

sending  out  a  response to each I frame  received  as  soon  as

possible.   With  non-zero  values for  RES,  the  responses  are

held,the   latest one replacing any one being held, until  no   I

frames  have  been  received for the time given. It  then   sends

out  its response. RESponstime can be set for individual ports.

REtries   Displays or sets number of retries before disconnecting

REQDIR      ON/OFF enables Request directory server

REQFIL      ON/OFF enables Request File server

REQMaxfil is used to set a limit on the size of files that may be

requested from the REQFIL server and also the  files that will be

displayed in REQDIR requests. The default is 5000 which is proba-

2-36

bly too high.

REQQTH      ON/OFF enables Request QTH server

RUn  that  allows  you a  DOS program with many limitations.  For

those of you  who  have asked for some sort of DOS shell capabil-

ity you may recall I have said it can't be done. But some of  you

said  all  you needed to  do was  output 8 bits to some  port  or

something like that.  Well  if you have something VERY SIMPLE and

VERY SMALL and very QUICK here is something that should work  for

you. Note: You must follow  all of  the rules given here or  very

strange things might happen  for which  I claim no  responsibili-

ty. If you use the RUn command  and it  doesn't work it is clear-

ly you who have made the error!  Here are the rules for  programs

you  might  consider using with the  run command  (if  you  don't

understand these rules or aren't sure  your program complies with

them, don't run it!):

1.  Program must be small (I'd say under 10K, if you  get   a

message that says not enough memory, the program is too big.

2. Program must be in default msys directory.

3.  Program  must be fast. If runs much over  1  minute   the

system  will  reboot.  While it is running  no  packets  will  be

transmitted of any kind.

4.  Programs must not use any of the  "standard"   pre-opened

files. Console I/O must be done using ROM BIOS only. If you  open

any  files, you must close them. If you get any memory, you  must

free  it. If you change anything in the computer you must put  it

back for the most part.

5.  You  should  write  your  programs  in  either  assembly

language or C. Here is an example of a C program that is suitable

for framing (I mean execution using the RUn command):

#include <stdio.h>

#include <stdlib.h>

main(int argc, char *argv[])

{

int i;

char *path;

path = getenv("PATH");

for (i=0;i<argc;i++)

cprintf("argv[%d] is '%s'\n",i,argv[i]);

if (path) cprintf("Path = '%s'\n",path);

exit(1);/* You didn't expect this to do anything useful I hope*/

}

SCreensave #  If the number given is zero (which is the  default)

the screen will not be blanked automatically. The method used  to

blank  the  screen will work with all display cards  (the  screen

content is saved and then blanks are written to the screen). When

you  press  a key to unblank the screen you will  always  get  to

screen 1 (the one you get with the F1 key).

SEnd      Parameters are an active channel number and the text of

the  message  to send. The text is prefixed  with  "MESSAGE  FROM

2-37

SYSOP AT id-callsign".  Use  this to give helpful hints to  users

you  see that are in trouble with the commands they are trying to

use.

SHow/   The various SHow, user commands for the DX Node that  can

be used as sysop commands (see DX Node).

SLottime  [port#] value (0 to 255)

SMTP     Allows local access to the SMTP server (mainly for  test

purposes). The data channel port number for SMTP transfers is not

correctly   displayed.  It does not affect operation and  may  be

fixed in a future release.

SNow      If  you have a CGA adapter and there is a lot  of  snow

during  screen changes (particularly when changing display  pages

with  F1-F4)  and you don't like it, set SNow  to  ON.  Otherwise

leave it off and the display will run faster.

SOrt  -  (filename)  Useful to sort your forward include files in

the /fwd directory so you can find things there more easily. Will

sort up to 200 eighty byte records.

STatus    I  put  this one in for my own debugging (like some  of

the  other   information   displays). The  current  time  is   in

something  like seconds since midnight of 1900. If  any  channels

are open you will learn very little by what you see, but state  5

is  the normal connected state number. The next to last  part  of

this  display  is somewhat important. FREE is what  is  currently

free  in  the   buffer used to get characters from  the  TNC  (an

interrupt  driven   routine is  used).  LEAST  is   the  smallest

amount   of  space  that  was available. If least is  small  (say

less than 100) you've got  some real problems. I don't think I've

ever  seen  it  go below 3000.  If it  is  small  it  means  that

characters are coming  from  the  TNC faster than they are  being

processed. You might try a lower  baud rate between the  computer

and TNC but this probably won't  really help the situation  much.

Dispatches per second is how many  times per  second  each  proc-

ess  was  given control of the CPU.  If  this goes  to  zero  I'd

worry a lot!Note: will only display information for real ports.

The  STatus  command  may be given the  operand  RESET  which

resets  the least free values to 4096, the other counters  (over-

run,  framing  and  discarded) to zero. It also  copies  the  lat

minute  dispatches to min and max. Also added to the STatus  com-

mand  is display of the average number of dispatches  per  second

since msys was started.

RESET    - clears forward abort msg as well as all counters

Channels - gives status for active channels only

Ints     - give counts of interrupts

Stats    - gives port statistics

SUspendtrace  #      Where # is the number of free bytes  in  the

input buffers below which the monitoring is turned off. There  is

1  buffer  of 4096 bytes for each serial port (tnc).  The  amount

2-38

free  at any time is given by the STatus command. When  the  free

space gets less than the SU value, F2 monitoring is turned off to

allow things to get processed more quickly. Default value is 2000

bytes. Again, F2 displays take a lot of time and when the  system

gets  behind  it is a good idea to turn off the updating  of  the

display. When the free space in all the buffers goes above the SU

value  tracing  is turned back on (unless  otherwise  suspended).

This function can be effectively disabled by setting SU to 0  and

then it will work as before.

TABs ON/OFF (when on tabs are expanded on local display; when off

a tab shows up as ^I). Default is ON.

TACK      sec   Default  3  Transport ACK delay (responsetime)

TBdelay   seconds Default 180  Transport busy delay

TElnet    Takes  an  IP address and  optional  server  number  as

parameters.

Examples:  TE [44.70.4.10]

te hsp             (hsp would have to be defined in

MSYSHOST.NET)

To allow Telnet access to the bbs, you must modify the  entries

in  your  MSYSHOST.NET  file. Basically you  must  add  the  call

(without  SSID) in square brackets of those stations that are  to

be allowed bbs access. Example:

44.70.4.5 eiw K8EIW  #this IP address would not have access to the BBS

44.70.4.32 [k8eiw] don #this IP address would have access to the BBS

Note: You can use any case within the [], do NOT use any SSID's!

TFrames  - Port# send test frames on a given port

TImezone  - Allows entry of your time zone. Set timezone  GMT  to

all caps to get lower case z on message time R: lines.

TNatime   seconds Default 600  Transport no activity timeout time

TOdo   Causes  the MSYSTODO.DAT file to be  scanned  immediately,

doing anything it is time to do and also setting the time of  the

next  thing  to do.  BE SURE TO USE THIS COMMAND EVERY  TIME  YOU

CHANGE MSYSTODO.DAT!

TPorts     Takes a port mask value as an argument. This number is

used  the  same way as in the other similar commands:  each  port

corresponds to a port (port 0 is the rightmost bit) and when  the

bit  is  set  to 1 for a given port that port  is  available  for

TCP/IP    use (particularly ARP broadcasts).

Trace25        Here  the parameters are on or off and if you  set

it on  you will  get  more stuff that you can't understand faster

than  you ever wanted to see it!

TraceIP       Enables/disables trace of TCP/IP

2-39

TREtries  #        Default 3  Transport retries

TRUncate filename Adds CR/LF to files as needed to make lines  no

longer than 80 characters so it can be editted with ED command.

TS-HHMMSS  TimeSet command - sets dos time to given value

(must be exactly 6 digits)

TTimeout  seconds  Default 60  Transport timeout (Net FRACK)

TXDelay   [port#] value (0 to 255)

TXTail    [port#] value (0 to 255)  This should be used with

TNC-2 clone tncs (any tnc that uses hardware switches to set baud

rate going to radio) when on HF. Without this parameter the tnc

keys up on the transmitter before the complete packet is sent. Try

a value of 20 as a starting point.

TYpe       Allows you to type a file. Use spacebar to  stop/start

and a control Z  or A to abort.

Users filename - writes out all users info to file

VErifysysop [ON or OFF]  If on then whenever any variation of the

id  call  is used to connect to the bbs (except  from  the  local

keyboard) then 4 characters from the remote sysop phrase must  be

entered.

VIdeo      You can change the monitor you want to use  while  the

system  is  running (using COLOR or MONOCHROME as  a  parameter).

I've  got both  connected  to my systems here (at the same  time)

so  it  is meaningful  to switch back and forth on rare occasions

(like  to see if it works). You probably ought to specify the one

you  want to use in your MSYS.OPT file and not otherwise use this

command.

WAtch     This  command  allows  you to see all that  occurs   in

both directions  for the channel you specify. Watching is  turned

off  when  the channel is closed or you use the WAtch  command  a

second  time   for that same channel. The lines that  go  to  the

screen   are  not labeled in any way, they are just sent  to  the

screen.  If  you WAtch  more than one channel at a time  it  will

work  but  you  may wonder  which line came from  which  channel.

Using  watch  may  have some  side  effects  like  resetting  the

window  being used  by  the process  using the channel  you  have

selected.  That's  a  "feature" of this program and will  not  be

corrected!

WBackgrnd     This  command requires two parameters to set  back-

ground  color: Window#  and  Color#. The color number  should  be

between 0 and 7.

WColors   Gives list of color names and their numbers

WForegrnd     This  command requires two parameters to set  fore-

ground  color: Window#  and  Color#. The color number  should  be

2-40

between 0 and 15.

WHO Displays current users and messages (Remote SYSOP only)

Window    Use  of this command is a good way to see what  numbers

mean in the various window definition commands.

WHY  msg#  wordfile   This command will search for the  words  in

wordfile  (same  format as in the hold file) in the  given  msg#.

Useful  when you do a LH and want to see which words  might  have

been caught.

WPAge       This  command  requires two parameters  to  set  page

number:  Window#  and Page#. The page number  must  currently  be

between 0 and 3.

WPEvery    Specifies  the  number of days between   sending   out

WP information  for  users who haven't  changed  their   informa-

tion. Default is 60 days.

WQupdate  #    Default 1  Worst quality DESt route to record

WRecord # (window record) which sends all output to a disk  file.

The  command  format is WRecord window# The first  time  use  the

command for a given window it begins recording everything sent to

that  window in a file called windowN.dat where N is  the  window

number.  The second time you use the command for the same  window

it turns off recording for that window. Using the command with no

operands  displays the status of recording. NOTE:  This  function

produces a lot of disk output quickly. It can fill up your  disk.

It  also slows down the system a fair amount due to all the  disk

writes. This command was added for debugging purposes and  should

not be used as a general purpose capture command. You can  deter-

mine the window numbers by using the W? command.

WSize     Specifies coordinates for specified window

This  command  requires 5 parameters. The first  is  the  window#

being defined. The remaining parameters are two pairs of  numbers

giving the upper left and lower right corners of the window.  The

pair  is  column  number  (1,80)  then  row  number  (1  to  25).

Overlapping  windows  may  produce amusing  results.  To  specify

window 3 as being the entire screen the following command may  be

used: WS 3 1 1 80 25

W?         Use  of  this command is the easiest way to find   out

which window is which!

<Esc>  Cancel partially typed command

Function key usage:

F1 through F4 select page to display (0 to 3 respectively)

F6  causes  the  following characters to the next  return  to  be

treated as a command even if keyboard currently connected to some

2-41

other process (such as local bbs, etc.).

F8  key can be used to blank the screen. Pressing any  other  key

will  unblank the screen. A new sysop command is also added  that

allows you to specify the number of minutes after last  keystroke

to blank the screen automatically:

Alt-F1 Alt-F2 Alt-F3 keys work like DOS F1 F2 and F3 commands  to

retrieve/edit last sysop command typed.

Ctrl/F2    suspends   activity  on  the  monitor   screen.    ^F2

restores normal  operation.  This is probably as close as   being

able  to

scroll the F2 screen that I will be implementing.

Ctrl/F4 terminates MSYS as if QUIT command was used

Ctrl/F5 is the same as the PRocesses command

Ctrl/F6 assigns keyboard to command processor

Most commands give information on their usage if you type command

name followed by a question mark. Example WS ?

2-42

MSYS BBS Commands #

(c) Copyright 1992 by HUB COMPUTERS, INC.

A - Abort                  N - Enter name/qth

B - Bye                    P - Path to station

C - Conference/DX          R - Read message

D - Download               S - Send Message

G - Search file            T -Talk to sysop

H - Help                   W - What files

I - Information            V - Version

J - Calls heard            X - Expert-Mode

K - Kill message           U - Current users

L - List Messages          ?x- Info about command

M - Message of the day     * - Comment line

The A (abort) command to the BBS. It can be used to abort  output

from most bbs commands, like Download, List and Read.

The Bye command disconnects you from the BBS. Use it when you are

done!

The CC command allows sending a "Carbon Copy" of a given  message

to  other stations. Only the originator of the msg or  the  Sysop

can  do  this  for a given msg. Syntax is:

CC msg# call1@BBS1; call2@bbs2 ...

The  @bbs parts are optional if the calls are users of your  sys-

tem. Example:

cc 4725 k8eiw@wb8bii; no8m

An alternate syntax for this command is:

CC msg# <filename

Where filename does not include a file type (.DIS is automatical-

ly appended). This file contains tocalls (and optionally @BBS) of

the  stations that should get the copies. There is one  call  per

line of the file. Example:

CC 3195 <locals

The file LOCALS.DIS could contain

k8eiw @ wb8bii

NO8M

The Conference command is used to enter the DX Node from the BBS.

The DX Node can also be entered by connecting directly to the  DX

Node callsign (see DXCall command).  When in the BBS itself,  the

SH/  family of DX Node commands may be used without  leaving  the

BBS  (SH/DX for example). The command syntax used once in the  DX

Node  is patterened after the commands familiar to  PacketCluster

users.  The  H command gives a list of the user  commands  and  a

brief  description of each. Its output is similar to  that  shown

below:

DX Node commands

Bye                         Disconnects from DX Node

BBS                         Returns to BBS

Conference                  Enters local conference

3-43

DX freq dxcall comments     Enters given data as DX spot

SH/CL                       Lists basic stats about DX network

SH/COnfig                   Lists DX nodes in network and users

on each

SH/Dx                       Lists last 5 DX spots

SH/Dx/#                     Lists given number of DX spots

SH/Users                    Lists local DX node users

SH/WWV                      Lists WWV reports (up to 5)

Talk callsign               Sends following lines to given user

call

Talk callsign msg           Sends single line msg to given user

call

WWV SFI=# A=# K=# comments  Enters given data as WWV report

Bye      Disconnects  from  DX Node. A  regular  disconnect  also

works.

BBS     Leave DX Node and go to BBS (even if original connect was

into DX node call directly)

Conference   Enters local conference. Following lines typed  will

be sent to all stations in the local conference.  Control-Z  (^Z)

is  used to exit the conference. When a station enters  or  exits

the conference a message noting the event is sent to the stations

in  the conference.  When in the conference the various forms  of

the  SHow/ command can be used by preceeding it with an  asterisk

(example: *sh/dx).

The Download command is used to read a file stored on the system.

The  format of the command is D filename  where filename  is  the

name of the file to download. To see what files are available for

downloading,  use  W or WN command. To   get   a  file   from   a

subdirectory   type   d   (directory name\filename)

The  G command (for GREP, a Unix utility) allows you to search  a

downloadable file for a specified string of characters,  printing

all lines that contain the string of characters. Case is  ignored

in  the comparison. No wild cards (like ? or *)  are  recognized.

When  you use this command you will be asked for a file name  and

then a search string. For file name enter the name that you would

use if you were downloading the entire file. If you do not give a

file name, Database.0 will be assumed.

When the command is used the user will be asked for the file

name  to  search  and then the search  pattern.  FILES\  will  be

prefixed to the file name the user gives; thus downloadable files

may be pattern searched.

The  G  command is not available when the BBS is  in  limited

service mode.

The H command will give you a short description of the  commands.

For more information about a particular command, type ?x where  x

is the letter of the command.

The  I  command by itself gives hardware  configuration  of  this

system. (the information displayed comes from HELP\INFO.HLP)

3-44

For example:

This system runs on an IBM PC type computer and is written in  C.

It  uses  TNCs running in KISS mode which allows  all  the  fancy

stuff it does.

Location is Port Huron, Michigan

Tnc's    KPC-4

PK-232

Frequency       Radio      Power      Antenna

145.03          MOCOM 70    60 W      Isopole at 80 ft

145.09          TR-7400A    30 W      Isopole at 30 ft

If you have any questions please leave a message or call

ID gives a list of the ports and digipeaters/gateways available.

The J command lists stations recently heard on the various  ports

and stations that recently connected. Use the P command for  path

to  station that have connected at some time. For  the  following

commands  the  sponsor's callsign is given in  {curly  brackets}.

This  is  normally NOT the call you would use. This  callsign  is

given  to help identify some of the "mystery" callsigns used  for

nodes and digipeaters such as "MYDIGI".

JB command gives a list of BBS's that have been heard along  with

date and time they were heard and the path to them.

JD  command gives a list of the Digipeaters that have been  heard

along  with  the date and time they were heard and  the  path  to

them.

JG  command  gives a list of the Gateways that  have  been  heard

along  with  the date and time they were heard and  the  path  to

them.

JK  command  gives a list of the KA nodes that  have  been  heard

along  with  the date and time they were heard and  the  path  to

them.

JM command List other MSYS systems that have been heard.

JN command gives a list of the NET/ROM nodes that have been heard

along  with the date and time they were heard, the real  callsign

of  the  NODe  and the path to them  (most  are  direct).You  can

specify a port number as an argument to limit the display to just

that port. For example, JK 1 would display the KA Nodes heard  on

port1 only.

The  K  command is used to kill (delete) old  messages  from  the

system. You can kill only those messages that are to or from your

station.   The format of the command is K, a space, and then  the

number  of  the  message to delete.   May  use  multiple  message

numbers separated by a space. Example: k 123 6789 1123.

3-45

KM - To delete all messages TO you.

KF - Kill messages that have been forwarded (sysop only)

KT - Msg# to kill NTS traffic you are going to deliver.

K* - (sysop only) Goes through ALL messages, allowing you to

kill,  keep or hold any of them. You can also read  a  given

msg  (it must be in the mail.bak directory). When reading  a

msg this way, use the space bar to pause the display.

K@  - kills all messages with given @BBS.(sysop only)  only  com-

pares the leftmost  part  of the  @BBS field (the normally

visible  part  of a  hierarchical address) instead  of  any

part which caused unexpected deletions.

Example:      K@ arrl

The  L  command  lists selected message  headers.  The  following

format are available:

L           -  List messages since you last used the B command

LH          -  (sysop  only)   List  held  messages.  A  message

gets held if any of the R: lines contain the idcall

of  the receiving BBS. This will  catch  forwarding

loops.

LM          - List messages to or from you  (List Mine)

LN          - List all messages not yet read

LT          - List NTS Traffic messages

LL #        - List the last # messages

Example: LL 10       - Lists last 10 messages

LO          - List messages older than date yymmdd given

LU          - Lists unread messages to you

LY          - Lists msgs that have been read but not deleted

LW          - Lists msgs with type W (for weather)

L?          - List msgs that will not forward

Also  reports msgs sent to your bbs for calls  that

are not registered as users of your bbs. For  exam-

ple, suppose someone sends a msg to W1AW@mybbs. The

msg  would show up in a regular L as going to  W1AW

and @BBS of --- (what you normally see for messages

to  local users). If W1AW is not a registered  user

of  my  system, then such a message would  show  up

when L? is used.

L< callsign - List messages from callsign

Example: L< WA8BXN

L> callsign - List messages to callsign

Example: L> WA8BXN

L@ callsign - List messages being sent to BBS callsign

Example: L@ WA8BXN

L #         - List messages >= given number

Example: L 827

L # #       - List messages from # to #

Example: L 50 75

3-46

L"string"   - Lists messages with given string in title

(case insensitive)

L'string'   - Lists messages with given string in title

(case insensitive)

Examples:  L'Pfb'   L"ID's"   l"Modem users"

L$          - List messages with type $

Example: L$

The L< and L> commands will also take wildcards as arguments.

Note:  Messages  that  are Private are shown only  to  Sender  or

Addressee.

The  M  command may be used to display the Message  of  the  day.

Normally  this  message  (if not empty)  is  displayed  when  you

connect  unless  you have selected Expert mode (?X for more  info

on  Expert mode). If you  have selected Expert mode, using the  M

command is  the  only way to see any Message of the day. If there

is no message of  the day,  you will simply get the next  command

prompt. The message of the  day  will  advise you of new features

and  other  important information.

N  command can be used to register your name or QTH.  You  should

enter both of these. To enter your name type N your name.

Example: N Mike

To enter your QTH, use the command NQ your qth.

Example: NQ Kirtland, OH

To enter your ZIP or Postal Code, use NZ code.

Example: NZ 44026

To  enter the primary BBS that you look for your mail on  use  NH

callsign.

Example: NH KB8CI

The P command can be used to find the path last used by a station

to connect to this system. The format is P callsign.  Example:  P

W1AW The information given comes from the station's use of the  N

commands  and  information  recorded by the  system.  Try  the  P

command  with your own call to check the information  the  system

has about you.

The  PF command can be used to find out if this system knows  how

to forward mail to a particular BBS (what you would enter as @bbs

in  the S command). The port number and call of the BBS  to which

the  message would be sent is displayed along with the  VIA  list

used for the connect. If Net/Rom or KA Nodes are used to  connect

to the BBS to which the message would be forwarded to the fact is

noted  but  the details are not given.  If this system  does  not

contain  the  requested  BBS  in its forward  file  you   get   a

message that the callsign is  unknown.  Example:  PF WA8BXN

The R command is used to read messages that appear when you do  a

3-47

L  command.  After the letter R and a space you should enter  the

number  of the message to read or multiple message numbers  sepa-

rated by spaces. Example:r 725 1012 892.

RE  - (for read export) This causes the given message(s)  to

be displayed with a S command line, followed by  title,

message  text and then /EX in place of the  normal  way

messages are displayed. If this form of output is saved

using  the output to file command [< filename] you  can

easily export messages to an ascii file.L

RH  -  Gives complete forwarding information.

RN - msg# list

This is like the R command but no headers are  printed,

just  the message text. Good for  marginal  connections

when you otherwise know the message number you want  to

read.

RM - Read unread messages addressed to you.

RP - Read Preview. It does a normal read operation but does

not  mark  the message as having been read so  it  will

show up if you do a LU command.

RS - Reads all mail for SYSOP.

Note  that when you read mail for SYSOP it gets changed

to  mail for Sysop so that it won't continue to show up

as a new msg.

R+ - Goes through all the held msgs using the hold file

trying  to identify why each was held and allowing  you

to  continue  to hold, kill, or release as  private  or

bulletin each msg.

R* -(sysop only) It allows you to read all held messages.

After  each  is  read you are prompted to  enter  H  to

continue  to holding the message, K to kill, N  to  re-

lease as private, or $ to release as a bulletin.

The REPly command is used to generate a reply to a message  with-

out  having to do a S command. The advantage of using the   REPly

command  is that it will usually pick up the correct @BBS  (often

with  proper  H  route) from the R: lines in  the  message  being

replied to. You can use either REPly msg# (example: REP 1234)  or

REPly  msg#.   (example: REP 1234.). In the first form  you  will

have to supply the title. In the second form, a title is automat-

ically generated of the form RE: the-original-title Messages that

passed  through SMTP systems will probably not generate the  cor-

rect reply address.

The S command format:   Sx  to call @bbs

Where  x  is  message type (P=private)

To  call  is  DEStination callsign  (without  -  number)

@bbs is DEStination BBS (again without any - number) [OPTIONAL]

--->  If @BBS part is omitted then if the station has  entered  a

Home  BBS  (NH command) that BBS will be used. Otherwise  the  to

station callsign will also be used as the @BBS.

Example: S W1XXX @W0YYY

You  will next be asked for a title for the message.  After  this

you type the message, keeping all lines less than 80  characters.

When you are done with the message type ^Z or /EX  on a new line.

3-48

If  you  type  ^A in place of ^Z when  you  are   done  typing  a

message, the message will not be stored.

Users  who send messages to stations without giving any  @BBS  or

giving unknown @BBS are warned of the fact.

Use SB to send bulletins, ST to send traffic, etc. All messages

sent  to  a specific callsign will be treated as if you  used  SP

(send  private).   To   make  messages  to  a  specific  callsign

readable   by everyone use SB instead of just S! Or make it to  a

non-callsign (such as all).

The T command  (talk to sysop). When a bbs user types T you  will

get the connect music and then have 60 seconds to type  something

or they will be put back to the bbs. If you do type to them  then

when  you  are done with your conversation and do ^Cd  then  they

will go back to the bbs. The keyboard call can still be used  for

incoming  connects; note that it is accessible to  all  stations,

even on closed bbs ports. Outgoing connects are the same as  they

have always been.  When an incoming keyboard connect occurs,  you

will automatically be taken to the F1 screen

The U command gives the current users of the bbs plus the  number

of active messages and users and what they are doing.

The Upload command  To use it, the user must be authorized

(see user flags ). To do an upload, the user types UP on the BBS.

MSYS  will then ask for the file name to be uploaded. If no  file

name is entered, or the file exists (in the FILES directory)  the

upload is terminated. If the filename is accepted, the user  then

sends  the ASCII file, ending with a line that contains only  ^Z.

Once the transfer is started, it may be aborted by sending a line

with ^A.

The  V  command gives the version of this software and  date  and

time MSYS was started.

The  W  command lists the available files  for  downloading.  The

formats:

W    - Lists all files with dates & sizes

W  (Directory)  - Lists all files in  a  subdirectory  under

files. The directory list provided includes the size of the files

listed.

The X command can be used to set or reset Expert status and  also

to  set  the  number of lines sent before  pausing  during  an  R

command.  If no argument is given, X simply toggles and  displays

the  new  Expert status. When Expert status is set,  the  initial

connect messages are replaced by a compatibility identifier  line

([MSYS-1.10-H$]) and the command prompt is simply >.

If  a number is given after X, this is the number of  lines  to

display  for  a  R  command  before  asking  More?  To   continue

displaying  the current message when More? is sent press  return.

To  abort the message, type N and then return. This facility  has

been  added  to prevent a long message from  scrolling  off  your

3-49

screen  faster  than you can read it. Experiment  with  different

values  until  you find one that  pleases you.  To not  use  this

facility, set the number of lines to  zero.

Examples:

X 20  (pause after every 20 lines)  X 0  (don't pause ever)

When you get More? in R cmd output(as a result of using X cmd  to

set  lines between pauses) you can  type C for  continue  without

any more stops.

Commands: XF and XS. XF sets "fast" mode, XS  sets "slow"   mode.

Fast mode means that multiple lines are  placed  in packets   for

some  commands  (like R and L). Slow mode  means  one  line   per

packet  as  it has been (the default). XF is  faster   for  users

with good paths. XS is better for users with poor paths (or  busy

channels!). Also works with the D command.

Putting * at the beginning of a line makes it a comment.  It

also  suppresses the next command prompt (but the system will  be

waiting for another command). * is useful to answer the  SYSOP if

you get a MESSAGE FROM SYSOP ...

<  filename      (sysop only) causes the commands  found  in  the

given  file to be processed as if they were typed from  the  key-

board. This may be used as a message import function if  messages

are placed in a file. A sample of this is:

SB ALL @ ARRL < W1AW $XYZ27

Sample title

This is the text of the message. It is short.

/EX

You  can  of course have many messages in the file. When  end  of

file is found, input is taken again from the keyboard.

> filename and >> filename

Both  of these commands cause output normally sent to the  screen

to be also saved in the indicated file. Using just one > causes a

new  file  to  be created (replacing any existing  file  by  that

name).  Use  of  two > before the filename causes  output  to  be

appended  to  the end of the filename given (if  it  exists).  To

terminate  writing  to  the file, use either  of  these  commands

without a file name. While output is being saved to a file, there

will be NO PROMPT FOR NEXT COMMAND.

\    To be used for sysop commands while in the BBS  (see  remote

sysop)

3-50

REQxxx SERVERS #

(c)  Copyright 1992 by HUB COMPUTERS, INC

A  set  of  REQxxx servers are available in  the  BBS  they  are:

REQDIR, REQFIL and REQQTH. Messages sent to your system to  REQa-

nything  will be processed. If a server is not available for  the

requested  function, a reply msg will be generated  stating  that

the server is not available. The servers named above are  enabled

by new commands of the same names. That is, using REQDir ON  will

enable  the REQDIR server, etc. Enabling the REQQTH  server  also

requires that a file name containing the Buckmaster CD ROM  ASCII

callbook  file (normally  S:HAM0\HAMCALL.129) be  specified  with

the CAllfile command.

The  REQDIR server responds with a message to the requester  con-

taining  either  the FILES directory listing or  specified  FILES

directory  subdirectories given in the message title. An @BBS  to

which  the message is to be returned may optionally be  given  in

the  title if the reply is to go to a bbs other than the  one  at

which the original message was entered. Examples:

SP  REQDIR  @

WA8BXN

(blank title)

^Z

would return a listing of the FILES directory to the bbs used  to

originate this message.

SP REQDIR @ WA8BXN

@NT8V             (title)

^Z

would  return  a listing of the FILES directory  to  the  sending

station @ NT8V.

Subdirectories  may be given in the title, proceeded by  a  slash

and  separated with blanks. The optional @BBS may also be  speci-

fied. Example:

SP REQDIR @ WA8BXN

/MODS /PGMS/BASIC

^Z

The  file REQDIR.HLP included on the distribution disk should  be

placed  in  the  HELP directory. The file  REQFIL.HLP  should  be

placed in the FILES directory.

The  REQFIL  server responds by sending the requested  file.  The

file name is specified in the title of the message.  Subdirectory

names  may be given if needed (don't include the leading  /).  An

@BBS can be specified in the title as described for REQDIR above.

Example:

SP REQFIL @ WA8BXN

mods/kam.1 mods/kam.2

^Z

3-51

The  REQQTH server responds with callbook information  for  calls

given  in the title. Currently only US hams are in  the  callbook

file. Calls must be separated by blanks. An @BBS may be specified

as described above.

Example:

SP REQQTH @ WA8BXN

nt8v no8m K8EIW

^Z

The callbook server has split out of MSYS. There are two  servers

available,  one for the Buckmaster CD ROM and one for  the  j-com

disk files. Note that the CAllfile command has been removed  from

MSYS. In its place you specify the name of the file (or directory

in case of j-com files) to be used when you start the appropriate

call server TSR.

To  use  either the Buckmaster CD ROM callsign  files,  or  those

available  from j-com, you must load the appropriate TSR  program

before starting MSYS. Once this is done accessing the call server

is  the  same  as it has been in the past (using  either  the  PC

command in the BBS or sending a msg to REQQTH @ the bbs call).

The TSR for the CD ROM files is CALLSRV1. It optionally takes one

pararmeter, the name of the callsign file. If none is given,  the

default is  S:\HAM0\HAMCALL.129.

The TSR for the j-com files is CALLSRV2. It optionally takes  one

parameter,  the name of the directory in which you installed  the

several j-com files. If none is given, the default is F:HAMBASE.

Things needed to get the CD ROM call lookup to work:

Put:

LASTDRIVE=W

DEVICE=SONY_CDU.SYS /D:MSCD210 /U:1 /B:340 /M:P /V /L:S /Q:* /T:*

in CONFIG.SYS  Reboot system.

Run  the following before MSYS (probably put these statements  in

AUTOEXEC.BAT):

MSCDEX /D:MSCD210 /M:4 /L:S /V

MSCDINFO

CALLSRV1 S:\HAMO\HAMCALL.129

Put:

REQQTH ON

in MSYS.DO

Notes:  You may have to make changes in the above  statements  if

you have different device drivers for your CD ROM drive or set up

the  hardware  options differently.  Running  without  interrupts

works pretty well on the CD ROM drive and if you are like me  you

don't  have  any interrupts to give to the drive anyway.  If  you

don't  want to respond to REQQTH queries but only enable  the  PC

command on the BBS, set REQQTH OFF.

For all the servers, the title must be less than 39 characters.

3-52

DATABASE HELP #

(c)  Copyright 1992 by HUB COMPUTERS, INC

Commands related to Database manipulation available on MSYS

MSYS  allows  BBS users to input information   and  search   for

information  in simple data bases. The names  of   the  databases

are  Database.0 through Database.999.  These  files  are  located

in  the  normal FILES directory, which can be accessed by  the  D

command (download) on the BBS.

These files can also be searched by the new G command on the

BBS.  To  use  this  command you enter the  command  G  (with  no

operands).  You  will  then be asked for the  file  you  want  to

search.  Simply  pressing  return  at  this  point  will   select

Database.0 by default. You are then asked for the search  string.

This is a set of characters that when found in lines of the  file

cause  those  lines to be displayed. Case is ignored  during  the

search. If a selected line ends with \ then the following line(s)

are also selected for display.

Data  is  entered into a data base by sending a  message  to

MSYS  with  DBADD in the title. The text of the message  will  be

added to the indicated Database (Database.0) by default. A header

line  is also inserted in the data base telling the station  that

made  the  entry,  along with the date and time.  Here  are  some

examples:

SP MSYS                   {this is a S command issued to the BBS}

DBADD 3               {the message title, Database.3 to be used}

wind damage mentor Ohio \

large tree down at 615 & 20 \

reported by mentor pd \

no injuries

^Z                       {normal way to end message}

The text of this message would be added to Database.3. Now if you

search  Database  3 for either     wind damage   or   mentor  for

example,  you would get to see these lines, along with any  other

lines that have    wind damage    or   mentor    in them.

Example of using search command:

G                        {BBS command to do search}

File search routine      {Response from bbs}

Enter file name (or just press return for Database.0): Database.3

Enter search argument: Mentor

{lines from message above displayed}

>                        {BBS ready for next command}

Another example:

Assume a file had the following lines:

Mary had a little lamb

its fleece was white as snow, \

and every where that Mary went

the lamb was sure to go.

3-53

It followed her to school one day

If you searched this for 'snow' you would get the following back:

its fleece was white as snow, \

and every where that Mary went

That's a brief outline of what can be done. Its simple yet very

flexible.  Let me know of any applications you can think  of  for

these facilities. Particular Database numbers can be assigned  to

different uses. Formats for information can be suggested for each

different Database.

3-54

Forward File Description #

(c) Copyright 1992 by HUB COMPUTERS, INC.

NOTE: TO FORWARD BULLETINS YOU MUST SET UP MSYSBBS.DAT!

The   MSYS forward file (MSYSFWD.DEF) is an ASCII file  that

can  be  prepared with any program editor. The file  consists  of

groups  of  lines separated by a line beginning with at least   3

dashes.  Each group of lines specifies what should be sent  to  a

given  BBS along with other information.

The first line has the following format:

Position  Content

--------  -------

1         The letter F

If you substitute R for F (F0 becomes R0 WA8BXN) then

it  enables reverse forwarding request. P0  Forces  a

reverse forward poll.

2         The port number to use (a single digit)

3         A blank

4 to N    Callsign of BBS (with SSID)

optional:

N+1       A blank

N+2 to ?  Via list of digipeaters

Examples:

F0 W1XX V W2XX,W1XX-1

F2 WA0XYZ-2

When no nodes are being used, this first line is used as  if

it  were a connect command to the desired system. When a NODe  is

used, the Via digipeater list will have no effect.

If  a NODe (KA NODe or NET/ROM) is to be used, the next  set

of lines are a script of how to use the NODe(s). This script  may

be up to 25 lines long. Its format will be described below.

Next comes a list of callsigns of BBS systems that should be

forwarded  to  the  system  given on  the  first  line  (the  one

beginning with F). There is one call per line. No SSID should  be

given. The callsign of the system given on the first line  should

normally be given in this list.

Example forward file that doesn't use any nodes:

F0 W1XX

W1XX

W2ABC

W2XXX

----------

F0 W1XX V W1XYZ

W1XX

W2ABC

W2XXX

---------

4-55

F0 W1XX V K1XXX,W1XXX

W1XX

W2ABC

W2XXX

--------------

F0 W8XXX-12

W8XXX

WA8XXX

K7XXX

------------

Note: Here we try three different paths to connect to W1XX. If we

get  the messages forwarded the first way the  other  connections

will not be attempted.

F0 W1XX

!01-04,12,21-23   This  will forward during  the  times indicated

W1XX              (Use  of this line is optional and  must  begin

W2ABC              with !)

W2XXX

----------

In the list of callsigns given indicating what BBS's msgs can  be

forwarded to a particular BBS if a second entry is  typed on  the

line  then the second entry must match some  part  of  a  message

title to allow it to be forwarded. For example,

F1 W1XX

ARRL DX

----------

@ARRL  will be forwarded only if DX appears in their title.  Case

of letters is ignored in the comparison.

You can use * in the forward file. If you have NTS* then all msgs

with NTS as the first 3 characters of the @BBS will be forwarded.

For example:

F1 W1XX

NTS*

----------

*  used alone is treated specially. Only messages that have  been

on  the  BBS for more than about 2 days will be  forwarded.  This

gives   time  for  them  to  be  forwarded  by  other   specified

connections.

IMPORTANT  NOTE:  In the forwarding file, use of *  for  the  BBS

names  cannot be the first call and the first bbs in the list  of

bbs's  that can be forwarded to through the given bbs MUST  begin

with a letter.

4-56

Example:

F0 WA8BXN

*              won't work

WA8BXN

-----------

F0 WA8BXN

44*            won't work

WA8BXN

-----------

F0 WA8BXN

WA8BXN         will work

*

44*

-----------

Note: That # is a wild card character. If you want to put #XYZ as

an entry in your forward file, you will have to use "#XYZ (the  "

is an escape character that says the character that follows  must

match  exactly and is not treated as a wildcard character  as  it

normally would be).

NODe Scripts:

The script consists of several sets of lines. The first line

begins  with  @  followed  immediately  by  the  call  (and   Via

digipeater  list if needed) of the NODe to initially connect  to.

The port that will be used is taken from the digit after the F on

the first card in the group. If the connection is established  to

the NODe, the  following sets  of  lines are used to talk to  the

first NODe  and  possibly other nodes called through it.

Each  set of lines for a given NODe begins with a line   that

has  a  period in position one. The following characters  on  the

line are sent to the NODe as a command.

The  next line in the set is optional and if used has  #   in

the  first position. Following this is the number of  seconds  to

wait  before  abandoning use of this NODe. If this  line  is  not

given then 60 seconds will be assumed.

The  next  line is required and begins with + in  the   first

position.  The  following characters are what is  looked  for  in

lines  that  come from the NODe to indicate that  connection  has

been  established  to  the station called (another  NODe  or  the

desired BBS). An exact match to the characters given on the  line

(including  case) must be found somewhere in a line  received  to

continue.

Next  come  two  lines  beginning  with  ?  in  the    first

position.  These cards contain strings  of  characters  that   if

found  anywhere  in a line that comes from the   NODe   that  the

connection was not made.

Thus  after we issue the connect command (given on the   line

beginning with .) one of three things can happen: a line from the

NODe  that matches the +string is found and we continue with  the

script or talk to the BBS, a line from the NODe matches a ?string

which  means we give up on making the connection, or  a  time-out

occurs waiting for a match which again means we will give up  the

4-57

connection.

Example script:

@ANOD          (issue connect command to ANOD)

.C            (issue connect to desired call)

#25            (time out after 25 seconds if no match)

+MADE          (if MADE comes from NODe, continue)

?BUSY          (if BUSY comes from NODe, abandon attempt)

?RETRIED       (if RETRIED comes from NODe, abandon attempt)

These  strings  are useful for KA Nodes.

For a NET/ROM  NODe  the following would be useful:

+to

?Failure

?Busy

Here is another example of a complete forward file:

F1 W8XXX

W8XXX

W5XYZ

---------

F0 W4XYZ

@W7XXX

.C W9XXX

+to

?Busy

?Failure

.C W4XXX

#120

+MADE

?RETRIED

?BUSY

W4XYZ

W4XXX

-------

F1 W9XXX-2 VIA K9XXX-3,KA9XXX

W9XXX

W1XXX

W3XXX

K0XXX

----------

NOTE:   The forward connect script for a given bbs is limited  to

25 lines.

The file MSYSBBSB.DAT (YOU MUST HAVE THIS FILE TO FORWARD

BULLETINS!)

This   file   contains  a  set  of  bit   position    numbers

and  corresponding  BBS calls to which $ BID   messages  will  be

sent.  The format of the records is MAX BIT# IS 31 Bit#  Callsign

(0-31)

An example file content is:

4-58

25 WB8JUI

26 KB8CI

27 WA8YWH

28 KQ8M

29 KB3X

(MSYSFWD.DEF):  If you put a line beginning with $  and  followed

by  a  file name (where BBS callsigns normally   go)  the   given

file will be read at that point as  more  BBS  calls.

Example:

F0 WA8BXN

WA8BXN

$BXN.DAT

ARRL

--------

In  this  example, the contents of BXN.DAT will be  used  for

more  calls  that  can be forwarded through WA8BXN. Due  to   the

extra  overhead  of opening, reading, and closing the file,  this

should only be used for more than just a few call signs.

MERge: (MUST DO THIS IN ORDER TO FORWARD MAIL)

This   command   reads   MSYSFWD.DEF   and   copies   it   to

MSYS.FWD, expanding any include files ($filename). A file is also

produced,  MSYS.BBS that is a sorted list of all  possible  BBS's

that can be forwarded to (including things like ARRL, 44*,  etc).

This command is for 2 reasons: if there were any include files in

the  MSYS.FWD files  they  did  not get read when a  pf   command

was  used  and secondly, the new file is needed for  hierarchical

forwarding.  The leftmost part of the hierarchical route only  is

used  for forwarding and the contents of the file is  needed  for

efficient processing.  Any  changes you make to your forward file

must  be done to MSYSFWD.DEF followed by use of the MERGE command

(IN  THE BBS).

Call/@BBS replacement facility:

Create  a  file  called  MSYS.REP In  it  place  lines  with  the

following format

OLDCALL OLD@BBS    NEWCALL NEW@BBS

For example:

ALL  ALLONT   ALL  NEOH    (messages  to  ALL@ALLONT   become

ALL@NEOH)

44* NTSOH   @1  ---        (messages to 44*@NTSOH become original

tocall@ ---)

NTS*  * @2 @1              (messages coming in to  NTS  anything

get tocall and @bbs swapped)

***  IMPORTANT  *** There can be no more than 150 lines  in  this

file!  The algorithm goes like this: When a message comes  in,  a

check is made comparing the to call and @bbs of the msg with  the

4-59

first two fields of the records in MSYS.REP. If a match is  found

(wild  cards are allowed in MSYS.REP) then the last 2 fields  are

used  to replace the tocall and @bbs of the message. @1 or @2  in

the last two fields in MSYS.REP have a special meaning. @1  means

the  original to call and @2 means the original @bbs.  Note  that

wildcards are not expanded in these last 2 fields.

To  support  this  facility  a new  command  is  also  added:

REPlace.

If an argument of L is specified, the current replacement list is

displayed.  If  no  argument is given, the  current  contents  of

MSYS.REP  are read and stored in memory. (This is done when  MSYS

is  started  also).  Use the REP command after  you  have  edited

MSYS.REP to make the changes effective.

REVERSE FORWARDING :

MSYS  can respond with messages to a connected BBS when it  gives

F>.  To  use this facility, you must have a  directory  FWD  that

contains files of the form callsign.REV. These files contain  the

@BBS  values that can be reverse forwarded to the given  station.

So you may want to set up your forward file as follows:

[file MSYSFWD.DEF]:

F0 W8XYZ

$fwd/w8xyz.rev

--------

F0 K8XXX

$fwd/k8xxx.dat

--------

[file FWD\W8XYZ.REV]:

W8XYZ

ARRL

[file FWD\K8XXX.DAT]

K8XXX

K9XYZ

ARRL

With these example files, reverse forwarding would be enabled

for W8XYZ but not K8XXX. Recall that $ lines in the forward  file

MSYSFWD.DEF  cause the given file to be included at  that  point.

Also  remember to be sure to use the MERGE command  whenever  you

make  any changes to the forward files! Reverse forwarding  of  a

given  message is not possible until there has been at least  one

normal  forwarding  (either  forced  manually  with  the  FOrward

command  or  at  your forward time). The other  half  of  reverse

forwarding  (where MSYS would send F> when it is done  forwarding

messages   to a particular station) is enabled with the R in  the

first  line of the forwarding file (F0 becomes R0  WA8BXN).  When

done forwarding MSYS just disconnects and does not send *** done.

Reverse  forwarding checks if the bbs is currently connected.  If

so MSYS will not request reverse forwarding.

Reverse forwarding poll (connects and requests reverse forwarding

even   if  you  don't have anything yourself to  forward) can  be

4-60

implemented. Simple place P where you had R or F before. Thus  F0

WA8BXN   becomes  P0 WA8BXN

A  check  is  made to see if asking for another  message  to  be

reverse forwarded will overfill the maximum number of messages.

Reverse forwarding will check if the required number of  attempts

at forwarding a msg have occurred before reverse forwarding. Thus

if  we  have  OH 25 in the reverse forwarding file,  a  msg  that

hasn't  undergone  25  forwarding attempts will  not  be  reverse

forwarded by this entry. NOTE that selective title forwarding  is

ignored during reverse forwarding.

SELECTIVE FORWARDING:

Selective  forwarding  by  title  contents  is  limited  to   ONE

selection  per route. Thus if you have both ARRL LETTER and  ARRL

PFB  in  your  forward  file for  a  given  bbs,  only  bulletins

addressed @ARRL with LETTER in their titles will be forwarded. No

other  bulletins  with @ARRL (including those with PFB  in  their

titles)  will  be forwarded.

In  addition  to  selective  forwarding  title  comparisons,  two

additional selection criteria may be specified: number of forward

attempts  that  must be made before using this  path  or  maximum

message size to be forwarded using this path. Here some examples:

F0 N8XYZ

ARRL

NEOH DX

N8HTG 10

NEOH '15

OKIPN <3000

OKIPN '<I

-------

This set of lines will cause the following to happen - All  @ARRL

messages will be forwarded. Messages with @NEOH will be forwarded

if  they  contain  DX (upper and/or lower  case)  in  the  title.

Messages  with @N8HTG will be forwarded if type is PN  and  there

have been at least 10 forwarding cycles with the message  present

on the BBS. Messages with @NEOH will be forwarded if there is  15

somewhere in the title. The single quote mark is used to indicate

that the number following is not the number of attempts that must

be  made  to forward the message by other  means.  Messages  with

@OKIPN that are less than 3000 bytes long will be forwarded.  And

finally  messages  with @OKIPN and <I in the title  (again,  case

ignored)  will  be  forwarded. The ability to  not  use  a  given

forwarding possibility until there have been the specified number

of  forwarding  attempts is included so that  preference  can  be

given to favored paths while still allowing alternate paths to be

automatically  used if the favored ones fail. Limits  on  message

size have been implemented for those who may forward to  mini-BBS

systems with limited storage capacity.

TCP/IP FORWARDING:

4-61

TO forward to VE3GKM-4's TCP/IP mailbox the file would look  like

the following.

F0 VE3GKM-4

@VE3GKM-4

.

+NET (may not be needed)

VE3GKM

(MORE @BBS CALLS)

---------------

SMTP FORWARDING:

Sample entry in MSYSFWD.DEF:

T 44 70 4 10 (DO NOT USE DOTS IN THE ADDRESS)

N8HSP

-----

Note that no port number is specified, it does an arp if  needed.

Also note no digis (put them in the arp table if you have to have

them).  No  nodes. Just T and the IP address of  the  DEStination

station  as  shown. This corresponds to the line like  F0  N8HSP.

Next comes the @BBS values to send to this station. There can  be

more   than one. Then the ------ that ends this entry.  The  same

forward  file  is used  for both  normal   forwarding  and  tcpip

stuff. I have also  made  some changes to the SMTP command. After

you  use  it  if you type  PRompt it  will  prompt  you  for  To:

(enter  call@bbs) Reply- to:    (just hit return if you like  the

default given) Subject:    (this is in the body of the text,  not

the  title) Then you enter your text and end  with . (not ^Z)  as

instructed.  The  result is a message  that  contains   all   the

fancy  extra  lines  often  found  in  tcp/ip messages.

Bulletins   may  be forwarded using SMTP. To do   so   you   must

assign  the smtp station an entry in the MSYSBBSB.DAT  file  like

you  would for normal forwarding (don't use any ssids). Also  add

the  callsign  of the station in the T line in the  forward  file

after the IP address. Example: T 44 70 4 15 W8XYZ

This  should  be  done  in  the  forwarding  file  for  all  SMTP

forwarding.

Bulletins   are  accepted from SMTP (a bulletin goes to  a   non-

callsign).  Bids  are  made  up based  on  your  msg  number  and

callsign.

Heirarichical addresses are added to msgs that come in via SMTP.

NOTE: Since Bids are not supported in this implantation of  SMTP,

bulletins  can either be sent to or received from TCP\IP  systems

but not both ways.

MSYS  will accept <RCPT TO>: lines in either the format a%b@c  or

b!a@c  where  c is your bbs call, a is the  destination  stations

call and b is the destinations bbs. Both of these get transformed

4-62

into  a@b on the msys msg list. The first form (the one with  the

%)  is  newly  added in 1.10 to help out in what seems  to  be  a

difference of opinion as to the correct way to do it.

*** FORWARDING NOTES

Make sure you don't have any blank lines in your various  forward

files (msysfwd.def and the files in the fwd directory).

Executing commands from within the forwarding file.

In  the forwarding file at the beginning of a set  of  forwarding

lines  (ie, at the beginning of the file or after a ------  line)

you   can  place any sysop command(s) by simply  putting   \   in

front of them.

The  MSYS.BBS  file contains a number after each "BBS"  that  the

system knows how to forward to. This is the ORing of all the  BBS

Masks  that can be used to forward the particular @BBS.  This  is

used   to   determine  when bulletins have  been  sent   to   all

stations  that  should be getting them; when such a  bulletin  is

found,  it  gets its type changed from $ to # so then  it  is  no

longer considered for forwarding.

Within  the forward file a !timestring line may be placed  before

any  callsign  to limit forwarding hours for  that  callsign  (or

bulletin route). The first ! line as used before limits all times

for  a bbs entry you forward to. The following ! lines remain  in

effect  until changed by another ! line. When you  start  another

bbs forward entry group the forwarding hour defaults to 0-23.  At

this  point I would like a little example of what this means,  so

here  it is (the comments and blank lines would NOT really be  in

the forward file):

F0 WA8BXN            {this is the beginning of a forward group}

!0-16,19-23          {don't initiate forwarding during hours of

5 or 6 P.M.}

WA8BXN               {forward the @BBS any time we connect}

!0-16,22-23          {don't forward the following during early

evening hours}

NTSOH

OH

!1-4                 {forward following only in early morning}

ARRL

ALLOH

-----

BID:

When a msg comes in to non-callsign without a bid a check is made

with  generated  BIDS  of  msg#_callsign  and  callsign_msg#   to

eliminate duplicates.

Messages  addressed  to other than a what looks like  a  callsign

automatically get a BID generated for them.

Mail forwarded to BBSs identifying with either [MBL or [... -  $]

4-63

get BID included in S command.

FORWARDING DISPLAY:

As  each of the BBS's to which msgs can be forwarded  is  scanned

for  a given bbs you will see it. When a msg is  being  forwarded

you will see a line that gives its number and the to station  and

@bbs of the message. In parenthesis you will see the entry in the

forward  file  that is being used. Thus if  the  forwarding  file

contained.

F0 WA8BXN

WA8BXN

44*

---------

and there was a message for K8ZZZ@44123 being forwarded you would

see something like Fwding msg # 123 K8ZZZ@44123(44*).

DROPING @BBS:

Messages  that  arrive  with @BBS of IDCALL get  the  @BBS  field

changed to  ---. This makes them distinctive

Messages  sent to REQQTH, REQDIR and REQFIL are left  private  if

they arrive that way.

***  There  can  only be up to 300 different  bbs  names  in  the

forwarding file. To check this do a LCount on the file MSYS.BBS.

Wildcards (where you can use *) have been enhanced to allow  more

than  just using *. Based upon the recommendations of  AA4RE  and

others, the following have been implemented:

* Matches 0 or more occurrences of anything

@ a..z, A..Z

# 0..9

+ a..z, A..Z, 0..9

? a..f, A..F, 0..9 (hex digits)

< a..z

> A..Z

$ =@#+?<>$         (any one of those characters, but I don't have

any guesses as to why this group was included)

= any one character

" the escape character: the next character must match exactly

( beginning of group. (abc) matches a b or c.

Examples: 9v1A2q   matches #@#@#@

95===   matches anything 5 characters long beginning

with 95

(WK)6    matches anything starting with W6 or K6 (case

is important)

95###    matches any 5 digi number beginning with 95

*** HF Forwarding regulations do not allow transmitting 3rd party

traffic  unless there is a control operator present, the  station

is  under  remote control, or a STA has been issued.  To  aid  in

4-64

compliance  with the regulations, the ATtended command  has  been

added.  When set to ON all forwarding is allowed. If  off,  ports

with radio speed of 300 will forward only those messages in which

the from call is the idcall (messages that you originate) or  the

to  station  in  the message is the station  you  would  directly

connect to for forwarding. HF reverse forwarding is also inhibit-

ed if ATtended is OFF. Note that the  regulations appear to allow

you  to receive messages unattended. So if you don't have  a  STA

for HF turn AT OFF when you aren't there and remember to turn  it

ON when you are. To be on the safe side, put ATtended OFF in your

MSYS.DO file should the system reboot when you aren't present.

*** MISSING TEXT TO MESSAGE

If  the  text  for a message is not available when  it  is  being

forwarded,  the  forwarding is aborted instead of sending  out  a

missing  text message  for the text. The STatus command will  now

indicate  how many times this has occurred and the  last  message

number  for which it happened. The  MISsing command may  also  be

used to determine how many times this occurred without generating

all of the other STatus output.

****  BE SURE TO INCLUDE MSYS in your forward files for all other

stations you forward to so I can send bulletins about MSYS to all

of you. The list of MSYS operators keeps growing!  ****

*** MSYS node forwarding

You can forward directly through your own MSYS node (assuming  it

is properly activated). Here are some examples how to do it:

F0  W8XYZ                {This is the station we  are  forwarding

to,the number after F is not important}

%W8XYZ                  {Do a network connect to W8XYZ - assuming

W8XYZ is a BBS that is in our local node

destination list}

W8XYZ                   {stuff to forward to this bbs}

-----

F0 W8XYZ

%W8XXX                   {connect to node W8XXX which is  in  our

local dest list}

+to

?busy

W8XYZ

-----

4-65

Hierarchical Forwarding and MSYS

(c) Copyright 1992 by HUB COMPUTERS, INC.

There are several (new) features in MSYS that combined  with

hierarchical forwarding should almost completely automate routing

of  incoming messages. These features act at two separate  times:

first when a message comes in and second during forwarding. First

let's review the way hierarchical forwarding works.

With hierarchical forwarding the @BBS field can contain more

than just the callsign of the BBS that is the DEStination of  the

message.  Routing information can also be placed in  this  field.

The  @BBS field thus has the syntax of part1.part2.part3.  ...  .

When it is time to forward a message, the @BBS field is inspected

and  the  leftmost part that is found in the forwarding  file  is

used to forward the message.

Consider  as an example @N8HTG.MI. If N8HTG is found in  the

forwarding file then it is forwarded that way and the MI part  is

ignored.  On  the  other  hand, if N8HTG  is  not  found  in  the

forwarding file, then if MI is found in the forward file then  it

is forwarded that way. If neither part is found in the forwarding

file,  then the message will not go anywhere. Such messages  will

show  up  (after at least one forwarding attempt after  the  last

time MSYS was started) if you do a L? command.

If  all  messages  had  both  the  actual  callsign  of  the

DEStination  BBS  and the appropriate xx part in the  @BBS  field

then  the forwarding file could be greatly shortened.  You  would

need  to have the calls of the BBS stations in your state in  the

forward  file,  along  with each of the xx  designators  for  the

remaining states. If there were say 50 BBS systems in your  state

then  perhaps  99 different entries would be sufficient  in  your

forward file (ignoring for the moment bulletin routes like ARRL).

Remember  that I have often said that the limit of 300  different

calls in the forward file is more than sufficient!

Messages that are going to a BBS in your state get routed as

specified  in your forwarding file; those going to another  state

get  routed to the BBS you have set up in the forwarding file  to

get  stuff  for  that state. What could  be  simpler?  You  could

include  a  few explicit calls for BBS systems in  nearby  states

that  you route directly to, if you want to override  the  normal

path for messages going to that state if you so desire.

Now for the second part, the action taken by MSYS when a new

message arrives. When a message comes in, MSYS looks at the  @BBS

field.  If it is absent, then if the to callsign  field  contains

something that MSYS knows the routing for, the to field is copied

to  the @BBS field. Otherwise the @BBS field is set to =====  and

the message won't go anywhere. It will show up as noted above  in

a L? command.

How  does  MSYS  determine  if it knows  the  routing  to  a

particular  callsign?  It uses a file  called  BBSTONTS.BIN  (new

beginning  with version 1.04). This file contains  BBS  callsigns

and  the  states  they belong in  (this  information  comes  from

several sources, more on this later).

Next  MSYS inspects the @BBS field to determine if it  knows

how  to forward what is in the given @BBS field. If some part  of

the possibly hierarchical specification is found in the .pa

4-66

forwarding  file, it is left alone and will be forwarded by  that

matching  specification. If no match is found in  the  forwarding

file, then the BBSTONTS.BIN file is searched to find some part of

the  given  @BBS field. If a match is  found,  the  corresponding

NTSxx is appended to the existing @BBS field.

Thus MSYS will attempt to lookup the right state to send the

message  to.  Assuming the state is found, then  forwarding  will

take  place as described above (see the N8HTG.MI example). If  no

part  of the @BBS field can be recognized, then the message  will

not  be  sent out, but will show up when you do a L?  command  as

described above.

The BBSTONTS.BIN look up will also handle zip codes given in

the @BBS field, making @12345 into @12345.xx as is appropriate.

Thus  far  we have examined what happens  when  to  automate

forwarding. In the event the proper routing cannot be  determined

by  MSYS  then you will have to use the sysop EM  (edit  message)

command to manually add the appropriate .xx designation, assuming

you  can guess it somehow. If you are lucky, the  originator  put

useful information in the title of the message or may have hidden

it in the text of the message. Maybe the callbook will be of some

use  to  you.  Or maybe you can just force  it  in  the  opposite

direction from which it arrived.

The  next  question that should be in your mind  is  how  to

create the BBSTONTS.BIN file. This file contains binary data  and

cannot  easily be created with a text editor. A utility  function

called  INDEXNTS (function 14 in MUTIL.EXE) has been provided  to

create  this  file.  INDEXNTS takes as its input  a  file  called

BBSTONTS.DAT and produces as output BBSTONTS.BIN.

How  do you create BBSTONTS.DAT? You run a  function  called

BBSTONTS  (function  13  in  MUTIL)  which  takes  as  its  input

BBSLIST.DAT.

BBSLIST.DAT  is  created  and/or  updated  using  data  from

message headers that have passed through your system and/or W9ZRX

bbs lists. Function 10 of MUTIL is used to use messages as input;

function 11 uses W9ZRX lists as input.

4-67

Lets look at a picture of these files and programs:

+---------------+

| Message files |

| from MAIL.BAK |

+---------------+

|

|

|

V

MUTIL function 10

|

|

|

+-------------+

| BBSLIST.DAT |

|   file      |

+-------------+

|

|

MUTIL function 13  <--- Other standard files:

|                   STATES.DAT

|                   ZIPCODES.DAT

V                   ROUTES.DAT

|

|

+-------------+

| BBSTONTS.DAT|

|    file     |

+-------------+

|

|

MUTIL function 14

|

|

V

|

|

+-------------+

| BBSTONTS.BIN|

|    file     |

+-------------+

4-68

MSYS Network NODe Introduction

(c) Copyright 1992 by HUB COMPUTERS, INC.

With version 1.06 is the ability to interface a variety of

the functions previously found in MSYS into networks built  using

any combination of other MSYS systems, TheNet nodes, G8BPQ  Nodes

and  Net/Rom nodes. This is last "major" enhancement planned  for

MSYS  at  this time. The following few releases are  expected  to

come  at about one or two month intervals and will add small  new

capabilities, correct errors and improve efficiency. The  network

NODe  support in MSYS is by no means complete or perfect at  this

time but it does seem to be usable.

Some basic information concerning the implementation of  the

MSYS  NODe  interface will help in making  decisions  related  to

configuration.  Two  terms  must  be  understood  by  the  system

administrator   (SYSOP).  These  terms  are  neighbor  NODe   and

DEStination  NODe. A neighbor NODe is a compatible NODe to  which

there  is  a  direct connection path.  Currently  MSYS  does  not

support use of digipeaters in the path to neighbor nodes.  Normal

AX.25 connections are made to neighbor nodes. The data  exchanged

by  these  connections consists of a mixture of  supervisory  and

data frames that together support the network operation.

DEStination  nodes are also network nodes and are the  nodes

listed  when  one  does a Nodes command.  DEStination  nodes  are

nodes that can be reached through the network either directly  if

they are also neighbor nodes, or indirectly through some neighbor

NODe.

Routing  of  frames in the network  is  conceptually  fairly

simple, handling all the details is not! Each NODe does a  "Nodes

Broadcast" periodically (usually once an hour) which is a list of

nodes  that  it  know how to route data to. This may  be  due  to

directly  hearing  a neighbor NODe or by hearing the  nodes  list

from  neighbor NODe that claims to have a route to each  NODe  it

lists in its NODe broadcast.

Information from the nodes broadcast is maintained by  MSYS.

When  it receives a frame that is not for itself, MSYS  looks  to

see  which neighbor NODe can handle the DEStination indicated  in

the frame and simply passes the frame on to some neighbor NODe.

Data frames while they are inside the network as handled  as

what  are  termed "datagrams". What this means is that  during  a

conversation  between two end users there is no guarantee by  the

network  that the data frames will always go by the same path  or

that  they  will  make  it  through  the  network.  Depending  on

conditions in the network the path taken by frames can change and

at times frames will be arbitrarily discarded.

This does not lead to chaos because the end nodes (the  ones

the  users connect to) are responsible for error free end to  end

services.  While the AX.25 protocol makes sure that  transmission

of frame between any two nodes is error free the network protocol

establishes error free operation between users.

MSYS  network  NODe functions include both  the  routing  of

frames  within  the network as well as the uplink/link  logic  to

support  the  error free end to end services.  Additionally,  the

TCP/IP  servers can make use of the network as an alternative  to

the existing IP routing facilities.

5-69

Connections from the network may be made directly to the BBS

through  the enhanced NODe command set. At this  time  forwarding

directly into the network is not supported but should be added in

a future release.

An attempt has been made to make the MSYS network NODe  user

friendly  while still retaining compatibility with other  network

nodes. To this end there is a help command and headings on output

generated by various information listing commands. Since MSYS  is

generally run at an attended site, a Talk command is available to

allow users to talk to the sysop directly from the network.

To  add all these enhancements, MSYS has grown a bit. It  is

now suggested that you have 640K ram available (more won't be  of

any  use).  There are also several new commands/options.  To  not

further delay distribution of version 1.06, the documentation for

all  the  changes is contained in this update  file  rather  than

being integrated into the existing documentation; the integration

should be done in the next release.

Basic Decisions About the Network NODe

The  really  big question is should you enable  the  network

NODe  functions of MSYS? Fortunately, you have more options  than

just  yes  or no! So perhaps the question should be  reworded  as

should  you run the NODe in full feature, fully  automated  mode?

The  answer  for  most stations will be NO. The  answer  in  your

particular  case will depend on the capabilities of your  station

and  the locations of other compatible nodes. If you run  a  high

profile station already (wide coverage as a digi/K-NODe) and have

compatible network nodes at reasonable distances from you  (maybe

at least 30 miles away), then you may fill a need by running with

everything  enabled. On the other hand, if the only way  you  can

talk to anything is through a digipeater (even when the batteries

in your ht are fully charged) and on a hazy day you can still see

the antennas of at least five network nodes, all still forwarding

the same frame they received 2 years ago, then may I suggest  you

run in a very controlled minimal mode. Make friends, not  enemies

through these latest enhancements to MSYS!

5-70

Network NODe Quickstart

This section tells the minimal things you must do to get the

NODe  up and running in some (probably not optimal) fashion.

You   MUST   define your network NODe callsign  using   the

NRC

command. If you are also active in TCP/IP  you should set  NRCall

and MYTcpcall to be the same thing.

You MUST use the NPorts command to enable the ports that you

want to be able to access the NODe. For example, NP 3 would allow

access to the NODe on ports 0 and 1 but no others.

You  SHOULD  define your network  NODe  mnemonic  identifier

using the NRI command.

You MUST enable the NODe using the NETNODE ON command.

You  should probably turn AUTONode and AUTODESt ON  and  set

OCM to 3.

This  should put your system into full service automatic mode  as

far  as the network NODe is concerned. The probably will  not  be

the way you will want to run but its a starting point.

5-71

Manual Operations on the Network NODe Database

To REMove a DEStination NODe completely, use

REMove DEStination nodecall

To REMove a DEStination for a given neighbor NODe, use

DESt DELete DEStcall neighborcall

To  REMove  a  neighbor NODe (and all  the  DEStination  routes

through

this neighbor NODe), use

NODe DELete nodecall port#

To add a new neighbor NODe, use

NODe ADD nodecall port# quality [digicall1 ... digicall8]

Note:   The  mnemonic identifier for the neighbor  NODe  will

be  obtained from the first nodes broadcast heard from the  NODe.

Use of the digipeater calls is optional. Although you can specify

up to  8 digipeaters, rarely will more than 1 or 2 be   practical

to use.

To add a new DEStination for a given neighbor NODe, use

DESt  ADD  [destid:] dest-call  neighbor-call  neighborport  obs#

quality#

Notes: The neighbor NODe callsign must be defined  before  any

DEStinations can be added for it.

mnemonic for the DEStination is optional.

obs# is the obsolescence count, 0 locks the DESt in  for

the NODe.

To change the quality of an existing neighbor NODe, use

NODe REPlace callsign port# new-quality [digi1 ... digi8]

To  change  the obsolescence count and quality of  a  DEStination

through some neighbor NODe, use

DESt  REPlace DEStcall neighborcall  neighborport  newobs#

newquality

5-72

How I Run My Network NODe

I  run my netnode in a moderate mode here that is  somewhere

between fully automatic and low profile. Here is some basic  info

that should help you understand my configuration:

Computer is an old original IBM PC 4.77 MHz clock 640 K ram,  two

10 M hard drives, 4 serial cards and a printer card.

Port Freq

0  223.7       High profile (150 W, beam at 80 ft. aimed west)

1  145.01      Quite high profile (200 W omni ant at 90 ft.)

2  145.05      Low profile (10 W omni ant at 60 ft.)

3  80 M

4  40 M

5  20 M

6  serial connection to another computer

Nearby existing netrom nodes:

Port Call     Distance

0  K8EIW-9  15 (Miles)

1  K8EIW-7  15

1  K8EIW-1  45

1  W8QLY    75

1  W3LIF    80

2  KB8AST-1 25

Nearby NET.EXE stations using netrom interface:

Port Call     Distance

1  N8HSP-4  10

1  WB8LYJ-4 15

All  of  these stations are usually reliable  connections.  Other

netrom  nodes  are  usually heard but are  not  always  reliable.

During  band  openings  many more are heard,  some  are  reliable

during the band opening.

Here are the network NODe related parameters I have in my MSYS.DO

file:

OCM  3                     Transmit DESt nodes  heard  that  have

obsolescence counts 3 to 6

NRI 460406                 Right 3 bytes of my IP address

[44.70.4.6]  expressed in  hex  (quite

useful this way)

NRC  WA8BXN-2              Callsign net NODe responds to,  should

be the same as TCP/IP callsign (MYTcp-

call)

NPorts $47               Network NODe uses ports 0,1,2 and 6 only

PQuality 100             Default port quality 100. A low value

assigned to nodes not manually entered

that connect to mine

NODe REStore             Read saved NODe file when MSYS starts

AUTONode OFF             Do NOT record NODe broadcasts from nodes

not already in neighbor lists. This

means that new nodes heard (during band

openings for example) will not be

automatically added, unless they do

5-73

actually connect to my NODe.

AUTODESt ON              DEStination NODe broadcasts (from nodes

already in my neighbor table) will be

recorded.

NETNODE ON               Enable network NODe

Here is the output I get from the JN sysop command (or R  command

in the NODe):

Port   Neighbor NODe Call  Quality DESts Heard    Digipeater(s)

1        AKR:K8EIW-1     179     66   23:43

1       CLE1:K8EIW-7     180     67   23:41

0     CLE220:K8EIW-9     180     67   23:34

2     PRYTWP:KB8AST-1    192      2   23:25

1      CLEIP:N8HSP-4     192      1   23:42

1     MERCER:W3LIF       180      6   23:44

1        YNG:W8QLY       180      7   23:36

1      PVLIP:WB8LYJ-4    192      1   22:46

The general philosophy I am using is that I only want to  attempt

to connect to network nodes that I have a good path to. These are

also the nodes that I broadcast myself (and the DEStinations they

claim they can handle). I have manually entered these nodes  into

my  neighbor NODe table (using NODe ADD). I see little  sense  in

saying that I can really connect to a NODe I hear once because  a

meteor  hit at the right time! This method does not  particularly

limit the number of DEStination nodes my NODe knows about because

the  nearby nodes claim they can handle everything they  hear.  I

just  let  the  nearby  nodes  worry  about  finding  routes   to

everything they claim to hear.

My  system  serves as a link from the one isolated  NODe  on

145.05 LAN frequency to the nodes on 145.01 which give access  to

the Ohio 4800 Baud UHF backbone frequency. I also serve as an  IP

switch to and from the network. I have adjusted the neighbor NODe

qualities  to favor nodes (primarily TCP/IP stations) to which  I

am a good route. I have set most of the normal netrom nodes to  a

lower quality value so that they will talk directly to each other

most   of  the time rather than trying to go  through  my   NODe.

Should  they  be unable to connect directly to each  other,  they

will still be able to go through my system.

If you want to run very low profile in terms of getting used

as  a network NODe, leave the default of 10 set for  OCMin.  This

means that your NODe broadcasts will only send out your own call.

This will get the network to recognize you but not make it  think

you  are  a good relay to anything. Keep AUTOD ON  so  that  your

system will know how to reach nodes in the network. Its  probably

a  good idea to set AUTON OFF and manually add the calls  of  the

best nearby nodes you can connect to (one is all you need).

If you are totally confused about the network operations, try  to

consult  with  who  ever runs your local  NODe(s).  Most  of  the

parameters  you  have,  they have (with  some  changes  in  their

names).

5-74

Miscellaneous Comments on Network NODe

If  you are low on SSIDs, you might consider the  following.

Set the IDCALL to your callsign without any SSID. Do not define a

BBS, Keyboard,  or  K NODe callsign. Pick an SSID  and  use  your

callsign  with it and set as the value for MYTcpcall, NRCall  and

FORWARD call.

To get to your BBS the network NODe BBS command can be used.

To  talk  to your keyboard the BBS T command or  network  NODe  T

command can be used. If you want to make outgoing calls you  will

have to define a keyboard callsign.

Should  you use the network NODe to route TCP/IP  frames?  I

would  say only if you have to! The normal IP routing  is  better

than  using the network NODe but in those cases where  using  the

network is the only way to reach some distant IP station then  do

use the network.

Network node connects are allowed only on those ports

enabled by NP command.

Knode  which  lists  K nodes that have been  heard.  An  optional

parameter may be given that specifies the port for which the list

is desired.

Justheard which lists stations recently heard. Here again a  port

number  may  be given as an argument. The N command  is  now  de-

scribed as Netnodes (lists destination network nodes known).

In  the  network node, using the C# form of the  connect  command

(example:  C2 WA8BXN) will always result in a normal  ax.25  con-

nect,  even if the destination station is a network node. We  are

getting very close to entirely removing the K node!

The network node does not record information in nodes  broadcasts

heard  on  ports that are not enabled for network  node  use  (NP

command).

5-75

NET NODe Parameters Summary

Default

PQuality  [port#] #       192  Quality for nodes hear on port

BInterval seconds        3600  Nodes Broadcast interval

OCMmin    #                10  Minimum obsolescence to broadcast

OCInit    #                6  Initial obsolescence count

WQupdate  #                1  Worst quality DESt route to record

NTtl      #                64  Network time to live (max hops)

TTimeout  seconds          60  Transport timeout (Net FRACK)

TRetries  #                3  Transport retries

TACK      sec              3  Transport ACK delay (responsetime)

TBdelay   seconds         180  Transport busy delay

NDWindow  #                 4  Net default window size (frames)

NETCon    ON/OFF           ON  Enable connect cmd in net NODe

NETBbs    ON/OFF           ON  Enable BBS cmd in net NODe

NETTalk   ON/OFF           ON  Enable Talk cmd in net NODe

AUTODESt  ON/OFF          OFF  Add new DESts for known neighbors

AUTONode  ON/OFF          OFF  Add new neighbors heard

NODe      SEnd                 Force known DESt transmission

"        SAve                 Write net NODe Database to disk

"        REStore              Read net NODe Database from disk

"        ADD call port qual [digis]    Adds neighbor NODe

"        REP call port qual [digis]   Changes quality of

neighbor NODe

"        DEL call port      Deletes neighbor nodes & its DESts

DESt      ADD  [id:] destcall neighbor call  neighborport   obs#

quality  Adds new DEStination

DEST      REP destcall neighborcall  neighborport  obs#  quality

Changes obs & quality

DEST      DEL  destcall neighborcall neighborport  Deletes  DESt

through given neighbor

REMove    DESt callsign       Deletes DESt through all neighbors

NInfo                         Gives net NODe status

NRCall    callsign            Sets net NODe callsign

NRid      mnemonic            Sets net NODe mnemonic identifier

NPorts    portmask         0  Ports to which nodes broadcast

TNatime   seconds        600  Transport no activity timeout time

C  BBS or C bbscallsign  act like BBS command taking you  to  the

bbs.  You also now get a connected to message when any  of  these

commands.

In  previous releases, network connects to the bbs  were  often

treated  as coming from port 0. This causes some  problems  since

the  port  0 options are not always what is  wanted  for  network

connections  (port  0 might be a closed HF  port,  for  example).

Actually  a  network connection shouldn't be considered to  be  a

connection on one of the ports (a network connection may in  fact

switch ports during its lifetime). For these reasons, a number of

new commands have been added to allow setting options for network

connections.  Here are the network command names and  their  port

related counterpart.

NETOnly         BPorts

NETRead         MAXRead

5-76

NETMax          BMaxcon

NETFbbstime    FBbstimes

5-77

MSYS DX Node #

(c) Copyright 1992 By HUB COMPURERS,  INC.

General Description

The  MSYS DX Node is a subset of and compatible with  the

PacketCluster  (trademark  of  Pavillion Software,  PO  Box  803,

Hudson,  MA 01749). The DX Node provides services of interest  to

DXers,  such as DX station spotting and WWV reports, in  addition

to a conference function. The DX Node may be used in  stand-alone

mode  or connected to a PacketCluster node. Currently  MSYS  does

not  support connects from other MSYS DX Nodes  or  PacketCluster

nodes.

User Interface

The  DX Node can be accessed from the Network node using  the

Dx command.

The  DX Node may also be entered from the BBS through use  of

the  C (for Conference) command or by connecting directly to  the

DX  Node callsign (see DXCall command).  When in the BBS  itself,

the  SH/ family of DX Node commands may be used  without  leaving

the BBS (SH/DX for example). The command syntax used once in  the

DX Node is patterened after the commands familiar to  PacketClus-

ter users. The H command gives a list of the user commands and  a

brief  description of each. Its output is similar to  that  shown

below:

DX Node commands

Bye                         Disconnects from DX Node

BBS                         Returns to BBS

Conference                  Enters local conference

DX freq dxcall comments     Enters given data as DX spot

SH/CL                       Lists basic stats about DX network

SH/COnfig                   Lists DX nodes in network and users

on each

SH/Dx                       Lists last 5 DX spots

SH/Dx/#                     Lists given number of DX spots

SH/Users                    Lists local DX node users

SH/WWV                      Lists WWV reports (up to 5)

Talk callsign               Sends following lines to given user

call

Talk callsign msg           Sends single line msg to given user

call

WWV SFI=# A=# K=# comments  Enters given data as WWV report

Bye      Disconnects  from  DX Node. A  regular  disconnect  also

works.

BBS     Leave DX Node and go to BBS (even if original connect was

into DX node call directly)

Conference   Enters local conference. Following lines typed  will

be sent to all stations in the local conference.  Control-Z  (^Z)

is  used to exit the conference. When a station enters  or  exits

5-78

the conference a message noting the event is sent to the stations

in  the conference.  When in the conference the various forms  of

the  SHow/ command can be used by preceeding it with an  asterisk

(example: *sh/dx).

DX freq dxcall comments     Enters given data as DX spot which is

sent to all station in the DX node locally (and the DX network if

connected to a PacketCluster station).  Some examples:

dx 14026.1 zp6cw

dx 3795 KP2A/KP5 Listening 3805 - 3810

SH/CL    Lists  basic stats about DX network. Output  looks  like

this:

Cluster: 20 nodes, 1 local / 30 total users  Max users 75

The Max users value is the highest number of users in the network

at one time. This value is reset each time MSYS is started.

SH/COnfig        Lists  DX nodes in network and  users  on  each.

Output might look like this:

NODE            USERS

N8HTT               KB8VN               WX8T                 W8NR

W8XYX

N8BLD                 WA8VPN                 NZ8O

KC8FS

N4RR

AD8J            K3UA            KA3JWJ          N3BPB

In  the  sample  above, N8HTT,  N4RR and AD8J are  DX  nodes  (or

PacketClusters).  The other stations listed are connected to  the

given nodes. N4RR shows no users connected.

SH/Dx  or SH/Dx/#  Lists previously broadcast DX spotting  infor-

mation  as shown below. By default 5 lines are sent. The  SH/DX/#

form may be used to specify more or less than 5 (up to the  limit

set with the DXSTart command).  Examples of this command:

sh/dx

SH/dx/10

Sample output from this command:

14214.5  CE0LJI      28-Aug-1991 0206Z Easter Is           NO9Z

14151.0  A41KJ       28-Aug-1991 0207Z                     AJ9C

14020.3  3B8FG       28-Aug-1991 0353Z QSY'd               WB3KKX

7116.1   WB9FIU      28-Aug-1991 0351Z GREAT SIGNAL!       N8LSQ

7006.6   FO4DL       28-Aug-1991 0350Z                     N9SW

SH/Users  Lists local DX node users

SH/WWV  Lists WWV reports (up to 5). Sample output:

Date        Hour    SFI  A  K      Forcast

27-Aug-1991   07      210  10  3  N24=M-H/UNS            ONKD2YP

27-Aug-1991   21      196  18  5  SA=L GF=U-A > SA=M GF=A.  K8IP

28-Aug-1991   00      196  20  4  mod mostly active SPE PCA K8DD

Talk callsign  All typing until ^Z is typed is sent to the  given

5-79

station.  The  station may be a local user or connected  to  some

other  node in a DX network. The ^Z must be the only  content  on

the line used to exit talk mode.

Talk callsign msg  Sends single line msg to given user call.  Use

of ^Z is not required (nor allowed) with this command. Example:

t kq8m Anybody there?

WWV  SFI=#  A=# K=# comments  Enters given data  as  WWV  report.

Example:

wwv sfi=210 a=10 k=3 N24=M-H/UNS

Sysop Interface

The  following commands controlling the DX Node are available  as

Sysop commands:

DXCall          callsign

DXDiddle        #seconds

DXEnable        call - list

DXNconnect      scriptfilename

DXPorts         portmask

DXRestore

DXSAve

DXSTart         #nodes #users #dxspots #wwv

SHow/...

DXCall callsign

where  callsign is the call, with ssid, that will be used by  the

DXNconnect  command to connect to a PacketCluster station.  If  a

connect is made to this callsign, it will take the user  directly

into the DX Node.

DXDiddle #

where # is the number of seconds of inactivity before a  'diddle'

packet  is  sent to connected dx node to keep  connection  estab-

lished.

DXEnable call-list

Enables  the  given callsigns (with ssids) to connect to  the  dx

node as dx nodes. This allows MSYS systems to link their dx nodes

together. Example:

DXEnable K8EIW-7 NO8M-2

DXNconnect   scriptfilename

where  scriptfilename is a connect script file found in  the  FWD

directory  using  the same format as the  keyboard  C  scriptfile

command.  Note  that connection to only  a  single  PacketCluster

station is allowed. Your DXCallsign must be enabled as a  Packet-

Cluster  node by the sysop of the node to which you will be  con-

necting.  The PacketCluster command that is used for this is  the

SET/NODE command (not supported in MSYS DX Node).

5-80

DXPorts portmask

One bits enable use of the DX node on that port. Rightmost bit is

Port 0.

DXRestore

Reads  the last recorded DX spotting and WWV information. If  the

current maximum number of DX spots or WWV reports (as set by  the

DXSTart command) differs from the values in effect when the  info

was  last stored this command will have no effect.  This  command

should  probably be put in MSYS.DO after the DXSTart  command  if

the DXNode is to be enabled.

DXSAve

Records  the  current DX Spotting and WWV information in  a  file

called   DXNODE.DAT. This function is done automatically  when  a

QUIT command is issued if the DX Node has been started.

DXSTart #nodes #users #dxspots #wwv

where

#nodes is max nodes in DX network (default 25)

#users is max users in DX network (default 100)

#dxspots is number of DX spots to be recorded (default 20)

#wwv is number of WWV reports to be recorded (default 5)

If the first parameter is a letter, most of the dx functions are

disabled, causing it to act like a the conference function  found

in the dx node. Example:

DXST Conference only

SHow/...

The  various SHow commands listed above as user commands  can  be

used as sysop  commands

Installation

To activate the DX Node, at least two commands are needed

(DXCall  and DXSTart). The DXCall defines the callsign with  SSID

that  users can use to connect directly into the DX Node as  well

as  the callsign that will be used if the DXNconnect  command  is

used  to  connect to a PacketCluster node.  A  DXRestore  command

should  probably also be used to recover the DX Spotting and  WWV

information saved at the last time QUIT was used.

These commands can conveniently be placed in the  MSYS.DO

file to  automatically start the DX Node each time MSYS is start-

ed. An example of the commands is

DXConnect WA8BXN-6

DXSTart

DXRestore

DXNconnect DX

An  example  of  the contents of the file DX (found  in  the  FWD

directory) is given below. It has the same format as the  connect

scripts  used  in  the forward file except that  the  first  line

begins  with  the letter C.  The file below first connects  to  a

network  node  CLE220  using port 0, then a  connect  command  is

issued  to CLE220 to connect to network node #CLEDX  and  finally

5-81

#CLEDX is told to connect to PacketCluster station KQ8M.

C0 KQ8M

@CLE220

+to

+to

When connected to a PacketCluster station there is the possibili-

ty  that the connection will be lost for some  reason.  Automatic

reconnection  may  be accomplished by using the  automatic  timed

sysop command facility in MSYS (the TO DO facility). For example,

the following line could be placed in MSYSTODO.DAT to attempt  to

connect  every 10 minutes. The command will fail (by  design)  if

the connection still exists.

???????0 DXN DX

The  interpretation  of  this line is to do the  command  DXN  DX

whenever  the time in minutes has a right digit of 0,  namely  at

00, 10, 20, 30, 40 and 50 minutes after each hour.

5-82

MSYS General information file #

(c) Copyright 1992 by HUB COMPUTERS, INC.

*** MEMORY MANAGEMENT:

Buffer  space for logical channels is allocated when the  channel

is  allocated  (and released when the channel is  released)  This

will cause the MEM command to show more available memory. Its  is

little deceptive though because when all the channels are in  use

the same amount of memory will be used as before.

*** Remote SYSOP:

/Sysop command:

It  gets you into remote sysop mode which allows you to use  most

of  the sysop commands through  the bbs.  The  command   must  be

typed  exactly  as  shown:  backslash, Capital S and  lower  case

ysop and then return.You will then get a line like

Usage stats: Users=4 Msgs rcvd=10 Msgs sent=5 Max=8

To this you must reply (there will be no prompt) four  characters

corresponding  to the numbers typed in order of characters  found

in  the file MSYS.RMT (case is important). For example,  if  your

MSYS.RMT  file contained MSYS is the greatest! then   you   would

type <space> e i t which are  the  corresponding characters  (the

first character is numbered 0). Thus  you  would type eit  (Don't

forget to count the spaces) You should now get a prompt cmd:

If  you are connected via radio, you issue \Sysop command  on

bbs to verify sysop authority (enter 4 letters) You  then get the

next bbs prompt (if you give correct letters or not). If you have

given   the correct reply then  you may  issue any sysop  command

by starting the line with  \  (while still  in the bbs). So  when

on the bbs from the  local  keyboard, you can now type  something

like \em 1234 and you will be able  to edit  message 1234 without

having  to leave the bbs. As  a  remote sysop, you  retain  sysop

status until you disconnect once you give the right 4 letters.

To  get a list of available commands, type h.  Most  of   the

commands  available  duplicate  those  found  normally  as  sysop

commands.  An added command, who, gives a list of who is  on  the

system and is the same as the new U command in the bbs.

NOTE: /Sysop is not required from local keyboard.

Make  sure you mark yourself a remote sysop, even if you  won't

be one.(User flag $10)

Automatic password reply:

If your system receives the password prompt (the Usage state:  ..

line)  from a system that you connected to during  forwarding  or

from  the Connect commands, MSYS will  automatically  supply  the

correct  response.  You must have a file called  PASSWORD.RMT  in

your  FWD directory that is the same as the MSYS.RMT file on  the

other system.

5-83

*** MSYS.DO:

A  command  file may contain the same things you  could  normally

type as commands. To execute a command file, type DO followed  by

the command file name.

Example:   DO cmds.dat

When the system starts it automatically tries to DO a command.

MSYSTODO.DAT

Timed  execution of commands has now been added to MSYS. You  can

create  a file called MSYSTODO.DAT with commands and  times  they

should  be done. Wild cards can be specified for the times.  Each

line consists of a time (given as exactly 8 characters)  followed

by  a space, and then the command to execute at the  given  time.

The wildcard character is ? and they may only occupy positions in

the  left  part of the time. The format of the time  is  MMDDhhmm

where  MM  is the month number (1 to 12), DD is the  day  of  the

month  (1 to at most 31), hh is the hour (0 to 23) and mm is  the

minute (0 to 59). Here are some time examples:

MMDDhhmm (pattern)

???????? every minute

???????0  every 10 minutes (times that are like 00 10 20 etc  for

the minutes)

??????00 at the beginning of every hour

????1200 at noon every day

????0000 at midnight every day

???10000 at midnight of the 1st, 11th and 21st days of each month

??010400 at 4 am the first of every month

01010000 at the beginning of every year

You can also specify a day of the week in the DD field, in  place

of  the day of the month. To do this you would put D# in  the  DD

field (# is from 1 to 7 for Sunday to Saturday). Examples:

??D11200 Every Sunday at noon

11D52300 At 11 PM on each Thursday in November

That is how the times are set up, now here is a complete  example

of things that could go in MSYSTODO.DAT:

????0000 nod sav

????1200 nod sav

??010000 log

??d10000 copy msys.usr msysusr.bak

???????0 dt

This  file would save the network node list twice a day,  make  a

new  log file at the beginning of every month, make a  backup  of

the  user file every Sunday, and display the time and date  every

10 minutes.

*** MONxx:

5-84

MONCC ON/OFF - enables/disables monitoring of TCP/IP packets

MONCF ON/OFF - enables/disables monitoring of NETROM packets

*** LINKED:

A  line of the form *** LINKED to callsign now recognized  as   a

connection  by  the  given callsign.  This   occurs   with   some

versions of RLI systems when a user uses the C command on the RLI

system  to  connect  to  the MSYS BBS.  ***  LINKED  to  callsign

requires sysop verification if sysop call is used.

*** KEYBOARD:

When  someone connects to your keyboard answer call you will  get

two  musical tune to alert you to the connect.

Any  stations connecting to your keyboard call when  you  are

already connected to another station will get a busy. The  actual

number of connects allowed before busy is given by the new  sysop

command  MCon. MCon 1, for example, only allows an incoming  call

to  the  keyboard to be answered if there are no  other  keyboard

connections. MCon 0 prevents all incoming keyboard requests (they

get busy). MCon N will allow N connections to the keyboard  until

busy is returned.

When  you  type past column 72 you will get a beep  for  each

character. Keep those lines less than 80 characters!

When a station attempts to connect to your keyboard via the

several ways possible but is sent a busy the bell will also beep

to alert you to this.

*** Packet size:

Data  in packets is limited to 256 bytes. Larger amounts of  data

in  a single packet will be discarded without causing the  system

to crash.

*** STACK requirement:

The stack space needed by the forward process has increased.  The

following   line   can be put in MSYS.OPT  (This   may   not   be

necessary):

STACK FORWARD 8000

A  smaller value (maybe 7000) might work. Try it if you want.  If

the system randomly crashed, make it 8000.

Modifications   have  been made to the R command  to  list   path

information  with  R: lines using either  @:bbscall  or  msg#@bbs

format.

*** Device errors

Device  errors are trapped by MSYS (you would previously get  the

Abort, Retry, Ignore? message). The first occurrence of the error

is  given a response of retry, the second ignore. This will  nor-

5-85

mally be caught by particular function being performed which will

terminate with an error message, but MSYS should continue  opera-

tion.

*** Disk full message

If the disk is full when a message is sent to the system it  will

give  a msg to the connection station that the disk is  full  and

then disconnect.

*** BBS Commands:

ST Command will not add bid to messages.

Download  requires that the first character of the file  name  be

alphanumeric (no more ..)

*** @BBS:

Automatic generation of @BBS field for S command in BBS: If the S

command contains an @BBS entry it is left untouched. If there  is

no @BBS as a parameter for the S command, the following  happens.

If the to station is found in the user file then the home bbs  of

that station is substituted for the given @BBS. If the to station

is not a known user then the to station callsign is placed in the

@BBS field.

Any messages entered into the system with no @BBS that don't  get

a  @BBS  from  the user file will have ==== for  the  @BBS.  Only

messages with second type character of N will cause the user file

to  be searched for @BBS. Thus bulletins will never get their  to

call sign (typically ALL) looked up in the user file.

In  the bbs, if a S command has an ssid in the @BBS filed  it  is

removed in the leftmost part of the hierarchical address. Thus  S

W1XYZ @ K1XXX-5 is taken as S W1XYZ @ K1XXX.

*** MSYSTNC.#:

If you use any tnc set up files be warned that the program  waits

1 sec after each line from the file is sent to the tnc to give it

time to think about it.

NOTE:

AEA tncs made after Jan 89 can probably be treated like KPC-x

tncs to get them into kiss mode. It should no longer be necessary

to  have  a  special tnc file for them (but if  it  doesn't  work

without it,it is suggested that the tnc file be used).

*** MAIL BEACON:

Messages  to  your IDCALL and to SYSOP are not displayed  in  the

MAIL FOR beacon. You will see them on the screen.

*** LOG:

Msgs sent, forwarded, and killed get logged to MSYS.LOG.

5-86

A new log file record is being generated. It is identical  to the

BBS  record  except it has BYE in place of BBS. It  is  generated

when  a user exits the bbs by any means (disconnect,  timeout,  B

cmd).

*** LOCKUPS:

If the system hangs an attempt is made to reboot. Thus you should

make  sure you don't have a diskette in A: and that you  have  an

appropriate autoexec file set up.

*** CHANNEL NUMBERS:

The  channel  numbers  use a rotational  assignment  rather  than

always starting from 0

*** USER BIT SETTINGS:

$01  -  Expert user

$02  -  BBS

$04  -  Disconnect immediately

$08  -  Limited BBS

$10  -  Remote sysop status

$20  -  Need to send WP Update

$40  -  Multiline packets (XF)

$80  -  Prevents use of the S command. Users so marked can only

read messages.

$100 -  Lock user

$200 -  Authorizes use of the Upload command in the bbs.

$400 -  Must have user password

NOTE: These settings may be added together.

When the BBS is in limited mode and the H cmd is used, the file

printed is HELP/HLIMITED.HLP in place of HELP/HELP.HLP

USER  PASSWORDS: If a user has their flag bits set with the  $400

bit on, when they connect they will get the Usage stats msg  line

and  must give the proper 4 characters just like a  sysop  would.

The password file for a user must be in the new PASSWORD directo-

ry and has a filename of the user's call (no ssid, no file exten-

sion).

When  a  user logs on to the bbs for the first time (or  after  a

long while (see DELUDays), the file HELP/NEWUser.HLP will be sent

to that new user (if it exists) on the first connect (only).

*** #:

Command  lines  beginning with # on the BBS  are   ignored  (they

sometimes come from KA Nodes).

*** MAX channels AND PORTS:

Maximum for MAXCHANS is  50   (MSYS.OPT)

5-87

Maximum number of ports is  7 (numbered 0 to 6)

MSYS supports all ints (0-7) for serial ports.  I suggest the use

of ints 2,3,4 & 7 on Pc class computers and 3,4,5 & 7 on at class

and  above computers.  Use of 01 & 6 are not recommended but  are

supported  in  the code.  Due to the use of these  interrupts  by

other hardware devices/functions results will generally be bad!

I  really  think that the MS-400 (and similar)  shared   inter-

rupt serial  interface  boards  should work properly.  Thanks  to

WB8CQV for loaning me one to test with!

*** END OF LINE BELLS:

When  you  type  past  column 72 you will get  a  beep  for  each

character. Keep those lines less than 80 characters!

*** WP SERVER:

Generation   of messages to the white pages master  server   is

included in MSYS. This sends the information users give in the N,

NH,  NZ and NQ commands to the WP server when it is  changed  (or

initially given) as well as periodically. Note that MSYS does not

respond  to  messages  to WP but merely passes  them  on  without

modification. WP messages are NOT generated for users with any of

the following characteristics: locked, homebbs of ?, bbs  system,

and/or  baduser.

*** FRACK

All   IP   (and  IP through netrom) frame retry  timing  uses   a

dynamic

FRACK value.

*** TESTING:

You  can connect two MSYS systems together through  their  serial

ports by the use of a NULL MODEM.

You  can also connect a loopback  by connecting pins 2 and  3  on

the same serial port. This will allow you to see what the  system

is doing without being on the air.

*** Automatic back-up of msys.msg and msys.usr has been removed.

If you still want to do these backups once a day, put appropriate

copy statements in your msyshcl.do file.

*** TCP/IP connects are rejected if not enough memory or MCON

exceeded.

*** SMTP

The \+\ filename include capability that is currently in the  BBS

S command to also be usable in SMTP with the PRompt command. Both

of these are available only from the local keyboard.

Typing  other than Y N C A or RETURN in response to More? in  the

5-88

BBS is taken as a new command.

Title of messages truncated after 37 characters.

***  MAIL.BAK

To reduce disk read/write delays when a message is received and

copied to the MAIL.BAK directory, subdirectories are automatical-

ly created in the MAIL.BAK directory. 100 messages are backed  up

in  each  directory. This can substantially reduce  the  time  it

takes  to make the backup copy of incoming messages because  much

smaller  number  of file names must be looked at to add  the  new

file name to the directory. The created subdirectory names are of

the  form MSGxxx where xxx is the message number divided by  100.

Thus  in MSG025 messages 2500 to 2599 would be found.  The  MUTIL

msg  restore  function  has been  modified  to  understand  these

changes  and  ask  for the main  mail.bak  directory  name.  This

addition  was needed for those times the mail.bak directory  name

is changed by the set command.

***  RS-232 connection to tncs running Net/Rom and TheNet.

Ports connected to such tncs should use > as the first  character

in the name given in the port statement in MSYS.OPT.

To  connect  the computer to a single tnc per  computer  port,  a

straight  through cable is used (with a minimum of pins  2,3  and

7).   At  the tnc end, pins 10 and 23 must be  jumpered.  DO  NOT

CONNECT PINS 10 or 23 at the computer end!

In  a  multiple tnc connection using the  standard  diode  matrix

interconnect,  the computer may be connected if you reverse  pins

2&3 and pins 5&20 at the computer connection.

Using  MSYS with a Net/Rom or TheNet equipped tnc can have  great

advantages  on a backbone system. The tnc handles traffic on  the

backbone  not  going  to/from the MSYS system  without  any  load

being placed on the MSYS computer.

Forwarding  may be accomplished by connecting to the RS-232  con-

nected tnc using normal script files. You can also beacon through

the  tnc using it as a digipeater. Many other interesting  possi-

bilities also exist. Have fun!

*** Type of a message (the "TR" field)

The  algorithm  uses 3 pieces of information  to  determine  the

appropriate TR to assign: the destination "callsign", the @"bbs",

and the character after the S in the Send command.

The to "callsign" is classified as one of the following:

0 - Looks like a real callsign

1 - A zipcode (5 digits)

2 - something else (like ALL, for example)

The @"bbs" is classified as one of the following:

5-89

0 - Looks like a real callsign

1 - none specified

2 - call of this bbs

3 - something else

The character after the S in the s command is classified as:

0 - B  (as in SB for send bulletin)

1 - P  (as in SP for send private)

2 - T  (as in ST for send traffic)

3 - none (as in just plain S)

4 - something else (SW for send weather bulletin, for

example)

The following message types are generated:

BN   For  bulletins going to a specific bbs -  these  are

forwarded to a single station that handles that bbs.

PN  For private messages

TN  For NTS traffic

xN  For Sx where x is an "something else"

N  For individual non-private messages.

B$  For bulletins going with @route (like ARRL)

x$  For bulletins sent with Sx (x is "something else")

P$  For private bulletins going with @route

Note:  [P]N below means that the type will be N is MAKEPRIVATE is

off,PN if MAKEPRIVATE is ON (default).

TR Values Assigned to Messages

For messages going to Callsigns:

Character after S ->    B       P       T       None    x (Other)

@BBS type:             ---     ---     ---      ----    ---------

callsign                BN      PN      TN      [P]N    xN

no bbs given            B$      PN      TN      [P]N    x$

this bbs                B$      PN      TN      [P]N    x$

something else          B$      PN      TN      [P]N    x$

For messages going to ZIP Codes:

Character after S ->    B       P       T       None    x (Other)

@BBS type:             ---     ---     ---      ----    ---------

callsign                BN      PN      TN      TN      xN

no bbs given            B$      P$      TN      TN      x$

this bbs                B$      P$      TN      TN      x$

something else          B$      P$      TN      TN      x$

For messages going to SOMETHING ELSE:

Character after S ->    B       P       T       None    x (Other)

@BBS type:             ---     ---     ---      ----    ---------

callsign                BN      PN      TN      BN      xN

no bbs given            B$      P$      TN      B$      x$

this bbs                B$      BN      TN      B$      x$

5-90

something else          B$      P$      TN      B$      x$

Messages  with  second character of type N are forwarded  to  one

place  (and then killed if AUTOKill is ON, the N is changed to  F

otherwise). Messages with second character of type $ are  flooded

to  all  stations that get the specified @BBS. When sent  to  all

such BBS's, the second character gets changed to #.

Messages  that  come in with an R: line that  contains  the  call

(actually HCALL) of this bbs are automatically held. The R:  line

scan is terminated by the first non-R: line found in the message.

Bulletins  that arrive with a bid on the S command line  are  re-

jected if their bid already exists in the bid file. Bulletins are

identified as those messages that arrived via the SB command or S

non-call.

All  messages  other than bulletins are  always  received  (never

rejected).  If  a message identifier ($string) exists  on  the  S

command line AND ACceptmid is ON, it is used. In all other cases,

a  message identifier is generated internally for all  non-bulle-

tins using the bbs call and message number from the last R:  line

scanned.  If the message identifier is found in the bid file  and

the message was not held as noted above, the message is not saved

(but it is acknowledged as being received entirely by sending the

> prompt when the ^Z is received).

The  processing of internal message identifiers is not  indicated

in  the  system identification line [MSYS-XXXX-H$].  If  you  set

MIDchar to something, then the character will appear between  the

H  and the $. For example, if you set MIDchar to M, then the  SID

line  will be [MSYS-XXXX-HM$]. This will be sent to all  stations

that connect to the BBS. Setting MIDchar to a non-null  character

will  also  cause  the MID to be sent during  forwarding  if  the

system to which you are forwarding has MIDchar in its SID.

Here  are  some  combinations of the parameters  to  do  selected

processing:

For no bid (mid) processing on non-bulletins, set

BIDall OFF

ACceptmid OFF

don't set MIDchar

To emulate AA4RE MID processing (I think), set

BIDallON

ACceptmid OFF

MIDchar M

To use a MID if provided, or generate one otherwise, set

BIDall ON

ACceptmid ON

MIDchar M

Currently there is still a lot of discussion going on over  MIDS.

Hopefully I have give enough parameters so that MSYS can be  used

with whatever standard wins out.

5-91

***     Automatic message holding

If you create a file called MSYSHOLD.DAT you can specify

characteristics  of messages to hold automatically using  parame-

ters   similar  to  those used in the house  cleaning  file.  The

available parameters are:

TO=           wildcard representation of To callsign

FROM=         wildcard representation of From callsign

AT=           wildcard representation of @BBS

SIZE=         number that is size of msg

CONNECTED=     exact  callsign of sending  station  (less

ssid)

PORT=         port number msg is coming in on

BID=           This  is  useful in  situations  like  the

following.    Normally   ARRL    bulletins

ARLP037.  These bids begin with a  letter.

Bids  automatically assigned to  bulletins

that don't have a bid usually begin with a

digit.  So we could use AT=ARRL BID=#*  to

hold  bulletins that probably  don't  have

the right bid. Note that  #* is a wildcard

specification  that matches anything  that

starts with a digit.

TITLE=         If used, this should be the last parameter

on the line. If the value given for TITLE=

is  found  any  where in the  title  of  a

message (and all other specifications  are

met) the message will be held.

Examples:

SIZE=2500               would hold any messages bigger than  2500

bytes

FROM=WA8BXN PORT=2      would hold any messages from WA8BXN  that

come in on port 2

CONNECTED=W8XYZ         would hold all messages sent by station

W8XYZ (connected to the bbs)

AT=MSYS                 would hold all messages with @MSYS

Note  the  difference between FROM= and CONNECTED=; FROM  is  the

from  call for the message (often supplied after < in the S  com-

mand) while CONNECTED is the call of the station connected to the

bbs sending the message.

There  is one other parameter that can be specified, a line  con-

taining only the letters BBS. The lines following such a line  in

the  file would not apply to messages that are forwarded  to  you

from  another  bbs. For this use, a bbs is defined as  a  station

that transmitted a [...$] line.

NOTE:  Held  messages are visible only to the SYSOP  and  sender.

They are killable only by SYSOP.

Other options on the lines in MSYSHOLD.DAT:

5-92

* at the beginning of the line makes it into a comment

TEXT=filename operand on a line

When specified, if other tests are met, the text and title of the

msg will be compared with the contents of the given filename.  If

any of the words in the file match, the msg will be held. Case is

ignored in the test.

Example: TO=ALL TEXT=WORDLIST.1 would cause all messages going to

ALL  to be compared with the file WORDLIST.1.  Put one phrase  or

word  per line in the file, enclosed in either single  or  double

quote  marks. You may also put a number between 1 and 255 at  the

beginning of the line before the quoted phrase to assign a weight

to  the  word. If no weight is given, the word or phrase  gets  a

weight of 255.  Here is a sample file of words:

'war'

"900"

"send check"

50 'send'

50 "check"

"That's it"

If the sum of the weights of these "words" found in the title or

msg text exceeds HOLdlevel (default 0) then the msg is held.

NOTE:  HOLdlevel  is a new sysop command!  Caution:  'war'  would

match  an  occurrence of the word 'forWARd' in a msg. Its  not  a

real  problem  as it just means the msg will be held  not  killed

off.

*** DOS

If you are using DOS 3.3, you may need to put STACKS=0,0 in your

config.sys file.

*** HEADERS

The  HDRS command has been removed. To create an ASCII file  con-

taining

all  the  active message headers, use the following  sequence  of

commands on the bbs:

X 0            (turn off the More? message temporarily)

>msghdrs       (this is the name of the file to be created)

L$ 0           (or L 0 if you don't want to see the bids)

>              (close the file)

X 20           (restore the More? message)

*** KANTRONICS 3.0 EPROM Upgrade users:

I am told you will have to make a tnc setup file and include  the

following:

INTFACE KISS

RESET

You  may also need to include a line with one or  more  control-c

characters before these two lines in your file.

EXPANDED MEMORY USAGE

If sufficient expanded memory is present (at least 320K must  be

available  to be used) then all stacks will be around 8000  bytes

5-93

and  come from expanded memory. This should free up about 50K  or

so  of memory as shown by the MEM command. Also each new  connec-

tion  will require less memory. If you have expanded  memory  and

don't want it used, start msys using:

MSYS NOEMM

Real expanded memory boards (that could be used with an 8086/8088

processor) will give better performance (faster) than simulating

expanded memory.

Expanded memory is  used (if present) to contain the msg  head-

ers.  Now  you  can have 600 msgs on your system  and  memory  to

spare. Be warned that with this many active msgs things will slow

down.

NOTE:  If you have problems running expanded memory try  using  a

different EMM program.

*** BBS ONLY ACCESS

Lets say you are running with ports 0 through 3 and want to  make

port  2  bbs only access with at most 1 connection allowed  at  a

time (either another BBS forwarding to you or you forwarding  but

not both at the  same time). You also don't want to generate MAIL

FOR beacon on port 2. Use the following:

FB 2 25                           turn off mail for beacon port 2

BP 4                              port 2 bbs only

BM 2 1                            max 1 connect port 2

MCF $B                      no simultaneous forwarding on port 2

Note that BP and MCF both have port masks as arguments. 73 - Mike

*** CONNECTS FROM NON-AMATEUR CALLSIGNS

Connects are no longer accepted from non-amateur callsigns  (like

PK-232 and NOCALL for example). You can still make connections to

non-call signs.

5-94

MSYS Utility Support Programs

(c) Copyright 1992 by HUB COMPUTERS, INC

MUTIL.EXE

Originally  many  of  the  functions  found  in  MUTIL  were

separate programs. When the amount of space on a single  diskette

was found to be scarce to include everything needed to distribute

MSYS a decision was made to combine these programs into a  single

menu driven utility program. This saves considerable space  since

each of these programs share many common routines. Now there  are

just two executable programs distributed, MSYS.EXE and MUTIL.EXE.

References  in  the  documentation for MSYS that  say  to  run  a

particular   utility  program  should  be  interpreted   as   the

corresponding function found in MUTIL.

When you run MUTIL you will get a screen that is similar to:

=================================================================

MSYS Utilities 1.04 by WA8BXN (C) 1989 Hub Computers, Inc.

1    Change maximum number of messages

2    Restore messages from backup directory MAIL.BAK

3    Print remote sysop phrase number table

4    Check Mail index file integrity

5    Test serial ports

6    Print forwarding tree

8    Run log file analyzer

9    Run SETUP program

10   Scan message headers to create/add to BBSLIST.DAT

11   Deletes old entries from BBSLIST.DAT and BBSTONTS.DAT files.

12   Fix mail index file (can be hazardous to use!)

13   Process BBSLIST.DAT into BBSTONTS.DAT file

14   Scan BBSTONTS.DAT to build BBSTONTS.BIN (indexed)

15   Build list of archived messages

16   Printer status

17   merge BBSLIST.DAT files

18   Check MSYSNODE.DAT integrity

19   Convert user files 1.05 & before <===> 1.06 and later

20   Sort calls in the BBSLIST.DAT file

21   Delete Old BIDS (and MIDS)

22   Specify file name to use in place of printer

99   Quit

Enter number of selection: _

6-95

MUTIL Function 1 - Change maximum number of messages

Input file(s):   MSYS.MSG

Output files(s): MSYS.MSG

The maximum number of messages that can be stored in MSYS is

determined  by  the MSYS.MSG mail index file.  When  MSYS  starts

running  it checks to see if there is an existing MSYS.MSG  file.

If there is one, it is used (and contains the header lines of the

messages that existed when MSYS was last stopped). If there is no

existing  MSYS.MSG  file, then an empty one is  created,  with  a

capacity of 100 messages.

This  utility function can be used to change the size of  an

existing MSYS.MSG file. You can increase or decrease the size  of

the  file.  You can't make it smaller than the number  of  active

messages.

NOTES:  There is a cost in memory requirements for  each  message

possible.  Each  message  slot  cost 138  bytes  of  memory;  100

messages (default maximum) thus takes 13800 bytes. A limit of 500

messages would take 69,000 bytes (even if there weren't that many

messages active).

VALUES  I USE: I run with a 300 message maximum size. Normally  I

have around 200 active messages.

DIALOG  WITH  FUNCTION: You are first asked for the name  of  the

MSYS.MSG file to modify:

Enter path to MSYS Mail Index file or just press return

to use MSYS.MSG: _

Normally  you  will  want to just press the return  key.  If  the

current  directory  from  which you are running  MUTIL  does  not

contain  the  MSYS.MSG  file that you want  to  modify,  you  can

specify  a different path (or even a different file name  if  you

have renamed MSYS.MSG for some reason).

This  function  will next copy the specified MSYS.MSG file  to  a

file called MSYS$$$.MSG which can be used to restore MSYS.MSG  if

for some reason this function fails.

Next  you  are  asked if you have run  the  Check  Mail  function

(function 4). You should run Check Mail just before this function

since  attempting to change the maximum number of  message  slots

can  produce  very interesting (and usually bad) results  if  the

original  MSYS.MSG  file is corrupt. If you respond N  then  this

function terminates and you can then do the Check Mail  function.

If  you have just run Check Mail, then press Y to continue.  Note

that you do NOT press return after either of these letters!

If  you  responded Y, then the function  continues,  telling  the

current maximum number of messages, the current number of  active

messages, and some other information. You are then asked for  the

6-96

new  maximum  number of messages you want. This  number  must  be

greater  or equal to the number of active messages. It must  also

be less than or equal to 1000.

If the value you ask for is acceptable, the file will be  changed

to accommodate the new number.

Next you will be asked to press a key to continue (try the return

key).  This pause is put in the function so that you can see  any

additional  messages  that  were produced before  the  screen  is

cleared.

6-97

MUTIL Function 2 - Restore Messages from backup mail directory

INPUT FILE(S):   MSYS.MSG

MAIL.BAK\MSG*.DAT

OUTPUT FILES(S): MSYS.MSG

MAIL\MSG*.DAT

This  function  allows  you to restore messages  that  have  been

deleted  or  otherwise lost from the normal MSYS.MSG  index  file

and/or  MAIL directory. This restoration is possible only if  the

directory  MAIL.BAK  exists  (and has existed  since  before  the

messages of interest arrived), and the messages have not yet been

removed from it.

The  function allows you to look at the messages in the  MAIL.BAK

directory and restore as active those you select.

NOTE:  The  format  of the messages  in  the  MAIL.BAK  directory

includes additional information not found in the messages in  the

MAIL   directory.  This  includes  the  normal   message   header

information  normally  found  in MSYS.MSG and  the  BID  for  the

message (if any).

NOTE:  You  are strongly advised to run the check  mail  function

(MUTIL function 4) BEFORE attempting to restore any messages!

NOTE:  Restored  messages  will  NOT  reflect  which  stations  a

bulletin  has been sent to. Generally a restored message looks  a

lot like it did when it first arrived. It may or may not have the

substitutions done to the @BBS or other fields done yet.  EXAMINE

each restored message with the EMsg command when you get MSYS  up

again!

DIALOG  WITH  FUNCTION: You are first asked for the name  of  the

MSYS.MSG file to modify:

Enter path to MSYS Mail Index file or just press return

to use MSYS.MSG: _

Normally  you  will  want to just press the return  key.  If  the

current  directory  from  which you are running  MUTIL  does  not

contain  the  MSYS.MSG  file that you want  to  modify,  you  can

specify  a different path (or even a different file name  if  you

have renamed MSYS.MSG for some reason).

Next you will get a menu with 5 functions:

F1  - Display a given message number

F2  - Restore a list of message #'s

F3  - Restore a range of messages

F4  - Display a range of message headers

F5  - Restore headers for message files without headers

F10 - Quit

F1  will ask you for a message number to display. If the  message

6-98

exists  in  the MAIL.BAK directory, its header and text  will  be

displayed.

F2  will ask you for the message numbers to restore. If the  mes-

sage already exists in the normal index file, or the message does

not  exist  in the MAIL.BAK directory, the restore  will  not  be

done.

F3 will ask you for a starting message number and ending  message

number  to restore. After this it acts much as if  each  possible

message in the given range was specified for F2.

F4 asks for a starting message number and ending message  number.

Messages  that  exist in the MAIL.BAK directory  will  get  their

headers  displayed. If no messages exist in the given  range  (or

the  MAIL.BAK directory is absent) you will simply get the  Press

Esc to continue message.

F10 terminates the restore operation. The MSYS.MSG file specified

at the beginning is sorted to get any restored messages in  their

proper place and then the file is written to disk.

F5 Restore headers for message files without headers. This can be

useful when MSYS.MSG is severely damaged (or deleted).

COMMENTS:  You  will  probably want to do F4 first  to  see  what

messages  are available to be restored unless you otherwise  know

which one(s) you want to restore or look at.

6-99

MUTIL Function 3 - Print remote sysop phrase number table

INPUT FILE(S):  MSYS.RMT

OUTPUT FILE(S): Hardcopy printer output (optional)

This function prints the contents of the MSYS.RMT file (used  for

remote  sysop verification). The output of this function  can  be

just  the screen or also to the printer. The output  consists  of

printing  the content of remote sysop phrase followed by a  table

that gives the character at each position in the phrase.

DIALOG WITH FUNCTION:

You  are  asked if you want hard copy output  (on  the  printer).

Respond Y or N as desired.

SAMPLE OUTPUT:

Sample phrase.

0    S

1    a

2    m

3    p

4    l

5    e

6

7    p

8    h

9    r

10    a

11    s

12    e

13    .

USAGE EXAMPLE:

If  you  did  the \Sysop command as a remote user  to  get  sysop

privileges and got the following reply:

Usage stats: Users=7 Msgs rcvd=3 Msgs sent=6 Max=12

you would type the following response:

pp e

6-100

MUTIL Function 4 - Check Mail index file integrity (CHKMAIL)

INPUT FILE(S):  MSYS.MSG

MAIL\MSG*.DAT

OUTPUT FILE(S): MSYS.MSG

This  function  checks  the  integrity of  the  Mail  index  file

MSYS.MSG. It checks for such things as message slots on both  the

free and active lists, loops in either of these lists and  active

message numbers that do not have text files in the MAIL directory

(and  vice-versa). It attempts to fix any errors if it  can  (and

its generally pretty good at doing it).

NOTE:  It  will  not delete any message text files  in  the  MAIL

directory  if  they  don't  have active  headers.  They  will  be

reported but you will have to delete them yourself if you  really

don't  want  them. You probably won't really want them,  but  you

decide!

NOTE:  This  function may be run in a batch  file  through  MUTIL

(without  having  to give its function number  interactively)  by

running:

MUTIL 4

SAMPLE NORMAL OUTPUT:

CHKMAIL - An MSYS Utility to verify mail index integrity

MSYS Mail Index Checker

head=0 free=1 next=2

Reading Index file:

There are 100 total message headers

Pass 1:

No cycles detected

No headers on both lists

All headers accounted for

No message files without headers

All active headers have files

Press a key to continue_

COMMENTS:  I would strongly suggest you run this  function  every

time  MSYS crashes for any reason (it should not crash often,  if

at  all). To be on the safe side, many MSYS sysops always run  it

when they start MSYS.

6-101

MUTIL Function 5 - Test serial ports

INPUT FILE(S):  None

OUTPUT FILE(S): None

This function provides a very basic test of serial interfaces. It

is  designed mostly to see if you got the  addresses/int  numbers

right for your machine. It is not intended to be a brutal test of

their operation!

DIALOG WITH FUNCTION:

After  a table is displayed that lists the normal  addresses  for

serial  cards and corresponding interrupts, you will be asked  if

your  machine  has an 8086/8088 CPU chip in it. If  its  a  basic

PC/XT compatible system, respond Y. If you have an AT (80286)  or

higher  level machine, respond with N.  If you have an  8088/8086

based  system (PC class) then it will look for ints  from  2,3,4,

and  7.   If you say not 8088/8086 (AT class and above)  it  will

look for ints from 3,4,5, and 7.

Depending  on your response, certain interrupt handlers  will  be

installed.  You will then be asked for the address of the  serial

card  you  wish  to  check.  The  answer  you  give  must  be  in

HEXADECIMAL. A valid response might be 3F8, for example. A simple

check  is  made  to see if there might be a serial  port  at  the

address  you  specify.  It is easily fooled if  you  specify  the

address of some other kind of I/O interface.

If  there seems to be a device at the address you gave,  a  quick

check  will be made to see if it looks like an 8250 UART chip  is

used in your serial interface. Internal loop back tests cannot be

done with the newer UART chips. If the test fails when you select

internal  loopback,  try  the  external  loopback  option.   This

requires  a  connector that will connect pins 2 & 3  together  in

place  of the tnc you would normally connect.  External  loopback

can be used to test all types of UART chips.

Next you will next be asked for a baud rate to test at. Here  you

give a decimal number as an answer. It probably doesn't make much

difference what you give. (Don't try 0!). I usually test at  1200

baud.

Now its your turn to type characters. As indicated, press Esc  to

terminate the test. Normal response should be something like

Had IRQ 4  1 time(s), A received

if  you press the letter a key. If interrupts aren't working  you

would see

Received A without interrupt.

NOTE: This is not a performance test, just an address test.

6-102

MUTIL Function 6 - Print forwarding tree

INPUT FILE(S):  MSYS.FWD

OUTPUT FILE(S): Printer hard copy (optional)

This  function displays your forwarding file in two ways to  tell

you  what  callsigns you forward to each bbs you forward  to  and

also to which bbs you forward each callsign.

DIALOG WITH FUNCTION:

You  are  asked if you want hard copy output  (on  the  printer).

Respond Y or N as desired.

The first part of the output consists of lines like

AD8I ---> KA8Z KD8GC

which  means that messages with @AD8I are sent to either KA8Z  or

KD8GC (which ever one you can connect to first).

The second part of the output consists of lines like

KA3NVP <---    KA3NVP NTSNJ NTSNY NTSPA

which means that you forward messages with KA3NVP NTSNJ NTSNY and

NTSPA in some part of the @BBS field to KA3NVP.

NOTE:  To  pause  the display, you must use  ^NumLock  (^S  won't

work). To resume scrolling, press any other key.

6-103

MUTIL Function 8 - Run log file analyzer

INPUT FILE(S):  MSYS.LOG

MSYS.MSG

OUTPUT FILE(S): Printer hard copy (optional)

This program produces a listing of BBS usage.

DIALOG WITH FUNCTION:

You  are  asked if you want hard copy output  (on  the  printer).

Respond Y or N as desired.

SAMPLE OUTPUT:

=================================================================

From Wed Mar 01 15:23:48 1989 to Mon May 229 00:57:03 1989

Station  Connects  Sends  Reads  Kills  Forwards

HClean          0      0      0     27         0

N8HTG           3      1      1      0        12

NT8V            0      1      1      0         0

WA8BXN          4      0      0      1         9

WB8ZZS        168     82     31     24         0

LAST

READ   MSG# TO       @BBS     Count   Title

May 18     1 TEST                  1   test

Mar 23     7 TEST                  2   test

May 28     2 TEST                  3   test

Mar 02  8073 TEST                  7   no title

MSGINs 84  MSGKILLs 97  MSGFWDs 21  MSGREADs 33

RESTARTs 331  QUITs 311  CONNECTS 175

The following messages were received but not killed:

1    2    3    20   21   8073

(These should be active messages on your BBS!)

=================================================================

NOTES:  This data is taken from my test system which uses  WB8ZZS

as its call (Thanks to my wife Ruth!) Many strange things  happen

on this system as I am testing new software changes.

The first line of the report gives the period of time covered  by

the contents of the MSYS.LOG file that was read.

The next section of the report lists stations that were active on

the BBS and what they did. The headings mean the following:

Station - Callsign of the station

Connects-  Number  of times the station  connected  (or  the

SYSOP used the BB command).

Sends   - Number of messages sent by this station

Reads   - Number of messages read by this station

Kills   - Number of messages killed by this station

Forwards- Number of messages forwarded to this station

NOTE:  The  callsign  HClean  indicates  activity  by  the  House

Cleaning routine.

6-104

The next section lists messages that were read during the period,

sorted in increasing order of number of reads. The meaning of the

headings is:

LAST READ - Date the message was last read

MSG#      - The number of the msg on the bbs

TO        - The "TO" station callsign of the message

@BBS      - The "@" field of the message.

Count     - The number of times the message was read

Title     - The title of the message

NOTE:  Private  messages  are not  shown,  only  bulletins.  This

section  is useful to see what interest there is in  the  various

types of messages.

The next section provides overall counts of various activities:

MSGINs    - The number of messages that were sent to the BBS

MSGKILLs  - The number of messages that were killed

MSGFWDS   - The number of messages forwarded by the  BBS  to

other systems. Note: If a bulletin is forwarded

to 4 different systems, it counts as 4 forwards

MSGREADs  - Total number of Read commands used on BBS

RESTARTs  - Number of times MSYS was started

QUITs      - Number of times MSYS was gracefully  shut  down

using ^F4 or Quit command. Restarts - quits  is

the number of crashes for whatever reason.

CONNECTS  - Number of user connects to BBS + SYSOP BB cmds

The  final section of the report gives a list of message  numbers

that were assigned to messages sent to the system but not killed.

Normally  these message numbers should be active messages on  the

BBS.  If they aren't, they may be messages to restore since  they

got lost somehow.

6-105

MUTIL Function 9 - Run setup program

INPUT FILE(S):  MSYS.OPT

OUTPUT FILE(S): MSYS.DEF

This function sets default initialization parameters. It must  be

run before you can use MSYS and any time you change MSYS.OPT.  It

is documented elsewhere in this document.

DIALOG WITH FUNCTION:

You  are  asked if you want hard copy output  (on  the  printer).

Respond Y or N as desired.

6-106

MUTIL  Function  10  -  Scan message  headers  to  create/add  to

BBSLIST.DAT

INPUT FILE(S):  MAIL.BAK\MSG*.DAT (or specified directory)

BBSLIST.DAT       (if it exists)

OUTPUT FILE(S): BBSLIST.DAT

This  program scans backup mail message files that are  (or  were

originally found) in the MAIL.BAK directory to determine what BBS

stations  have been "seen" (using the R: lines added by each  BBS

as it forwards a message).

DIALOG WITH FUNCTION:

You will be asked to give the name of the directory that contains

the messages to be scanned or to press return to use the MAIL.BAK

directory directly.

NOTE: This function may be run from a batch file by using either

MUTIL 10

--- or ---

MUTIL 10 dirname

In  the first form (MUTIL 10) it will be assumed that  the  input

files are in the MAIL.BAK directory. In the second form (MUTIL 10

dirname) the files are assumed to be in the given directory name.

OUTPUT FILE FORMAT:

The  first line of BBSLIST.DAT gives the date this  function  was

run to create or add to the file.

The remaining lines in the file consist of the following fields:

callsign of bbs

location of bbs

postal code of bbs

number of times this call has been seen

Note: If you choose to edit this file, be sure to keep things  in

the same columns. Also note that the callsigns are lined up using

the  digit  of  the  callsign. The list is  sorted  by  digit  in

callsign,  letters after the digit, and then letters  before  the

digit (pretty much callbook order). Calls that do not begin  with

A,  K, N, or W are grouped together at the end of the list,  with

an * in the first position in the record (non-US calls).

6-107

MUTIL  Function  11 - deletes old entries  from  BBSLIST.DAT  and

BBSTONTS.DAT files.

6-108

MUTIL Function 12 - Fix mail index file

***** WARNING ***** This function can DEStroy the mail index file

INPUT FILE(S):  MSYS.MSG

MAIL\MSG*.DAT

OUTPUT FILE(S): MSYS.MSG

This function allows manual inspection, checking and modification

of  the  mail  index file. It is a utility I wrote  to  see  what

DEStroyed  a mail file and perhaps fix it. There is one  function

that may be useful to most sysops, that allows you to specify the

next message number to be assigned to incoming messages. You  may

want  to do this if you were running some other bbs and  want  to

continue   using  message  numbers  from  where  you  left   off.

Personally  I  would suggest that you don't do  this  but  rather

start with the default value 1. But its up to you!

NOTE:  Make  a copy of your existing MSYS.MSG file  before  using

this function!

NOTE:  Most  of  the  checking and  repair  functions  have  been

automated in the CHKMAIL function (MUTIL Function 4).

DIALOG  WITH  FUNCTION: You are first asked for the name  of  the

MSYS.MSG file to modify:

Enter MSYS message header file name or simply press

return for MSYS.MSG: _

Normally  you  will  want to just press the return  key.  If  the

current  directory  from  which you are running  MUTIL  does  not

contain  the  MSYS.MSG  file that you want  to  modify,  you  can

specify  a different path (or even a different file name  if  you

have renamed MSYS.MSG for some reason).

To  change the number that will be assigned to the next  message,

use the following procedure:

Select F8 (Manual changes)

Select F5 (Change Next)

Enter new value for next message number to be used.

Select F10 (Return to previous menu)

Select F9 (Save changes and quit)

Renumbering Messages

All  active messages may be renumbered using MUTIL  function  12.

You  should renumber your messages when you reach  about  message

number  64000. As message numbers increase above this  they  will

wrap around back to 0 if you don't use this function. Some of the

L  command options will not work properly if newer messages  have

numbers smaller than older ones.

To renumber the messages, go through the following steps:

1 - Start the MUTIL program

2 - Type 12 and return

6-109

3 - Press return to accept use of MSYS.MSG

4 - Press F8 for manual changes

5 - Press F6 for Renumber messages

6  -  Type new starting message number (1 is  good)  then

press return

7 - Press F10 to return to previous menu

8 - Press F9 to save changes

9 - Press return to go back to main menu

10 - Type 99 and return to exit

MSYS.MSG FILE FORMAT: (For the adventuresome only!)

(If you know what you are doing,

this will make some sense to you)

int head;      /* slot number of head of active list */

int free;      /* slot number of head of free list */

int next;      /* next message number to be assigned */

/* Each slot has the following definition: */

struct msghdrdef {

unsigned  mmsgnr;

unsigned char mmsgtype[3];

int  mmsgsize;

unsigned char mmsgto[7];

unsigned char mmsgfrom[7];

unsigned char mmsgatbbs[39];

unsigned char mmsgdate[7];

unsigned char mmsgtitle[40];

long mmsgbits;

unsigned char mmhbbs[8];

unsigned char mmsgtries;

unsigned char mmsgbid[15];

int  mmsglink;

};

/* Note: word alignment is forced for all ints & longs */

6-110

MUTIL Function 13 - Process BBSLIST.DAT into BBSTONTS.DAT

INPUT FILE(S):  BBSLIST.DAT

ZIPCODES.DAT

STATES.DAT

ROUTES.DAT

OUTPUT FILE(S): BBSTONTS.DAT

This  function  builds a list of BBS calls  (and  optionally  zip

codes)  and the corresponding xx designator to which they  should

be forwarded.

If you select processing US BBS calls by state the data found  in

BBSLIST.DAT  is  scanned  for state and zip code.  The  state  is

identified as being something after a comma in the location field

that  matches  the list of state two letter codes  found  in  the

STATES.DAT file. If only one state is found and it corresponds to

the  zip code found, then the BBS call is automatically  assigned

to  xx  where xx is the state abbreviation. If  either  of  these

pieces  of information is missing, or they do not match the  test

fails and you are asked to decide what state the bbs belongs in.

DIALOG WITH FUNCTION:

You  will  be  asked if you want to include  state  routings  for

ZIPCODES. You should reply Y the first time you are creating  the

BBSTONTS.DAT file. Otherwise reply N. If you delete  BBSTONTS.DAT

for  some reason, you should reply Y the first time you create  a

new one.

If  the state of the BBS cannot be automatically determined,  you

will get a screen that looks like this:

=================================================================

WB8ZZS    KIRTLAND OH              44026     (4)

State/ZIP check failed - Zipstate OH

Enter  two letter state code then <Return key> or  press  <Escape

key>  then <Return key> to ignore or just press <Return  key>  to

use OH

=================================================================

The  first line of this screen is the line from BBSLIST.DAT  that

is  being  considered.  The  next  line  gives  the  state   that

corresponds to the zip code (if available). The third line  gives

the  guess that can be accepted if you just press return. If  the

guess is good, hit return. If you want to use a different  state,

type the two letter code for that state and then press return. If

you want to omit this call from the output file, press some  non-

letter key before pressing return. Space bar, return works well.

In this example, the state could not be identified because  there

is no comma between the city and the state. (It would be nice  if

6-111

everyone  used standardized R: lines!) The guess of OH  was  made

from the zip code alone.

When  the US calls have been processed, you will be asked if  you

want to process non-US calls. If you have earlier choosen not  to

process  US  calls  by  states/zipcodes, the  US  calls  will  be

processed with all the other calls if you choose to process  non-

US calls. This feature is useful for non-US hams that handle  all

US messages as just going to the US.

If  you  choose to process the calls at this point, you  will  be

presented  with  screens  that  list an  entire  entry  from  the

BBSLIST.DAT file. Following this line will be the callsign of the

BBS  identified from the line. Using the call prefixes  found  in

the  ROUTES.DAT  file, a guess will be made as  to  what  routing

should be used for the given call. You may then choose to  ignore

the  call, to use the routing obtained from the call  prefix  (if

any) or to type the complete route you desire.

A sample screen might look like this:

=================================================================

*   WB8ZZS    KIRTLAND OH              44026     (4)

Callsign:       WB8ZZS

Possible route: USA.NA

Press <Escape key> <Return key> to ignore

Type desired routing then press <Return key> or

just press <Return key> to use USA.NA

=================================================================

BBSTONTS.DAT FILE FORMAT:

This  is an ASCII file containing in bytes 1 to 9 a BBS  call  or

zipcode (left justified, possibly with wildcard * indicator), and

the corresponding designator to which messages for that BBS  call

or zip code should be forwarded beginning in position 10. Example

lines:

89*         NV

WB8ZZS      OH

VK1XX    AUS.AS

NOTE:  Bulletin routing designator (like ARRL) should NOT  appear

in this file.

NOTE:  The  file is sorted in ascending order  using  the  entire

record as key.

NOTE:  There should be no duplicates of calls/zip codes found  in

the first 9 bytes of the records.

6-112

ROUTES.DAT FILE FORMAT:

This  is  an  ASCII  file that  contains  callsign  prefixes  and

routings.   The  callsign  prefix  begins  in  the  first   byte.

Comparisons  are  made with the BBS call for the  length  of  the

prefix from this file. The routing field begins in byte 9 (a  tab

character can be used to get here). Here are some sample entries:

4X      ISR.AS

9M2     MYS.AS

A       USA.NA

The idea here is that incoming messages for a given bbs will  get

the  routing appended to the @BBS field. Thus if someone  sent  a

message   to   4X1ABC  @  4X1XYZ  the  @   field   would   become

@4X1XYZ.ISR.AS

It  is  expected  in  your forward file  you  would  either  have

provisions  to route the country part (ISR) to some bbs,  or  the

continent part (AS) to some bbs. Thus to route foreign  messages,

you  might just have all the continent designators (like EU,  AS,

NA  etc)  in your forwarding file or you can be  more  selective.

Remember  that  you  can  have  some  countries  along  with  the

continent  routings  in your forward file. The part of  the  @BBS

from  left to right that is found in the forwarding file  is  the

one that is used. If you do a lot of direct forwarding to various

countries that can be accommodated; if you just dump all  foreign

stuff  on a single station that is easy too. Feel free to  modify

the  contents of ROUTES.DAT as you see fit; I have just  included

country  prefixes for countries for which I seen message  headers

pass through my system.

The  following program may be used to create a  BBSLIST.DAT  file

from the BBSTONTS.DAT file supplied on distribution disk:

10 OPEN "BBSTONTS.DAT" FOR INPUT AS 1

20 OPEN "BBSLIST.NEW" FOR OUTPUT AS 2

30 IF EOF(1) THEN CLOSE:END

40 LINE INPUT #1,A$

50  I=1:C$="":  WHILE  MID$(A$,I,1)<>"  "    :C$=C$+MID$(A$,I,1):

I=I+1: WEND

60 D$=MID$(A$,35,6)

70 H$=MID$(A$,10,25)

90 IF LEFT$(D$,2)="99" THEN 30

100 X$=LEFT$(C$,1): IF X$>="0" AND X$<="9" THEN 120

110 X$=MID$(C$,2,1): IF X$>="0" AND X$<="9" THEN C$=" "+C$

120 PRINT #2,C$;TAB(8);H$;TAB(74);D$

130 GOTO 30

You may use a text editor to extract the program from this file.

The  result of this program is put in a file called  BBSLIST.NEW.

It  may be renamed to BBSLIST if you don't have one  already.  Or

you  can  merge your file with this one by  using  the  following

commands:

copy BBSLIST.DAT+BBSLIST.NEW TEMP

del BBSLIST.DAT

ren TEMP BBSLIST.DAT

6-113

Then  run  MUTIL  function 20 to sort the file  and  discard  the

duplicates from the BBSLIST.NEW file.

6-114

MUTIL Function 14- Scan BBSTONTS.DAT to build BBSTONTS.BIN

INPUT FILE(S):  BBSTONTS.DAT

OUTPUT FILE(S): BBSTONTS.BIN

This  function builds an index of the calls in  the  BBSTONTS.DAT

file,  placing  the index at the beginning  of  the  BBSTONTS.BIN

file. After the index, the contents of BBSTONTS.DAT is copied  to

BBSTONTS.BIN.

DIALOG WITH FUNCTION:

This function does not require any keyboard input.

NOTE:  The format of the resulting file is such that  no  attempt

should be made to edit it. With care, BBSTONTS.DAT may be  edited

and the result then processed by this function.

NOTE:   The  file  BBSTONTS.BIN  is  used  by  MSYS.  The   files

BBSTONTS.DAT  and BBSLIST.DAT are NOT used directly used by  MSYS

and  may  be  stored  on a diskette, if  desired,  when  MSYS  is

running. I just keep them in the normal MSYS directory.

6-115

MUTIL Function 15 - Build list of archived messages

This  newly documented function will look at messages found in  a

given directory (in MAIL.BAK format) and construct a file of  the

message  headers. The result is a file of all your messages  like

doing  a L command on the BBS for every message that  has  passed

through your system. You can use the grep function on it to  find

messages  with  some  particular string of  characters  in  their

header line. I use it to find a message when I think to myself "I

remember  there was a message about ... one time".  Then  knowing

the message number you can go back to your archive diskettes  and

extract the complete text.

6-116

MUTIL Function 16 - Printer status

This  function provides a very simple printer test to see if  its

connected and can be initialized.

6-117

MUTIL Function 17 - merge BBSLIST.DAT files

This  function takes as its input your current  BBSLIST.DAT  file

(which  it  first renames to BBSLIST.BAK) and  another  file  you

specify  (in the same format as BBSLIST.DAT) and merges  the  two

files  into one called BBSLIST.DAT. I use this function to  merge

the BBSLIST.DAT file that some of the users of MSYS have sent me.

This allows me to include in my file some bbs calls that normally

would not have been seen in messages passing through my system.

6-118

18   Check MSYSNODE.DAT integrity

6-119

19   Convert user files 1.05 & before <===> 1.06 and later

6-120

20   Sort calls in the BBSLIST.DAT file

6-121

MUTIL Function 21 - Delete Old BIDS (and MIDS)

This  function  deletes  old bids/mids from  the  file  BID-

LIST.DAT.  It  asks  for the number of days worth  of  bids  that

should be kept. Run it periodically (maybe once a month). The bid

file  can hold up to 6500 entries; the more it has the slower  it

works. The maximum number of bids in the bid file is  settable.

6-122

MUTIL Function 22 -  Specify file name to use in place of printer

The  file  you specify will be opened and  will  receive  any

output that would normally go to the printer.  Any existing  data

in the given file will be lost.  If you want to abort this  func-

tion,simply press return when asked for the file name.

6-123

Information for Advanced Users

I. Replaceable directory names

Beginning  in MSYS 1.04 you can specify alternate  directory

names  to be used by MSYS in place of the standard ones by  using

appropriate  DOS  set commands. The directory names that  can  be

replaced and their default values are:

Default             Set DOS variable

Dir Name            to replace

--------            ----------------

FILES               MSYS:FILES

FWD                 MSYS:FWD

HELP                MSYS:HELP

MAIL                MSYS:MAIL

MAIL.BAK            MSYS:MAIL.BAK

As  an example, assume you have a RAM disk at drive  E:  and

have  set up a directory called HELP on E: and copied the  normal

help  files to this directory. You can then issue  the  following

DOS command before starting MSYS:

set MSYS:HELP=E:HELP

and now MSYS will get the help files from the E:HELP directory.

NOTE:  You  must  be very careful in using a  RAMDISK  since  its

contents  are  easily lost. Also, you should probably not  use  a

RAMDISK that takes space out of the normal 640K address space.

NOTE:  You  are not limited to using RAMDISKS in  specifying  the

disk  and/or directory to be used for the above directories.  You

could specify floppy and/or hard disks as well.

II. Replaceable file names.

You  can  also  replace the following file  names  of  files

normally  found  in  the "MSYS"  (default)  directory.  Judicious

choices  can  speed  up  operation and/or let  you  make  use  of

multiple disk drives. Poor choices can be a disaster!

Default File        Set This DOS

Name                Parameter to replace:    Notes

-------------       ---------------------    -----

BBSTONTS.BIN        MSYS:BBSTONTS.BIN        (1)

MSYS.FWD            MSYS:MSYS.FWD            (2)

MSYS.HRD            MSYS:MSYS.HRD            (3)

MSYS.LOG            MSYS:MSYS.LOG            (3)

MSYS.MSG            MSYS:MSYS.MSG            (4)

MSYS.USR            MSYS:MSYS.USR            (4)

MSYSHOST.NET        MSYS:MSYSHOST.NET        (5)

MSYSBBSB.DAT        MSYS:MSYSBBSB.DAT        (2)

MSYSPASS.DAT        MSYS:MSYSPASS.DAT        (5)

6-124

Notes:

1 - Very good candidate for ram disk (read only)

2 - Good to put on ram disk if you are careful (changed by MER)

3 - These can be made NUL if you don't want them

4 - If you like to live very dangerously, consider putting them

on ram disk. Otherwise keep them on hard disk. Updated

frequently during normal operation, they are very important

files.

5 - Good for ram disk if you run tcp/ip much. Read mostly.

Example:  Assume you still have a ram disk at E: and have  copied

the  normal contents of BBSTONTS.BIN to that disk. You  could  do

the following DOS command

set msys:bbstonts.bin=e:bbstonts.bin

to cause MSYS to read the file from your ramdisk.

NOTE: With all these changes you can make you can configure a lot

in  terms of files now. Be careful of file sizes (MSYS.LOG  grows

endlessly) when thinking about where to put things.

NOTE:  The  programs  in MUTIL assume the files  are  where  they

should  be and totally ignore any replacements you may have  made

of file names and/or directory names.

NOTE: If you are not absolutely sure of what you are doing, don't

(do anything with these facilities).

NOTE:  If  you crash your system, don't blame it on  me!  If  you

think one of these replacements doesn't work right let me know.

NOTE: Before you try any of this, MAKE BACKUPS OF EVERYTHING!

NOTE: Good luck!

NOTE: This function hasn't been exhaustively tested yet!

6-125

EDITVIEW (ED Command) #

(c) Copyright 1992 by HUB COMPUTERS, INC.

The  ED command used from the MSYS command mode can be  used

to edit small text files. The file is limited to records each  80

bytes  or less in length. By default up to 200 records may be  in

the  file. This can be modified by giving the maximum  number  of

lines  after  the file name. The intent of this editor is  to  do

little  things  without having to take down the system to  run  a

more powerful editor. These might include editing message  texts,

the forward file, etc.

ED is a full window editor (it adjusts its operation to fill

the  window it is run in). It must be run in a window that is  80

characters wide. Thus using the default window definitions it can

be  run  in  the normal command window (0) or  you  can  use  the

CMdwindow  command to first go to window 4 (the one you get  when

you press F4) and then use the ED command. You will then be using

the entire screen for your editing.

ED  is  pretty much a what you see is what you  get  editor.

There  are two modes of operation. When you start the editor  you

are in full screen or edit mode in which changes you make to  the

screen are also made to the file. In this mode you get a line  at

the  bottom  of  the window that gives such  information  as  the

current line number and the name of the file being edited.

The other mode is command mode in which you give commands to

be  done  by the editor. To enter command mode press  the  Escape

key.  The  bottom line will change to Edit Cmd:  and  await  your

command. The command you type will appear on this bottom line. To

get  a  list  of available commands type  Help.  You  should  see

something that looks like the following:

Commands

Find - locates given string. Example:  F wa8bxn

Notes: Not case sensitive. F10 from edit mode is find

same string again.

Insert - puts blank line at beginning of file

QUIT - Abandons edit

SAVE - Saves file and exits

DELete  -  Delete specified # of lines from  file  beginning

with current line

Return key in edit mode with insert on inserts blank lines

Press a key to return to edit

Use  of  the keypad to move around in the file  is  what  is

expected  of the labeled keys, with the  following  enhancements.

If you are somewhere in a line and you press the Home key you  go

to   the   beginning  of  the current line.  If   you   are   the

beginning  of  the  current line and press Home  you  go  to  the

beginning  of  the  screen. If you are at the  beginning  of  the

screen, pressing Home takes you to the beginning of the file.

7-126

EDITFILE (EF Command) #

(c) Copyright 1992 by HUB COMPUTERS, INC.

The  EF command used from the MSYS command mode can be  used

to edit small text files. The file is limited to records each  80

bytes  or less in length. By default up to 200 records may be  in

the  file. This can be modified by giving the maximum  number  of

lines  after  the file name. The intent of this editor is  to  do

little  things  without having to take down the system to  run  a

more powerful editor. These might include editing message  texts,

the forward file, etc.

This  isn't  a full screen editor but rather a  line  editor

that  was influenced in its design by the UNIX editor. Plain  and

simple,  nothing fancy is found in it. To begin editing, use  the

command  EF filename when you have the cmd: prompt. The  filename

can  contain  drive and path information. If the  file  does  not

exist you will be given the option to create one by that name.

To create a new file this way, use the Append command.  Type

the  lines of the new file and then a line with a period  at  the

beginning   to   exit append mode. Then use the  commands   Write

and Quit (each followed by the return key) and you have created a

new  file.  All  of the commands can be shortened  to  the  first

letter  of  the  command,  except for the DElete  command   which

requires  at least the first two letters.

Here is information about the commands:

#   (i.e.,  a line number) makes that line the current  line  and

also displays it

+ # Makes the current line # more than it is now. If # is omitted

then the current line is incremented by one.

- # Same as + # except current line number is decremented.

<ESCAPE> Using the Escape key followed by the Return key abandons

edit  with no changes made to the file (unless you  used  the

Write command yourself).

<RETURN> Pressing the Return key alone displays the next line and

makes it the current line.

Append  enters input mode, placing the following lines  of  input

AFTER  the  current  line. A line  beginning  with  a  period

terminates append mode.

Bottom makes the last line the current line and displays it.

Change can be used to replace part of the current line. The first

non-blank character after the command is the quote character.

The  operands  are quote-character  string-to-replace  quote-

character replacement-string quote-character. For example,

C /YXZ/abc/

would  replace XYZ in the current line with abc. The line  is

7-127

printed after it is displayed.

DElete  can  be used to delete the current line. If a  number  is

specified as an operand, it is the number of lines to  delete

beginning with the current line.

Down  moves the current line down the number of  lines  specified

(or 1 if none is specified).

Find uses the first non-blank character after the command word as

the  beginning of the search string. The search is  from  the

current  line  on in the file. Case is NOT  important.  If  a

match  is found the line is displayed and that  line  becomes

the  current  line.  If no match is found  the  current  line

remains unchanged.

Help displays the list of commands.

Insert is like Append except that the new lines are placed BEFORE

the current line.

Near displays the contents of the file 5 lines before and 4 lines

after the current line which remains unchanged.

Print with no operands displays the current line. Give 1  operand

it  displays  the  line with that number.  Given  2  operands

(separated with a blank or comma) it displays from the  first

to  the second line number. The current line  number  remains

unchanged.

Quit is used to exit the editor after the file has been written.

Top  makes  the  first line (numbered 0)  the  current  line  and

displays it.

Up decrements the current line number by the number given or 1.

View  displays the current line and the 9 lines that  follow  it.

The current line remains unchanged.

Write causes the file to be written to disk with any changes that

have been made.

^F  is used to search again for the same search string give  with

last use of Find.

To start the editor, type EFile filepathname [# of lines max].

Examples:

ef msys.fwd 250

ef mail/msg25.dat

efile a:f1.c

7-128

HOUSECLEANING (HC command) #

(c) Copyright 1992 By HUB COMPUTERS, INC

MSYS.HCL

You create a file called msys.hcl. In it you put lines describing

classes  of  messages  you want to eradicate.  Each  line  should

contain one or more of the following keywords:

AT= TO= FROM= AGE= TITLE= TYPE= KEEP= SYSOP=

So for example:

AT=ARRL AGE=30

would delete all messages with @BBS of ARRL that are over 30 days

old.  If  specified,  the values of AT TO  and  FROM  must  match

exactly for a message to be deleted. TITLE and TYPE if  specified

must  match  some  part of  their  corresponding  message  header

fields; case is ignored.

KEEP can be used to specify a number of msgs that match the other

values given that should be saved anyway.

Example: TO=ALL AT=ARRL TITLE=PFB KEEP=2

Would keep the newest two messages to ALL@ARRL with PFB somewhere

in their title. The value of title given cannot contain blanks.

SYSOP=Y  has to be specified in any line that you want to  delete

messages that are to the idcall of the bbs.

Thus to delete all read messages (including one to you) you could

have  a  line: TYPE=PY SYSOP=Y The following  line  deletes  read

messages, except those to you: TYPE=PY

The  effects of these lines can be far reaching. For example,  if

you put in the line: KEEP=5

Then it will delete all but the 5 newest messages!

All this action takes place when you type HClean NOW

or  at a time specified as a numeric parameter to HClean. HC  120

would cause it to happen at 02:00  (number given is minutes after

00:00 [midnight] to begin housecleaning).

msys.hcl file as an example:

at=arrl age=21

at=okipn age=10

at=panet age=3 type=#

at=alloh age=10 type=#

at=neoh age=10 type=#

at=allbbs age=7 type=#

at=rlibbs age=7 type=#

at=amsat age=7 type=#

at=midnet age=3 type=#

at=usnet age=3 type=#

at=wpab age=3 type=#

at=arrl title=pfb keep=1 type=#

8-129

at=arrl title=dx keep=1 type=#

at=arrl title=keplerian keep=1 type=#

type=# title=sale age=2

type=# title=need age=3

type=# age=35

type=py keep=1

type=pf keep=2

type=tn age=21

type=pn age=60

to=sysop age=7 type=#

At the end of housecleaning, the file MSYSHCL.DO is done (as a DO

file) if it exists.

8-130

MODEM Support

(c) Copyright 1992 by HUB COMPUTERS, INC.

One  or more modems may now be connected to MSYS.  Each  re-

quires  its  own port. In the port command in MSYS.OPT  for  each

modem,  the speed value is the rate at which the modem is  to  be

operated on the phone line.

Note  that only one speed is possible for each port.  You  should

set radio speed to 1200 baud independent of phone line speed  and

also  select a tnc file. Here is a sample MSYSTNC.9 file  I  used

for  testing with comments (not actually in the file) that  begin

with :

ATZ             : reset modem

ATS0=1          : answer on one ring

ATQ1            : don't send any result codes

ATM1            : turn on speaker during call setup

When  the  computer  answers the caller will  have  to  give  two

values, one for what is called "call" and another for "password".

These  values,  in the case typed, must be found on a line  in  a

file called MSYSPASS.MOD. A third entry in each line of this file

is  required and is the callsign that the caller will  appear  to

have  connected with on the bbs. Thus the first response  doesn't

have to actually be a callsign. All functions normally  available

on  the bbs from a radio connection are available via  the  modem

including remote sysop if so authorized. If the correct  informa-

tion  is not provided in two attempts the modem hangs up,  as  it

does when the Bye command is used by the caller or the channel is

disconnected  by  the  sysop. There is no timeout  timer  on  the

modem.

In the PORT statement in the MSYS.OPT file mentioned above, a

modem is identified by giving a port the name MODEM.

Carrier Detect must reflect real CD. That is be true when someone

is connected and false otherwise.

DTR must also be connected.

A parameter can be added at the end of the lines in  MSYSPASS.MOD

(after  the "password"). If NETNODE (must be all caps) is  given,

then the caller will be connected to the network node instead  of

the BBS. Use this option with CAUTION! Here is a sample line:

wombat poobear WA8BXN NETNODE

Modem forwarding support. In your forwarding file, you put a  F#,

R#,  or P# line followed by the things to forward. The format  of

the  F,R  and P lines are identical except for the  letter.  This

format is different from what would be used for packet forwarding

and is as follows:

F# callsign phone# callword password

where  # is the port number, callsign is the call of the  station

you are forwarding to, phone number is the string to send to  the

modem to dial, callword is the reply to give to Call:, and  pass-

word  is the reply to give to Password: (questions asked  by  the

8-131

destination MSYS system when you call it). Example:

F1 WA8BXN ATDP5551212 swiss normal

Note: Everything is converted to upper case. Thus the station you

are  calling  must have UPPER CASE values for your  callword  and

password.   The station you are calling must be running  1.11  or

higher.

Modem activity is monitored on the F2 screen.

You can create a file called MSYSMOD.MSG to display lines when  a

caller  first  calls the modem, that contains ascii  text  to  be

displayed to anyone connecting to the modem port before they  get

the  call  and password prompts. The normal message  of  the  day

should work with the modem after the bbs is entered.

8-132

Sysop commands related to TCP/IP #

(c) Copyright 1992 by HUB COMPUTERS, INC.

MYIpaddr  takes four integers in the range 0 to 255. It sets  the

IP address that the system will respond to. Example: MYI 44 70  4

6  If  MYIpaddress  is not specified, or set to  all  zeros,  the

TCP/IP support will be inactive.

MYTcpip  takes  a  callsign (with SSID) that will  be  used  when

sending arp and TCP/IP frames. It can be the same as the ID  call

in most cases.

TElnet  takes  an  IP  address  and  optional  server  number  as

parameters.  A symbolic IP address may given to be looked  up  in

the file MSYSHOST.NET. Default server is 23. This command is used

to  establish  keyboard  to keyboard  connection  with  a  TCP/IP

station.

Examples:  TE [44.70.4.10]

te hsp             (hsp would have to be defined in

MSYSHOST.NET)

TElnet  operates much like C# normally used to connect  to  AX.25

stations. ^Cd is used to disconnect.

TPorts  takes  a port mask value as an argument. This  number  is

used  the  same way as in the other similar commands:  each  port

corresponds to a port (port 0 is the rightmost bit) and when  the

bit  is  set  to 1 for a given port that port  is  available  for

TCP/IP use (particularly ARP broadcasts).

ARP used without an argument lists the known callsign/IP  address

correspondence   determined  by  TElnet  connects.  An   optional

argument, CLEAR, may be used to erase all of the ARP entries.

FTP  takes  a  host-id as an argument (it can  be  either  an  IP

address  in  the square brackets or a symbolic  IP  address).  It

establishes  a  connection  to the FTP server  of  the  specified

system. FTP is the File Transfer Protocol used in TCP/IP.

SMTP  allows  local access to the SMTP server  (mainly  for  test

purposes).

Note: The following servers are currently implemented in MSYS:

7       Echo

9       Discard

21      BBS

23      Telnet

25      Simple Mail Transfer Protocol

87      Local keyboard

*** Files related to TCP/IP:

MSYSHOST.NET  This  is an Ascii file you create  with  an  editor

(such as the MSYS ef command). Each line in the file begins  with

8-133

an  IP  address  that includes the periods  but  not  the  square

brackets  around it. Following this there is at least  one  space

and/or  tab   character,  then  the list of  symbolic  names  (or

aliases,  or nicknames,  what ever  you want to call  them).  You

may place comments on the  line by  beginning the comments with a

pound sign (#).  Example  entry line for this file: 77.70.4.6 bxn

mike wa8bxn # kirtland, oh This line would allow "bxn" "mike"  or

"wa8bxn" to be used as arguments  in the TElnet and FTP commands.

Note that WA8BXN  here just  happens to look like a callsign. The

actual   callsign   (and SSID)  associated with a  particular  IP

address  is  determined  by ARP  (address   resolution  protocol)

broadcasts.   These   symbolic names  you  define  in  this  file

simply relieve you of  having  to remember (and type) complete IP

addresses. Nothing more,  nothing less!

MSYSPASS.DAT This file contains the user names and passwords for

FTP  users.  Each line contains 3 or 4 items:  username  password

access directory Username  is  the  name the user will use to log

in.   It  can  be  anything  for  a  given  user. It   could   be

their   callsign  or  something  else. The password can  also  be

anything.  If you put  *  as the password in the file for a  par-

ticular, any password  given by  the  user  will be accepted. The

third  entry  on  each  line, access, is a number that  says  how

much  access  the user will  have to  your  system: 1  means  can

only read files, 3 means  read  and write  (but  not  replace  or

delete)  and   7  means  read,  write, replace  and  delete.  You

shouldn't  set 7 for any user  since  the passwords  given  by  a

user  can  be monitored by others.  The  last item  which  should

always be given is the root directory that  the user  will  have.

They  may never go closer to the real  root  than what  you  give

here. Here are some examples:

anonymous * 1 files        --- this allows user anonymous to read

any  files in MSYS/FILES and  any  of

its subdirectories. Any password  can

be  given  for this  user.  Anonymous

with   *  for  the  password  is   an

expected "standard" entry for  TCP/IP

systems  to  give access  to  totally

public files. Include it in your

system.

apple runtime 3         --- This allows user apple (who must give

password  runtime) read/write  access

to   your   entire   disk    (DEFAULT

DIRECTORY    IS   THE    REAL    ROOT

DIRECTORY!)

*** USER ACCESS NOTE

Access level chart

Level 1: CD DIR GET PWD QUIT

Level 3: PUT MKDIR

Level 7: PUT (to replace existing file) DELE RMDIR

8-134

If you are going to allow level 7 access (ie, can do  everything)

I  would suggest you also assign a directory that  doesn't  allow

access  to  any needed files. For example, you might set  up  the

following   entry  in  the  MSYSPASS.DAT  file:

WA8BXN  MIKE   7 /MSYS/FILES/WA8BXN

You  will  have  to  create  a  directory  WA8BXN  in  the  files

directory.  Now  stations  can log in with  WA8BXN  MIKE  and  do

anything in /MSYS/FILES/WA8BXN and any directories found in  this

directory.

If  you  set  up  ANONYMOUS * 1  /MSYS/FILES  then  the  user

ANONYMOUS  can  read anything put in the WA8BXN  subdirectory  of

FILES  (as  well as in FILES itself) but not write  or  otherwise

change  anything.  The rest of your files should be  pretty  well

protected.

*** FTP COMMANDS:

When you use the FTP command to connect to another system you can

type HELP to get a list of the supported commands.

They currently include:

CD dirname  To move around in the directory structure

CD          with no arguments gives the current directory

DELE filename  To delete given file

DIR filespec gives a directory listing

GET filename   downloads a the given file

MKDIR       makes a directory

PUT filename   uploads the given file

syntax is put [localfilename] remotefilename

If the optional localfile name is omitted it is assumed

to be the same as the remote file name.

PWD         gives the current directory

QUIT        disconnects

RMDIR dirname  removes the given directory

FTP  function  will  give an estimate on the  number   of  bytes

transfered during GET and PUT commands.

*** IP Routing:

What  does  this mean? Its sort of like digipeaters  for  the

TCP/IP  world.  If a TCP/IP user sets you up as a  route  for  IP

frames going to a particular IP address MSYS will now repeat them

if  it has (or can get through an ARP request broadcast) a  known

route  to  the  DEStination IP address.  Consider  the  following

fictitious example: W9AAA [44.60.1.1] is in Indiana and wishes to

connect  to  W3AAA [44.80.1.1] in Pennsylvania but  can't  do  it

directly.  W9AAA can reach W8AAA [44.70.1.1] in Ohio who  can  in

turn reach W3AAA.

W9AAA  will set up the route to [44.80.1.1] to be W8AAA (an  MSYS

system).  When attempting to connect to W3AAA (using Telnet,  for

example)  will  send  out a SYN IP  frame  (connect  request)  to

8-135

callsign  W8AAA  but with an IP address of  [44.80.1.1],  the  IP

address  of  W3AAA. W8AAA upon hearing this will  check  its  ARP

table to see if it knows how to get to [44.80.1.1]. If the  entry

is  not  there, W8AAA will do an ARP broadcast on all  the  ports

enabled  for TCP/IP. W3AAA will respond on one of them  and  this

will put an entry in the ARP table. Now when a SYN is heard  from

W9AAA it will be repeated by W8AAA and thus heard by W3AAA.

To  use IP ROUTING yourself in MSYS you use the arp add  command.

When  adding  a given IP address you would put the  call  of  the

TCP/IP station you want to use as an IP repeater in place of  the

call  that belongs with the IP address. You can still use  normal

ax.25 digipeaters (up to 8) to reach the TCP/IP station.

If  the  path from W8AAA to W3AAA in the above  example  was  not

direct,  then  the following arp entry could be  done  at  W8AAA,

assuming that K8AAA is a normal ax.25 digipeater:

arp add W3AAA port# 44 80 1 1 K8AAA

NOTE:  For  more on TCP/IP refer to The  KA9Q  Internet  Software

Package.

In the TCP/IP related ARP command, use -2 as the port number  for

routing through the network interface.

8-136

Samples of MSYS files #

(c) Copyright 1992 by HUB COMPUTERS, INC.

*** MSYS.OPT:

PORT 0 AT $3F8 INT 4 SPEED 4800 NAME  145.090

PORT 1 SUBPORT 1 OF PORT 0 NAME 220.52

PORT  2  AT  $2f8 INT 3 SPEED 4800 NAME  145.030  RADIOSPEED  300

TNCTYPE 1

NUMCHANS 10

DIGI N8HTG-1 FROM 0 TO 1

DIGI NT8V-1 FROM 0 TO 1

DIGI N8HTG-1 FROM 1 TO 0

DIGI NT8V-1 FROM 1 TO 0

DIGI N8HTG-3 FROM 1 TO 2

DIGI NT8V-3 FROM 1 TO 2

DIGI N8HTG-3 FROM 2 TO 1

DIGI NT8V-3 FROM 2 TO 1

DIGI N8HTG-6 FROM 0 TO 2

DIGI NT8V-6 FROM 0 TO 2

DIGI N8HTG-6 FROM 2 TO 0

DIGI NT8V-6 FROM 2 TO 0

bbs alias NT8V

BBS CALL N8HTG

ANSWER CALL N8HTG-13

KANODE CALL N8HTG-8

SYSOP NAME Lee

SYSOP QTH PORT HURON, MI.

SYSOP ZIP 48060

ID CALL N8HTG

FORWARD CALL N8HTG-15

STACK FORWARD 8000

STACK DECODE 1500

STACK CALL 4000

FORWARD TIME 10

ID EVERY 45

MONITOR OUTGOING ON

MONITOR IFRAMES ON

MONITOR UFRAMES ON

MONITOR BFRAMES ON

AX25 RETRIES 10

WINDOW 0 BACKGROUND 0

WINDOW 5 BACKGROUND 0

WINDOW 6 BACKGROUND 0

WINDOW 2 BACKGROUND 0

WINDOW 3 BACKGROUND 0

WINDOW 5 FOREGROUND 10

WINDOW 6 FOREGROUND 9

WINDOW 2 FOREGROUND 12

MAKE

*** MSYS.DO:

WS 0 1 1 80 17

8-137

WS 5 50 19 80 25

WS 2 1 19 48 21

WS 6 1 22 51 24

retries 5 5 5

maxframes 4 4 4

frack 15 15 15

paclen 128 128 128

BD 0 NM8X

BT 0 N8HTG PBBS -- PORT HURON MI.

BD 1 KV8G-3

BT 1 N8HTG PBBS -- PORT HURON MI.

MAKEP OFF

TIMEZONE UTC

MONST OFF

MYI 44 102 0 75

MYT N8HTG-4

TP 2

host xt.n8htg.ampr

BELL ON

*** MSYS.OTD:

<<< K-NODe = N8HTG-8 #### GATEWAY = N8HTG-1 145.03 <-> 145.09 >>>

*** MSYSK.OTD:

< Please try N8HTG PBBS ### N8HTG-1 Gateway 145.030 <-> 145.090 >

*** MSYS.REP:

48060 NTSMI 48060 N8HTG

48040 NTSMI 48040 N8HTG

48001 NTSMI 48001 N8HTG

48002 NTSMI 48002 N8HTG

*** MSYS.RMT:

TEST PHRASE

*** MSYSBBSB.DAT:

25 VE3WZL

26 WA8BXN

27 WB8I

28 N8HTG

29 VE3GYQ

30 WA8OOH

*** MSYSFWD.DEF:

F1 WA8BXN

WA8BXN

----------

F0 WB8I V NM8X-1

8-138

WB8I

----------

*** MSYSHOST.NET:

# IP address         host name         alias      # comments

#

44.102.0.74         xt.nt8v.ampr       nt8v       #roy

44.102.0.75         xt.n8htg.ampr      n8htg      #lee

*** MSYSPASS.DAT:

guest * 3 public

nt8v roy 3

n8htg lee 3

k8dd hank 3 public

*** MSYSTNC.1 (AEA PK-232):

VHF OFF

HB 300

AWLEN 8

PARITY 0

CONMODE TRAN

TRACE OFF

HID OFF

BE 0

passall off

RAWHDLC ON

HP OFF

PP ON

KISS ON

HOST ON

host on

8-139

BIT PATTERN CHART #

(c) Copyright 1992 by HUB COMPUTERS, INC.

To be used for DPorts, KPorts, MONPorts, NPorts and TPorts

[ PORT     3     2     1     0 ]       1=ON     0=OFF

0               0     0     0     0

1               0     0     0     1      0-ON

2               0     0     1     0      1-ON

3               0     0     1     1      1,0-ON

4               0     1     0     0      2-ON

5               0     1     0     1      0,2-ON

6               0     1     1     0      2,1-ON

7               0     1     1     1      0,1,2-ON

8               1     0     0     0      3-ON

9               1     0     0     1      3,0-ON

A               1     0     1     0      3,1-ON

B               1     0     1     1      0,1,3-ON

C               1     1     0     0      3,2-ON

D               1     1     0     1      0,2,3-ON

E               1     1     1     0      1,2,3-ON

F               1     1     1     1      ALL ON

8-140

MSYS AMTOR Interface

(c) Copyright 1992 by HUB COMPUTERS,  INC

Introduction

------------

NOTE:  The AMTOR interface is still  undergoing  develop-

ment.  Additional  features (in particular  forwarding)  will  be

added in later releases. Currently only Kantronics KAM tncs  with

Host Mode are supported. When used for AMTOR, the VHF portion  of

the  KAM cannot be used (this limitation will probably  never  be

removed  unless  Kantronics allows simultaneous use of  KISS  and

Host Mode).

The  AMTOR  interface in MSYS allows  AMTOR  stations  to

connect  to  the normal MSYS BBS. From there the DX Node  may  be

accessed  if  enabled. The connect commands may also be  used  to

make outgoing keyboard connects to other AMTOR stations.

AMTOR has some major differences when compared to  packet

operation. The two most important is that when AMTOR is used only

two  stations  can be connected together at one time on  a  given

frequency and secondly that AMTOR uses a greatly reduced  charac-

ter set when compared to ASCII used with packet.

Another  difference  is  that AMTOR  will  probably  work

better  than packet in marginal signal situations. AMTOR is  also

much  slower (around 100 bits per second) that packet  operation.

This  combined with the fact that only 3 bytes of text  are  sent

before  an acknowledgement is obtained account for  the  improved

performance on HF.

AMTOR operates in a half duplex mode; only one of the two

connected  stations  can  send data at a time. Thus  there  is  a

sending  and  receiving station at any given time  unlike  packet

which  is more full duplex in nature. This description of  course

ignores  that  fact that at any given instant  usually  only  one

packet  station  can transmit at a time. With AMTOR  an  explicit

action must be taken to reverse data flow direction. This is usu-

ally  done  by sending +? at the end of a group  of  text  lines.

Think  of  the +? sequence as the "go ahead, its your  turn  now"

signal.  IMPORTANT NOTE: There can be no space between  the  plus

and  the  question mark. Also note that if you happened  to  send

data that contained the sequence the direction of data flow would

be changed when you might not have expected it.

MSYS  handles the direction of data flow control for  the

most  part. When an AMTOR station connects to MSYS either the  +?

sequence or CR LF can be used to indicate when MSYS should  begin

transmitting. If MSYS does not have anything to send (as would be

the  case  if you are in the process of   entering  a  multi-line

message)  MSYS will again reverse the direction of data  flow  to

allow the station to continue sending.

Much  more  can be said about AMTOR. Refer  to  your  TNC

manual for more information.

8-141

Setting up an AMTOR port

------------------------

AMTOR  ports  are identified by the port  name  beginning

with the ~ character.

An  example of a PORT statement in MSYS.OPT for an AMTOR port  is

PORT 0 AT $3F8 INT 4 SPEED 4800 NAME ~20M RADIOSPEED 300

The use of RADIOSPEED 300 will shorten some of the messages  sent

by the bbs to the user. The TNCTYPE parameter should not be used.

Before  starting MSYS with the AMTOR port for  the  first

time,  be sure that the SELCAL parameter in your TNC is  porperly

set.

Note  that  the AX.25 parameters (such as FRACK)  can  be

issued for an AMTOR port but they are ignored in operation.  For-

warding  should not be attempted on an AMTOR port.  Reverse  for-

warding will be attempted but probably won't work if a connecting

station requests it.

Turning an AMTOR port off will not prevent stations  from

connecting to it; turn the radio off instead! You cannot  monitor

general activity on AMTOR ports on the F2 screen. Only the infor-

mation exchanged with a station connected to your system is shown

(along with various status messages the TNC sends to the  comput-

er).

AMTOR Users and the BBS

-----------------------

Due to the character set limitations of AMTOR, the famil-

iar > prompt character is replaced by the +? sequence which  also

allows the user station to enter their reply.

Most  of  the  normal BBS commands  should  work  without

modification from the way they would be used on packet. A notable

exception  are the various S commands used to send  messages.  In

place of the @ character the word AT (with blanks around it) must

be used. Since the $ character is not in the AMTOR character  set

(if used it may have strange results), BIDS cannot be  explicitly

entered. The < character is also invalid in AMTOR so it can't  be

used to indicate the originating station. Here are some  examples

of S commands:

SP WA8BXN AT WA8BXN.#NEOH.OH.USA   {valid}

SP WA8BXN@WA8BXN                   {invalid because of @}

SP WA8BXN < WB8ZZS                 {invalid because of <}

ST 44026 AT NTSOH                  {valid}

SB ALL AT NEOH $SOMEBID            {invalid because of $}

SB ALL AT ALLOH                    {valid}

SB ALL ATALLUS               {invalid, no space after AT}

The  Talk command will work. You will have to use the  +?

sequence to turn it over to the user station as needed. The  user

will also have to use the +? to turn it back to you.

8-142

Frequent problems and notes from the author #

(c) Copyright 1992 by HUB COMPUTERS, INC.

IMPORTANT:  You must use the new version of MUTIL with this  ver-

sion  of  MSYS (ie, run setup, function 9). Its probably  also  a

good idea to delete MSYS.HRD when going to a different version of

MSYS or changing the port statements in MSYS.OPT!

If you have problems getting MSYS going here are some things to

try that have helped many who have called:

1.  Make  sure you have FILES=20 and BUFFERS=20  in

CONFIG.SYS

2. Try adding STACKS=0,0 to CONFIG.SYS

3.  Get rid of anything resident (check AUTOEXEC.BAT  and

CONFIG.SYS)

4. Run MUTIL 4 (do this every time you start MSYS)

5. Simplify MSYS.OPT as much as possible; remove all PORT

statements

6. Delete your MSYS.DO file (after making a backup!)

These  items  should get you to the point where MSYS  will  work.

From here try putting things back and see what causes failure.

--- The system will not forward.

Make sure you executed the MERGE command. Without doing this  the

system will never forward.

NOTE:  You  must have the file MSYSBBSB.dat in order  to  forward

bullitens.

--- The system appears to work but nothing is on the screen.

Make  sure  you set the display type correctly  in  the  msys.opt

file. The command is WINDOW DISPLAY COLOR for color monitors  and

WINDOW DISPLAY MONOCHROME  for monochrome monitors.

--- The system comes up and appears to hang.

This may be caused by a non-existent com port, check the lines in

your msys.opt to make sure the port addresses are correct.

--- The system boots after forwarding

This may be caused by having too many entries in your forwarding

file remember you may only have up to 300 different bbs names in

your file.

--- Performance hint:

Don't let too many messages pile up in mail.bak <DIR> before  you

delete  them (after archiving if you wish) so disk  doesn't  fill

up.

"I Haven't tried it, but maybe you will want to" Section

8-143

Here  are  some suggestions from users of msys  that  they  claim

work. Use at your own risk!

To cure the problem of certain tncs that don't transmit  properly

on  HF  use  radiospeed of 1200 in place of 300  in  msys.opt.  I

can't  imagine how this could help, but someone says it  did  for

them (they weren't using MFJ tncs which seem to have this kind of

problem).  One  thing  you might want to try if  you  are  having

problems  with  a tnc on HF is to get a copy  of  KA9Q's  net.exe

program  and  see if you can use its ax.25 connect  command  with

your  tnc on hf. If that fails as well the problem definitely  is

in the tnc.

To  run msys under dv on a 386 machine, try these values  in  the

"performance" section:

foreground 9

background 6

common memory 15

dos buffer for ems 5

optimize communications Y

allow swapping of programs N

manage printer contention N

To put a MFJ Tnc in kiss mode, try this in your msystnc.# file

ax off

parity 0

awlen 8

conmode trans

kiss on

restart

Paccom rom version 1.1.5 may work better than 1.1.662

If  you are just bring up msys for the first time and  experience

crashes when mail is forwarded to you, or the size of BIDLIST.DAT

is 0 bytes, do the following to solve the problem: Shut down msys

(quit), delete BIDLIST.DAT, start msys, shut it down (the dos dir

command  should now show a size of 130000 for  BIDLIST.DAT).  You

should  now be ready for normal operation.  ANOTHER NOTE TO  ALL:

From  time to time do a BID # command and see how many  bids  are

active.  If  its over 3 or 4 thousand its probably  time  to  run

MUTIL function 21 to get rid of the older bids.

---  If  you are using DOS 3.3, you may need to  put  STACKS=0,in

your config.sys file.

If  you find that the displayed time is different from  what  you

have set the time to be in DOS, put the following in AUTOEXEC.BAT

and reboot: SET TZ=EST5

Home BBS replacement occurs only if @BBS is blank or the call  of

your bbs.

Note:  If you run into problems with newer versions of  QEMM  and

MSYS, try turning off the stealth option in QEMM.

8-144

Here  is some useful information about running MSYS  under  Desq-

view, provided by my local DV guru, K8EIW (Thanks, Don!):

To  run MSYS under DesqView on a 386 or a 486 machine, try  these

values  in  the  "Add Program" (AP) setup: O (for Other)  F1  (to

specify Program Information)

PROGRAM NAME = MSYS

Keys To Use = MS (or your choice!)

Memory Size = (as big as you can make it - try 585K)

Program  = C:\MSYS\MSYS.EXE (be sure to specify proper drive  and

sub-directory)

Directory = C:\MSYS  (you DO use subdirectories, don't you?)

Writes text directly to screen.......: [Y]

Displays graphics information........: [N]

Virtualize text/graphics (Y,N,T).....: [Y]

Uses serial ports (Y,N,1,2)..........: [Y]

Requires floppy diskette.............: [N]

F1 for advanced options

Starting Height:  25

Starting Row...:   0

Starting Width.:  80

Starting Column:   0

Close   on   exit   (Y,N,blank)......:  [   ]

Uses its own colors..............: [Y]

Allow   Close  Window  command.....:  [Y]

Runs in background (Y,N,blank)...: [ ]

Uses  math  coprocessor..........:  [Y]

Keyboard conflict (0-F)..........: [0]

Share   CPU   when   foreground......:  [Y]

Share EGA when foreground/zoomed.: [Y]

Can  be  swapped  out (Y,N,blank).: [  ]

Protection level (0-3)...........: [0]

In the performance section:

Foreground 9

Background 6

Allow swapping Y

If  you choose to use QEMM as your memory manager, the  following

line in your  CONFIG.SYS file should give satisfactory results:

DEVICE=C:\QEMM\QEMM386.SYS RAM ROM

A WORD OF CAUTION:

When  using QEMM-386, the /ST:M option in the DEVICE=  line  will

cause  MSYS  to  not load properly. There may  be  other  options

which will also cause grief.

These  suggestions are not necessarily the optimum  settings  for

your system, but they have been known to work.

A useful utility called PAK is found on the primary MSYS  distri-

bution disk. This clever program will take any file (binary, text

or otherwise) and both compress and convert the result into lines

of printable ASCII characters suitable for downloading. I suppose

they  could  also  be sent in msgs but  use  careful  forethought

8-145

before doing it! The same program is used to extract the original

file  from the encoded version. This program is somewhat  similar

to R95 but includes quite effective data compression.

I am still looking into using data compression during forwarding.

Notes on sending me messages with questions/suggestions/etc: I do

try  to respond to your messages. Keep in mind I do get  quite  a

few at times. Messages that contain demands (add this feature  or

correct this bug immediately and put out a new release or I won't

use your program) generally are saved in the bit bucket. If I was

getting $500 dollars a copy from everyone using MSYS it might  be

a  different story but for a free program be considerate! Try  to

put just one thought/question in each message. If you bundle up a

whole bunch, I don't respond until I can address all your issues.

If  you  send follow-up messages, please give a sentence  or  two

summary  of  the problem so that I may understand  what  you  are

talking  about.  Just saying "I tried what you suggested  and  it

still  doesn't work, anything else to try?" doesn't give me  much

of  a  clue about the situation. Although you may have  only  one

problem  you are dealing with and can easily remember its  entire

history,  I  don't have that luxury here! If you  find  what  you

think  is a bug, please try to tell me how to reproduce it,  what

you  are  seeing,  and what you think should  be  happening.  The

harder  the  problem, the longer it takes me to work  on  it  and

respond.  When you run into any sort of problem, simplify  every-

thing as much as possible. Remove all statements from config.sys,

autoexec.bat,  msys.opt  and msys.do that are  not  essential  to

reproduce  the bug. Believe me, this will probably help you  find

at least half of the problems and their solutions yourself.

8-146

INDEX

A (abort), 43

ABortfwd, 10, 17

ACceptmid, 10, 17

ADDUser, 10, 17

Alt-F1 Alt-F2 Alt-F3, 42

AMTOR, 141

ANSWER CALL, 6

APpend, 10, 17

ARChive, 10, 17

ARP, 10, 17

ARP add, 10, 17, 18, 136

ARP DEL, 10, 18

AT number, 3, 4

Atabase, 34

ATtended, 10, 18

AUTODest, 10, 18, 71, 74, 76

AUTOHold, 10, 18

AUTOKill, 10, 18

Automatic back-up, 88

Automatic generation of @BBS field, 86

Automatic message holding, 92

Automatic password reply:, 83

AUTONode, 10, 18, 71, 73, 76

AX25 CHECK, 7

AX25 FRACK, 6

AX25 MAXFRAMES, 6

AX25 RETRIES, 6

AX25Backoff, 10, 18

AX25L2V2, 10, 18

AX.25 version 2 protocol:, 18

BACkuptim, 10, 18

BADuser, 10, 18

BBs, 10, 19, 43, 44, 78

Bbs alias, 4, 6, 13, 33, 137

BBS CALL, 6

BBS ONLY ACCESS, 94

BBSLIST.DAT, 67, 68, 95, 107, 111, 112, 115, 118

BBSLIST.DAT file from the BBSTONTS.DAT, 113

BBSNode, 10, 19

BBSTimeout, 10, 19

BBSTONTS.BIN, 66, 67, 68, 95, 115, 124, 125

BBSTONTS.DAT, 67, 68, 95, 111, 112, 115

BDigi, 10, 19

BElloff, 10, 20

BIDall, 10, 20

BINterval, 10, 20, 76

BMaxcon, 10, 20

BOOT, 10, 20

BPorts, 10, 20

BText, 10, 20

BUDCalls, 10, 20

BUDList, 10, 20

8-147

Bye, 43, 44, 78

C BBS or C bbscallsign, 76

Cache, 2

CAPTURE.DAT, 23

CC command, 43

CHANNEL NUMBERS, 87

CHAnstat, 10, 21

CHEck, 10, 21

CL B #, 10, 21

CLrscrn, 10, 21

CMdwindow, 11, 21

Conference, 43, 44, 78

CONNECTS FROM NON-AMATEUR CALLSIGNS, 94

COpy, 11, 19, 21

CRetries, 11, 21

Cscript, 11, 21

Ctrl/F2, 42

Ctrl/F4, 42

Ctrl/F5, 42

Ctrl/F6, 42

CTS, 11, 22

C#, 11, 22

Database, 44, 53, 54, 76

DEBug, 11, 23

Default directory, 1

DELEfile, 11, 23

DELUDays, 11, 23, 87

DELUSer, 11, 23

DESt, 11, 14, 15, 24, 34, 36, 41, 72, 73, 76

DESt ADD, 72

DESt DELete, 72

DESt REPlace, 72

DEStination, 17, 19, 24, 34, 48, 62, 66, 69, 72, 74, 76, 135

DEStination NODe, 69, 74

Device errors, 85

DFree, 11, 24

DIGI statement, 5

DIGipeats, 11, 24

DIRectory, 11, 24

DISconnec, 11, 24

Disk full message, 86

Displayed time, 144

DOcmdfile, 11, 24

DOS, 14, 24, 25, 37, 93, 124, 125

Download command, 44

DPorts, 11, 24

DROPING @BBS, 64

DRoute, 11, 24

DRoute DELete, 24

DRSI CARDS, 9

DTimedate, 11, 25

DUmpbbs, 11, 25

DX freq dxcall comments, 44, 78, 79

8-148

DXCall, 11, 25, 80

DXConnect, 81

DXDiddle, 80

DXDiddle #, 11, 25, 80

DXEnable, 11, 25, 80

DXNconnect, 11, 25, 80, 81

DXPorts, 11, 80

DXPorts portmask, 25, 81

DXRestore, 11, 25, 80, 81

DXSAve, 11, 25, 80, 81

DXSTart, 11, 25, 26, 80, 81

EDit, 11, 26

EFile, 11, 26

EMsghdr, 11, 26

END OF LINE BELLS, 88

EUser, 11, 26

Executing commands from within the forwarding file, 63

EXPANDED MEMORY USAGE, 93

F2 time, 11, 26

F8, 15, 42

FBbstimes, 11, 26

FCall, 11, 27

FFile, 11, 27

FIle, 11, 27

Files related to TCP/IP, 133

FMsg, 12, 27

FOrward, 12, 27

FORWARD CALL, 6

FORWARDING DISPLAY, 64

FORWARDING NOTES, 63

FRAck, 12, 27, 88

FREE, 12, 27

FSize, 12, 28

FTIme, 12, 28

FTP COMMANDS, 135

FTPgm, 12, 28

FUlldup, 12

FUllduplex, 28

Function key usage:, 15, 41

G command, 44

GEtmsgotd, 12, 28

GRep, 12, 28

H command, 44

HApnstat, 12

HCAll, 12, 28

HCLean, 12, 28

HDRS command has been removed, 93

HEard, 12, 29

Help, 12, 29

HELP\MSYSMSGS.DAT, 1

HF Forwarding, 64

8-149

HOLdlevel, 12, 29, 93

Home BBS replacement, 144

HOStname, 12, 29

How I Run My Network NODe, 73

HReplace, 12, 29

I command, 44

ID, 29

ID CALL, 6

ID EVERY, 6

ID *, 12

IGnore, 12, 29

Information for Advanced Users, 124

INitpr, 12, 29

INT number, 3, 4

IP Routing, 135, 136

IPNCheck, 12, 29

IPNFrack, 12, 29

IPNRetry, 12, 29

IPTtl #, 12, 29

J command, 45

JBbs, 12, 29

JDigipeat, 12, 29

JGateways, 12, 30

JHeard, 12, 30

JKanodes, 12, 30

JMsys, 12, 30

JNetrom, 12, 30

JTcp/ip, 12, 30

Justheard, 75

K command, 45

KANODE CALL, 6

KANTRONICS 3.0 EPROM Upgrade users:, 93

KEyboard, 12, 30, 85

KILLproc, 12, 30

Knode, 75

KPorts, 12, 30

L command, 46

LCount, 12, 30

LINKED, 85

LOCKkbd, 12, 31

LOCKUPS, 87

LOG, 86

LOGclose, 12, 31

LUsers, 13, 31

M command, 47

MAIL BEACON, 86

MAIL.BAK, 35, 68, 89, 95, 98, 99, 107, 116, 124, 143

MAKEPriva, 13, 31

Manual Operations on the Network NODe Database, 72

MAStersrv, 13, 31

8-150

MAX channels AND PORTS, 87

MAXBullag, 13, 31

MAxFrames, 13, 31

MAXRead, 13, 31

MCForward, 13, 31

MCOn, 13, 32

MDir, 13, 32

MEMory, 13, 32

MEMORY MANAGEMENT, 83

MERge, 1, 13, 32, 59, 60, 95, 118, 143

Message of the day, 12, 28, 43, 47

MEXpert, 13, 32

MHclear, 13, 32

MIDchar, 13, 32

MINmem, 13, 32

Miscellaneous Comments on Network NODe, 75

MISsing, 13, 32

MISSING TEXT TO MESSAGE, 65

Modem forwarding support, 131

MODEM Support, 131

MONBframe, 13, 32

MONIFrame, 13, 32

MONITor, 13, 33

MONITOR BFRAMES, 7

MONITOR IFRAMES, 7

MONITOR OUTGOING, 7

MONITOR PORTS, 7

MONITOR SFRAMES, 7

MONITOR UFRAMES, 7

MONOutgoi, 13, 33

MONPorts, 13, 30, 33

MONSFrame, 13, 33

MONSTats, 13, 33

MONUFrame, 13, 33

MONxx, 13, 33, 84

MSgtraile, 13

MSgtrailer, 33

MSYS command (typed at DOS prompt), 2

MSYS DX Node, 78

MSYS node forwarding, 65

MSYS under DesqView, 145

MSYSBBSB.DAT, 1, 58, 124, 138

MSYSFWD.DEF, 1, 13, 32, 59, 60, 62

MSYSHOLD.DAT, 92

MSYSHOST.NET, 39, 124, 133, 139

MSYSK.OTD, 138

MSYSMOD.MSG, 132

MSYSPASS.DAT, 124, 134, 135, 139

MSYSPASS.MOD, 131

MSYSTNC.1, 139

MSYSTNC.#, 4, 86

MSYSTODO.DAT, 39, 82, 84

MSYS.BBS, 32, 59, 63, 64

Msys.def, 1, 3, 4, 8, 106

MSYS.DO, 17, 81, 84, 137

8-151

MSYS.FWD, 13, 32, 59, 103, 124, 128

MSYS.HCL, 28, 129

Msys.opt, 1, 2, 3, 4, 8, 40, 85, 87, 106, 137, 143

MSYS.OTD, 28, 138

MSYS.REP, 59, 60, 138

MSYS.RMT, 83, 100, 138

MUTIL.EXE, 67, 95

MUTIL.EXE Function 9, 3

MYAlias, 13, 33

MYBbs, 13, 33

MYCall, 13, 33

MYIpaddr, 13, 33

MYKnode, 13, 33

MYTcpcall, 13, 33

N command, 47

NAME description, 3

NDwindow, 13, 33, 76

Neighbor NODe, 69

NET NODe Parameters Summary, 76

NETBbs, 13, 33, 76

NETCon, 13, 34, 76

NETFbbstime, 13, 34, 77

NETMax, 77

NETNode, 13, 34, 71, 73, 74

NETOnly, 13, 34, 76

NETRead, 13, 34, 76

NETTalk, 13, 34, 76

Network NODe Quickstart, 71

NEWUser, 13, 34, 87

NInfo, 13, 34, 76

NNode, 13, 34

NOBEacons, 13, 34

NOBUlls, 14, 34

NODe, 1, 5, 6, 12, 13, 14, 30, 33, 34, 35, 45, 55, 57, 58, 69, 70,

71, 72, 73, 74, 75, 76, 138

NODe ADD, 72

NODe DELete, 72

NODe REPlace, 72

Nodes Broadcast, 69

NODETimeout, 14, 34

NPorts, 14, 34, 71, 73, 76

NRCall, 14, 35, 71, 75, 76

NRId, 14, 35, 76

NTtl, 14, 35, 76

NUMCHANS number, 5

OCInit, 14, 35, 76

OCMmin, 14, 35, 76

P command, 47

Packet size, 85

PAClen, 14, 35

PATh, 14, 35

Performance hint, 143

8-152

PF command, 47

PForward, 14, 35

PIng, 14, 35

PMessage, 14, 35

PORT number, 3

PORT statement, 4

POrts, 14, 35

PPersist, 14, 35

PQuality, 14, 35, 73, 76

PRInt, 14, 35

Problems getting MSYS going, 143

PROCESS processname, 8

PROcesses, 14, 36

PRompt, 62

PRTmsgs, 14, 36

PWindow, 14, 36

QEMM, 144

QUIT, 14, 36

R command, 47

RDir, 14, 36

Readbackup # Command, 36

Readbakup #, 14

Remote SYSOP, 83

REMove, 2, 14, 23, 34, 36, 76

REMove a DEStination NODe, 72

REName, 14, 36

Replaceable directory names, 124

Replaceable file names., 124

REPlaces, 14, 36

REQDIR, 14, 36

REQDIR server, 51

REQDIR.HLP, 51

REQFIL, 14, 36

REQFIL server, 51

REQMaxfil, 14, 36

REQQTH, 14, 37

REQQTH server, 52

Required subdirectories:, 1

RESET, 38

RESpons #, 14, 36

RETries, 14, 36

REVERSE FORWARDING, 60

Routing, 69

RS-232 connection to tncs running Net/Rom and TheN, 89

RUn, 14, 37

R*, 48

R+, 48

S command, 48

SCreensave #, 14, 37

Scripts, 57

SELECTIVE FORWARDING, 61

SEnd, 14, 37

8-153

Set DOS variable, 124

SET/NODE, 25, 80

SHow/, 14, 38

SHow/..., 80, 81

SH/CL, 44, 78, 79

SH/COnfig, 44, 78, 79

SH/Dx, 44, 78, 79

SH/Dx/#, 44, 78

SH/Users, 44, 78, 79

SH/WWV, 44, 78, 79

SLottime, 14, 38

SMTP, 14, 38, 88

SMTP FORWARDING, 62

SNow, 14, 38

SOrt, 14, 38

SPEED number, 3

STACK processname, 8

STACK requirement, 85

STACKS=0 in your config.sys file., 144

STatus, 14, 38

SUBPORT number, 3

SUspendtr, 14

SUspendtrace, 38

SYSOP NAME, 8

SYSOP QTH, 8

SYSOP ZIP zip code, 8

T command, 49

TABs, 14, 39

TACk, 14, 39, 76

Talk callsign, 44, 78, 79

TBdelay, 15, 39, 76

TCP/IP FORWARDING, 61

TCP/IP Time to live, 12

TElnet, 15, 39

Terminate the connection, 23

TESTING, 88

TFrames, 15, 39

The REPly command, 48

Things needed to get the CD ROM call lookup to wor, 52

Timed execution of commands, 84

TImezone, 15, 39

TNatime, 15, 39, 76

TOdo, 15, 39

TPorts, 15, 39

Trace25, 15, 39

TraceIP, 15, 39

TREtries, 15, 40, 76

TRUncate, 15, 40

TS-HHMMSS, 15, 40

TTimeout, 15, 40, 76

TXDelay, 15, 40

TXTail, 15, 40

TYpe, 15, 40

Type of a message (the "TR" field, 89

8-154

Type of monitor, 1

U command, 49

Upload command, 49

USER ACCESS, 134

USER BIT SETTINGS, 87

USER PASSWORDS:, 87

Users, 15

Users filename, 40

V command, 49

VErifysysop, 15, 40

VIdeo, 15, 40

W command, 49

WAtch, 15, 40

WBackgrnd, 15, 40

WColors, 15, 40

WForegrnd, 15, 40

WHO, 15, 41

WHY, 15, 41

Window, 15, 41

WINDOW DISPLAY, 8

WINDOW number, 7

WP SERVER, 88

WPAge, 15, 41

WPEvery, 15, 41

WQupdate, 15, 41, 76

WRecord #, 15, 41

WSize, 15, 41

W?, 15, 41

X command, 49

"Carbon Copy", 43

#msgs/use, 10, 17

(MUTIL.EXE Function 9), 1

* at the beginning, 50

*comment, 10, 17

*** HEADERS, 93

/Sysop command, 83

< filename, 50

<Esc>, 41

> filename and >> filename, 50

@BBS, 86

@home, 10, 17

\+\ filename, 19, 23, 88

8-155

�