ICOM Programmer

written in TURBO C 1.5

by John Lundy - N5MHZ

This program is my answer to an online radio control program for

my ICOM radio.  Although originally written for the 735 it has been extensively

modified to work with most ICOM rigs with computer interfaces.  For those

of us who just want to load-and-run, this program may be your answer.

There are 2 modes of operation for this program: stand alone and

memory resident.  Be sure your radio is hooked to the COM port.

Stand alone mode: at the DOS prompt type ICOM.  You then get a

menu and a display of your radio's status (i.e Frequency and Mode).

Memory resident mode: at the DOS prompt type ICOM R.  After an

identification is printed on the screen, you're back at the DOS prompt.

The ICOM programmer is now memory resident and can be called up by pressing

the CTRL and Right-Shift keys at the same time.  Otherwise, the function of

the two modes is the same.

Play around with the program.  It's really simple to use.  Select

from the memu items and see what they do.  The only place a little more info

might be helpful is in the Frequency option.  This option has the ability

to change the frequency by an offset.  When prompted for the frequency,

enter +/- and the offset in KHz.  In this way you can move around by a

few KHz without typing the whole frequency.

You might ask, "How does the program know what COM port my rig

is hooked to?"  Or, "What ICOM address to use?"  Well, it doesn't.

It assumes COM1 and the IC735.  To change this and some other options do

the following:  Enter the program as outlined above, Use the "Save List Disk"

option to create the ICOM.INI file, and then Edit this file changing the

parameters in the file to your preferences.  This ICOM.INI file will be

created in whatever directory ICOM.COM was loaded from by DOS.  So, if you

run ICOM.COM and the DOS PATH is used to find the executable, ICOM.INI will

be created in the same directory.

Some other special features that you might like to know about are:

1) 64 frequency/mode memories that are accessed sequentially in a round-robin

method, both forward (-->) and backward (<--).  These memories are saved

with the setup info by using the the "Save List Disk" option.  They are

automatically reloaded the next time the program is run or by using the

"Load List Disk" option.

2) frequency rounding, round the present frequency to the nearest 1KHz

boundary (.)

3) up and down frequency stepping in .1KHz, .5KHz, 1KHz and Scan Incr steps

4) up and down frequency scanning with selectable increments

5) 32 programmable band limits (see the included ICOM.INI file), next and

previous band selection, top and bottom commands for the present band

6) hitting Enter will reset the COM port to present desired values and

refresh the screen

Release revision information

1.0	Initial release

1.1	Found problem with interaction with some programs and the use

of keyboard interrupt trapping.  Had to change the method of

exiting and memory resident entry to Ctrl-RShift.  No longer trap

keyboard interrupts.

2.0	Added ability for user to specify radio address in initialization

file for virtually any ICOM radio.

Added user specifiable band limits loadable/saveable in the

initialization file (total of 32 limit sets)

Added more keypad frequency stepping capability

If you have any difficulties, please let me know about them.  I'm

always tinkering with programs and would appreciate useful feedback I can

incorporate into future programs.

vy 73, John