The K1EA CQ WW Logging Program - Version 4.18

TABLE OF CONTENTS

Introduction

The First Steps

Make a Copy

Print CT.DOC

Print CT.HLP

Country Lists

Boot and Set Time

Start the Program

Set the Band

Start Making Contacts!

Namimg Conventions

Hard Disk Users

Floppy Disk Users

Starting the Program

Exiting the Program

Editing Commands

Check Sheet Commands

Logging Commands

Meaning of Marks in the Band Country Column

WAE QTC

Packet Users

Sending CW

Keyboard Mode

General Information

Time

Summary

Country Lists

CQ WW QSO Points

Frozen Keyboard

Band Change Errors

The Real Time Logging File

Logs by Band

A Printed Dupe Sheet

Save and Quit

Send Copy of name.BIN to N2AA, K1EA

Color

Updates

The Future

After the Contest

Copy to Floppy

Run Breakdown

Run WriteSum

Run WriteMult

Run WriteLog

Hints and Kinks

YCCC Users

Revision History

This program is designed for real-time logging.  To enjoy it, a

quiet computer, one which does not interfere with reception at HF,

must be used in the shack.  Be sure that hardware problems are solved

first!

THE FIRST STEPS

Make a Copy.  To begin, label a new disk (using the DOS command

LABEL) CT, with the version number (example: CT4_15), and copy this

disk.  That way, you won't have to torture K1EA if you trash your

original.  Remember, he isn't going to take your telephone call to

fix anything during the contest.  Put the original disk aside, and

insert the new program disk.  Now you are ready to begin.

CT requires 512K of memory. Practically speaking, 640K is needed

for a good size log.

Print CT.DOC.  Using your DOS command, PRINT CT.DOC, or COPY

CT.DOC PRN, print a copy of this introductory document (CT.DOC) so

that you'll have a hard copy of the instructions.

Print CT.HLP.  Using your DOS command, PRINT CT.HLP, or COPY

CT.HLP PRN, print a copy of the help window. Now you'll have a hard

copy to insert in a page protector, which is particularly handy.

Country Lists.  The prefix and country information the program

needs is in the file CQWW.CTY, WAE.CTY or ARRL.CTY.  If countries or

zones have errors, fix them and inform K1EA, so the author can

correct his copy too. Some countries are not in the "standard"

continent for the zone; for example, OX is in North America while the

rest of zone 40 is in Europe. A country can be assigned to a

continent in the .CTY file by adding the two character abbreviation

for the continent as the LAST two characters on a country line (after

the semi-colon). For example:

Greenland:          40:  OX:    OX;     NA

Lebanon:            20:  OD:    OD;     AS

Accepted abbreviations are: NA, SA, EU, AS, AF, OC.

Boot and Set Time.  Now, begin.  From a cold boot, bring up

your computer DOS.  If you have a built in clock on a battery, the

program will prompt you.  Just remember the zone to which your clock

is set.  If you must enter the time each time you boot up, you may

enter either UTC, or one of the four continental U.S. time zones

(EST, CST, MST, or PST). For those four time zones, the program will

adjust for daylight time, so don't do anything fancy.  For those of

you who are really nervous at this point, don't worry.  The program

will display UTC in the middle of the screen once you get going.

You'll be able to check on yourself then.

Enter CT <CR>.  Enter CQ88CW (or other file name) <CR>. Fill in

the Information Sheet, moving from field to field with the TAB key.

Hit CTL ENTER when done.  If you've made a mistake on the Information

Sheet, ESC will permit you to abort the program and start again.

Alternatively, enter CT CQ88CW (or other file name) <CR>.  Note:

for a quick short cut, you may get into the program with a one line

entry.  Just be sure to put space between CT and NAME.  A piece of a

K1EA log is on the distribution disk as EXAMPLE.  Type CT EXAMPLE to

run it.

Set the Band.  What band are you on?  Using ALT F1 or ALT F2,

set the band.

Start making contacts!

NAMING CONVENTIONS

NAME (call it what you like--CQ88CW, AR87PH, etc.) is the base

name from which all output files are derived. For example, starting

the program with the base name CQCW88 results in CQCW88.BIN,

CQCW88.NOT, CQCW88.ALL.  WAE logs create a .QTC file.

HARD DISK USERS

A hard disk is definitely preffered for reasons of space and

speed.  This program can be run from a directory other than the

current one, just make sure the PATH environment variable can find

CT.  If you wish to clean off your hard disk after the contest (or

return the computer to work!), you'll need only copy	the .BIN file

to a floppy for storage.  All other files can be recreated from the

.BIN file.

FLOPPY DISK USERS

You can run this program from a floppy, but large logs may run

out of room.  The .BIN file takes about 40 bytes per QSO.  Look at

the program size, your disk size and your potential log size and

decide for yourself.  You are most likely to run out of room when

making a log per band (WRITELOG) as this causes lots of output.  You

might want to replace the program disk with a fresh floppy just

before typing WRITELOG.  Remember to put the program disk back in

before making any more contacts.

STARTING THE PROGRAM

The program is run by typing: CT NAME (example: CT AR87CW), where

NAME is a base file name of your choosing (most likely the name of

this particular contest).

An "Information" screen will come up.  Fill in the fields.  ENTER

or TAB to change fields.  Shift TAB will move you backwards through

the fields.  The UP and DWN keys move you around the pop up menus.

CTRL-ENTER (when NOT in a pop-up menu) exits the first screen and

starts the program.  One last ENTER, after the startup messages appear,

gets the real program going.

For the CQ WW Contest, type in callsigns and hit return, as 599

and the zone are automatically filled in.  If it is an ARRL DX Test

(contest type set to ARRL on the Information Sheet), use the TAB key

to get to the power field (TAB twice), fill in power as sent by the

DX station and hit return. A 599 will be automatically inserted in

the RST field. In an ARRL DX Test, tabbing to the power field will

cause the power to be filled in automatically, if the station has

been worked on another band, or is a dupe. Alternatively, to save a

keystroke, the SPACE BAR will toggle you from the callsign field over

to the power or zone field, skipping the RST field.

EXITING THE PROGRAM

You may exit the program by entering ALT-Q (hold down the ALT

key while typing Q), or typing QUIT.  This causes all log info to be

saved to disk before the program quits.  You can restart from this

point simply by starting the program again (CT <CR>).

EDITING COMMANDS

These are line editing commands that can be used instead of the

cursor keys.  If you have used the EMACS editor, they are familiar.

Up and down cursor (arrow) keys work as you might expect.

PgUp and PgDn work as you might expect. You can page back as far as

you want.

CTL A               Go to beginning of line

CTL E               Go to End of line

CTL D               Delete character under cursor

CTL W               Wipe (delete) everything inside field

CTL K               Kill (delete) to end of line

CTL F               Forward one character

CTL B               Backward one character

Tab                 Change to next field

Return              Log a call

CTL G               Go to log entry.  If a number is in the call field,

go to that QSO number, otherwise go to the end

of log.  For example, you wish to change the

DX station's power in the ARRL DX Test.  Type

G3FXB then F9 to find his QSO number. Now type

CTL-W to remove G3FXB from the current call field.

Type QSO number (example: 1209 from the CHECK CALL

window) and CTL-G. You may now edit the QSO.

CHECK SHEET COMMANDS

The following functions are used in lieu of a check sheet during

the test. Type a call or partial call then hit:

F8        Check Partial Call (against all calls worked)

Calls worked on current band will appear in bold.

For color monitors, bold = white.

F9        Check Call (for dupes)

F10       Check Country (for new country)

ALT M     Hit it once, and it enables the Multiplier Check Sheet.

Hit it a second time, and it scrolls through the

continents.

LOGGING COMMANDS

You may wish to fabricate a template to go over your function keys.

ALT F1    Band up

ALT F2    Band down

ALT F3    Announce DX into PacketCluster (TM)(if a TNC is connected)

ALT F4    Grab latest DX spot from PacketCluster (TM).  Note to TS-940

users: A TS-940 will QSY if connected.  Hit ALT F4 a second

time, and you are toggled back to your running frequency.

ALT F5    Note to TS-940 users:  After an ALT F4 use, ALT F5 will leave

you on the new band (for a new running frequency).

MEANING OF MARKS IN THE BAND COUNTRIES COLUMN

*  New Country (But Not a New Zone)

#  New Country and New Zone

+  First QSO with that Country on Any Band

^  Not a New Country, But a New Zone

WAE QTC

ALT S     Starts the QTC form.  Follow directions.

Enter call of station receiving the QTC.  Press ENTER.  ALT S

picks up the current call, or the last one worked (if the current

line is empty) to use in the QTC form.

PACKET USERS

CT can talk to a TNC via your computer's COM1 port. Configure

your TNC to:

4800 baud, 1 stop, no parity, 8 bits

The TNC should be set to ECHO characters.  Set the TNC field in the

Information Sheet to Y, to tell the program that a TNC is available.

ALT P     Brings up a monitor window to watch packet activity.

ALT T     Brings up the monitor window and puts the cursor in the

window so you can "type to the TNC."  A second  ALT T gets

you back out, as this is a toggle function.  Make sure that the TNC

is left in CONVERSE MODE so that your outgoing announcements get

sent to the PacketCluster (TM) (software by Pavillion Software--

AK1A).

ALT F3    Causes a DX spot to be sent to PacketCluster (TM).

The call announced (sent) is the call under the cursor or

the last call worked.  If a TS940 is both connected to COM2

and entered in the "RADIO" field of the Information Sheet,

the spot frequency is taken from the radio.  If not, the

user is prompted to type in a frequency to be sent with the

spot.  When announcing a split receive frequency (for example,

G3FXB transmiting 7091.3 and listening 7213.0) separate

the TX and RX frequencies with a slant bar (/), i.e. 7091.3/7213.0.

[Note that when a DX spot comes in, the program checks the log

to see if that band country is needed. If so, it rings

the computer's bell and puts a message on line 25 of the screen.]

ALT F4    Pulls the call of the latest DX spot into the call field so you

don't have to type it in.  If a TS-940 is configured, you can send

the radio to that frequency by hitting ALT F4.  Hitting ALT F4 again

puts the radio back where it started, as this is a toggle function.

ALT F5    Note to TS-940 users:  After an ALT F4 use, ALT F5 will leave

you on the new band (for a new running frequency).

ALT A     Brings up an abbreviated list of DX spots recently heard.

SENDING CW

Morse code is sent by keying the DTR line of COM1, COM2, or the

STATUS line of LPT1 or LPT2.

The circuit needed to key positive keyed radios from COM1 or

COM2 is a 1K resistor from DTR (pin 20 on a DB25, pin X on a DB9) to

the base of an NPN small signal transistor (for example, a 2N4400, a

2N3904, etc.).  The emitter of the transistor and the ground lead of

the transmitter keying cable are connected to SIGNAL GROUND (pin 7 on

a DB25, pin X on a DB9).  The collector of the transistor is connected

to the transmitter keying cable (hot, not ground side).

The circuit needed to key positive keyed radios from LPT1 or

LPT2 is a 1K resistor from SLCT (pin 17 on a DB25) to the base of an

NPN small signal transistor (for example, a 2N4400, a 2N3904, etc.).

The emitter of the transistor is connected to STROBE (pin 1 on a

DB25).  The ground lead of the transmitter keying cable is connected

to SIGNAL GROUND (pin 18 on a DB25).  The collector of the transistor

is connected to the transmitter keying cable (hot, not ground side).

For technical reasons, the minimum keying speed is 22 WPM.  This

will be fixed in a later release.  The maximum keying speed is 50

WPM, because I said so.  The computer's speaker is enabled by typing

SOUND in the call field and hitting return. (This is like trying to

log the call SOUND).  The speaker is disabled by typing NOSOUND in the

call field and hitting return.

The following keys act much like a memory keyer:

INS       (The Insert key) Send his call followed by exchange.

F1        Send: CQ

F2        Send: exchange

F3        Send: QSL de CALL    (Your CALL taken from information sheet.)

F4        Send: CALL           (Your CALL taken from information sheet.)

F5        Send: his call

F6        Send: QSO B4 de CALL

F7        Send: QRL QSY de CALL

ALT F7    Send: QRL? de CALL

ALT F9    CW sending speed down by 2 WPM.

ALT F10   CW sending speed up by 2 WPM.

SHIFT F1  Change CQ message.   (You will be prompted.)

SHIFT F2  Change exchange message.

SHIFT F3  Change QSL message.

SHIFT F4  Change your call message.  (Does not change the Information Sheet.)

ALT K     Toggle into and out of Keyboard mode.

ALT V     Change CW speed by taking the number typed in the callsign field.

CTL C     Cut off sending currently in progress.

Keyboard Mode.  If there is no contest going on, or if you merely

wish to engage in idle chatter during a contest ("Hr 900 QSO's.  Hw u

doin?"), ALT K will toggle you into keyboard mode.  This is

particularly handy for those of you who never could handle a paddle.

To leave keyboard mode, type ALT K again.

GENERAL INFORMATION

You may wish to print the file CT.HLP for quick reference.

ALT H     Think of it as ALT HELP.  It brings up an abbreviated command set

window. Hit any key to get back to the main program.

You may wish to do a Print Screen (SHIFT PrtSc) of this window.

ALT M     Think of it as ALT MULT.  Hit it once, and it enables the

Multiplier Check Sheet.  Hit it a second time, and it scrolls

through the continents.

ALT N     Think of it as ALT NOTE.  It prompts you for a brief note which is

sent to a file called NAME.NOT, along with the time and callsign then

under the cursor. For example, you are astounded when you work P40GD

and write this NOTE:

"Sending 48 WPM ... and I can copy him!"

or: "Power failed at 1415z. I'll never catch W1RM now."

QUIT      Save to disk and exit to DOS.

WRITELOG  Generates the logs that contest organizers require.

BREAKDOWN Generates a rate sheet. showing QSO's and multipliers by

hour (NAME.BRK).

DUPESHEET Generates Dupe Sheets by band, in compressed mode (NAME.DUP).

SOUND     Enables computer's speaker when sending CW.

NOSOUND   Disables computer's speaker when sending CW.

WRITESUM  Generates a summary sheet (NAME.SUM).

WRITECLIST Generates a log by country (just because it is interesting)

(NAME.CTY).

Time.  As currently written, this program does not permit you to

enter times after the contest.  However, you could still use this

program to get dupe, count multipliers and QSO points, and multiply

out your score.

Summary.  The numbers under the Q's, Z's and Multipliers display

are:  QSO points, total multipliers and score. The scoring system is

based on the contest type found in the CONTEST field of the

Information Sheet.

Country Lists. For the ARRL contests, this program uses the file

ARRL.CTY.  For CQ WW Contests, this program uses the file CQWW.CTY.

For DARC WAE, this program uses the file WAE.CTY.

CQ WW QSO Points.  When using CT for CQ WW, the information sheet

CALL field is used to compute your country and continent. All points

are computed based on this information. For example, NP4A is counted

as 2 points if your call is K1EA, but is 3 points for G3FXB.

Similarly, ON4UN is 1 point for G3FXB, but 3 points for K1EA.

Frozen Keyboard.  If you manage to freeze the keyboard, do not

panic.  Hit CTL-ALT-DEL (or push your "RESET" button).  Go through

DOS (setting the time to EST again, if necessary) and start again

with CT NAME <CR>.  You're back in business.

Band Change Errors.  Should you forget to change bands (in the

logging program), after changing bands with your radios, there is no

harm done.  Simply go up with your cursor and change bands with

either ALT F1 or ALT F2.

The Real Time Logging File.  This logging program creates, and

restarts from, a file called NAME.BIN.  This is a binary file and

cannot be edited with a text editor.  All changes to the log should

be done from within CT.

Logs by Band.  Dumping the full log to disk (WRITELOG), makes six

files:  NAME.10, NAME.15, NAME.20, NAME.40, NAME.80, and NAME.160.

These are the separate logs CQ Magazine requires.  Do not type

WRITELOG until after the contest.  The ARRL DX test requires a single

log for all bands.  WRITELOG provides this in the form of NAME.ALL.

A Printed Dupe Sheet.  If a sorted list of calls on each band is

needed, use DUPESHEET after the contest.  It may take a few minutes

with 5000 or more calls.  However, few of us will have to wait that

long.  Except in Multi Single operation, for the multiplier station,

do not hit ALT F5 until after the contest.  The CT program will

produce a dupe sheet (which is a file called NAME.DUP) for the

individual bands 10 through 160.  The NAME.DUP file has control

characters (special instructions) embedded in it to cause dot matrix

printers to go into "compressed mode".  If your printer doesn't

support this, you will get lines that wrap around and are hard to

read.

Save and Quit.  The program is exited by typing QUIT.  This saves

the log to disk and returns you to DOS.

Send Copy of .BIN to K1EA.  He is creating a database of "known

good calls."  A future version of this program will access using

Check Partial (F8).  N2AA and N6AA are also creating large databases

in an attempt to understand the logging error rates we contesters

make.  N2AA and N6AA would also like copies of NAME.BIN.

Color.  Plain vanilla mono screens and color screens should work

just fine with no operator intervention. The strange and murky world

of LCD displays, black and white emulating color, ATT and other

combinations can cause unexpected results. There are two DOS commands

which set up the display hardware: MODE BW80, and MODE CO80. Try one

of those commands before running the program if the screen doesn't

look reasonable with the default. You can ask the program to try to

force the use of the mono display or, failing that, color display in

black and white by using the switch -m in the command line which

starts CT.  For example: CT -m EXAMPLE <CR>   starts CT in mono mode.

If you start a color setup with the -m switch on, you will have to

run MODE CO80 after ending the program to get your color back.

Updates.  K1EA has donated CT to the Yankee Clipper Contest Club

(YCCC). K1VR has taken over the documentation chores.  K1GQ will

publish the manual.  KC1EO will maintain the user registration list,

and take care of distribution.  K1EA continues to support the code.

The registration fee is being raised to $25. This will get

you a major revision and serious bug fixes for that revision.  It also

funds updates and better documentation. In the past, the fee was $8 for

the latest revision, regardless of how often you got an update.

If you use the program, and wish to receive updates, send $25 to:

YCCC

c/o Bill McGowan KC1EO

33 Truell Road

Hollis, NH  03049

U.S.A

The Future.  Major features planned for Release 5 and 6 are:

* Support for extended memory, and thus 14,000 QSO's per extra megabyte.

* Support for multi-single and multi-multi operations. This will

require multiple computers and com hardware.

* Support for the DRSI packet controller, which is an IBM compatible

plug-in board. This frees a com port for use in slaving computers,

together in the multi categories.

* Support for ICOM radios.

* Off-line, after the contest, time editing (for those of you who didn't

bring a computer to Aruba).

* On-Line "uniques" and "uniques plus one" checking (with extra memory).

This feature is similar to "Check Partial", but it will check a partial

call against a data base of calls that have been worked in this or

prior DX tests. If you are registered, and send K1EA a copy of your

.BIN file, it will be used to help create the master data base. You

will then be entitled to the data base for use during the next test,

and recognition as a certified good guy.

AFTER THE CONTEST

Exit the Program.  Type QUIT <CR> in the call field.

Copy to Floppy. Immediately, before you are tempted to screw up,

copy name.BIN to a:name.BIN (a floppy disk which you should immediately

label and store safely).

Run BreakDown. Enter the program by typing CT name <CR>. Go

through the Information Sheet, using CTL ENTER. Make a breakdown

sheet by typing BREAKDOWN <CR> in the call field.  This writes out a

DOS file called name.BRK which contains breakdowns by QSO count and

by rate in each hour and on each band.

Run WriteSum. Make a summary sheet by typing WRITESUM <CR> in the

call field.  This writes out a DOS file called name.SUM which contains

a summary for the contest committee.

Run WriteMult. Make a multiplier check sheet by typing WRITEMULT <CR>

in the call field.  This writes out a DOS file called name.MUL which

should be printed using the DOS command PRINT name.MUL. This conatins

a multiplier checkoff sheet in compressed form yielding over 300 countries

and 40 zones on one page.

Run WriteLog. Make logs for the contest committee by typing

WRITELOG <CR> in the call field. This writes out a DOS file per band

called name.160, name.80, etc.

HINTS AND KINKS

Check Partial (F8) is the only way to go "search and pounce".

When you hear a call, automatically type 2 or more characters and hit

F7. Unless you typed a commom prefix like G3 or DJ, you will rarely

get more than a few calls to choose from.  Calls which have been

worked on the current band are highlighted.  You will soon find that

you are automatically hitting 2 characters plus F8 as you tune the

band, just as you automatically used to glance at the checksheet.  I

see no time difference between this and the paper method.  When you

only hear the suffix, Check Partial has a big advantage over paper

dupe sheets.  You won't have to mentally associate a prefix with the

suffix, then find it on the paper.

Try to get the habit of looking at the Check Country display

that comes up after each QSO.  This forces you to know what you need

on what band.  For example, after working a dozen GW's on 20 and 15,

you "just know" you need a GW on 40, because it's been displayed at

you so often.

For those idiots who answer your CQ with only the last two letters

of their call, enter those two letters and CTL-A to place the cursor

at the begining of the callfiled.

PACKETCLUSTER (TM) USERS

This program should make DX spotting easier for single ops who

wish to contribute multipliers without taking any.  Tell your

PacketCluster (TM)  SysOp to exclude you from getting DX spots.

Naturally, multi single and single op plus packet types can use the

incoming spots too.

REVISION HISTORY

Rev 3.14

*  F7 (Check Partial) now sorts partial list and highlights calls

worked on the current band. - Thanks K5ZD.

*  Two bugs which crash the program: F8 with no TNC, and WA8ABC/QRP

entered as a call.  Both fixed.

*  Program now properly senses mono and color video adapters. If both

are installed, mono is used.  If color only is installed, color monitor

is used, but written black and white. A future rev will allow user to

specify b/w or real color.

Rev 3.15

*  ALT M, for Multiplier Check Sheet added.

*  Lingering DUPE information now wipes with CTL W in call field.

Rev 3.16

*  Check Country now displays the name of the country in text at the

bottom of the window.  This way the operator knows exactly what

country the program has interpreted the prefix to be. - Thanks K5ZD.

Rev 3.17

*  Check Partial accepts a wildcard (*) character.  G3FXB can be

found by checking G*F or G3F*B. - Thanks K1KA.

*  Specify B/W or COLOR in NAME file.

Rev 4.07 - This a major revision.

*  The "hesitation" problem is fixed. Greenleaf confessed and has a new

windows package.

*  .RES is no longer used, all restart data is kept in .BIN, a binary

file.  Calls, RST, BAND and INFO can all be changed at any point in

the log. It is no longer necessary to edit a text file.

*   The startup text file is no longer used, instead a menu driven form is

filled in by the user.  This information is kept at the start of the .BIN

file.

*  WAE is supported, in all it's complexity.  Works for non-EU stations,

mostly works for EU stations.  A separate .QTC file is kept.

*  WPX is supported, still a little buggy I'm afraid.

*  The program now sends morse code. External hardware (a transistor)

required.

*  CQWW zones are correct except for W4's. UA9, UA0 find the correct

zone based on suffix. - Thanks YU3EA.

*  Two extra tools are provided: R2B and B2R. R2B converts an

existing .RES file to a .BIN format, and B2R converts a .BIN to a

.RES. If you simply can't control yourself and have to use an editor

on your files (like to rubber clock the time column), run R2B, edit

to your hearts content, then B2R it back.

Rev 4.08 - Minor bug fixes.

*   F6 now assigned to sending "QRU" in WAE, ANNOUNCE DX reassigned to

ALT F6.

*   Serial number now takes 4 digits.

*   When editing non-Eu calls in WAE, a trap was hit and program stopped

(ASSERT statement).  Now fixed.

*   IN QTC screen '-' didn't work right, and F10, 0 (for 10) didn't work,

Now fixed.

*   In CW mode, the Information Sheet allows a choice of COM1 or COM2 for

sending.

Rev 4.09 - Many bug fixes.

*   Times put back in time field!

*   Program crash after: modify call, F5, enter. This is fixed.

*   Times in the output logs (from WRITELOG) now read properly.

*   The TS940 support had been inadvertantly removed.  Now working.

*   Flow control for the packet port had been disabled causing some machines

to lose characters.

Rev 4.10 - More bug fixes.

*   When editing old calls, time on/off got screwed up.  Fixed.

*   ALT S now picks up the most recent call for QTC form.

Rev 4.14 - Substantive changes.

*   The use of color has changed: in Check Partial, white means needed,

black means worked on this band.

*   The startup screen accepts UTC, the 4 continental USA timezones or

any arbitrary offset from UTC.  When in USA timezones and daylight

savings in effect, offset to UTC is corrected automatically.

*   Real time clock added for convenience.

*   In CQWW, spots from packet which are not new band countries, but are new

zone countries (VK6, UA0Y, VO2 etc), are sent to the screen.

*   Flow control with packet is now fixed, no more dropped characters.

*   F2 and F3 are now swapped: F2 sends report, F3 sends QSL QRZ de CALL.

*   At K5ZD's suggestion, several commands are now "text commands"; i.e.,

type in a command in the call field and hit return.  Examples are:

QUIT, WRITELOG, BREAKDOWN, SOUND, NOSOUND, WRITECLIST.

*   F4 Send His Call.

*   F5 Send My CALL.

*   F6 Send QSO B4

*   F7 Send QRL QSY de CALL, in WAE send QRU (no QTC).

*   F8 Check Partial

*   ALT F3 Announce DX (when TNC connected)

*   ALT F4 Get Packet Spot and QSY TS-940 if connected. Second time:

will QSY TS-940 back.

*   ALT F5 Make current frequency the running frequency.

*   ALT F7 Send: QRL?

*   ALT F9 and F10 now change CW speed

*   CW sending keys work while in the multiplier or help windows.

*   Scan through the multiplier window with repetitive ALT-M.  Other keys

switch back to the main window.

*   QTC field now takes 4 digits in EU serial number - Thanks K1VR

Rev 4.17

*   Check Partial fixed (again).

*   CTL-BS does what CTL-W does, wipe out an entry.

*   Fixed the "changed bands, but it didn't take" bug.

Rev 4.18

*   New feature: WRITEMULT produces a multiplier checksheet file name.MUL

PRINTMULT sends a multiplier checksheet to the printer.

Tnx W6GO.

*