DAYTIME

Copyright (c) 1988

By :Richard Tashner N2EO

163-34 21 Road

Whitestone New York 11357

Daytime.asm is written in Intel 8088 assembly language

and is intended to be used with the IBM PC/XT/AT and most

compatibles.

The ARC file you just downloaded contains three files.

1 - DAYTIME.CMD        ( The executable program )

2 - DAYTIME.ASM        ( The source code )

3 - DAYTIME.DOC        ( This file )

This program has been tested with the A.E.A. PK-232 and

should work with most if not all TAPR clones as well.

DAYTIME.COM will set the real time clock contained in the TNC

to the proper date and time and display the values on the

screen.

This is useful as each time the TNC is turned off at the

end of a packet session the data in the TNC,s real time clock

is lost.

You can load DAYTIME.COM and a program such as YAPP.COM

from a batch file. In this way you automatically set the

TNC.s clock each time you enter your packet communication

program.

Just be sure and load DAYTIME.COM first then your

communications program (such as YAPP) second.

Sample Batch file

-----------------

The following example batch file shows how to configure

DAYTIME.COM to automatically load with YAPP.COM... Please note everything

shown within the < > are the commands used to write the batch file and are not

actually entered into the batch file... Do not type in the < >!!

<COPY CON YAPP.BAT>  <ENTER>

ECHO OFF

CLS

DAYTIME.COM

YAPP.COM

CLS

DIR

<CTRL Z>

The above example should be typed in from the DOS prompt and the

batch file should be located on the same disk that contains the DAYTIME.COM

and YAPP.COM files (or what ever communications program you are using...)

Using the above example, when you type 'YAPP', you will see the

DAYTIME.COM screen come up and the TNC,s clock will now be set. Pressing any

key will cause YAPP.COM to come up and load. As an added bonus, when you leave

the Yapp program, the screen will clear and a disk directory will appear. This

is due to the last two lines of the batch file.

If your using some other communications program other than YAPP

simply substitute the file name of that program in place of YAPP.COM...

How the program works

---------------------

DAYTIME.COM uses DOS interrupts to check the real time

clock in the IBM computer. These interrupts return the

year,month,day and time in various registers of the

8088/80286 microprocessor. The values returned in these

registers are in binary. The program then converts these

binary values to "Unpacked B.C.D." and then finally converts

these numbers to their ASCII equivalents. Once converted to

their ASCII equivalents, these numbers are stored in a buffer

inside the computer for future use.

The program then issues the TAPR command 'DAYTIME'

followed by a space (20 HEX) to the TNC via the serial port.

Next the program spits out the data in the afore-mentioned

buffer. As an example the data stream might look like this:

DAYTIME 8808292310

In this example we would have just set the real time

clock in the TNC to a date of August 29,1988 and the time to

2310.

The program also displays the time and date on the

screen just to let you know that it has in fact done its job!

Finally the program sounds a bell and waits for keyboard

input. Once the program senses keyboard input the program

terminates and returns to DOS. If the program was executed

from inside a batch file then the next command in the batch

file will be executed once the program has terminated. (I.E.

next line could be used to tell the computer to load a

communications program such as YAPP.COM.

To take advantage of the real time clock feature you

must have the commands CONSTAMP,MSTAMP and or DAYSTAMP

turned on in your TNC. This will allow time and date stamping

of connected and or unconnected packets.

I suggest you read the section in your TNC,s manual that

deals with the commands DAYTIME,CONSTAMP,MSTAMP and DAYSTAMP.

Changing parameters

-------------------

The program as supplied is set up to communicate with

the TNC at 1200 Baud,8 Bit words,No Parity,1 Stop bit

which is the most popular combination The program is also

set up to communicate via comm port number 1.

There is no provision to change these parameters from

within the program. However I have included the source code

DAYTIME.ASM. You can change the following equates in the

source code and re-assemble the program using the microsoft

macro assembler. The following information shows how to

change the program to run on comm port 2 and also shows you

how to set the program to other baud rates.

At the top of the source code listing you will see the

following equates:

BAUDRATE     EQU     083H       ;BAUD RATE SELECT

COMMPORT     EQU     0          ;COMM PORT SELECT

ATTRIB       EQU     07H        ;VIDIO COLOR ATTRIBUTE

To change to comm port 2 change the '0' to a '1' and re-

assemble the source code..

To change the baud rate to other than 1200 baud, 8 bit words,

1 stop Bit No parity see the following chart..

300 Baud 8 Bit Words 1 Stop Bit No parity =  043H

1200 Baud 8 Bit Words 1 Stop Bit No parity =  083H

2400 Baud 8 Bit Words 1 Stop Bit No parity =  0A3H

4800 Baud 8 Bit Words 1 Stop Bit No parity =  0C3H

9600 Baud 8 Bit Words 1 Stop Bit No parity =  0E3H

Make the appropriate changes to the BAUDRATE EQU and then

re-assemble the code.

I have set the ATTRIB EQU to a "normal attribute" 07H.

I did this so as to insure compatibility with a monochrome

monitor.

If you have a color monitor, you can change this equate

to give the program some interesting colors.

Below is a short list of colors that I have found

pleasing to the eye..

00EH = Black background / Yellow foreground

00FH = Black background / Bright white foreground

00BH = Black background / Light cyan foreground

01FH = Blue background / Bright white foreground

Any comments/suggestions please feel free to write me.

Enjoy...

Richard Tashner N2EO

163-34 21 Road

Whitestone N.Y. 11357