KAMterm:  Copyright 1992 by Jim Graham, N5IAL (All Rights Reserved)

PREFACE

As part of their more recent firmware in their TNCs, Kantronics offers

a special mode of operation known as Host Mode.  Host Mode differs from

the ``normal'' operating mode in many respects, and offers a great

deal of additional capabilities when using Host Mode terminal software.

This section is a general introduction to what Host Mode is and how it

operates, what it buys you, and some special considerations for using

Host Mode as opposed to the normal command mode.

What Is Host Mode?

Before going into any detail on the benefits of using Host Mode, it

is important to first have a general idea of what Host Mode actually

is, and how it operates, as opposed to the normal command mode you are

probably more accustomed to using.  It is not necessary that you fully

understand the details behind this, but you should have somewhat of a

feel for what's going on behind the scenes.  The detailed description of

the data format is in the Kantronics documentation, and only a general

summary will be listed here.

In normal command mode on TNCs, the text you type is transmitted

directly to the TNC exactly as you typed it.  This does not require any

special software on your computer other than a simple dumb terminal

emulator.  In fact, in normal command mode, you can attach a plain

terminal (as opposed to a computer running terminal emulation software)

to the TNC.

In Host Mode, however, the text you type is not transferred to the TNC

directly.  It is first enclosed in blocks of data, which include a

header indicating the type of data being transmitted (commands, data to

be transmitted, etc.) and the port and stream to which the data is

addressed, and a flag marking the end of the data block.

WHEN THE TNC IS IN HOST MODE, A SIMPLE TERMINAL EMULATOR WILL NOT

BE ABLE TO COMMUNICATE WITH THE TNC DIRECTLY, AS IT WILL NOT PUT THE

DATA IN THE REQUIRED DATA BLOCKS.  Likewise, when the TNC is in normal

command mode, software attempting to run Host Mode communications to

the TNC will also fail to function.

The data format used within Host Mode is actually very simple when

compared to that used in other data communications protocols (such as

X.25, AX.25, TCP/IP, etc.).  The blocks of data are delimited by a

flag, called a FEND (C0H, or decimal 192).  In most types of data

blocks, the first FEND is followed by 3 characters, indicating the

type of block, the port, and the stream, respectively.  This is

followed by the actual data, and finally another FEND which signals

the end of the block.

What Does Host Mode Buy You?

First, you can (assuming you have the KAM, as opposed to the KPC, etc.)

operate in a non-packet mode on HF, while operating packet on VHF.

(Without Host Mode, attempting to operate non-packet modes on HF while

connected on VHF packet will get you a ``NOT WHILE CONNECTED'' response

from the KAM.)

In addition, regardless of the type of Kantronics TNC, the special block

format of Host Mode allows you to do many things in Host Mode that can

not be done in normal command mode.

For example, in normal command mode, all data, regardless of the

port/stream it is from, is simply dumped to your screen, and each time

you wish to change streams, you must issue a command to the TNC to

change it to the new stream.

In Host Mode, however, the data block identifies the stream from which

incoming data is arriving, which provides for the design of a windowed

environment, such as KAMterm.  In such an environment, the data for

each stream can be given its own window, thus keeping things cleaner on

the screen.  In KAMterm, you need not keep telling the TNC which stream

you wish to work with --- the Host Mode data format takes care of this

for you.  Outgoing data is automatically set to the port/stream

associated with the window you are currently viewing.

A related feature is the ability to have an entirely separate screen

for command entry (same as you would have with the TNC at the cmd:

prompt) and for monitor output, once again keeping the screen much more

organized.

All of this boils down to being able to have multiple connections open,

with a different screen for everything.  To change from one conversation

to another, you simply move to the next screen and the software takes

care of the rest for you automatically.

There are other features derived from this in KAMterm, such as the

ability to keep an eye on one stream while in another stream's window,

the AMTOR XMITECHO window (which displays the text as it is transmitted

without interfering with other operations).  These and other features

will be described in more detail later.

Special Considerations When Using Host Mode

In general, Host Mode can make things much simpler for you, but there

are a few things you need to keep in mind, or you may find yourself in

some confusing situations.

First, and most important, if the TNC is set to use normal command mode,

and the software is trying to use Host Mode (or vice-versa), neither one

will recognize the other.  While you are in KAMterm, if you change

between Host Mode and normal command mode, KAMterm will take care of

changing the TNC's setting for you.  If you also run other terminal

software, however, it is important to keep this in mind when configuring

and using KAMterm.

In general, if you expect to use any other terminal program, possibly

including other Host Mode software, you should configure KAMterm *NOT*

to automatically start and exit in Host Mode.  This is, therefore,

the default configuration.

For example, if you exit from KAMterm and leave the TNC in Host Mode,

and then try to use a plain terminal program, it simply will not work,

because the TNC is expecting Host Mode data blocks, and the terminal

program is not providing them.  To prevent this situation, do not leave

the TNC in Host Mode if you expect to use a regular terminal program.

Similarly, if you have instructed KAMterm to immediately start off in

Host Mode (see the section on the configuration file for details), and

the TNC is in normal command mode, KAMterm will be presenting Host

Mode data to the TNC, and the TNC will not recognize the data format.

You can re-establish communications with the TNC by telling KAMterm to

return to normal mode.

NOTE:  It has come to my attention that the HostMaster software from

Kantronics automatically returns the TNC to normal command mode when

it exits, so if you use KAMterm after using HostMaster, do not

assume that the TNC is in Host Mode --- it probably isn't.

A second thing to keep in mind is the fact that the command screen is

always associated with some port/stream combination.  If you are

connected on streams A, B, and C on the VHF port, and you wish to

disconnect stream C (using the DISCONNECT command), you would return to

the command screen and enter that command.  You must, however, make sure

that the command screen is pointing to VHF Stream C, or you might end

up disconnecting the wrong stream!  The status bar will always indicate

the port/stream associated with each window.

A third thing to keep in mind is the fact that when you make a

connection from the command screen, you will see a status message

informing you of the connect, but from that point on, all data from

that connection will be displayed on a different screen.  This differs

from what you would expect when using normal command mode, where the

data from the connect is dumped straight to the same screen (the only

screen).

1.0)  INTRODUCTION

KAMterm is the result of my not being able to find a terminal program

that take advantage of the KAM's host mode functionality and maintain a

friendly user interface, without sinking large amounts of cash to get

it.  So, after getting tired of being told (by the KAM) that I can't do

this or that while connected, I decided it was time to do something

about it.  So here it is!

KAMterm is my answer to this problem.  I've included a lot of the standard

bits that you see in most terminal programs (i.e., split-screen, scrollback

buffers, logging to a file, etc.).  I've also added a lot of things that,

in my opinion, have been seriously lacking in everything I've tried for

packet terminal programs.

KAMterm is designed around several basic concepts:

*) EACH ACTIVE STREAM HAS ITS OWN WINDOW.

*) THERE IS A SEPARATE SCREEN FOR COMMAND MODE AND FOR MONITOR MODE.

*) You can switch between screens with a single keystroke.

*) There is a ``priority window'' that you can use to display any

incoming data on a given stream while working with another stream.

*) KAMterm is designed to be extremely simple to operate.

*) KAMterm WILL NOT MODIFY YOUR TNC's PARAMETERS UNLESS YOU ASK IT TO.

*) IF YOU CHOOSE TO DO SO, you have the ability to create startup and/or

exit command files that will configure the KAM for you.

*) When logging to a file, KAMterm will try to keep things organized by

adding a line above any text it logs indicating who said what.

*) In some menus, if you have a mouse, the mouse can be used for those

menus.

*) Normally, host mode does not have some of the things normal command

mode does --- these are ``faked'' in KAMterm, and the appropriate

action is taken.  These include:

*) fake command prompt (hcmd:)

*) |n to switch to VHF stream n for command window i/o commands

*) ~n --- same idea, except for HF

*) a simple [CR] will result in new hcmd prompt (for aesthetics only)

*) Scrollback buffers are setup for each window if memory permits. Size

is adjustable via configuration file (kamterm.cfg).

*) KAMterm will notify you of incoming connects no matter where you

happen to be at the time.

*) Programmable function keys --- [F1] is reserved for HELP, but

otherwise, any of the function keys ([F1] through [F10]) can be used

alone or in combination with [SHIFT], [ALT], and/or [CTRL] for your

own macros, etc.  The limitation here is that the string is limited

to 80 chars.

*) Macro command files --- just as you can have setup files for start

and exit, you can send macro command files to the KAM at any time

while running KAMterm.

*) ``Brag'' files are supported, too.  You will be asked for the name of

the file you wish to transmit.

2.0)  DISCLAIMER

KAMterm is provided AS IS with no warranty, expressed, written, or

implied.  While every effort has been made to keep KAMterm free of bugs,

individual systems do vary, and it is beyond the author's ability to

test KAMterm in all possible system configurations.  Under no

circumstances shall KAMterm's author be liable for incidental or

consequential damages that may be incurred by the performance or use of

this product.

3.0)  DISTRIBUTION

You are free to distribute UNMODIFIED copies of KAMterm, providing

that all documentation, sample files, and other related files are

also distributed unmodified.  This applies ONLY to the shareware

distribution --- registered versions of KAMterm, as well as the

typeset documentation, are not to be distributed, period.

KAMterm may, if needed, be re-packaged by another archiving program

(e.g., ZIP, ZOO, ARJ, etc.) for distribution.  This represents the only

acceptable ``modification'' for its distribution.

Under *NO* circumstances is the Copyright notice within the program

or its documentation to be altered in any way or removed.

4.0)  NS16550AN/AFN  SUPPORT

KAMterm provides full support for the NS16550AN and NS16550AFN UARTs.

These UARTs (made by National Semiconductor), provide a much greater

degree of reliability at higher speeds (the definition of which depends

on the speed of the computer, and ranges from 4800 bps in severe cases

to 38,400 bps or higher on faster computers).

The 16550 does this in two ways.  First, it provides a 16 character FIFO

buffer, as opposed to the 1 character FIFO in older UARTs such as the

8250.  This provides some breathing room for your program. The second

thing the 16550 does is to allow a single interrupt for multiple

characters.  In other words, with only one interrupt, a communications

program can read several characters from the port at a time.  This

substantially reduces the interrupt loading on the computer, and thus

improves performance of serial I/O.

KAMterm automatically detects either the NS16550AN or NS16550AFN if it

exists, and takes advantage of the FIFO and the modified interrupt

structure described above.

Note, however, that while KAMterm does support speeds up to (and

including) 115,200 bps, the KAM currently only supports speeds up to

9600 bps.

If you do not have 16550 UART (you would know if you did --- they are

more expensive, and are not standard stock in most serial cards), and

particularly if you have a slower computer, you may run into problems

running at 9600 bps.  If you're seeing dropped characters (normally only

a problem when there is a continuous stream of incoming data), try

dropping the speed down to, say, 4800 bps or lower.  Remember to change

the speed on the KAM (this requires a reset) before changing KAMterm.

5.0)  KAMterm  OPERATIONS

This section will describe some operational aspects of KAMterm.  These

include startup options, screen layout, and what happens internally when

new connects become active in Host Mode.

One thing worth mentioning, even though this is not a function of

KAMterm, is the KAM's handling of the LEDs in Host Mode.

In version 4.x of the firmware, some LEDs function differently in Host

Mode than they do in normal command mode.  The CON LED on the VHF side

indicates that data is being transmitted from the KAM to the computer,

and the STA LED on the VHF side indicates that data is being received

from the computer.  The normal command mode functions of these LEDs does

not apply in Host Mode.  This includes the usage of the STA LED to

indicate the status of the PBBS.

It is my understanding that in version 5 of the firmware, PBBS status,

and possibly even connect status and outstanding packet status are

available from the LEDs even under Host Mode.  More information on this,

and if needed, updates to KAMterm, will be added once I get the new

firmware.

5.1)  STARTUP  OPTIONS

There is only one startup option:  -h   (start immediately in Host Mode).

This assumes that the KAM is already in Host Mode, and does not send the

commands to the TNC to initialize Host Mode.  This overrides the

STARTHOST parameter in the kamterm.cfg configuration file (described

later), and starts in Host Mode regardless of its setting.  This is

primarily intended for times where you have left the KAM in Host Mode,

but normally do not do so.

5.2)  SCREEN  LAYOUT

The physical screen is divided into 2 regions.  The first is the main

output window.  This takes the majority of the screen.  The second is

the local window, which is the first place where the text you type will

be echoed. The format shouldn't be new to anyone who has used popular

terminal programs for packet radio.  The status bar in between these

which indicates the stream, who (if anyone) that stream is connected to,

etc.

5.2.1)  THE  SCREEN  ITSELF

When you first start KAMterm (assuming you start in non-Host Mode), you

will see a screen which looks something like the following:

��������������������������������� TOP OF SCREEN �����������������������������ͻ

�                                                                             �

� This is the MAIN WINDOW.  All text coming FROM the KAM is placed in this    �

� window, as is the local echo (if it is turned on).                          �

�                                                                             �

� This window uses lines 0 through 19 on the screen.                          �

�                                                                             �

�                                                                             �

�                                                                             �

�                                                                             �

�                                                                             �

�                                                                             �

� The line below (it is only one line in the program) is line 20, which is    �

� the STATUS LINE.                                                            �

�������������������������������������������������������������������������������

F1=HELP COM1 9600 8N1             NORMAL MODE             09 May 1992  12:33:37

�������������������������������������������������������������������������������

�                                                                             �

� This is the LOCAL WINDOW.  Text you type goes here.  This window uses       �

� lines 21 through 24 on the screen.                                          �

�                                                                             �

������������������������������� BOTTOM OF SCREEN ����������������������������ͼ

So, with that picture in mind, let's take a look at what we've got.

First, the main window contains all text coming in from the TNC for the

stream assigned to the current window.  Because the KAM is not yet in

Host Mode, this means all data coming in from the TNC, period.

The last window on the screen is the local window.  This is where text

you type is first echoed.  In here, you can edit a line of text to send

to the KAM before actually sending the data.  If the line exceeds 78

characters, everything that has been typed to that point will be

transmitted to the KAM.  Word wrap is not currently implemented.

5.2.2)  THE  STATUS  BAR

The status window provides some information as to where you are, how

things are configured, etc.  Reading from left to right, we first see a

reminder that F1 can be used for HELP.  Next, we see that the serial

port is configured for COM1, 9600 bps, 8 data bits, No parity, and 1

stop bit.

Off to the right a bit in the status bar, we see an indication that we

are in normal mode, as opposed to Host Mode.  Note that this does not

indicate the KAM's configuration, i.e., the KAM may really be in Host

Mode.  If there is a mis-match between KAMterm and the KAM, things won't

work.

At the far right of the status bar we have the current date and time,

according to your computer's clock.

When you enter Host Mode, the status bar will be modified, and you will

see the following:

�������������������������������������������������������������������������������

F1=HELP COM1 9600 8N1          HOST CMD:  VHF/A           09 May 1992  12:33:37

�������������������������������������������������������������������������������

This is now telling us that we are in Host Mode.  Furthermore, this is

the HOST COMMAND WINDOW, which is where we issue actual commands to the

KAM (e.g., CONNECT FOO-BBS).  It then tells us that commands typed here

are currently affecting VHF stream A.  For many commands, this is not

important (e.g., DAYTIME, MYCALL, MONITOR, etc.), but for others (e.g.,

CONNECT, DISCONNECT, etc.), this does matter.  In addition, if you

intend to set the KAM's HF port to a non-packet mode (RTTY, CW, etc.),

the port needs to be HF, not VHF.  See the section on Host Mode commands

for more information about changing the current stream, etc.

From here, if you hit [PgDn], you will see the following:

�������������������������������������������������������������������������������

F1=HELP COM1 9600 8N1            HOST MONITOR             09 May 1992  12:33:37

�������������������������������������������������������������������������������

This window is the HOST MONITOR WINDOW, and all monitor output will be

placed in this window.

From here, if you hit [PgDn] again (assuming there are no other windows

opened), you will go back to the HOST COMMAND WINDOW.  If, on the other

hand, you have a connection open (let's say your connected to my PBBS,

N5IAL-1, on VHF stream A), you would see the following:

�������������������������������������������������������������������������������

F1=HELP COM1 9600 8N1           VHF/A (N5IAL-1)           09 May 1992  12:33:37

�������������������������������������������������������������������������������

The KAM is indicating here that the current stream is VHF stream A, and

that it is connected to N5IAL-1 (or at least, KAMterm thinks it is).  If

you were to then disconnect from N5IAL-1, the status bar would change to

the following:

�������������������������������������������������������������������������������

F1=HELP COM1 9600 8N1                VHF/A                09 May 1992  12:33:37

�������������������������������������������������������������������������������

This indicates that the current stream is VHF stream A, and as far as

KAMterm knows, this stream is not connected to anyone.

KAMterm attempts to keep track of who you are really connected to at all

times.  So, for example, if you connect to a local digipeater node (we'll

call it FOO-DIGI), and then from there, you connect to a local BBS (we'll

call it FOO-BBS), you would see the status bar change to indicate the new

callsign you're connected to.  This assumes, however, that the format of

the status message from FOO-DIGI is along the lines of:

FOO-DIGI:callsign} Connected to FOO-BBS

The only thing that is really important is that the FOO-DIGI string

appear in the first string on the line, and that the next bits are the

``Connected to'' message, followed by the new callsign/alias.  This is

NOT case sensitive.

It is possible, however, that KAMterm will miss one of these status

messages and not correctly track who you are connected to.  If this is

the case, you can use the [ALT][Z] command to correct it.  This command

is described later.

When you change screens, the main window and the status bar will be

updated, but the local window will remain the same.  This way, you can

type the connect command, etc., while still looking at the information

on the screen.

5.2.3)  LOCAL  ECHO  vs.  INCOMING  TEXT

KAMterm allows you to use a different color for locally echoed text than

that used for incoming text from the TNC.  This is defined in the

configuration file, kamterm.cfg, which is discussed later. The default

is to use YELLOW for locally echoed text, and LIGHTBLUE, RED, OR GREEN

for incoming text in the COMMAND WINDOW, MONITOR WINDOW, and STREAM

WINDOWS, respectively.  These are also configured in the configuration

file.

5.3)  NEW  CONNECTS  (Host  Mode  ONLY)

In Host Mode, a new connect will cause a new window to be created if the

stream connected is not currently allocated to an active window.  So,

for example, if the only windows open are the COMMAND WINDOW and the

MONITOR WINDOW, and you connect to someone, a new window will

automatically be created to handle that stream.  If, on the other hand,

you made the connect on, say, VHF stream A, and a window was already

active on VHF stream A, no window would need to be created.  This

applies for incoming connects as well.

In addition to this, KAMterm will notify you of incoming connects.  This

requires that the TNC parameter RING be set to ON (the ^G characters

sent from the KAM on incoming connects are used as the indicator that

the connect was not originated by you).

6.0)  KAMterm  COMMANDS

6.1)  GENERAL  COMMANDS

In KAMterm, some commands are available all of the time, and others are

available only while in Host Mode operation.  In this section, those

commands which are available at all times will be discussed.  They are

covered in order by the key that calls them (i.e., [ALT][B] will be

covered before [ALT][F], etc.).

6.1.1)  [ALT][B]

This command is used to transmit a ``brag file'' to the connected

station. A window will popup and ask you for the name of the file you

wish to transmit.  If this file is found, it will be transmitted just as

if you had typed the information.  If, on the other hand, it is not

found, KAMterm will complain about not finding the file, and return to

normal operation.

During each phase of the file transmission, KAMterm will check to see

that the transmit buffers have been cleared before proceeding to the

next part of the file.  This is to help insure that the buffers in the

KAM are not overrun by trying to send information faster than it can

transmit it to the remote station.  If the KAM's buffers fill up, and it

tells your computer to stop sending data, the buffers within KAMterm

will not empty, and KAMterm will wait until it can move on.

This function is not really intended for use to upload large text files

to a BBS, etc., though it could probably be used for such applications.

It is intended for short files describing your station, and does not

implement any error control with the remote station, and assumes that if

the KAM is happy, the remote end is happy.

Functionality for uploading actual files will be added when I get the

protocol specifications for file transfer programs used on packet BBS

systems.

6.1.2)  [ALT][C]

This command clears the screen in the current window.

6.1.3)  [ALT][E]

This command toggles the local echo of characters to the screen (from

KAMterm, not from the KAM).  Local echo from KAMterm is normally used

when the KAM is not echoing characters you type.  In its normal

operating mode, the KAM uses the ECHO command to determine whether or

not you want input echoed back.  In Host Mode, the KAM does not echo

characters back to you.  Therefore, KAMterm's default action is to echo

characters back only in Host Mode operations.

It is important to note that this command affects the current operating

mode only.  That is, the setting of the local echo parameter for normal

mode and for Host Mode are two different settings.  [ALT][E] toggles the

setting for the mode you are in when you use this command, without

changing the other mode's value for this parameter.

The default values for echo in both normal and Host Mode operations can

be set from the configuration file, as well.  This is discussed later.

6.1.4)  [ALT][F]

This command is used to send a macro command file to the KAM.  You will

be asked for the name of the file you wish to send to the KAM (the

default will be the name of the startup command file, if one is

defined).  This command can be used to change the KAM's configuration

while running KAMterm, if desired.

The format of the macro file is the same as that for the startup and/or

exit command files, which will be discussed later.

6.1.5)  [ALT][G]

This command is used to call KAMterm's internal QSO logger (or,

alternatively, an external logging program).  In the log function, you

will be asked for the normal items you would enter in a logbook, such as

the person's callsign, the start/end times for the QSO, the frequency,

and so on.  If you are in Host Mode, KAMterm will maintain a different

set of log information for each window.

The internal log function for KAMterm is far from elegant.  It is

primarily intended as a quick and easy way to document the QSO

information without having to switch between a note pad, a logbook, and

the keyboard.

When entering QSO log information in the internal logger, do not save

it to a file until the entry is complete.  Once you save an entry to

disk, KAMterm assumes that you have completed that entry, and the next

time you access the log in that window, it will be blank.

If you wish to set a default filename for the logfile, you can do so via

the configuration file, kamterm.cfg.  This is also where you can

instruct KAMterm to use an alternate logging program.  See the section

on the configuration file for more details.

6.1.6)  [ALT][H]

This command toggles between normal mode and Host Mode.  This command

DOES change the KAM's configuration, as this is REQUIRED to change

to/from Host Mode.

IMPORTANT NOTE:  Changing the KAM's state to/from Host Mode requires a

reset, which will DROP ANY ACTIVE CONNECTS!!!

While in Host Mode, KAMterm keeps track of which windows' streams are

connected, and will warn you if it knows you have connects active.  In

normal mode, KAMterm doesn't know you're connected, and will NOT WARN

YOU.  Also, if you have entered the program in Host Mode, you may have

connects active that KAMterm hasn't detected, and it will not warn you

if this is the case.  It is also quite probable that the reset will drop

any connection to your PBBS that is active.

YOU ARE THEREFORE ADVISED to use the STATUS command to check to see if

there are any connects active before using this command.  STATUS will

tell you if any connects are active, including connects to your PBBS.

6.1.7)  [ALT][I]

This command provides some general information about KAMterm,

registration, etc.

6.1.8)  [ALT][K]

This command brings up a menu which allows you to change the function

key definitions.  The menu will appear in the upper left corner of the

screen, and its exact nature will vary, depending on whether or not a

mouse was detected on the system.  The items in the menu are as follows:

����������������������������Ŀ

� F2 through F10             �

����������������������������Ĵ

� ALT F1 through ALT F10     �

����������������������������Ĵ

� CTRL F1 through CTRL F10   �

����������������������������Ĵ

� SHIFT F1 through SHIFT F10 �

����������������������������Ĵ

� Load new function key file �

����������������������������Ĵ

� Save function keys to file �

����������������������������Ĵ

� Exit                       �

������������������������������

The first four options allow you to change the characters that will be

sent to the KAM for the function keys themselves.  Note that plain [F1]

is not included in this list --- [F1] itself is reserved for HELP.

The fifth option, ``Load new function key file'', allows you to load a

new set of function key definitions from a file.  You will be asked for

the name of the file, and if it exists, it will be loaded.  Function key

macro files can either be created through the program, or manually.

The sixth option, ``Save function keys to file'', allows you to save the

function key definitions you have defined to a file.  You will be asked

for the name of the file.   The results will then be stored to that

file.

The last option simply exits the menu.  Pressing [ESC] will do the same

thing.

The format for the function key file will be discussed in the section

entitled FUNCTION KEY DEFINITION FORMAT.

6.1.9)  [ALT][L]

This command toggles logging of the current window to a file.  If

logging on the current window is not active, you will be asked for the

name of the file to log to.

In Host Mode, KAMterm will attempt to keep things organized in the log

file by indicating who said what.  It will use the callsign it last

detected as the current connected call for the remote end, and the

callsign you defined (if any) in the configuration file.  If you have

not defined a callsign, it will simply use ``YOU'' to refer to you.  If

KAMterm doesn't know who you are connected to, this will not happen.

You can manually define the callsign of the station you are connected to

(if you need to) by using the [ALT][Z] command (discussed in the Host

Mode Commands section).

6.1.10) [ALT][M]

This gives a quick snapshot of the amount of memory available to KAMterm

on your system.

6.1.11)  [ALT][P]

This command allows you to change the communications parameters from

within KAMterm.  You will be given the following list of options:

�������������������������������������������������Ŀ

�         Port [1-4]:                             �

�         Port Speed:                             �

� The KAM's Host Mode *REQUIRES a port setting    �

� of N81 and bidirectional hardware flow control. �

���������������������������������������������������

The port option selects the communications port, and valid inputs are

[1], [2], [3], and [4], for COM1, COM2, COM3, and COM4, respectively.

The port speed option is just that --- port speed.  The KAM does not

support port speeds greater than 9600 bps.  KAMterm will, however, allow

speed settings up to (and including) 115,200 bps.

Parity, data bits, and stop bits are automatically set at No Parity, 8

bits per character, and 1 stop bit.  These settings are required for the

KAM's Host Mode.

In addition, the KAM's Host Mode requires that flow control be in the

form of bidirectional hardware flow control.

In this mode of flow control, the CTS (Clear To Send) control line is

used to inform the computer that the KAM is ready to receive data, if

the computer has any to transmit, and the RTS (Request To Send) control

line is used by the computer to inform the KAM that it is ready to

receive data, if any is available.  These operate independently of each

other.  This mode of flow control is the preferred mode for use with

high-speed modems, etc., as it adds the ability for the computer to stop

data flow from the connected device when it is not ready.

6.1.12)  [ALT][R]

This command allows you to execute an external DOS command.  You will

be asked for the name of the command you wish to execute.  If you simply

press [CR], you will simply shell out to DOS.  If you decide not to do

anything here, simply press [ESC] to abort.

6.1.13)  [ALT][X]

Exit KAMterm.  If you are in Host Mode, and have connects active (and if

KAMterm knows about them) you will be asked if you REALLY want to exit.

If you are in Host Mode, regardless of whether or not you have connects

active, you will be given the option of leaving the KAM in Host Mode

(press [h]).

6.1.14)  [ALT][Y]

This command is used to set the date/time on the KAM.  It uses the

current date and time setting on your computer.

6.1.15)  [F1]

View HELP SCREEN.

6.1.16)  [UP-ARROW]

If scrollback is enabled, and if scrollback buffers could be allocated

to the current window, this will pull up the scrollback buffer for the

current window.  Within scrollback, [PgUp], [PgDn], [HOME], [END], [UP],

and [DOWN] will move you around in the scrollback buffer. [ESC] exits

back to KAMterm's normal operation.

In addition, if you wish to save the entire scrollback buffer to a file,

press [S].  KAMterm will then ask you for the filename that you wish to

save the buffer to.  Press [ESC] instead of entering a filename to abort

this operation.

6.2)  HOST  MODE  COMMANDS

In KAMterm, some commands are available only while in Host Mode

operation. In this section, those commands which are available only

while in Host Mode be discussed.

6.2.1)  [ALT][A]

This command loads a primative protocol analyzer.  This function was

originally built for testing purposes within the program, but users may

find it interesting, and as a result, it has remained in the program

code.

6.2.2)  [ALT][D]

This command will delete the current window.  It cannot be used on

either the COMMAND WINDOW or the MONITOR WINDOW.  If KAMterm is under

the impression that the stream associated with window you wish to remove

is connected, it will ask you if you are really sure you want to delete

it. Doing so will not affect the connect itself, but as soon as

information is received for that stream, the window will be created once

again (except that it will no longer have the connected callsign

associated with it, and will not even show up as being connected in the

STATUS BAR).

6.2.3)  [ALT][N]

This command allows you to change the stream associated with the current

window.  If the stream associated with that window is connected (or at

least, if KAMterm believes it is), you will be warned that a connect is

active, and asked if you really want to do this.

This command can be used under the COMMAND WINDOW, but you will probably

find it easier to use the ``faked'' TNC stream switch commands (see the

information on faked commands later in this section).

6.2.4)  [ALT][S]

This command is used to send special non-packet HF port commands to the

KAM.  This is needed due to the fact that in Host Mode, the KAM does not

use the normal CTRL-C,n directives.  This will bring up a menu of the

directives you can send.  All non-packet HF port directives documented

for Host Mode (Kantronics firmware version 4.0) on page 30B of the

OPERATIONS MANUAL are implemented.

6.2.5)  [ALT][T]

In Host Mode, the KAM does not use the PACLEN parameter as it would in

normal command mode.  Instead, the length of transmitted packets is

determined by the block size of Host Mode data blocks.

KAMterm uses two parameters (defined in the configuration file, and

discussed in the section on setting up the configuration file) called

VHFPACLEN and HFPACLEN to determine the default packet length to be

transmitted.  This option allows you to override the default values you

select for a given window.

You will be asked for the new value to be used within this window when

you use this command.  The current maximum is 78, as this is the maximum

number of characters you can type on one line in KAMterm.  If you ask

for a higher number, KAMterm will currently back it off to 78. When the

ability to transmit entire files using the various protocols used on

packet BBS systems is added, this will be modified accordingly to avoid

artificially limiting the efficiency of such transfers.

6.2.6)  [ALT][W]

This command is used to create a new STREAM WINDOW. If you already have

too many windows open, KAMterm will complain if you (or an incoming

connect) try to open another window.  Also, if a STREAM WINDOW is

already open for the port/stream combination you request, the window

will not be created. Otherwise, the window will be created with the

information you give it.

BE ADVISED, however, that as you open more windows, things slow down a

bit when switching from one window to another.  On the system where this

was developed (20 MHz 386 w/ 2 Meg RAM, running Digital Research Dos

6.0), this did not become noticeable until about 8 to 10 windows were

open (this includes the COMMAND WINDOW and the MONITOR WINDOW).  If

things get too slow, and you have inactive windows, you may wish to

close them.

6.2.7)  [ALT][Z]

This command is used to manually override KAMterm's concept of who you

are connected to on a given stream, or even whether or not you are

connected at all.  You will first be asked if the window's stream is

connected.  If you respond NO ([N]), all connect information for that

window will be cleared.  If, on the other hand, you respond YES ([Y]),

you will then be asked for the correct callsign of the person/bbs you

are connected to.

This command is primarily intended for either of two purposes.  First,

if you start KAMterm in Host Mode, and there is already a connect

active, KAMterm will not see the incoming status message, and will not

register the connect --- this allows you to manually correct for this

problem. Second, if (and I have never seen this happen, but it is

possible) KAMterm should miss an incoming connect message from a

digipeater, you can insert the correct information yourself.

6.2.8)  [ALT][=]

This command is used to tell KAMterm that the window you are currently

in is the PRIORITY stream, and that you want to keep tabs on what is

going on in that window.  Using this command will create a PRIORITY

WINDOW, which uses the top four lines of the screen when any other

window is displayed on the screen to display incoming text that is being

displayed on the window you have defined to be your PRIORITY stream.

This could be used, for example, for sitting on a DX cluster while

working packet, RTTY, etc., over in another window.  With the PRIORITY

WINDOW, you don't have to check the other window every few minutes to

see if anything interesting has come in --- you can have a smaller

version of that window at the top of your screen to keep an eye on.

This command can also be used when the PRIORITY WINDOW is active to

CHANGE the stream that it monitors (i.e., you don't have to delete an

existing PRIORITY WINDOW just to change the stream you wish to keep tabs

on).

6.2.9)  [ALT][-]

This command is used when you want to temporarily hide the PRIORITY

WINDOW, but you still want to keep it active.  This simply toggles

whether or not the priority window is actually displayed on the screen.

6.2.10)  [ALT][0]

This command completely deletes the PRIORITY WINDOW.

6.2.11)  [HOME]

This command takes you immediately to the COMMAND WINDOW.  It will also

set the stream and port associated with the COMMAND WINDOW to that which

is associated with the window you were in when you used the HOME key.

(This last bit does not, however, apply to the MONITOR WINDOW, or the

PROTOCOL ANALYZER window if it is open, as these have no actual

stream/port association.  In this case, the stream/port associated with

the COMMAND WINDOW is left as it was.)

6.2.12)  [PgDn]

This takes you to the next window active in KAMterm.  If you are at the

last window, it will wrap you around to the COMMAND WINDOW.

6.2.13)  [PgUp]

This takes you to the previous window active in KAMterm.  If you are at

the COMMAND WINDOW, it will wrap you around to the last active window.

6.2.14)  ``Faked''  Commands

Normally, Host Mode doesn't have some of the things normal command mode

does, and this can be rather annoying at times.  To make things easier,

KAMterm ``fakes'' some of these things, and takes the appropriate action

for you.  These pseudo-commands are only effective in the COMMAND

WINDOW.

Fake command prompt (hcmd:)

Normally, the TNC gives you a cmd: prompt telling you it is ready to

accept commands.  When you are used to this, it can be distracting to

simply get a blank screen staring at you in Host Mode.  To eliminate

this problem, KAMterm generates its own prompt, hcmd: (the 'h' is added

to avoid confusion with the real cmd: prompt). A simple [CR] will result

in new hcmd prompt.  This is done for aesthetics only, and has no impact

or interaction with the TNC.

|n and ~n

In Host Mode, the KAM does not respond to the stream switch commands as

it does in normal command mode, because each block of data transmitted

to and from the KAM contains its own stream/port information, and

setting the KAM to a stream/port would have no meaning.

KAMterm will, however, allow you to use the stream switch commands as if

you were in normal command mode.  The difference here is the result.  In

normal command mode, using these changes all I/O with the KAM over to

another stream, which will be affected by what you type.  In Host Mode,

KAMterm uses these commands internally to change the stream/port

combination associated with the COMMAND WINDOW.  This will have

basically the same effect, since your commands will now impact the new

stream you have selected --- it is simply handled within KAMterm instead

of by the TNC.

6.2.15)  AMTOR  SPECIAL  COMMANDS

To make working AMTOR a bit simpler, three special commands have been

created in KAMterm.  These are @AMTOR, @FEC, and @LAMTOR.  These special

commands are entered directly as if they were text to be transmitted

(i.e., in the LOCAL WINDOW --- similar to the faked commands '|n' and

'~n'), and will only work under the following conditions:

*) You must be in Host Mode

*) The current port/stream must be HF/0

*) You are assumed to be planning to work AMTOR

Note that these commands are not implemented as function key macros at

this time, nor are they in effect when transmitting macro files. This

may be added in the future, if there are requests to do so.

Normally, when working AMTOR, you might listen for a CQ using the KAM's

LAMTOR command, and then call the remote station using either Mode A

(ARQ) or Mode B (FEC).  To do this, however, you must first put the KAM

back into packet mode (to remove it from LAMTOR mode), and then go to

the COMMAND window and enter either ``AMTOR selcal'' or ``FEC selcal''

to connect to the remote station.

These special commands allow you to switch from whatever mode you might

be in to the appropriate AMTOR mode, unless you are already connected on

an HF packet stream (in which case, the KAM will probably not allow you

to do this).

@AMTOR is used to immediately enter ARQ mode.  If it is followed by the

SELCAL of the remote station you wish to connect with, it will cause the

KAM to attempt to connect to that station.  If it is not followed by any

additional arguments, it will simply place the KAM in ARQ mode.

Likewise, @FEC can either be used to connect with another station in

AMTOR FEC mode, or simply to enter that mode.

@LAMTOR takes no arguments --- it simply puts you into the LAMTOR mode,

where you can monitor both ARQ and FEC transmissions, but cannot enter

transmit mode.

7.0)  THE  CONFIGURATION  FILE

KAMterm uses a configuration file called kamterm.cfg, which allows you

to change most aspects of its configuration.  This file must exist,

however, it need not have all configuration options in it (or any of

them, for that matter).  Any options which are not set within this file

simply revert to their default values.  If the file is not found at all,

the system will create one for you with only the communications

parameters.

Within the configuration file, anything following a '#' character is

treated as a comment, blank lines will be ignored, and commands within

the file are not case-sensitive.  Only one command is allowed per line

in the file.  In the examples shown below (and in the sample

configuration file), a colon follows the field name --- this is

optional, and is included for readability only.  KAMterm ignores this

completely.

Throughout this section, example commands will be represented as:

INITFILE:  startup.kam

There may be several commands grouped together, but the groups of

commands (or the command) will have a blank space above and below it,

and will be indented.  If KAMterm does not understand the input in any

line, it will advise you of this, and tell you the offending line number

within the configuration file.

7.1)  GENERAL  OPTIONS

There are several types of options which may be set from the

configuration file.  This section will cover those which are general in

nature, and don't apply to any specific grouping.

7.1.1)  CAPBUFF

CAPBUFF is the size of the scrollback buffer for EACH WINDOW.  Take care

when increasing this number --- remember that each stream will have this

allocated to it, and that eats memory.  I suggest you start with the

default (100 lines) and gradually increase.  If a window is opened, and

there isn't room to allocate this number of lines, that window will not

have a scrollback buffer, but will be opened (provided there is enough

memory left to do so).

Here are some sizes for 100 line increments of the scrollback buffer

(note that there is clearly a pattern here):

0 lines:  scrollback disabled

100 lines:  18,400 bytes per active window

200 lines:  28,400 bytes

300 lines:  38,400 bytes

......      .......

800 lines:  88,400 bytes

The example below sets the scrollback buffer size to 200 lines.

CAPBUFF: 200

7.1.2)  INITFILE  and  EXITFILE

These options set the filenames for the optional command (macro) files

to be sent to the KAM when starting and exiting KAMterm.  In the

following example, when KAMterm is loaded, it will transmit all commands

in the file called startup.kam to the TNC, and right before exiting,

will do the same for the file finish.kam.

INITFILE:  startup.kam

EXITFILE:  finish.kam

The filenames can be anything you choose, provided of course that it is

a legal dos filename, and that the total length of the path/filename

does not exceed 80 characters.  The format for these files will be

covered in the section called FORMAT FOR INIT/EXIT COMMAND FILES.

7.1.3)  QSO LOG OPTIONS

There are several options which can be used to customize the operation

of the QSO logging functionality in KAMterm.  These include setting a

default file to log information to, setting up an external logging

program, asking KAMterm to automatically (and silently) record the times

for connects and disconnects, etc.

The first option is LOGFILE.  This sets the default filename that

KAMterm will use when logging QSOs to a file.  You will still be asked

for a filename, and can choose an alternate file if you so desire. This

is optional, and if this is not found in your configuration file,

KAMterm will simply present a blank field when asking for the filename.

The next option is AUTOLOGFILE, which tells KAMterm the name of the file

to which connect/disconnect times will be (silently) recorded. If this

option is not included, this information simply will not be recorded.

In the following example, the default filename to use for manually

entered logs is kamterm.log, and the filename for the automatic

recording of start/end times is autolog.kam.

LOGFILE: kamterm.log

AUTOLOGFILE: autolog.kam

In addition to these options, if you wish to use an external logging

program, you would identify it to KAMterm by using the LOGGER option.

With this option, when you press [ALT][G], you will execute the external

logging program, and when you exit from it, you will return to KAMterm.

If you want KAMterm to check with you before calling the external

logging program (e.g., if you sometimes use the internal logger), you

can use the ASKLOGGER option.  If ASKLOGGER is not used in your

configuration file, KAMterm will run the external logging program every

time you press [ALT][G].

In the following example, the external logging program is called by the

command ``mylogger logfile.txt'' and we do want KAMterm to ask before

calling the external logger.

LOGGER: mylogger logfile.txt

ASKLOGGER: YES

7.1.4)  MORSESPEED

KAMterm uses Morse code for a few minor status messages and error

messages. This option allows you to determine the code speed at which

Morse code messages will be sent from within KAMterm.  The default is 20

WPM. If this parameter is set to 0 (zero) words per minute, Morse

messages will be disabled.

The example below shows the default setting of 20 WPM:

MORSESPEED: 20

NOTE:  According to the documentation for Turbo C++, the functions used

for determining the delays used in generating the Morse code messages do

not vary with the CPU speed of the computer the program is run on. Tests

run on multiple computers have verified this, however, if you should

happen to find this to be in error, you may need to experiment a bit.

7.1.5)  MORSETONE

This option allows you to change the frequency of the tones used for

Morse code messages.  The default is 600 Hz, as represented in the

example below:

MORSETONE: 600

7.1.6)  MYCALL

MYCALL is the callsign that will be used for logging to a file. This

does not affect the value in the TNC in any way.  The example below sets

MYCALL to N5IAL.

MYCALL: N5IAL

7.1.7)  NORMALECHO  and  HOSTECHO

NORMALECHO and HOSTECHO are used to determine if KAMterm should default

to echoing text locally in normal command mode and Host Mode,

respectively.  The values selected for these must be either 1 for on or

0 (zero) for off.

The default for normal mode is NOT to echo text locally, and the default

for Host Mode is to echo text.  The examples shown below illustrate how

those parameters would be set if they were included in the startup file.

NORMALECHO: 0

HOSTECHO: 1

7.1.8)  VHFPACLEN  and  HFPACLEN

In Host Mode, the KAM does not use the settings defined by its PACLEN

setting.  This is left up to the Host Mode software.  Therefore, KAMterm

provides two parameters, VHFPACLEN and HFPACLEN, which correspond to the

VHF and HF portions of the PACLEN setting, respectively.

In KAMterm, a line of text is limited to 78 characters (limited by the

screen size), and the maximum setting for each of these options is,

therefore, 78 (a higher value would have no meaning).  When the various

file transfer protocols used on packet BBS systems is added, this will

be changed to avoid artificially limiting the throughput on file

transfers. Currently, the default for each of these settings is 78

characters, as shown in the examples below.

HFPACLEN:  78

VHFPACLEN:  78

7.1.9)  MARGINBELL

If you choose to do so, you can activate a margin bell that will sound

when you reach the specified number of characters from the left margin

(similar to the margin bell on actual mechanical typewriters).  The

default option is for this to be turned off, and if nothing is setup

in the configuration file, there will be no bell.  If, however, you wish

to set the margin bell to sound at a given column, you would setup

something like the example below, which assumes you want the bell to

sound 5 spaces from the right margin (the right margin is at column 78,

therefore, you would want it to sound when you hit column 73):

MARGINBELL:  73

7.2)  COMMUNICATIONS  PARAMETERS

This section discusses the communications parameters that can be set

from the configuration file.

7.2.1)  PORTSPEED

PORTSPEED sets the default serial port speed which KAMterm will use, and

must be one of 1200, 2400, 4800, or 9600.  As this is being written, the

KAM does not allow port speeds higher than 9600 bps.  The example below

shows the default setting, 9600 bps, and how it would be set in the

file.

PORTSPEED: 9600

7.2.2)  COMMPORT

COMMPORT sets the default serial port which KAMterm will use, and must

be one of 1, 2, 3, or 4, for COM1 through COM4, respectively.  In the

example below, COM1 (the default) is selected.

COMMPORT: 1

7.3)  COLOR  SETTINGS

COLOR commands take exactly 2 arguments, and they must be in the right

order.  The first argument is the parameter which we wish to associate

the color with, and the second is the color itself.

A program is included which dumps all color combinations to the screen

and identifies the foreground and background colors in effect at that

time. Use this to see how different color combinations look, and decide

what you like/don't like.

MONOCHROME USERS:  The windowing library used with this program is smart

enough to know that you have a monochrome monitor, and does not use the

majority of these colors.  Depending on what combinations you select, it

will set things up to either do normal, reverse, or high-intensity

video.

Choices for colors are as follows:

BLACK         DARKGRAY

BLUE          LIGHTBLUE

GREEN         LIGHTGREEN

CYAN          LIGHTCYAN

RED           LIGHTRED

MAGENTA       LIGHTMAGENTA

BROWN         YELLOW

LIGHTGRAY     WHITE

NOTE:  If the BACKGROUND color is set to anything in the second column

(DARKGRAY through WHITE), the foreground will be flashing.  This is true

for both color and monochrome monitors.

The first set of color parameters deal with the colors assigned to the 3

key areas of the screen in normal operations (i.e., not in a menu,

etc.). The second set of color parameters deal with the colors used for

popup windows, menus, text entry (e.g., when you are asked for a

filename), and so on.  These are shown in the following tables.

�����������������������������������������������������������������ͻ

�          Screen Colors for Normal Operations                    �

�����������������������������������������������������������������͹

�  STATFG              �  STATUS LINE foreground                  �

�����������������������������������������������������������������Ķ

�  STATBG              �  STATUS LINE background                  �

�����������������������������������������������������������������Ķ

�  PRIFG               �  PRIORITY WINDOW foreground              �

�����������������������������������������������������������������Ķ

�  PRIBG               �  PRIORITY WINDOW background              �

�����������������������������������������������������������������Ķ

�  LOCALFG             �  LOCAL WINDOW foreground                 �

�����������������������������������������������������������������Ķ

�  LOCALBG             �  LOCAL WINDOW background                 �

�����������������������������������������������������������������Ķ

�  MAINFG              �  MAIN COMMAND WINDOW foreground          �

�����������������������������������������������������������������Ķ

�  MAINBG              �  MAIN COMMAND WINDOW background          �

�����������������������������������������������������������������Ķ

�  MONFG               �  MONITOR WINDOW foreground               �

�����������������������������������������������������������������Ķ

�  MONBG               �  MONITOR WINDOW background               �

�����������������������������������������������������������������Ķ

�  STREAMFG            �  STREAM WINDOW(s) foreground             �

�����������������������������������������������������������������Ķ

�  STREAMBG            �  STREAM WINDOW(s) background             �

�����������������������������������������������������������������Ķ

�  ECHOFG              �  local echo foreground (any window)      �

�����������������������������������������������������������������Ķ

�  ECHOBG              �  local echo background (any window)      �

�����������������������������������������������������������������ͼ

����������������������������������������������������������������������������ͻ

�                     Popup Menu/Window Colors                               �

����������������������������������������������������������������������������͹

�  POPUPFG                  � popup window foreground                        �

����������������������������������������������������������������������������Ķ

�  POPUPBG                  � popup window background                        �

����������������������������������������������������������������������������Ķ

�  HELPFG                   � help screen foreground                         �

����������������������������������������������������������������������������Ķ

�  HELPBG                   � help screen background                         �

����������������������������������������������������������������������������Ķ

�  SBFG                     � scrollback screen foreground                   �

����������������������������������������������������������������������������Ķ

�  SBBG                     � scrollback screen background                   �

����������������������������������������������������������������������������Ķ

�  MENUFG                   � menu foreground                                �

����������������������������������������������������������������������������Ķ

�  MENUBG                   � menu background                                �

����������������������������������������������������������������������������Ķ

�  MENUBDRFG                � menu border foreground                         �

����������������������������������������������������������������������������Ķ

�  MENUBDRBG                � menu border background                         �

����������������������������������������������������������������������������Ķ

�  MENUSELFG                � menu selected item foreground                  �

����������������������������������������������������������������������������Ķ

�  MENUSELBG                � menu selected item background                  �

����������������������������������������������������������������������������Ķ

�  MENUFIRSTFG              � foreground for highlighted single key in menu  �

����������������������������������������������������������������������������Ķ

�  MENUFIRSTBG              � background for highlighted single key in menu  �

����������������������������������������������������������������������������Ķ

�  ENTRYFG                  � text entry foreground                          �

����������������������������������������������������������������������������Ķ

�  ENTRYBG                  � text entry background                          �

����������������������������������������������������������������������������Ķ

�  ENTRYKEY                 � text entry key text (labels)                   �

����������������������������������������������������������������������������Ķ

�  ENTRYMSG                 � text entry message text (messages to user)     �

����������������������������������������������������������������������������ͼ

The examples below show the default assignments for each of these items.

COLOR: STATFG BLUE

COLOR: STATBG LIGHTGRAY

COLOR: PRIFG BLUE

COLOR: PRIBG LIGHTGRAY

COLOR: LOCALFG GREEN

COLOR: LOCALBG BLACK

COLOR: MAINFG LIGHTBLUE

COLOR: MAINBG BLACK

COLOR: MONFG RED

COLOR: MONBG BLACK

COLOR: STREAMFG GREEN

COLOR: STREAMBG BLACK

COLOR: ECHOFG YELLOW

COLOR: ECHOBG BLACK

COLOR: POPUPFG YELLOW

COLOR: POPUPBG BLUE

COLOR: HELPFG BLUE

COLOR: HELPBG LIGHTGRAY

COLOR: SBFG YELLOW

COLOR: SBBG BLACK

COLOR: MENUFG BLACK

COLOR: MENUBG LIGHTGRAY

COLOR: MENUBDRFG RED

COLOR: MENUBDRBG LIGHTGRAY

COLOR: MENUSELFG YELLOW

COLOR: MENUSELBG CYAN

COLOR: MENUFIRSTFG RED

COLOR: MENUFIRSTBG LIGHTGRAY

COLOR: ENTRYFG YELLOW

COLOR: ENTRYBG LIGHTGRAY

COLOR: ENTRYKEY BLUE

COLOR: ENTRYMSG RED

If the color selected or the parameter to assign that color to are not

valid, KAMterm will advise you that it did not recognize what you were

trying to do, and the line number in the configuration file where the

mistake was found.

8.0)  FUNCTION  KEY  DEFINITION  FORMAT

If you are creating a file containing function key definitions by hand,

it must follow a specific format.  First, in most cases, anything

following a '#' character is treated as a comment, and ignored by the

program.  The only exception to this is the line of text associated with

the function key, where the '#' character is treated as any other

character would be.

8.1)  GENERAL  FILE  FORMAT

When definining text to be transmitted, the ']' character is used for a

carriage return.  This is not required at the end of the line, as a [CR]

will automatically be sent at the end of the macro key.  It is primarily

used for multiple commands being placed within one macro key. At this

time, the notation '^n' to send a CTRL-n key sequence is not supported.

To define the text to be sent to the KAM when a function key is pressed,

you first need to tell KAMterm what key you are defining.  To do this,

simply enter one of the following on a line by itself:

{KEY Fn}

{KEY ALT Fn}

{KEY CTRL Fn}

{KEY SHIFT Fn}

where 'n' is a number from 2 to 10 for the function keys by themselves,

or from 1 to 10 for [ALT], [CTRL], or [SHIFT] and the function key.

Again, [F1] is reserved for the HELP command within KAMterm.

The next line of text is the text that will be sent to the KAM when you

press that function key.  If the next line of text is blank, it will be

ignored.  Function key definitions cannot extend beyond one line, and

are limited to 80 characters.  In this line, the '#' character is NOT

treated as a comment.

So, for example, if we want to define [F2] to connect to FOO-BBS via

FOO-DIGI, we would enter something like the following:

����������������������Ŀ

� KEY F2               �

� C FOO-BBS v FOO-DIGI �

������������������������

As for comments within the above, take a look at the following example:

��������������������������������������������������Ŀ

� # This is a valid comment, and is ignored        �

� KEY F2     # This is also a valid comment        �

� C FOO-BBS v FOO-DIGI    # This is NOT a comment! �

����������������������������������������������������

In the above example, pressing [F2] would result in the following

being transmitted to the KAM:

``C FOO-BBS v FOO-DIGI    # This is NOT a comment!<CR>''

Note that the <CR> is automatically added at the end of the line.  If

the connect string itself had been followed by a ']' character, (i.e.,

``C FOO-BBS v FOO-DIGI]''), an additional <CR> would have been inserted

there, as well, thus sending TWO commands to the TNC (the second of

which would be ``# This is NOT a comment!'').

8.2)  INTERNAL  COMMAND  MACROS

Function key definitions within KAMterm can, to some degree, call

internal functions within KAMterm and thus provide single-key access

to some commands.  These are primarily intended to simplify operations

used while in a QSO, where you may often use the same command with the

same parameters (e.g., sending a ``brag'' file), or where using the

menu structure may be considered inconvenient or overly time-consuming.

As with any other command in the configuration files, these options are

not case-sensitive.  Only one internal command macro is allowed for a

function key --- if more exist on the line, they will not be recognized

by KAMterm.

Some of these macros are available only in Host Mode.  If you attempt

to use them in normal command mode, KAMterm will respond with an 'H'

in Morse Code, reminding you of this (assuming you have not disabled

Morse messages).

8.2.1)  @BRAG

This macro is used to transmit a ``brag'' file, as if you had entered

the [ALT][B] command.  This command takes one optional argument, the

filename for the ``brag'' file to be transmitted.  If the filename is

not given, you will be asked to provide the filename.

8.2.2)  @MACRO

This macro is used to transmit a macro command file to the KAM, as if

you had entered the [ALT][M] command.  This command takes one optional

argument, the filename for the macro file.  If the filename is not

given, you will be asked to provide the filename.

8.2.3)  @DAYTIME

This macro is used to set the time and date on the KAM, as if you had

entered the [ALT][Y] command.  It accepts no arguments.

8.2.4)  @EXEC

This macro is similar to the [ALT][R] command.  The difference here,

is that you specify the command you wish to execute directly in the

function key macro line.  Like the [ALT][R] command, if you do not

provide a command to execute, you will simply shell out to dos.

8.2.5)  @SHELL

This macro is used to immediately shell out to DOS.  It takes no

arguments.

8.2.6)  @SET_PRIWIN

This macro is the equivalent of the [ALT][=] command, which sets the

PRIORITY WINDOW to the current stream.  It accepts no arguments. This

internal macro is only available in Host Mode.

8.2.7)  @TOGGLE_PRIWIN

This macro is the equivalent of the [ALT][-] command, which toggles the

display of the PRIORITY WINDOW.  It accepts no arguments. This internal

macro is only available in Host Mode.

8.2.8)  @DELETE_PRIWIN

This macro is the equivalent of the [ALT][0] command, which deletes

the PRIORITY WINDOW.  It accepts no arguments. This internal macro is

only available in Host Mode.

8.2.9)  @HFSPECIALS  Macros

These internal macros duplicate the functionality of selections under

the [ALT][S] menu, which transmits non-packet HF port directives to the

KAM in Host Mode.  These macros are, therefore, only available in Host

Mode.  There are five parameters which can be used with @HFSPECIALS.

These are XMIT, RCVE, RCVI, PACKET, and CLRXMIT.  If you use any other

parameter with this macro, KAMterm will respond with a '?' in Morse

Code, indicating that it doesn't understand what you want it to do

(assuming you have not disabled Morse messages).

@HFSPECIALS XMIT

This command causes the KAM to enter transmit mode.

@HFSPECIALS RCVE

This command causes the KAM to enter receive mode when the transmit

buffer is empty.

@HFSPECIALS RCVI

This command causes the KAM to enter receive mode immediately.

@HFSPECIALS PACKET

This command causes the HF port on the KAM to return to packet mode.

@HFSPECIALS CLRXMIT

This command clears the transmit buffer for the KAM's HF port.

9.0)  FORMAT  FOR  INIT/EXIT  COMMAND  FILES

When using the startup and exit command files, there are only two key

things you must remember.  First, comments MUST have the '#' in the

first column on the line. This is done here in order to allow you to use

this character in TNC commands, if you so desire.

Second, if you wish to continue a command on to the next line (e.g., for

CTEXT), all you have to do is make sure the last character on the line

is a '\'.  This character is ignored anywhere else on the line, but in

this position, it will result in sending the ^V^M sequence to the KAM to

tell it to continue the command on the next line.

The following is a sample of what you might wish to place in the startup

file:

����������������������������������������������������������Ŀ

� CTEXT Howdy!  if I don't show, pse lv a msg on n5ial-1.\ �

� 73, de n5ial, Jim in Chicago.                            �

�                                                          �

� CMSG ON                                                  �

������������������������������������������������������������

This sets the connect text (CTEXT) to send the message shown, with the

``73, de n5ial'' part starting on a new line, and then sets the TNC up

to send CTEXT on a new connect in the normal manner.

The following is a sample of what you might wish to place in the exit

file:

���������������������������������������������������������������������Ŀ

� ctext Hmmm.....I don't seem to be around. pse lv a msg on n5ial-1.\ �

� 73, de n5ial, Jim in Chicago.                                       �

�                                                                     �

� cmsg pbbs                                                           �

�����������������������������������������������������������������������

This sets the connect text (CTEXT) to send the message shown, with the

``73, de n5ial'' part starting on a new line, and then sets the TNC up

to send CTEXT on a new connect, and then transfer the user to the PBBS

if it is available.

10.0)  DEVELOPMENT  ENVIRONMENT

For those who may be interested, the following is the environment under

which KAMterm was developed, and initially tested:

*) COMPUTER / SOFTWARE

*) 20 MHz 386 running Digital Research Dos version 6.0

*) Serial port uses National Semiconductor NS16550AFN UART

*) Turbo C++ Version 1.0 and Turbo Assembler Version 2.0

*) Windowing library is UltraWin Version 2.10, which is

available from EnQue Software (BBS:  816-353-0991).[1]

*) Communications library is Encom Version 1.0, which is

also available from EnQue Software.

*) Kantronics All Mode (KAM) with firmware version 4.00.

*) Printed documentation formatted using emTeX version 3.0 [3A]

and printed on Panasonic KX-P1124 24-pin dot matrix printer at

360 dpi (typical desktop laser printer is 300 dpi).

[1] This may seem strange to some, but I'd like to include a small advert

for another shareware product, the UltraWin library.  For those who

wish to use windowing type functions in any C code, check this library

out!  It is very nicely done, and is very powerful.  Support from EnQue

has been very good WITHOUT EXCEPTION, and has been very helpful here.

11.0)  ADDITIONS  FOR  FIRMWARE  VERSION  5.0  (AND  UP)

Several new capabilities have been introduced with version 5.0 of the

firmware for the KAM.  Most of these require that the Host Mode software

know how to take advantage of them.

11.1)  CONFIGURATION  FILE  OPTIONS

In order to prevent problems with TNCs running older versions of the

firmware, KAMterm requires that you specifically tell it if you are

running revisions above 4.x.  To do this, use the following in the

configuration file:

{KAMVERSION: 5}

The above tells KAMterm that you are running under version 5 of the

firmware.  This number must be an integer number (i.e., do not use 5.1,

etc.).  If you are not running version 5 of the firmware (or higher,

when it exists), this need not be included, as the default is to assume

that you are running firmware prior to version 5.

One of the nice things that version 5.0 of the KAM firmware added is the

ability of the LEDs on the front panel to act as they would in normal

command mode.  So, for instance, CON now indicates that the currently

active stream is connected.  KAMterm will send an update command to the

KAM when you change windows to cause it to show the current status for

the stream associated with the window you change to.  This does not,

however, apply to the MONITOR window or to the PROTOCOL ANALYZER

windows, as these have no stream associated with them.  When changing to

these windows, the KAM will remain on the last active stream.

11.2)  HOST  MODE  COMMANDS

There are several changes and additions to the commands which are only

available in Host Mode.

11.2.1)  HF  SPECIALS  MENU  ADDITIONS

The first of these changes affects the HF SPECIALS menu, which is

accessed by using the command, [ALT][S].  The new options are:

*) Lock HF port CW speed

*) Unlock HF port CW speed

These allow you to lock and unlock the CW speed on the HF port as in

normal command mode.

11.2.2)  AMTOR  XMITECHO  WINDOW

Another addition in version 5 of the KAM's firmware is the ability to

use the XMITECHO parameter on the KAM when running AMTOR under Host

Mode. If this window is active, it will display the KAM's status on

transmitting the text you type. Unlike normal mode operations, this does

not impact the normal echoing of characters to the screen as you type

them.  Instead, this will take the form of a small window in the upper

portion of the screen (just like the PRIORITY WINDOW described earlier).

This option requires that the KAM's XMITECHO parameter be turned on to

operate.

This option is not documented in the version 4 documentation, but may

work there as well.  It is not restricted within KAMterm to version 5.

To create this window, you press [ALT][1].  The port/stream of the

current window must be HF/0 (HF PORT 0).  This small window will not be

displayed in other streams' windows.  If the PRIORITY WINDOW is also

active, this will override that window.

If you wish to toggle the display of the AMTOR XMITECHO WINDOW, you can

press [ALT][2].  To close it completely, press [ALT][3].

If the AMTOR XMITECHO WINDOW does not exist, and XMITECHO is turned on,

incoming data blocks identified as XMITECHO data will be treated as any

other data coming from the KAM, and displayed in the appropriate window

along with other incoming text.

11.3)  FUNCTION  KEY  MACROS

The only additional function key macros for version 5 correspond to the

above AMTOR XMITECHO WINDOW.  These are shortcuts for the commands

associated with that status window.

11.3.1)  @SET_AMTORWIN

This macro is the equivalent of the [ALT][1] command, which sets the

AMTOR XMITECHO WINDOW to the current stream.  It accepts no arguments.

This internal macro is only available in Host Mode.

11.3.2)  @TOGGLE_AMTORWIN

This macro is the equivalent of the [ALT][2] command, which toggles the

display of the AMTOR XMITECHO WINDOW.  It accepts no arguments. This

internal macro is only available in Host Mode.

11.3.3)  @DELETE_AMTORWIN

This macro is the equivalent of the [ALT][3] command, which deletes the

AMTOR XMITECHO WINDOW.  It accepts no arguments. This internal macro is

only available in Host Mode.

8.0)  SUPPORT

I will provide support for problems, bug reports (what bugs?), and

suggestions to registered users via several modes:

8.1)  ELECTRONIC  MAIL

The best and FASTEST way to reach me is via electronic mail.  My addresses

are as follows:

*) INTERNET

*) jim@n5ial.chi.il.us

*) grahj@gagme.chi.il.us

*) j.graham@ieee.org

*) UUCP: gagme!n5ial!jim@clout.chi.il.us

The last Internet address, j.graham@ieee.org, is an alias provided by the

Institute of Eletrical and Electronics Engineers (IEEE) --- this will

follow me even if I move --- if all else fails, use this.  Right now, this

address points to grahj@gagme.chi.il.us, which will then forward mail to me

at home, at jim@n5ial.chi.il.us.

In addition, you can reach me from COMPMAIL using another version of the

above IEEE alias:  j.graham.

NOTE:  support will NOT be available via Amateur Radio --- the FCC would

probably have some very bad things to say about that.

8.2)  LANDLINE

Another way to reach me is via landline.  This will probably be a bit more

difficult, as I'm not always around the phone, and don't currently have

an answering machine.  I now have an AT&T EasyReach number, which will

always point to my actual phone number, even if I move.  This number is

0-700-JGRAHAM (0-700-547-2426).  Please note that this is *NOT* a

toll-free 800 number.

When you dial this, you will get an AT&T menu that will ask you to press

the # sign and then ask if you want to charge to call to the number

you're calling from or your AT&T calling card (if you have one).  If you

don't have a touch-tone phone, just wait a few seconds, and the system

will just charge the call to the number you're calling from.

When using this EasyReach number, there are a couple of things to keep

in mind.  First, you must dial 0-700, and not 1-700.  Second, if you

are not an AT&T subscriber, you must dial 10-288-0-700 (10-ATT-0-700)

to get an AT&T line.  Admittedly, this is a hassle, but at least you

will be able to reach me via telephone even if I move and change

phone numbers.

If you have questions or problems with the EasyReach number, you may

contact the AT&T EasyReach customer service department 24 hours/day

at 1-800-982-8480.

The number this points to is currently also my data number.  If you get

a busy signal, or if a modem answers, you'll want to try back later.

This number is virtually certain to be in data-mode during the following

hours (all times are Central Time, and daylight or standard time,

whichever is in effect):

2300 to 0600 Hrs 7 days a week

0700 to 1600 Hrs Monday through Friday

During these hours, the modem will be setup to receive incoming UUCP

connects to support e-mail and Usenet News.  If you have the ability to

use UUCP or UUPC, we can set you up there, too.

8.3)  SNAIL  MAIL

If all else fails, send your questions to the snail mail address shown

in the REGISTRATION section.  This will take longer, as there will be a

delay involved.  I won't get it until my dad forwards it to me, but I

WILL get it.

I will keep registered users aware of my current mailing address, too.

9.0)  REGISTRATION

If you received this documentation with your registration, this section

does not apply to you, and can be skipped.

KAMterm is the result of a large investment of time and money spent in the

development process, and is NOT freeware.  If you have not registered

this program, please send $ 25 (US) to:

Jim Graham

C/O James A. Graham (my Dad --- this address is more stable than mine)

519 Benning Dr.

Destin, FL 32541

US users, add $ 4 (US) shipping/handling.  International users, add

$ 9 (US) shipping/handling.

If you would like a copy of the printed, bound documentation, please add

an additional $ 10 to your registration.  This is primarily to cover the

cost incurred at the printshop (would be much worse if I was giving them

anything less than camera-ready art) and the additional cost of mailing.

With your registration, you will receive the latest release of KAMterm,

as well as a registration program which can be used to automatically

register any new releases of KAMterm that you receive.  (Registered

users will also receive announcements of new releases in the mail.)

This registration program requires a set of ``keys'' which will be

provided with the program.

The copy of KAMterm that will be shipped with your registration will

not yet be registered --- this is to allow you to distribute the most

current version as shipped to you if you care to do so.  You will,

therefore, need to register it upon its arrival.

By default, this will be shipped on a 360 k 5.25" disk.  If you prefer,

you can have the program shipped on a 720k or 1.44 Meg 3.5" disk.

In addition, you will be eligible for full support from me, and will get

priority for any enhancement requests you may have.

Please fill in the following form for my records --- this will help me to

support you better.

�����������������������������������������������������������������������������

NAME: ___________________________  CALL: ____________

ADDRESS: ___________________________

___________________________  PHONE (day):   (___) ___-____

___________________________  PHONE (eve.):  (___) ___-____

INTERNET E-Mail Address (if any):

OTHER E-Mail Addresses (specify type, e.g., UUCP, Compuserve, BITNET, etc.):

�����������������������������������������������������������������������������

E-mail addresses will currently only work if I can get to them via

INTERNET, but try anyways. You never know --- I might even have access

to them right now. I am including a list of other networks which can

exchange e-mail with INTERNET sites.  This list is available for

anonymous FTP from ftp.msstate.edu (130.18.80.11), and is found in

ftp/pub/docs/internetwork-mail-guide.  Other methods of distribution

are described within the file as well.  In this distribution, the file

is named mail.gui.