R T T Y   2 2 0 0 . 0 0   for IBM-PC MSDOS SYSTEMS

BY Perry Taylor, W0SE

JAN 1st., 1986

The  RTTY  2000  series of Amateur Radio Teletype Programs

were originally developed for  Heathkit  Computers,  and  are

based  on much of the original work of Irv Hoff, W6FFC, Keith

Peterson, W8SDZ, and others, on the Heath Computer net.  This

series of programs has added split screen,  and  consolidated

some of the concepts and programs, originated by the group on

the net.

One  of  features  of the program, that may be new to many

who first use this program, is the 3rd and 4th level transmit

and receive of ASCII characters, via the BAUDOT  level  code.

By  adding a third and fourth level shift, to the two already

available in the BAUDOT code, it is possible to send all  the

ASCII  characters,  over the BAUDOT channel.  This is done by

making a third shift of BLANK-FIGS, and a  fourth  of  BLANK-

LTRS.   The  use  of this feature is explained in the body of

this file.

Prior  to  using this version of RTY.EXE, please remove or

erase any previous RTY.CFG file. Set TIME/DATE for MS-DOS, as

this  is used in the RTY program.  Use all "UPPER-CASE" while

configuring the RTY program.

This  version  of  RTY  requires  192k or more of RAM, and

version 2.0 or higher of MSDOS.  The FILES parameter  in  the

CONFIG.SYS  file  should  be  reviewed.  See page 9.19 in the

MSDOS version 2.0 Heath Manual for details.  This program can

use as many as four files at one  time,  and  the  CONGIF.SYS

file should be set for 4 or more FILES.

OPERATION:

When  calling  up  RTY.EXE  for  the first time, the first

thing that will be shown is the "TITLE SCREEN".  Hit any key,

and the "CONFIGURATION" screen will be available.  Enter  the

values  on  this  screen to fit your requirements and station

hardware.  This builds a configuration record  which  can  be

saved  on  disk  by pressing the "f7" key.  The next time the

program is called, this configuration  record  will  be  read

from disk.

Page 2 - RTTY 2200 IBM PC (MSDOS)

CONFIGURATION ITEMS:

Using all capital letters, fill in the items shown on the

screen, customized as desired.

ITEM:                               COMMENTS:

File Name for Sending:              File name of file to be

sent or saved with "Alt-N"

File Name Disk Out:                 File name for file copy

Max File Size:                      File copy max size

Margin:                             Custom left margin

Baud Rate Index (RTTY):             Index from chart at right

Transmit Line Length:               Length of line sent

Call Sign:                          Your station call sign

Time Zone:                          Your time zone

Pace:                               Delay between chars sent

Length of Diddle:                   Number of chars in callup

diddle line

RTTY Port (A or B):                 Port A=COM1, B=COM2

ASCII Printer Port (A, B, or Z):    Which port for Printer?

Baud Rate Index (Printing):         From index at right

Printer Ready Low(Y or N)           DTS High="N"

Printer Line Length:                How long a printer line

Print Received (Y or N):            Print incoming data

Print Sent (Y or N):                Print outgoing data

Disk Out Mode ( R, T, B, of N):     What do you want to copy?

Pix Mode (Y or N):                  Pix, anyone?

Auto Run Mode:                      Start with RUN SCREEN

New Line for BAUDOT:                What will you have?

New Line for ASCII:                 Ditto.

Screen Saver:                       "Y" for 10 min. blank

Split Line Location:                Split line location

WRU on callsign only (Y or N):      Yes=response on ZW only

Page 3, RTTY 2200.00 IBM PC (MSDOS)

DISCUSSION:

DISK COPY:

This  program  can  "LOG" both incoming and outgoing copy.

In the field "DISK OUT MODE", "R" means "RECEIVE", "T"  means

"TRANSMIT",  "B"  means  "BOTH"  and "N" means "NEITHER".  If

this field has been programmed "N", and when  in  "RUN",  the

CONTROL-Y is pressed, the "N" will change to "R".  "DISK COPY

MODE"  comes  up automatically if "CONFIGURED" for "R","T" or

"B" and a fname.ext has  been  entered  in  the  "DISK  COPY"

field.   It is suggested that a fname.ext such as "JAN01.AAA"

be  used  so  that  each  day's  DISK  COPY  can  be   easily

identified.

MAX DISK OUT SIZE:

The  number  of  characters  for a disk output file can be

specified, to allow editing of the  file  by  small  editors.

When  the  output  file  reaches  this  size it is closed and

another is opened, with an incremented fname.ext.  If set  to

zero  of  blank, then the maximum will be the amount of space

left on the disk.

FILE NAMES :

All disk file names can include directory names.  The only

restriction is that the fully qualified name length can be no

more  than 26 characters.  For directory display, a path name

can be used and all files at the end of that  path  name  are

listed.   A blank path name defaults to the current drive and

directory.  If all files on a non-default  drive  are  to  be

listed, then the colon must be used.  e.g.  "d:"

OUTPUT FILE NAME:

The output file name is incremented in the last character

shown, even if there is no extension.

e.g. FILEA, FILEB, FILEC,......,FILEZ if named without an

extension.

FILEA.RYA, FILEA.RYB,......,FILEA.RYZ if named with an

extension, such as RYA...

END OF LINE SEQUENCES:

This  controls  the  sequence  sent whenever a new line is

started from the transmitted text.  It is  triggered  by  the

RETURN  key,  automatic wrap of a line being entered into the

buffer, or by a NEWLINE (0AH, same as  LINE  FEED)  character

from  a disk input file.  Baudot allows CARRIAGE RETURN (CR),

LINE FEED (LF), LETTERS (LTRS), or FIGURES (FIGS).  ASCII

allows CARRIAGE RETURN (CR), LINE FEED (LF), or NULL (00H).

If left blank, the default sequence is LINE FEED (one).

PROGRAMMABLE LINES:

Program the "PROGRAMMABLE LINES "A" and "B".  Line "A" and

"B" will be put into the "CONFIGURATION" record, but each can

be  changed  by  going  to  the "CONFIGURATION" screen at any

time.

Page 4, RTTY 2200.00 IBM PC (MSDOS)

AUTO RUN:

A  new  option has been added on the CONFIGURE screen that

allows specifying "AUTOMATIC or MANUAL" operation,  when  the

program  is  called.   This  will make possible the automatic

re-booting of the program, after a power glitch  or  failure,

and  continued operation of the RTY program.  The RTY program

will start with the RUN screen, if the item "AUTO  RUN  MODE"

is  a "Y".  To modify the configuration, the CONFIGURE screen

is still available by hitting the F1 key.  Of course, the RTY

program must be called from an "AUTOEXEC.BAT" file,  for  the

"AUTO RUN MODE" to be effective, upon a re-boot.

BAUD RATES:

The  lower  right  corner of the screen will show "Receive

Mode" if receiving and "Send Mode" if transmitting.  Just  to

the left of the Mode indication there will be a letter "B" or

"A"  and  a baud rate.These are from the configuration screen

and indicate Baudot or ASCII and the baud rate for  the  RTTY

input  and  output  speed.  The input and output speed can be

set from the "RUN" screen by hitting keys  "f2  or  f3".   f2

will program 45 baud BAUDOT, and f3 will step through all the

normal BAUDOT and ASCII baud rates.

PRINTER:

If  the printer has been programmed "ON" and the middle of

the 24th line shows "PRINTER", this means that the printer is

either not "READY" of the "HANDSHAKE" line  is  not  properly

set.

3RD AND 4TH LEVEL OPERATION:

One  of the newest features of this program is called "3rd

and 4th level BAUDOT".   This  is  a  programming  method  of

sending  the  full ASCII character set over a BAUDOT channel.

This  program  always  transmits  with  3rd  and  4th   level

characters, and is always available.

Unlock the "CAPS LOCK", to send lower case data.  The print

on  a TTY machine, or on a CRT (with other than this program),

will show as if it were  normal  Baudot.   Of  course,  normal

ASCII at 100 WPM can also be used, if desired.

Toggle  3rd  and 4th level receive by hitting the "f7 key",

(while in RUN) and note the "3/4" on the  left  of  the  Split

line,  which  means  that  the  3rd  and  4th level receive is

active.  The program comes up in 3/4 mode.

Irv  Hoff's  IRTTY  HDOS  program will receive 3rd and 4th

level BAUDOT and some of his HDOS and CP/M drivers will  send

3rd and 4th level.

DISK OPERATIONS:

When using "DISK COPY or DISK TRANSMIT", a word in the left

half  of  the 24th line will show "IN" or "OUT", meaning "Disk

INput of Disk OUTput".  If there is an error, then  the  error

will be indicated next to the "IN" or "OUT" depending on which

has the problem.

Page 5, RTTY 2200.00 IBM PC (MSDOS)

From  the  "RUN" screen, "Alt-N" will allow you to enter a

"d:fname.ext"  to  send  without  going  to  the  "CONFIGURE"

screen.   The entry made will be placed into the field called

"Filename for Sending".  End by typing RETURN.

"CTRL-D"  calls  up  the  file  to  be  sent, and a second

"CTRL-D", stops or aborts the file.  Call up the  file  where

ever in the pre-typing or transmit, the file should be sent.

SAVE  PRE-TYPE  BUFFER  TO  DISK.   Prepare the doc in the

pre-type buffer that you wish to put into a special fname.ext

file.  Then hit "Alt-N", and then follow  directions  on  the

lower  area  of  the  screen.   The  contents of the pre-type

buffer will remain unchanged but the data will be written  to

the  disk.  The pre-type buffer can then be cleared by typing

"Alt-V".  After that, the file can be reloaded, at  any  time

to send it, by typing "CRTL-D" in the normal manner.

DELETE  FILE  FROM  DISK.  To delete a file from disk, hit

the "Alt-N", and follow directions given on the lower lines.

HELP SCREEN:

From  the  "RUN"  screen,  hit  "Alt-H"  to see all of the

programs controls that are available.  Type "RETURN" to  exit

the  Help screen.  No incoming data will be lost when looking

at the control  menu.   The  unsent  pre-type  data  will  be

scrolled  back  into  the  pre-type area when the "RETURN" is

hit.  Transmit will stop while in the "HELP" screen, but will

resume when "RETURN" is hit.

DISPLAY OF DISK DIRECTORY.  Type "CTRL-A" while the "HELP"

screen  is displayed.  Then type the ID (in UPPERCASE) of the

drive on which there is a disk  mounted,  that  you  want  to

display the directory.Type "A","B","C", or "D" for the MS-DOS

drive  directory.   Type  "RETURN"  to  return  to the "HELP"

screen, and a second "RETURN" to return to the "RUN" mode.

FAST BREAKIN:

This  mode  is  toggled by "Alt-B".  The letters "bk" will

appear  on  the  left  of  the  split  line,  when  operating

"BREAKIN".   The  "CTRL-G" will send "-BK-", and turn off the

transmitter with out the 4 N's.  "CTRL-X" will  send  only  4

diddles,  and then the pre-typed data, when in this mode.  Do

not use "BREAKIN" mode unless you are truely using  "BREAKIN"

as  most  demodulators  can  not  start  printing with so few

start-up diddles.

RELAY:

In   the  "RELAY"  mode,  there  is  a  timer  to  prevent

untransmitted  relay  messages  to  linger  in  the  Pre-type

buffer.   Ths  timer  is  one  hour long, which prohibits any

relay message longer than 30 minutes.  If the timer  expires,

the  relay message is cleared and the relay function is armed

for another usage.

Page 6, RTTY 2200.00 IBM PC (MSDOS)

NOTE: To use the relay system, follow these steps:

1.   Send  call  of  station having a RTY program operational

with ZC.

callsignZC "CR" (puts relay station into RELAY MODE)

callsignZC "CR" (send a second time to assure reception)

2. Send normal message to station: "W0NW de W0SE etc."

3. Send "CR" then "four N's" and a "CR"

4. Send call of station being used for relay,with ZR

callsignZR (tells relay station to relay)

5. Hit CTRL-V to leave the air immediately

6. Watch relay being made.

PIX:

In  the  "PIX  MODE",  the "UNSHIFT ON SPACE" is disabled.

The shift stays the same as it was before the space.

BUFFERS:

Depending  of the amount of RAM available, the size of the

buffers is displayed at the  bottom  of  the  "CONFIGURATION"

screen.   There  are  three  buffers  of  this size: Received

Characters, Pretype for sending, and DISK OUTPUT.

LENGTH OF DIDDLE:

The number of diddles (LTRS in BAUDOT, NULL in ASCII),

sent at the start of each "non break-in" mode transmission,

can be specified. The value can be from 1 to 99, and if left

blank or set to zero, then the default value of 32 is used.

WRU on CALLSIGN:

If this item is set to "Y", the WRU will not respond to

the :DISK of :TIME command, but only to callsignZW. This mode

is indicated on the left of the split line by a reverse video

"I". A count is kept of the number of times a WRU is requested

of the station (whether it is sent of not). The count is dis-

played on the right of the split line. This count will alert

the operator of possible messages in the copy.

PACE:

To  smooth  out the transmitted output, when a slow typist

is typing in "real time", a delay between characters  can  be

programmed  on the line called "PACE".  Any number from blank

to 999 will  add  from  none,  to  999  millaseconds  between

characters, thus smoothing out the output. Characters are

still send at the programmed rate, but the average speed is

reduced to smooth out the typing.

Page 7, RTTY 2200.00 IBM PC (MSDOS)

Following are the controls for RTTY 2200.00 (MS-DOS)

CTRL-x means key x, while holding down the CONTROL key.

CTRL-A - Play string A

CTRL-B - Play BELL

CTRL-C - Enter twice to return to MS-DOS

CTRL-D - Toggle Disk Input to Transmit ON/OFF

CTRL-E - Play String B

CTRL-F - Replay Previous or Manual (ZR)

CTRL-G - Go to Receive After NNNN

CTRL-K - Kill Transmitter Immediately

CTRL-L - Turn up newline/fancy margin

CTRL-N - Play Station ID

CTRL-R - Play RYRY (U*U*U* in ASCII)

CTRL-T - Play Time/Date

CTRL-U - Send WRU

CTRL-V - Go to Receive Mode without NNNN

CTRL-W - Force DISK COPY write

CTRL-X - Go to Send Mode with X diddles

CTRL-Y - Toggle Disk Copy ON/OFF

CTRL-Z - Toggle Gimic ON/OFF (fig in upper right)

CTRL-_ - Clear Receive Screen

f1 Key - Toggle RUN/CONFIGURATION Screens

f2 Key - 45.45 baud (Baudot 60 WPM)

f3 Key - 50 baud to 9600 baud stepping

Alt-A - Toggle 3/4 Level Receive

Alt-B - Toggle Break Mode

Alt-C - Same as Ctl_C

Alt-F - Force Relay Record

Alt-H - Display This Screen (HELP)

Alt-N - Enter DIsk Input Fname

(or Save Pretype BUffer

or Delete File)

Alt-P - Clear Print Buffer

Alt-Q - Play "Quick Brown Fox etc."

Alt-R - Toggle Relay Mode

Alt-V - Clear Pretype Buffer

ESC-RETURN - Send Bare CR (PIX ONLY)

This version of RTY will respond to the following incoming

commands, if preceeded by either a CR of LF, and the Pre-type

buffer of the called station, is clear or empty.

callsignZC - Set RELAY to copy incoming

callsignZR - Relay message, with VIA etc.

callsignZP - Replay previous transmission

callsignZD - Put incoming to disk, if in DISK COPY

callsignZW - Play WRU

callsignZL - See Below Explanation for Remote WRU

callsignZM - See Below Explanation for Remote WRU

:TIME - Play all WRU'S

:DISK - Play all WRU'S if in DISK COPY

:xTIME - Play x call area WRU's (use 0 to 9 for x)

:xDISK - Play x call area WRU's (use 0 to 9 for x)

Page 8, RTTY 2200.00 IBM PC (MSDOS)

REMOTE WRU FUNCTION:

To  determine  if a remote station is active, when you can

not hear the  station  on  a  direct  path,  the  REMOTE  WRU

function is available.  The function will only work when this

version of the RTY program is in use at the relay station and

the  remote  station.   Any  local  station  can activate the

function.

By  the use of specific "Z" signals, a remote, but unheard

station, can be triggered to send its WRU,  through  a  relay

station, and thus be heard by the originating station.

Following is the procedure to activate this feature:

------------        -----------          -----------

: Original :        : Relay   :          : Remote  :

: Station  :----\---: Station :-----\----: STATION :

:          :        :         :          :         :

------------        -----------          -----------

Callsigns:

aaaaa              bbbbb                ccccc

Action:                             Comments:

aaaaa Sends:                        Calling station

bbbbbZLccccc<ret>                   Request bbbbb to call WRU

NNNN                                Hit CTRL-G, for NNNN's

-

bbbbb Sends:                        Response from bbbbb

cccccZMbbbbb                        Ask ccccc to relay WRU

NNNN                                Normal 4 N's

-

ccccc Sends:                        Response from ccccc

bbbbbZC                             Set relay at bbbbb

WRU for ccccc                       Normal WRU from ccccc

bbbbbZR                             Request bbbbb relay

-

bbbbb then relays the WRU of ccccc, for all to hear.

-

No operator intervention is required at any station, other

than  at the originating station.  The remote station must be

"ON", and its Pre-type buffer empty.

Have FUN!!!!  And have any kind of a day you would like.....

de W0SE AND W0NW...

##################################################################################################