Description of TTY Program

TTY requires advanced basic 1.1, communications adapter,

and 80 character wide screen.

Comm adapter pins used are (2) xmit data, (3) rcv data, and (20) DTR.

DTR is active (plus) when the program is in transmit mode, and may

be used to control t/r switching in the TU or transceiver.

TTY transmits and receives 5 bit teleprinter code (murray/baudot)

at 45.45 baud (60 wpm) in half duplex mode and is intended for use

in the amateur radio service.

This documentation assumes the reader is familiar with amateur radio

teleprinter conventions and practices.

CAUTION: The IBM PC comm adapter uses rs 232 input and output voltage

levels of +12v/-12v. Most commercial TUs use TTL levels 0v/+5v.

Conversion circuitry may be required.

The TTY menu provides for selection of receive or transmit mode.

Switching between transmit and receive modes is done by calling the

menu, using the F10 key.

In receive mode, the normal mode of operation is to ignore incoming

cr and lf characters, and print full lines of 80 characters to the

display and printer. However, the F1 key may be used to select a mode

of operation in which cr/lf characters are recognized, and normal

teleprinter operation is simulated. F1 toggles this function on/off.

F2 toggles the PC line printer on/off.The printer is used in receive

mode only.

In receive mode, the program is arranged for "unshift" on space,

cr and lf, as well as "ltrs".

In transmit mode, ltrs and figs shift characters are inserted

where needed by the program.Ltrs and figs cannot be sent directly

from the keyboard. The ascii cr (enter) key sends a cr/lf/ltrs

sequence.The program assumes that the receiving terminal is

equipped with unshift on space, and inserts figs accordingly.

F3,F4,F6,and F7 are used to send preprogrammed messages. These

messages reside at lines 17000,18000, 20000,and 21000, and may

readily be changed to meet the needs of your station.

F5 sends the required Morse code station identifier. This subroutine

starts at line 19060. Morse code is sent by calling dit, dah, and

intercharacter space subroutines at 19500,19600,and 19700 in sequence.

Change lines 19060 thru 19080 to send your callsign.

The program will accept either upper case or lower case keyboard entry.

Other ascii characters which have no equivalent in murray code are

ignored.

Hints and Kinks:

The `ignore cr/lf' mode is very useful when receiving conditions are

less than perfect, and produces much more reliable copy. However,if the

chap on the other end wants to send you some teleprinter art, use the

cr/lf `on' mode.

If the other station is using a conventional teleprinter, his line

length is probably 70 or 72 characters. TTY lets you send 80 character

lines which will overprint at the other station. Be careful!

In transmit mode, characters appear on the screen when they are sent

to the output buffer. The screen may be up to 128 characters ahead of

the comm adapter output! F5 or F10 (cwid or return to menu) are stored

and do not take effect until the buffer is empty. Hence the program

may appear to `hang' while the buffer is emptying. Be patient! If it

didn't work this way, some of your deathless prose would be lost down

the bit bucket!

73 es GL de Bob Johnson AA4L, 11305 Rums Hill, Raleigh NC 27614

phone 919 847 5606