WHATS-UP.DOC Page 1.

WHATS-UP (c) Joe Kasser, G3ZCZ, 1990

Member ASP

Joe Kasser G3ZCZ/W3

11241 Fairoak Drive

Silver Spring, Md. 20902

(301) 593 6136

G3ZCZ @ N4QQ

This is an interim documentation file.  A fuller document will be

published with release 1.00.

This version of WHATS-UP is a concept demonstration version of  a

Telemetry   Decoding  and  display  Program  for   the   MicroSat

Spacecraft  (OSCARs 16, 17, 18 and 19).  It is table  driven  via

the configuration files to allow maximum flexibility.

It contains the following features:

*    Real time and Playback modes.

*    Automatic Capture-to-disk of raw telemetry.

*    Link quality measurement.

*    Capability to print the raw telemetry as it is received.

*    Up to 16 user configurable display pages (screens).  You set

the  position on the page (width of engineering unit  field,

and number of decimal places) that a parameter is  displayed

at.

*    Wild card page (parameter shows up on all pages).

*    Selectable display of Engineering units or Hex byte for each

display page.

*    Display of raw packets (i.e. STATUS)

*    Color  changes if a parameter value changed between  succes

sive frames.

*    Audio and visual alarms if a parameter value exceeds,  falls

below or falls outside a preset limit value(s).

*    Dumb split screen terminal mode (a la LAN-LINK).

*    Customizable  colors, PC to TNC baud rate, data  parity  and

stop bits.

*    Default spacecraft configuration files.

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP.DOC Page 2.

*    Time of day clock display (in HH:MM:SS format)

WHATS-UP  has three basic modes of operation; Interactive,  real-

time and playback.

The  Interactive mode is a dumb terminal. You can use it to  give

commands  to the TNC. The usual one is to set the TNC  date  from

the  computer's  clock.   You  should also  use  it  to  set  the

'HEADERLINE  ON'.  In this mode, you will see the raw packets  on

the channel.  You can also use this mode as a regular TNC program

(If you do, you ought to get your head examined, because LAN-LINK

will  do the job much better).  The Real-time mode  converts  and

displays engineering data.  You can display up to 16  (configured

by  you) pages of information.  Information that changes  between

successive  frames, is shown in a different  color.   Information

that  has exceeded a preset (by you) limit is shown in  an  alarm

color (default: blinking red).  The capture-to-disk will turn  on

when  the first packet is copied, and will turn off  two  minutes

after the last.

The Playback mode allows you to play back captured telemetry with

4 speeds (speedy, slow, slower and snail's pace).

Starting the program.

You may start the program by typing in three ways as follows.

Default

Type  'whats-up'  and return (without the 's).  This  brings  the

program  up in the default mode. It will read  the  whats-up.sys

file to determine the spacecraft being monitored, and then prompt

you for the mode.

User Chosen spacecraft

Type  'whats-up  spacecraft' and return (without  the  's).   The

program reads the spacecraft.sys file to load the parameters for

the MicroSat of choice, and then prompts you for the mode.

Examples of the command are :

'WHATS-UP DOVE' or 'WHATS-UP PACSAT'

Custom Mode

Type  'whats-up  spacecraft mode' and return  (without  the  's).

This  brings the program up in the custom mode. It will read  the

whats-up.sys  file to determine the spacecraft  being  monitored,

and then start up in the mode you set.  Valid modes are 'p',  'i'

and 'r'.

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP.DOC Page 3.

Examples of the command are :

'WHATS-UP DOVE R' or 'WHATS-UP PACSAT R'

If you place a command line like this in your autoexec.bat  file,

should  you  be copying telemetry in an unattended manner  and  a

power  failure take place, the system will boot up into the  cor-

rect WHATS-UP mode when power is restored.

Contents of the SPACECRAFT.SYS file

In the telemetry processing part, only the quadratic equation  is

hard  coded, the coefficients are table driven from a  configura-

tion file. In fact the contents of the configuration file are  as

follows:-

Spacecraft ID.

Selected or default display page number

Number of display pages (maximum = 16)

Then follows the page definitions, as in for example;

4

SPACECRAFT HOUSEKEEPING, 30,31

PAYLOAD, 30, 31

BATTERY STATUS, 30, 30

MISC,30,2

the number above the page definitions must be set correctly.

For each page:

Page title

Page color

The  next set of items is the Telemetry parameter  configurations

(maximum = 64). You must have at least one of these lines in  the

file.   If  you  want a value to show in in more  than  one  page

(other than the wild card [0]) you must enter it twice (once  per

page). Typically each row looks as follows:-

00,Rx E/F Audio(W):,+0.000,+0.0246,0.000,V(p-p),2,11,1,6,1,0,0,0

01,Rx E/F Audio(N):,+0.000,+0.0246,0.000,V(p-p),2,11,32,6,1,0,0,0

For each telemetry parameter:

hexadecimal channel number (2 bytes, e.g. '0F' )

Description text string (e.g. '+Z Array Temp.')

Equation Coefficient C

Equation Coefficient B

Equation Coefficient A

Units text string (e.g. '.C')

Display page number

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP.DOC Page 4.

Display page row

Display page column

Display width for Engineering Units

Number of digits after the decimal point

Limit check flag (0 = do nothing, 1 = check below low limit,

2= check above high limit, 3 = check

for  [below low limit] or [above high

limit])

Low limit value (e.g. -4.00)

High limit value (e.g. +10.6)

A  line  with  an  '*' as the  first  character  terminates  that

section.

Packet types:-

packet types are set as follows:-

name of UNP address (e.g. WASH, BCXRT)

Display page for the contents of the packet

Display color

Display row

Display column

Display page for the cumulative count of the packet type

Display row

Display column

Typical entries are as follows:

WASH, 0, 30,4, 1, 0, 5, 1

TIME, 0, 31, 1, 1, 0, 5, 12

STATUS, 0, 29, 2, 1, 0, 5, 24

BCRXMT, 0, 28, 3, 1, 0, 5, 37

TLM, 99, 31, 3, 1, 0, 5, 50

These also terminate with an '*' character.

Limits are only checked for parameters being displayed (in  Engi-

neering  units).   With this approach, you can set  up  different

pages  for  different on-board subsystems, you can  also  set  up

different  display  pages of the same  parameters  for  daylight,

darkness  and  terminator crossing passes, with  different  limit

values to draw your attention to changes.

The  link quality part allows the following to be done.  You  can

define  which  of the packets you want to display/count  (If  you

just want to count them and not display them, set the page  value

to  99).   You can then view an incrementing count  counter  each

time  a  selected packet is received. You can  also  display  the

contents  of  the STATUS, WASH or BCXRT packets  in  any  display

page.

My  version (the default) is set up to display those  packets  as

wild  cards  (i.e will show up on all pages),  and  then  display

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP.DOC Page 5.

several temperatures and solar cell array currents.

Program Requirements

Hardware

IBM PC or clone with 256k memory.

TNC of some kind, I've tested it with a TNC2, a KAM and a  PK-232

so far.

Data Format

The  program  DOES require that the telemetry  be  received,  and

captured-to-disk  with the packet header on a different  line  to

the  contents of the packet (HEADERLINE ON). I also suggest  that

you  turn  the date/time stamp on so that when you  get  Revision

1.00, you will be able to playback your data and extract selected

values and their corresponding time codes into a file that can be

read into your spreadsheet program for further trend analysis.

A typical display is shown below.

SPACECRAFT HOUSEKEEPING

PHT: uptime is 046/12:38:33.  Time is Tue Jan 30 03:07:27 1990

80 00 00 89 B0 18 DD 02 00 B0 00 00 B0 00 00 00 00 00 00 00

vbat= 10.864 vlo1= 10.540 vlo2= 10.040 vmax= 11.540 temp=  6.661

wash addr:2a80:0000, edac=0x70

WASH    28 TIME    22  STATUS    21 BCRXMT     2 TLM    17

+Z Array Cur   :-0.011 A +Z Array Temp  :  -6.7 C  +Z Array V     :   0.1 V

-Z Array Cur   :-0.017 A +Y Array Temp  :  10.9 C

-Y Array Cur   :-0.012 A Baseplt Temp   :   6.7 C  BCR Load Cur   : 0.164 A

+X Array Cur   :-0.013 A Array V        :   9.8 V

-X Array Cur   :-0.011 A +5 Volt Bus    :   4.9 V  +8.5V Bus      :   8.5 V

+10V Bus       :  10.8 V  8.5V BUS       :   8.4 V

TX#1 RF OUT    :   0.0 W +8.5V Bus Cur  : 0.053 A

TX#2 RF OUT    :   3.7 W +5V Bus Cur    : 0.208 A  BCR Input Cur  : 0.191 A

BCR Output Cur :-0.017 A

Bat 1 Temp     :   6.7 C Bat 2 Temp     : -20.6 C

IR Detector    :     0 Counts

BCR Set Point  :   134 Counts

WHATS-UP.SYS file format

The contents of the WHATS-UP.SYS file are as follows:

Your callsign           (e.g. G3ZCZ)

Default configuration file) (e.g. Dove)

station latitude        (e.g. 35.00)

station longitude       (e.g. 74.00)

station all.            (e.g. 100)

capture-to-disk file    (e.g. dove.RUN)

playback file           (e.g. dove.run)

PC serial port          (e.g. 1)

PC serial baud rate     (e.g. 9600)

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP.DOC Page 6.

data bits               (e.g. 8)

Stop bits               (e.g. 1)

parity                  (e.g. 0)

status (top) window color (e.g. 79)

Telemetry Page color    (e.g. 14)

outgoing window color   (e.g. 14)

incoming window color   (e.g. 30)

prompt window color     (e.g. 15)

alarm window color      (e.g. 15)

bottom window color     (e.g. 79)

Emphasis color          (e.g. 14)

prompt color            (e.g. 14)

option color            (e.g. 78)

parameter changed color (e.g. 95)

parameter limit exceeded color  (e.g. 14)

The callsign and geographic parameters are not used in this  ver-

sion.

Some typical color combinations are

Function Keys

The following function keys are active:

FK 1 capture to disk

FK 2 type of display Engineering Units/Hex Byte

FK 3 select display page

Alt-B send a 'break' to the TNC, interactive mode only.

Alt-P Printer on/off

Alt-S Sound on/off

Alt-X Quit

left arrow decreases playback speed

right arrow increases playback speed.

This  version  as I mentioned above is  a  concept  demonstration

version,  and  contains a few cosmetic display bugs.    I  expect

(but don't promise) to put out Revision 1.00 with more  features,

(such as extraction capability to interface the extracted data to

a spreadsheet program for further analysis) and proper documenta-

tion, sometime between March and June 1990.  This program is  not

in  the  public domain but may be copied and passed on.   If  you

would  like  a copy of Revision 1.00 which will  be  released  as

shareware, send a FORMATTED disk and sase to

Joe  Kasser  G3ZCZ/W3, 11241 Fairoak Drive,  Silver  Spring,  Md.

20902.

Better yet, why not register your copy and support development of

this  kind of software.  Registered users will receive  all  ver-

sions 1.00 to 2.00 automatically.

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP.DOC Page 7.

If you have captured any telemetry to disk, especially daytime or

southern  hemisphere passes, please put them on the disk.   Don't

let  the  data files become too long. I suggest that  you  rename

them  every  couple of days or so, such as  DOVE2-16.RUN,  DOVE2-

18.RUN, etc.

Suggestions for improvements and additions are always welcome.

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP.DOC Page 8.

Other Amateur Radio Software by Joe Kasser G3ZCZ

PC-HAM 3.3 BY G3ZCZ

LOGBOOK

Full  blown  logging package.  With automatic check of  logs  for

awards such as DXCC.  Allows you to recall any entry by call sign

within  seconds. Indexed displays, QSL'ing, Contest  mode  QSLing

(prints  the lot) and lots more written in dBASE3, but a compiled

LOGBOOK.EXE  file is  supplied  together  with  the  source code.

Export your  LAN-LINK.LOG file and put this package to work.

Version 3.2 DOES NOT NEED DBASE TO RUN.

To see what this program can do, extract the documentation file in

LOGDOC and read it.

CONTEST

Keeps Dupes in memory, logs QSO's to disk in format which can  be

processed by the LOGBOOK package.

CQSS

Sweepstakes  game.   Work the ARRL Sweepstakes  contest  on  your

computer.   You  are  located  just  outside  Washington  DC.   A

propagation  model is built in to the program.  This  program  is

REQUIRED  training  for all sweepstakes  operators.   Program  is

based on the one described in detail in 'Software for Amateur

Radio' by Joe Kasser G3ZCZ, published by TAB Books, Blue Ridge

Summit, PA. 17214.

WHATSON

Predict HF Propagation. Contest mode with printout to whole world

at hourly intervals. Needs BASIC

This  software  AND MORE comprises PC-HAM which is  available  as

Shareware  from G3ZCZ.  Send a disk, SASE, and a QSL card  for  a

copy.   Users  are asked to register for $36.50  (half  of  $73).

Request PC-HAM.

For an evaluation copy (unregistered full blown version) of each

software send a formatted disk and SASE to Joe Kasser.  Overseas

users,  send  $3.00  instead. Try  the  software first, and only

register your copy if you make use of it.  If you like LAN-LINK,

you will like PC-HAM.

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP.DOC Page 9.

LAN-LINK 1.55  Registration Fee $35.00

PC-HAM   3.3   Registration Fee $36.50

WHATS-UP 0.3   Registration Fee $35.00

Evaluation Copies free, you pay only for the disk/postage.

To: Joe Kasser G3ZCZ

P O BOX 3419

SILVER SPRING

MD 20918.

CALL ________________   TODAY'S DATE _____________

NAME ______________________________________________

ADDRESS ___________________________________________

___________________________________________

CITY    ___________________________________________

STATE   _________  POSTCODE  ______________________

TNC TYPE _____________________ Home BBS __________

I enclose a check for ______.  Please send me the latest  version

of ______________,  and register me as  a user.

I enclose a disk/sase or $3.00. Please send me the latest version of

______________________ to evaluate.  If I like it and use it, I plan

to become a registered user.

Disk Size 5.25 ______ 3.5 ______

I AM NOT/AM currently using Shareware Version _____

which I obtained from  _________________________________________.

Signature

COPYRIGHT Joe Kasser, G3ZCZ 1990.

############################################################################################