W0RLI MailBox and GateWay	- Configuration File	  Page 1

Sysop Information.

The basic information about the system owner and system operation is

contained in the file INIT.MB. Replace the information with your own.

The Login Message.

The login message is contained in the file MOTD.MB

Variable text fields can be used in MOTD.MB, just as in CONFIG.MB

The Configuration File.

The file CONFIG.MB is a text file that contains all site-specific

parameters. Edit it to have the proper parameters for your site.

The form $x is a variable text field. The "$x" is replaced by

the current value for that text.

$B - Users home bbs.

$D - The current date.

$F - Name of the users port.

$f - Name of the "other" gateway port.

$H - Hang at end of line (suppress carriage return). Use at end of line only.

$h - Number of held messages.

$I - Sysops name.

$L - Number of the last message in the MailBox

$M - Message number from current msg header.

$N - Number of active messages.

$n - Number of killed messages.

$O - Sysops callsign.

$P - Port ID

$Q - Sysops QTH

$T - The current time.

$t - Task ID.

$U - User callsign.

$V - Software version.

$W - Users name.

$X - Date user last logged in.

$Y - Time user last logged in.

$Z - Users zip code.

Any line beginning with ";" is ignored.

This can be used to comment CONFIG.MB.

W0RLI MailBox and GateWay	- Configuration File	  Page 2

The Configuration File, section one.

The first section (to the first *** EOF) is the port configuration info.

There must be one line defining your printer:

Type can be one of LPTx, COMx, or NONE

Examples:

Printer NONE

Printer COM5

Printer LPT2

There are three (or more) lines for each communications port.

The first line contains the port definition.

The second line is the port name.

The third line contains the port privileges and timing information.

Any lines following the third line are commands to send to the

TNC at the time the software is initialized.

The port definition line is made of three fields, seperated by blanks:

The first field contains the word "Port".

The second field is the port identifier.

The third field is the type of device connected to the port:

AEA     - This port has a TNC with AEA commands.

Console - This port is the local console.

KAM     - This port has a TNC with KAM / KPC-4 commands.

Node    - This port connects to G8BPQ switch software.

Serial  - This port has a modem, computer, terminal, or printer connected.

TNC2    - This port has a TNC with TAPR commands.

Examples:

Port A TNC2

Port L Console

Port G Node

The second line contains the port name.

Examples:

144.91 Mhz

Local LAN link frequency.

The third line is made of several fields, seperated by blanks:

Field 1:

A list of characters that give information about the port:

B - Only BBS may connect on this port.

D - File download allowed from this port.

F - The TNC on this port will be put in "converse" mode

instead of "semi-transparent" mode.

G - GateWay allowed from this port.

I - Kick off user that connects using illegal call.

M - Monitoring allowed from this port.

P - If remote sysop is allowed, password is required.

R - Remote sysop allowed on this port (if user marked as sysop).

U - File upload allowed from this port.

1 - Echo monitored packets to the console.

2 - Echo user data and forwarding to the console.

3 - Echo TNC commands to the console.

4 - NEVER monitor (i.e. monitor always set to OFF).

Field 2: Connect timeout, in seconds.

Field 3: Disconnect timeout, in seconds.

Field 4: Monitor time, in seconds.

Field 5: Monitor lines.

Field 6: Number of digipeaters allowed on connect.

Field 7: Minute of the hour to attempt forwarding.

Note that any value above 59 will disable forwarding on the port.

Field 8: Number of command errors allowed before user kicked off.

Field 9: Maximum users in RoundTable. Zero disables for this port.

W0RLI MailBox and GateWay	- Configuration File	  Page 3

The Configuration File, section two.

The second section (to the second *** EOF) is the user file directory

path definitions. Three lines per path.

The first line is a single character path ID,

followed by "D" if downloading is allowed, and "U" if uploading is allowed.

The second line is the path, with trailing '\'.

The third line is the name of the path, as shown to the user.

The Configuration File, section three.

The third section (to the third *** EOF) is the designator translation list.

Each line in this section instructs the MailBox to take some action

when it recieves some specific kind of message. Each line has several

fields. The first field tells WHAT action to take (@, D, H, or T).

The second and following fields tell HOW to take that action.

Action code   Meaning

@	      Translate @ BBS field of message.

B	      File message by BID

D	      Use the distribution list given in field 4,

if TO matches field 2 and @ BBS matches field 3.

H	      Hold any message with this TO, FROM, or @ BBS.

T	      Translate TO field of message.

The examples below show how to use this feature:

@ W0RLI 		 - Remove the "@ BBS" if it is this BBS.

@ NCNET ALLCAN		 - Any message "@ NCNET" becomes "@ ALLCAN".

T SYSOPS SYSOP		 - Any message "TO SYSOPS" becomes "TO SYSOP".

H N1NERD		 - Hold any message to/from/at N1NERD

D * ALLUSA WORLD.DIS	 - Any message @ ALLUSA go to all calls

in file WORLD.DIS

D * NORCAL NCNET.DIS	 - Any message @ NORCAL go to all calls

in file NCNET.DIS

D * ALLCAN NCNET.DIS	 - So does any messages @ ALLCAN ...

D AMSAT *  NCNET.DIS	 - ... and all messages TO AMSAT ...

B ARL* \bull\arrl\	 - File ARRL fulletins.

*** EOF

There can be up to 35 destinations in a distribution list.

My file NCNET.DIS contains:

kb6irs

n6iya

ki6eh

aa4re

kb6owt

wa6rdh

w6pw

w0rli	 <-   I put my own call in the list so the message will

not automatically kill until after I have read it.

The message is not forwarded to these stations in any special order.

If a station is busy then the MailBox will try again the next hour.

An "L" listing of a message with a distribution list shows the sysop the

status of forwarding to each station on a second "cc:" line. The calls to

which the message have been sent have an asterisk before them.

The Configuration File, section four (remainder of file).

Each item goes on one line. Various prompts, messages, and constants.