1/1/92

DigiBoard Non-Intelligent board OS/2 Device Driver    Version 1.4.1

Installation Guide

Files

There are 2 files on the distribution diskette that you will need to

install the PC/X driver:

DGX.SYS	        - The executable device driver.

INSTALL.DOC	- this file.

Additionally, a utility program to assist in establishing communications

settings on the additional serial ports is included.

DMODE.EXE	- a 'MODE' work-alike utility

DMODE.DOC	- related documentation for above program.

Note that if the driver file (DGX.SYS) is not placed in the root

directory, its full path must be given on the line in the CONFIG.SYS

file.

In order to have the driver load, the following line must be included

in you 'CONFIG.SYS' file.

driver=dgx.sys

Without any arguments thus the driver comes up with the following defaults:

[ Port addresses in Hexadecimal ]

1 8-channel PC/8 board

Status Port = 140

Channel I/O port addresses 100, 108, 110, 118, 120, 128, 130, 138

Device names are assigned to COM3 COM4 COM5 COM6 COM7 COM8 COM9 COM10

The default modes for each of the channels follow the MS OS/2 standards of:

1200 Baud, 7 data, 1 stop, Even Parity, RTS control-mode.

GLOBAL Command line arguments:

/n:device_names		sets up the naming algorithm to be used to

generate device names.  Consists of an

optional new stem followed by the starting

device number. Without the new stem, 'COM'

will be used; without a number 3 will be used.

Numbering of channels will continue contiguously

from one board to the next in the order that

the boards are specified on the command line.

/n:"digi"0	DIGI0,DIGI1,DIGI2,....DIGI7

/n:4		COM4,COM5,COM6,....COM11

/b:baud_rate		OS/2 by default sets up all serial devices

at 1200 baud at initialization-time.  Use this

option to override that value.  Valid values

are:   300,1200,2400,4800,9600

/b:9600		[ all channels initialize to 9600 baud ]

/d:data_type		OS/2 by default sets up all serial devices

to Even Parity, 7 data bits, 1 stop bit at

initialization-time.  Use this option to

override those value.   Syntax is

ParityDataStop

[E|O|N] [8|7|6|5|] [1|2]

All three values MUST be provided.

/d:N81		[ all channels initialize to No parity, ]

[       8 data bits, 1 stop bit         ]

PER-CARD arguments to the driver MUST be in complete sets indentifying

Port address range and IRQ number. It is HIGHLY reccomended to

fully specify ALL boards in your system right on the command line

to ensure that current settings are plainly visible.

----------------

PC/16 Exception:

----------------

Since the I/O port settings for the PC/16 are set at the factory and

are NOT configureable by the user, any PC/16 MUST !! be specified

with the following set of arguments:

device=dgx.sys /p:140 /c:16 /i:[interrupt_line]

The '/p:140' and '/c:16' arguments are very important as they tell

the driver that this is a 16-channel board!!

If any of the below arguments ar given, ALL cards in the system must

be thus fully described.  In other words, if you are running ANY

other configuration except the default (see above 'Default

Configuration'), then the each card in your system MUST BE FULLY

DESCRIBED by these arguments.

/p:io_port_address	set the I/O port address range ceiling.

This argument specifies the top of the

[Hexadecimal] 40-port range of I/O port

addresses that will be used by the PC/X.

This address should correspond to the

dip-switch setting of the Status Port.

Individual channel port addresses should then

begin [hex] 40 BELOW this address, occupying

a congiguous block, each channel a [hex] 8

step from the previous.

/p:280		[ set card to Status port address 280 (hex)]

[ IO Port for Channel 1 = 240 (hex)        ]

[ IO Port for Channel 2 = 248 (hex)        ]

[ IO Port for Channel 3 = 250 (hex)        ]

[ IO Port for Channel 4 = 258 (hex)        ]

[              ....  etc.   .......        ]

/i:IRQ_num		set the IRQ line number from among the legal

options for the PC/X.

/i:12		[ set card to IRQ 12 (decimal)		   ]

/c:channels		set the number of channels (4, 8 or 16)

This is required for each card that does NOT

have 8 channels on it.

/c:4		[ assume only 4 channels on card	   ]

[ NOTE: The driver may crash the System    ]

[       if you only have a 4-port PC/X    ]

[       and you do not specify this option ]

Trouble-shooting:

If the driver comes up with a problem message please check that your

configuration matches the switch settings on the card.  If it still

fails, try actually powering down the system and then back up.  If it

STILL fails, give DigiBoard Technical Support a call and talk it thru.

The switches for default configuration are as follows:

1    2    3    4    5    6    7    8    9   10	address

DS1	    OFF       OFF                               0x140 (status)

DS2	    OFF                                         0x100 (COM3)

DS3	    OFF                      OFF                0x108 (COM4)

DS4	    OFF                 OFF                     0x110 (COM5)

DS5	    OFF                 OFF  OFF                0x118 (COM6)

DS6	    OFF            OFF                          0x120 (COM7)

DS7	    OFF            OFF       OFF                0x128 (COM8)

DS8	    OFF            OFF  OFF                     0x130 (COM9)

DS9	    OFF            OFF  OFF  OFF                0x138 (COM10)

J1 thru J8 should be strapping 1-2 (top)

J9 and J10 should be strapping 2,3 (bottom)

IRQ jumper should be on J86 (for IRQ5)

In general, a good way to test the basic functionality of the driver is

to hook up a terminal to one of the ports; (say, for example, COM3)

and set it up for the correct Baud rate, Handshake, and Data type

characteristics; and simply try to re-direct output to it from the

command prompt.

C:\>dir > com3

If the terminal is set up correctly (typically the above defaults) you

should get the directory listing comming out on it.  If there is no

output, or if the above commands hangs,  then you may need some technical

support.  It is possible that the board is not functioning properly.