IBM Token-Ring Network 16/4 Busmaster Server Adapter/A

NDIS Device Driver User's Guide

(C) Copyright IBM Corp. 1990, 1991.

TRADEMARKS

3Com is a trademark of 3Com Corporation

Microsoft is a trademark of the Microsoft Corporation

Micro Channel is a trademark of the IBM Corporation

OS/2 is a trademark of the IBM Corporation

Page 1

IBM Token-Ring Network 16/4 Busmaster Server Adapter/A

NDIS Device Driver User's Guide

INTRODUCTION

The IBM Token-Ring Network 16/4 Busmaster Server Adapter /A Network

Driver Interface Specification (NDIS) device driver for the OS/2

program provides the interface between the IBM Token-Ring Network 16/4

Busmaster Server Adapter /A and a NDIS protocol stack. The device driver

delivers received frames from the adapter card to the protocol handler

and sends frames from the protocol handler to the adapter card to be

transmitted on the token-ring network.  The device driver does not

examine frame contents.

The IBM Token-Ring Network 16/4 Busmaster Server Adapter /A NDIS device

driver supports the NDIS interface specified in the 3COM/Microsoft

LAN Manager Network Driver Interface Specification, version 1.0.2.

INSTALLATION

Device Driver Files

To install the required device driver files:

1.  Create a directory on the target machine

2.  Obtain the following files from the IBM Bulletin Board:

*  IBMTRBM.OS2

*  IBMTRBM.286

*  LT4.MSG

*  LT4H.MSG

3.   Copy the files listed in Step 2 into the newly created directory.

4.   Copy the following files from the options diskette distributed with

the IBM Token-Ring Network 16/4 Busmaster Server Adapter /A into

the newly created directory:

*  WRTRAM.BIN

*  MONT400.BIN

NOTE -  The IBMTRBM.OS2 file copied from the IBM bulletin board

supports only 80386/80486 processors.  If the target machine

contains a 80286 processor, then follow these steps to install the

proper version of the IBM Token-Ring Network 16/4 Busmaster Server

Adapter /A NDIS device driver:

1.  Rename IBMTRBM.OS2 to IBMTRBM.386

2.  Rename IBMTRBM.286 to IBMTRBM.OS2

Page 2

IBM Token-Ring Network 16/4 Busmaster Server Adapter/A

NDIS Device Driver User's Guide (con't)

CONFIG.SYS File

Add a "DEVICE =" statement after the Protocol Manager device driver

statement in the CONFIG.SYS file.  A single "DEVICE = " statement is

required for the IBM Token-Ring Network 16/4 Busmaster Server Adapter /A

NDIS device driver, even when supporting multiple adapters.

Add the following "DEVICE =" statement to the CONFIG.SYS file to allow

the IBM Token-Ring Network 16/4 Busmaster Server Adapter/A NDIS device

driver to initialize:

DEVICE = d:\pathname\IBMTRBM.OS2

where d:\pathname\ is the drive and path where the IBM Token-Ring

Network 16/4 Busmaster Server Adapter /A NDIS device driver has

been installed.

CONFIGURING THE DEVICE DRIVER

The configuration file for the IBM Token-Ring Network 16/4 Busmaster

Server Adapter /A NDIS device driver is the PROTOCOL.INI file.  The

following information describes the configuration parameters

accepted by the IBM Token-Ring Network 16/4 Busmaster Server Adapter

/A NDIS device driver.  For more information about the PROTOCOL.INI

file, see the 3COM/Microsoft Lan Manager Network Driver Interface

Specification.

Page 3

IBM Token-Ring Network 16/4 Busmaster Server Adapter/A

NDIS Device Driver User's Guide (con't)

Default PROTOCOL.INI Configuration

The device driver accepts 10 parameters (see Modifying PROTOCOL.INI

Parameters) in the PROTOCOL.INI file.  The PROTOCOL.INI file should

contain a section for each IBM Token-Ring Network 16/4 Busmaster

Server Adapter /A installed in the target machine that will be used

for token-ring communications.

The following is an example of a section in a PROTOCOL.INI file that

provides the configuration parameters to load and initialize an

IBM Token-Ring Network 16/4 Busmaster Server Adapter /A.

The section name (in the following, TOKEN1) must be different for

each adapter's configuration.

[TOKEN1]

DriverName   = IBMTRBM$

BootStrap    = C:\IBMLAN\DRIVERS\TOKENRNG\WRTRAM.BIN

MicroCode    = C:\IBMLAN\DRIVERS\TOKENRNG\MONT400.BIN

AdapBuffSize = 1032

AdapMinTran  = 16

AdapMaxTran  = 40

MinRcvBuffs  = 16

SizWorkBuf   = 2048

MaxTransmits = 10

NetAddress   = "400000000077"

Page 4

IBM Token-Ring Network 16/4 Busmaster Server Adapter/A

NDIS Device Driver User's Guide (con't)

The following is a example of a PROTOCOL.INI file that contains two

adapter configuration sections:

[TOKEN1]

DriverName   = IBMTRBM$

BootStrap    = C:\IBMLAN\DRIVERS\TOKENRNG\WRTRAM.BIN

MicroCode    = C:\IBMLAN\DRIVERS\TOKENRNG\MONT400.BIN

AdapBuffSize = 1032

AdapMinTran  = 16

AdapMaxTran  = 40

MinRcvBuffs  = 16

SizWorkBuf   = 2048

MaxTransmits = 10

NetAddress   = "400000000077"

[TOKEN2]

DriverName   = IBMTRBM$

BootStrap    = C:\IBMLAN\DRIVERS\TOKENRNG\WRTRAM.BIN

MicroCode    = C:\IBMLAN\DRIVERS\TOKENRNG\MONT400.BIN

AdapBuffSize = 1032

AdapMinTran  = 16

AdapMaxTran  = 40

MinRcvBuffs  = 16

SizWorkBuf   = 2048

MaxTransmits = 10

NetAddress   = "400000000078"

Page 5

IBM Token-Ring Network 16/4 Busmaster Server Adapter/A

NDIS Device Driver User's Guide (con't)

Modifying PROTOCOL.INI Parameters

Edit the PROTOCOL.INI file to modify the parameters.  All the keyword

parameters for the IBM Token-Ring Network 16/4 Busmaster Server Adapter

/A NDIS device driver can be modified except the DriverName parameter.

The following information describes each keyword parameter and dis-

cusses the possible values.  When modifying any vaules, use caution

to select a new value close to the defined default.  Using the maximum

values for several of the parameters at the same time may cause the

device driver to fail to load.  It is difficult to define the upper

bound that is valid.  Use values near the accepted default as a

guideline.

PROTOCOL.INI Parameters

DriverName

This required parameter indicates the network adapter driver

name for the module. This parameter must be set to IBMTRBM$.

Numerics before or after the $ are not supported.  If omitted,

the IBM Token-Ring Network 16/4 Busmaster Server Adapter /A

NDIS device driver will ignore that section of the PROTOCOL.INI

file.

BootStrap

This parameter specifies the path to the network adapter card

loader program, WRTRAM.BIN.  The network adapter card loader

program is downloaded onto the network adapter card.  Then

the network adapter card loader program downloads the network

adapter card program onto the IBM Token-Ring Network 16/4 Bus-

master Server Adapter /A.  If this parameter is omitted, the

default C:\WRTRAM.BIN is used as the location and name of the

adapter card loader program.

Page 6

IBM Token-Ring Network 16/4 Busmaster Server Adapter/A

NDIS Device Driver User's Guide (con't)

MicroCode

This parameter specifies the path to the network adapter card

program, MONT400.BIN.  If this parameter is omitted, the de-

fault C:\MONT400.BIN is as the location and name of the adap-

ter card program.  Refer to the BootStrap parameter for more

information.

AdapBuffSize

This parameter specifies the size of the buffers on the net-

work adapter card.  The value of this parameter must be a

multiple of 8.  The size of the buffers is actually 8 bytes

less than the amount specified, because there are 8 bytes of

overhead associate with each buffer.

Default value:  1032

Minimum value:    96

Maximum value:  2048

If this parameter is omitted the default value of 1032 is used.

AdapMinTran

This parameter specifies the minimum amount of buffer resources

reserved for transmit buffers.

Default value:    4

Minimum value:    4

Maximum value:  676

If this keyword is omitted the default value of 4 is be used.

AdapMaxTran

This parameter specifies the maximum amount of buffer resources

reserved for transmit buffers.

Default value:    6

Minimum value:   40

Maximum value:  676

By limiting the maximum number of buffer resources used for

transmission,  the buffer resources used for receiving frames

is defined. If this parameter is omitted the default value of

40 will be used.

Page 7

IBM Token-Ring Network 16/4 Busmaster Server Adapter/A

NDIS Device Driver User's Guide (con't)

MaxTransmits

This parameter specifies the maximum number of transmit queue

entries. The network adapter driver allocates approximately

260 bytes for each transmit queue entry.  The value of this

parameter should be high enough to accommodate the sum of all

MaxTransmits parameter values for all protocol drivers using

the adapter concurrently.

Default value:     10

Minimum value:      1

Maximum value:  65535

If this keyword is omitted a value of 10 is used.

MinRcvBuffs

This parameter specifies the minimum number of receive buffers

that are allocated.  For a 4MB token-ring network, the value

of this parameter must have the following relationship:

minrcvbuffs x sizworkbuf >= 4500 bytes

For a 16MB token-ring network, the value of this parameter

must have the following relationship:

minrcvbuffs x sizworkbuf >= 18000 bytes

If the number of buffers is excessive and the size of the

buffers unusually large (see SizWorkBuf), then the device

driver may fail to load.  The values selected for these key-

word parameters should be within close range of the provided

defaults.

Default value:     16

Minimum value:      1

Maximum value:  65535

If this keyword is omitted a value of 16 is used.

Page 8

IBM Token-Ring Network 16/4 Busmaster Server Adapter/A

NDIS Device Driver User's Guide (con't)

SizWorkBuf

This parameter specifies the size of each receive buffer.

The value of this parameter should be high enough to accomodate

the largest frame of any protocol driver using the network

adapter driver.  Refer to the MinRcvBuffs parameter for more

information about the value of this parameter.

Default value:   2048

Minimum value:      1

Maximum value:  65535

If this keyword is omitted a value of 2048 is used.

NetAddress

This parameter overrides the network address of the network

adapter card.  Specify this parameter only if there are dup-

licate addresses on the netowrk.  The value of this parameter

is a hexadecimal string of 12 digits enclosed in quotes, as

in "400001020304".  The address must be unique among all other

network adapter addresses on the network. Specify the network

address in IBM Token-Ring network format.  If the protocol

driver specifies a network adapter address for the network

adapter driver, that value becomes the default value for the

network adapter driver.

Default value:   Universially administered address (burned-in address)

Minimum value:   "400000000000"

Maximum value:   "40007FFFFFFF"

Configuring Multiple Adapters

The PROTOCOL.INI parameters for the first DriverName = IBMTRBM$

applies to the adapter in the lowest Micro Channel slot number.

The second DriverName = keyword applies to the adapter card in

the next higher slot, and so on.

Adapters are initialized based on occurrences of the DriverName =

IBMTRBM$ statement in a section in the PROTOCOL.INI file, regardless

of bind specifications.

Resource Allocation

Parameters are processed for each adapter.  Resources allocated for

each adapter are reserved for use by only that adapter.

Page 9

IBM Token-Ring Network 16/4 Busmaster Server Adapter/A

NDIS Device Driver User's Guide (con't)

RESTRICTIONS

Initialization Failure

If a section in the PROTOCOL.INI file is incorrect and a particular

adapter cannot be loaded, the IBM Token-Ring Network 16/4 Busmaster

Server Adapter /A NDIS device driver will not load any of the adap-

ters and will not complete initialization.  In this situation, the

IBM Token-Ring Network 16/4 Busmaster Server Adapter /A NDIS device

driver cannot be used by a protocol stack until the PROTOCOL.INI

section is fixed.

If a adapter card cannot be loaded due to a hardware fault, the

adapter should be removed. Otherwise, the faulting adapter will

prevent all the other IBM Token-Ring Network 16/4 Busmaster Server

adapters from being loaded and being used by the protocol stack.

DOS Mode Support

The IBM Token-Ring Network 16/4 Busmaster Server Adapter /A NDIS

device driver will execute in a protect mode system only.  The

PROTECTMODE = YES statement should be included in the CONFIG.SYS.

Interrupt Level

All IBM Token-Ring Network 16/4 Busmaster Server adapters must exe-

cute using the same interrupt level. SETUP should be used to change

the rings to use the same interrupt level.  By default, SETUP places

adapters on different interrupt levels.

The interrupt level that is chosen may be shared with other devices.

NDIS Restrictions

The following NDIS general requests are not supported by the IBM

Token-Ring Network 16/4 Busmaster Server Adapter /A NDIS device

driver:

*  ResetMac

*  Interrupt

*  StartReset

*  EndReset.

The following NDIS general requests are supported but with

restrictions:

*  TransmitChain - Only physical addresses are supported

*  SetStationAddress - May only be issued before the adapter is

opened using the NDIS OPENADAPTER general request.

Page 10

IBM Token-Ring Network 16/4 Busmaster Server Adapter/A

NDIS Device Driver User's Guide (con't)

MESSAGES

LT40004E   The microcode file xxxxxxxx could not be opened.

Cause:  An unexpected error occurred when the

program attempted to open the microcode file.

Action:  Check the drive and directory to ensure that

the microcode file exists.  Check the microcode file name

specified in PROTOCOL.INI to ensure that it is correct.

If the problem persists, reinstall the microcode file in

the proper directory.

LT40005E   The microcode file xxxxxxxx could not be read.

Cause:  An unexpected error occurred when the

program attempted to read the microcode file.

Action:  Reinstall the microcode file in the proper

directory.

LT40006E   The bootstrap file xxxxxxxx could not be opened.

Cause:  An unexpected error occurred when the

program attempted to open the bootstrap file.

Action:  Check the drive and directory to ensure

that the bootstrap file exists.  Check the bootstrap

file name specified in PROTOCOL.INI to ensure that

it is correct.

If the problem persists, reinstall the bootstrap file

in the proper directory.

LT40007E   The bootstrap file xxxxxxxx could not be read.

Cause:  An unexpected error occurred when the

program attempted to read the bootstrap file.

Action:  Reinstall the bootstrap file in the proper

directory.

Page 11

IBM Token-Ring Network 16/4 Busmaster Server Adapter/A

NDIS Device Driver User's Guide (con't)

LT40008E   The parameter specified in PROTOCOL.INI for

keyword xxxxxxxxxxxx is not the proper character

type.

Cause:  The value provided for the keyword in the

IBMTRBM module in PROTOCOL.INI is not the correct type.

Action:  Change the value in PROTOCOL.INI to

the correct type for the parameter.

LT40009E

An unrecoverable error occurred in the

IBM Token-Ring Network 16/4 Busmaster Server Adapter/A NDIS

device driver program.

Cause:  An internal software error occurred.

Action:  Contact your vendor sales representative.

LT40010E   All IBM Token-Ring Network 16/4 Busmaster

Server Adapter/A adapters must be set to

the same interrupt level.

Cause:  One or more IBM Token-Ring Network 16/4 Busmaster

Server Adapter/A adapters are set to a different interrupt level.

To function properly, these adapters must all be set to the

same interrupt level.

Action:  Use the reference diskette that came with your

computer to ensure that all IBM Token-Ring Network 16/4 Busmaster

Server Adapter/A adapters are set to the same interrupt level.

LT40011E   A data-check error occurred while attempting

to load the bootstrap code onto the adapter.

Cause:  The bootstrap code was unable to load properly

onto the IBM Token-Ring Network 16/4 Busmaster Server

Adapter/A adapter.

Action:  Contact your vendor sales representative.

Page 12

IBM Token-Ring Network 16/4 Busmaster Server Adapter/A

NDIS Device Driver User's Guide (con't)

LT40012E   A checksum error occurred while attempting

to load the bootstrap code onto the adapter.

Cause:  The bootstrap code was unable to load properly

onto the IBM Token-Ring Network 16/4 Busmaster Server

Adapter/A adapter.

Action:  Contact your vendor sales representative.

LT40013E   This version of the IBM Token-Ring Network

16/4 Busmaster Server Adapter/A NDIS device

driver will not run on an 80286 machine.

Cause:  The 80286 processor is not supported for this

version of the device driver.

Action:  Either install the 80286 version of the

device driver on this machine or remove the statement

that installs the device driver from CONFIG.SYS.

LT40014E   The parameter values have forced the

control-block size to exceed the allowable

limit.

Cause:  The parameters specified require more space for

processing than can be handled.  The maximum allowable

control-block size is 64KB.

Action:  Reduce the value specified for the SIZWORKBUF

or MINRCVBUFFS parameter to change the amount of storage

required for the control-blocks.

LT40015E   The adapter is not responding.

Cause:  The adapter is not responding to a request to load

the bootstrap code.

Action:  Run the hardware diagnostics for the IBM Token-Ring

Network 16/4 Busmaster Server Adapter/A adapter.  Contact

your vendor sales representative.

Page 13

IBM Token-Ring Network 16/4 Busmaster Server Adapter/A

NDIS Device Driver User's Guide (con't)

LT40016E   A 2KB window error occurred while attempting

to load the bootstrap code onto the adapter.

Cause:  The adapter is not responding to a request to load

the bootstrap code.

Action:  Run the hardware diagnostics for the IBM Token-Ring

Network 16/4 Busmaster Server Adapter/A adapter.  Contact

your vendor sales representative.

LT40017E   An error occurred while attempting to run

the bootstrap code.

Cause:  The adapter is not responding to a request to run

the bootstrap code.

Action:  Run the hardware diagnostics for the IBM Token-Ring

Network 16/4 Busmaster Server Adapter/A adapter.  Contact

your vendor sales representative.

LT40018E   A parameter error occurred while attempting

to run the bootstrap code.

Cause:  The adapter is not responding to a request to run

the bootstrap code.

Action:  Run the hardware diagnostics for the IBM Token-Ring

Network 16/4 Busmaster Server Adapter/A adapter.  Contact

your vendor sales representative.

LT40019E   A write error occurred while attempting

to load the bootstrap code onto the adapter.

Cause:  The bootstrap code was unable to load properly

onto the IBM Token-Ring Network 16/4 Busmaster Server

Adapter/A adapter.

Action:  Run the hardware diagnostics for the IBM Token-Ring

Network 16/4 Busmaster Server Adapter/A adapter.  Contact

your vendor sales representative.

Page 14

IBM Token-Ring Network 16/4 Busmaster Server Adapter/A

NDIS Device Driver User's Guide (con't)

LT40020E   A checksum error occurred while attempting

to load the microcode onto the adapter.

Cause:  The microcode was unable to load properly

onto the IBM Token-Ring Network 16/4 Busmaster Server

Adapter/A adapter.

Action:  Contact your vendor sales representative.

LT40021E The microcode encountered a program-status

error while attempting to write to the adapter.

Cause:  The microcode was unable to write properly

to the IBM Token-Ring Network 16/4 Busmaster Server

Adapter/A adapter.

Action:  Contact your vendor sales representative.

LT40022E   A reset error occurred while attempting to

initialize the adapter microcode.

Cause:  The adapter is not responding to a request to initialize

the microcode.

Action:  Run the hardware diagnostics for the IBM Token-Ring

Network 16/4 Busmaster Server Adapter/A adapter.  Contact

your vendor sales representative.

LT40023E   A parameter error occurred while attempting

to run the microcode.

Cause:  The adapter is not responding to a request to run

the microcode.

Action:  Run the hardware diagnostics for the IBM Token-Ring

Network 16/4 Busmaster Server Adapter/A adapter.  Contact

your vendor sales representative.

Page 15

IBM Token-Ring Network 16/4 Busmaster Server Adapter/A

NDIS Device Driver User's Guide (con't)

LT40024E The burned-in address could not be found

on the IBM Token-Ring Network 16/4 Busmaster

Server Adapter/A adapter.

Cause:  The burned-in address on the adapter could not be located.

Action:  Either use the set network address (NETADDRESS)

parameter in PROTOCOL.INI to override the burned-in address or

run the hardware diagnostics for the IBM Token-Ring Network

16/4 Busmaster Server Adapter/A adapter and contact your

vendor sales representative.

LT40025W   Duplicate device driver statements exist

in CONFIG.SYS.

Cause:  Only one occurrence of the device driver statement

for the IBM Token-Ring Network 16/4 Busmaster Server Adapter/A

NDIS MAC device driver is required to support multiple adapters.

Additional instances of the device driver statement in CONFIG.SYS

will be ignored.

Action:  Remove the duplicate device driver statements from

CONFIG.SYS.

LT40026E   The required parameter DRIVERNAME was not

found in PROTOCOL.INI.

Cause:  The parameter specifying the driver name (DRIVERNAME) was not

found in PROTOCOL.INI.  This parameter is required.

Action:  Change PROTOCOL.INI to specify the proper information.

LT40027E   The protocol manager could not be opened.

Cause:  An unexpected error occurred when the

program attempted to open the protocol manager.

The IBM Token-Ring Network 16/4 Busmaster Server Adapter/A

NDIS device driver has been removed.

Action:  Check the drive and directory to ensure that

the protocol manager is located in the specified path.

Page 16

IBM Token-Ring Network 16/4 Busmaster Server Adapter/A

NDIS Device Driver User's Guide (con't)

LT40028E   An unrecoverable error occurred while

attempting to register the IBM Token-Ring

Network 16/4 Busmaster Server Adapter/A NDIS

device driver program with the protocol manager.

Cause:  The device driver was unable to register with

the protocol manager.

Action:  Reinstall the protocol manager and

try the operation again.

LT40029E   No physical adapter was found in this machine.

Cause:  There was no adapter found in this machine.

IBM Token-Ring Network 16/4 Busmaster Server Adapter/A adapters

are required to run the MAC device driver.

Action:  Install an IBM Token-Ring Network 16/4 Busmaster

Server Adapter/A adapter in this machine, or remove the device

driver statement from CONFIG.SYS.

LT40030W   The unrecognized parameter xxxxxxxxxxxx was

found in PROTOCOL.INI.

Cause:  An unrecognized parameter was found

while processing the IBM Token-Ring Network 16/4 Busmaster

Server Adapter/A section of PROTOCOL.INI.

Action:  Correct the parameter or remove it from

PROTOCOL.INI.

Page 17

IBM Token-Ring Network 16/4 Busmaster Server Adapter/A

NDIS Device Driver User's Guide (con't)

LT40031W   The value specified for network address

in PROTOCOL.INI is not valid.  The burned-in

address will be substituted.

Cause:  The value specified for the network address parameter in

PROTOCOL.INI is not valid for one of the following reasons:

1.  Invalid local station address

2.  Too long

3.  Too short

4.  Contains non-hexadecimal characters

5.  Not enclosed in double quotes

Action:  Remove or correct the network address parameter

(NETADDRESS). A valid value for this parameter is 12 characters

long, is enclosed in double quotes, and contains only valid

hexadecimal characters within the range of 400000000000 through

40007FFFFFFF.

LT40033E   A duplicate network address was found

while inserting onto the ring.

Cause:  The network address that the adapter wants to use on the

token-ring is being used by another node.

Action:  Either modify PROTOCOL.INI to provide a different

value for the network address (NETADDRESS) parameter, use the

default burned-in address, or remove the other node that is

using this network address from the ring.

LT40034E   Ring failure or beaconing was encountered

while inserting onto the ring.

Cause:  The ring speed is different from the speed of the

node attempting to insert onto the ring.

Action:  Use the reference diskette that came with your

computer to ensure that the adapter is configured at the same

speed as the token-ring.

Page 18

IBM Token-Ring Network 16/4 Busmaster Server Adapter/A

NDIS Device Driver User's Guide (con't)

LT40035E   A hardware failure occurred while attempting

to open the adapter.

Cause:  When the adapter tried to insert onto the token-ring, a

hardware error occurred on the adapter.

Action:  Run the hardware diagnostics for the IBM Token-Ring

Network 16/4 Busmaster Server Adapter/A adapter.  Contact

your vendor sales representative.

LT40036E   A lobe media error has occurred.  Ensure

that the token-ring cable is connected

to the adapter.

Cause:  While attempting to open the adapter, an indication

was received that the adapter is not properly connected to

the token-ring.

Action:  Ensure that the token-ring cable is connected to

the adapter and to the token-ring multistation access unit (MAU).

If the error persists, use a different token-ring cable

and check to ensure that the connection to the MAU is

functioning properly.  Then run the hardware diagnostics

for the IBM Token-Ring Network 16/4 Busmaster Server Adapter/A

adapter.

LT40037W   The version of the installed device driver is not

optimized for the 80386/80486 processor.

Cause:  The version of the IBM Token-Ring Network 16/4

Busmaster Server Adapter/A NDIS MAC device driver is

built for an 80286 machine.  The version will function

correctly on this 80386/80486 machine, but performance

benefits will not be available.

Action:  Install the optimized version on this machine.

Page 19

IBM Token-Ring Network 16/4 Busmaster Server Adapter/A

NDIS Device Driver User's Guide (con't)

LT40038E   The installed IBM Token-Ring Network 16/4

Busmaster Server Adapter/A NDIS device driver

cannot run on this operating system.

Cause:  The device driver installed is designed to run on

a different operating system.

Action:  Do the following:

1.  Locate the current version of IBMTRBM.OS2 on the machine.

2.  Erase IBMTRBM.OS2.

3.  Copy IBMTRBM.286 from the installation diskette

into the directory.

4.  Rename IBMTBM.286 to IBMTRBM.OS2.

5.  Restart your system.