|======================================================================|

| This is an abstract of this programs documentation used as a short   |

| review. To get the full concept of this program you should download  |

| all concerned files and read the programs full documentation.        |

|                                                                      |

|    Please register your favorite programs, keep shareware alive!     |

|==============================================================(SDN)===|

HDTEST was originally written to allow comprehensive Read/Write testing

of hard disks on IBM PCs, XTs, ATs, and "true" compatibles.  The V4.10

release relaxed many of the hardware restrictions imposed by earlier

(V2.xx and V3.xx) versions of the program and made it useful for testing

most MS-DOS-compatible storage devices currently in use on IBM-compatible

computers, including removable (floppy and hard) disks and logically

partitioned hard disks. This release (V4.45) includes BIOS-level support

for DOS 3.3 "extended devices" and more options for the handling of

totally unreadable sectors, and recognizes (but does not support) large

(> 32 Mb) devices under IBM DOS 4.0 and Compaq DOS 3.31.  Although many

programs, including IBM's diagnostics, allow "surface scanning" and seek

testing of hard disks, none that are available to the average user test

whether the whole disk can retain the data that is or will be written to

it.  HDTEST's principal "claim to fame" is the ability thoroughly to test

the magnetic media in a device which contains data while preserving the

data (although data loss is unavoidable if an area which is in use has

become unreadable).

HDTEST tests the disk much more thoroughly than programs such as DFixer,

Mace, and Norton's DT. Mace and DT, like many public domain "disk test"

programs, simply read every cluster on the disk and report on any errors

encountered.  DFixer does better, writing 4 test patterns to every free

cluster and checking that they can be read back correctly, but it makes

no attempt to test clusters that are already in use - the very ones which

contain your precious data!

In addition to its testing function, HDTEST has been reported to be

useful as an automatic disk repair utility for both hard and floppy disks

which have developed (especially) intermittent or persistent read errors,

preventing or impeding their normal use. Although HDTEST cannot read the

totally unreadable, it tries much harder that DOS does and will almost

always, in the course of its operation, allow you to recover the entire

readable portion(s) of a file with one or more unreadable clusters in it,

optionally either remapping all of what it reads from the disk or

replacing bad sectors (the default) or clusters with NULLs.  HDTEST is

not an alternative to backing up your floppies and/or hard disk it may

sometimes be able to save your neck when you forget or your backup goes

bad too! On hard disks which have developed intermittent read errors,

running HDTEST can sometimes totally cure the problem, presumably because

the "flaky" sectors get rewritten and the minor misalignments and/or

weakening of the magnetic "image" which were causing the trouble are

corrected   ... (continued)...

|=======================================================================|

| This abstract is taken from a portion of this programs documentation  |

| and is intended to give an idea of the program. It does not reflect   |

| the entirer capabilities and features of the program and the full     |

| documentation should be consulted.   (SDN)                            |

|=======================================================================|