��������������������������������������������������������������������������������

� HOW TO USE THIS TROUBLE-SHOOTING GUIDE                                       �

��������������������������������������������������������������������������������

�                                                                              �

� This guide is contained in the file  SAPPHTS.DOC.  You can print it out, but �

� it works best if you view it via the SYSOP menu, which uses our SEE program. �

� To do that, enter SYSOP at the DOS prompt, select INFO, then select TROUBLE. �

� This lets you search the guide for keywords whenever you have a problem.     �

�                                                                              �

� Each problem described in this file is  preceded by a list of keywords, each �

� of which is preceded by  a slash character  (/).   Thus,  if you had a modem �

� problem, you could press "F" (for Find) then search for  /MODEM.  By repeat- �

� edly searching  for  /MODEM  (press  F3  after  the initial search)  you can �

� quickly scan the file for any related problems.                              �

�                                                                              �

� When in doubt about  which keyword to choose,  use the shortest one that can �

� describe your problem.  For example,  if a user has trouble connecting, look �

� for /CONNECT rather than /CONNECTION PROBLEM,  since /CONNECT would match on �

� /CONNECT or /CONNECTING or /CONNECTION and so on.  Most important, avoid any �

� plurals:  search for /MODEM, not /MODEMS.                                    �

�                                                                              �

� You can also search major categories.  See the INDEX section, below.         �

��������������������������������������������������������������������������������

��������������������������������������������������������������������������������

�                                                                              �

�                                 == INDEX ==                                  �

�                                                                              �

��������������������������������������������������������������������������������

Major categories in this guide are presented in alphabetical order, as

follows:

� COMMANDS (STANDARD)

� COMMANDS (SYSOP)

� DISPLAY

� DOORS

� EVENTS

� FILE TRANSFER

� FRONT END

� INPUT

� MAINTENANCE

� MESSAGES

� MODEM

� NOISE

� PROGRAM FAILURE

Specific headings are preceded by two equals signs and a blank (see INDEX

heading, above).  Thus, to find the NOISE section, press F for Find, type

"==", then a blank, then "NOISE".

��������������������������������������������������������������������������������

�                                                                              �

�                          == COMMANDS (STANDARD) ==                           �

�                                                                              �

��������������������������������������������������������������������������������

KEYWORD:  /COMMANDS/SET/NAMES/TEXT

PROBLEM:  How do SET, NAMES or TEXT work?

These commands "tune" other commands to restrict or control what they report.

SET controls them by time, NAMES by the name of the person or file being

referenced, and TEXT by any text (apart from name-oriented text) that might

be presented by the command.

��������������������������������������������������������������������������������

�                                                                              �

�                           == COMMANDS (SYSOP) ==                             �

�                                                                              �

��������������������������������������������������������������������������������

KEYWORD:  /COMMANDS/USERS/CLEAN/CLEAN-UP/CLEANUP

PROBLEM:  How do I access clean-up mode?

To clean up the user list, you have to be in UltraMode (turned on and off with

Alt-F10).  Once you're in UltraMode, select the USERS command.

��������������������������������������������������������������������������������

�                                                                              �

�                                == DISPLAY ==                                 �

�                                                                              �

��������������������������������������������������������������������������������

KEYWORD:  /SEE/VIEW/43/50/EGA/VGA

PROBLEM:  When viewing text files with SEE, I no longer switch to 43- or

50-line mode

The version of SEE that accompanies Sapphire is a custom version designed for

Sapphire.  If you find a more recent version of SEE, don't install it in your

Sapphire directory.

��������������������������������������������������������������������������������

�                                                                              �

�                                 == DOORS ==                                  �

�                                                                              �

��������������������������������������������������������������������������������

KEYWORD:  /DOORS/OTHER BBS/INTERFACE

PROBLEM:  How do I use doors that weren't specifically designed for Sapphire?

Several interface programs are available which will interface doors for one BBS

to another BBS.  For example, the shareware programs SP2QBBS and DOORMASTER can

be used.  Both of these programs may be obtained from our support system.  If

you use them, please support the author's efforts by registering them.

�������������������������������������������������������������������������������

KEYWORD:  /DOORS/REMOTE

PROBLEM:  The door comes up but the users don't see anything

Here are a few things to check out:

1.  Are you running a "FOSSIL" driver?  If so, why?  Does the door NEED the

fossil?  Can you disable the fossil when it's not being used?  If the door

documentation doesn't explicitly say that it needs a fossil driver, try

removing the fossil COMPLETELY before trying the door (i.e. reboot and

don't install it).  Then, if that proves that the fossil was the cause of

the problem, read the fossil documentation to see if there is some way to

disable the fossil when it is not needed.

2.  Does the door know which port it is supposed to be using?  You may have to

use a door interface program (described above) to let it know.

3.  Some programs (such as Pinnacle Software's Pyroto Mountain program) can

accept the port information on the command line, in which case you should

make sure that you are not pointing at the console.  Taking Pyroto as an

example, if the command line specifies a port of 0 (zero), it means the

console.  If it specifies a port address, it means the serial port.

�������������������������������������������������������������������������������

KEYWORD:  /DOORS/RETURN

PROBLEM:  How does Sapphire know we are coming back from a door?

When Sapphire creates the batch file EXTERNAL.BAT, it writes the part where it

starts itself back up as follows:

SAPPHIRE DOOR

This lets Sapphire know that it should not initialize the modem.

��������������������������������������������������������������������������������

�                                                                              �

�                                 == EVENTS ==                                 �

�                                                                              �

��������������������������������������������������������������������������������

KEYWORD:  /EVENTS/RETURN

PROBLEM:  How does Sapphire know we are coming back from a event?

When Sapphire creates the batch file EXTERNAL.BAT, it writes the part where it

starts itself back up as follows:

SAPPHIRE EVENT

This lets Sapphire know that it should not initialize the modem.

��������������������������������������������������������������������������������

�                                                                              �

�                             == FILE TRANSFER ==                              �

�                                                                              �

��������������������������������������������������������������������������������

KEYWORD:  /DOWNLOAD/UPLOAD/ABORTS

PROBLEM:  File transfer aborts immediately

Here are a few things to check out:

1.  Are you running a "FOSSIL" driver?  If so, see the discussion of fossils

in the DOORS section.

2.  If you are running on COM3 or COM4, you must tell DSZ this by specifying

the DSZPORT parameter.  For example, if you are on COM3, you would have

to enter the DOS command SET DSZPORT=3.  The DSZ manual explains more.

��������������������������������������������������������������������������������

�                                                                              �

�                               == FRONT END ==                                �

�                                                                              �

��������������������������������������������������������������������������������

KEYWORD:  /FRONT END/FIDO/NETWORK

PROBLEM:  Does Sapphire support Fido or other networks?

Not at the moment.  We are looking into the possibility of creating a truly

maintenance-free network.  We have heard from some Fido sysops that they spend

up to 3 hours a week keeping their Fido system up to date.  This is not in keep-

ing with the "install and forget" design of Sapphire.

�������������������������������������������������������������������������������

KEYWORD:  /FRONT END/INSTALL

PROBLEM:  How do you install a front-end program (to answer the modem)?

Another program can call Sapphire with the command:

SAPPHIRE FRONT <baudrate>

This will start up Sapphire without touching the modem.  If the baudrate is 0

(zero) or omitted, it means that the user is at the console.

��������������������������������������������������������������������������������

�                                                                              �

�                                 == INPUT ==                                  �

�                                                                              �

��������������������������������������������������������������������������������

KEYWORD:  /USER/ENTER/INPUT/FREEZE

PROBLEM:  User has to press Enter twice after each input.

The symptom is that the user will type some input, hit Enter, and the cursor

will "freeze" in column one -- before doing a linefeed -- and will not move

on until he hits another character.

This happens when the user sets his terminal program to send a CTRL-S whenever

he presses Enter.  For some reason, some Macintosh terminal programs seem to

come pre-configured this way.

Over the phone, it is very difficult to explain to a user how to solve this

problem.  In brief, however, he should look through his terminal program's

configuration section for something labelled "handshaking" or "ASCII protocol",

or something like that, and disable handshaking there.

�������������������������������������������������������������������������������

KEYWORD:  /USER/INPUT/CHARACTERS/MISSING

PROBLEM:  Some of the characters the user types are lost.

Sapphire can have problems with multi-tasking software such as DesqView.  The

Sapphire serial routines are poll-driven rather than interrupt-driven.  What

this means is that, if you're running multi-tasking software, your users may

lose characters when they type.

Poll-driven I/O offers total port compatibility with all hardware platforms

upon which Sapphire is likely to be run, but it does make it inappropriate for

multi-tasking machines.

��������������������������������������������������������������������������������

�                                                                              �

�                              == MAINTENANCE ==                               �

�                                                                              �

��������������������������������������������������������������������������������

������������������������������������������������������������������������������ͻ

� SEE ALSO:  COMMANDS (SYSOP)                                                  �

������������������������������������������������������������������������������ͼ

KEYWORD:  /FILE LIST/FULL/DISK/SPACE

PROBLEM:  File list is full

In most cases, the file list is self-maintaining, but if ALL of the files have

been on the list for less than 14 days, auto-maintenance is disabled.

When this happens, you can manually delete some files, using the FILES command

(when logged in at user level 8), or you can wait for a few days for some files

to time out.

��������������������������������������������������������������������������������

�                                                                              �

�                               == MESSAGES ==                                 �

�                                                                              �

��������������������������������������������������������������������������������

KEYWORD:  /MESSAGES/READING/MISSING/NEW

PROBLEM:  The number of "new" messages reported is different from what is

actually shown.

1.  If the message just before the first one in that message base is yours,

yours will be diplayed, in order to put the replies in context.  This is

not, however, a "new" message, so it isn't counted.

2.  Messages with no viewable text (or just the summary line viewable) due to

use of <<privates>> or ><exclusives>< are not displayed, though they are

counted in the "new" messages.  (It would take too long for Sapphire to

look at each message in the message base, in advance, to see which ones

are visible.)

3.  If you physically delete message files (instead of using the ERASE

command), Sapphire will keep them listed in the message file, but won't

be able to open the file when it is time to display it.

�������������������������������������������������������������������������������

KEYWORD:  /MESSAGES/READING/MISSING/NEW/MINE/MOVE

PROBLEM:  When I send a message, I can't see it, even if I use the MOVE command

to move it to a date ahead of my read date.

In normal reading, Sapphire avoids showing you your own messages unless they

serve to put other messages in context (see the previous item).  You can see

your own message if you do a Temporary SET, however.  But since SET can't be

entered before you logon, this means that you never see your own logon

messages (unless somebody else is also writing logon messages).  If you wish

to check your messages, use the REVIEW or CHANGE command.

��������������������������������������������������������������������������������

�                                                                              �

�                                 == MODEM ==                                  �

�                                                                              �

��������������������������������������������������������������������������������

This categories is divided into the following sub-categories:

� CONNECTING

� DISCONNECTING

� MISCELLANEOUS

��������������������������������������������������������������������������������

�                               = CONNECTING =                                 �

��������������������������������������������������������������������������������

KEYWORD:  /MODEM/ANSWER/CONNECTING/CALL

PROBLEM:  Modem doesn't answer incoming calls.

This happens if the modem is not configured correctly, either in hardware or

modem settings.  Check all of the following suggestions; one of them may solve

your problem.

�������������������������������������������������������������������������������

HARDWARE PROBLEMS

�������������������������������������������������������������������������������

1.  If, when you start up Sapphire, it does NOT have an error-free dialog

with the modem (i.e. if it complains about a carrier, or modem not

ready, or if the modem never responds "OK"), refer to the trouble-

shooting section concerned with those problems (do a keyword search for

(slash)CARRIER, (slash)NOT READY or (slash)OK).

2.  If you have an external modem (a box outside your computer, as opposed

to a card inside your computer), check the following:

A.  Is your modem plugged in?  (Are lights glowing on the front panel?)

B.  Is the modem plugged into the phone line?

C.  Is the phone line plugged into the CORRECT plug on the modem?

(Some modems have one plug for the line and one for a handset)

3.  As an aid to understanding the problem, try calling the modem on a voice

phone.  Does it pick up the line?  If it does pick up the line, do you

hear a carrier?

�������������������������������������������������������������������������������

MODEM SETTINGS

�������������������������������������������������������������������������������

1.  If you have an external modem, it may have some lights which indicate how

it is set, or what signals it is receiving.  Check these lights and

settings, and compare them with the informatin in your modem manual.

LIGHT                  USUALLY AFFECTED

LABEL  MEANING         BY MODEM COMMAND  RELATED ITEMS IN THIS GUIDE

�����  ��������������  ����������������  ���������������������������

AA     Auto-Answer     ATS0=1            Search for (slash)DIP

MR     Modem Ready     AT&D1             Search for (slash)DIP

TR     Terminal Ready  N/A

OH     Off-hook        ATH0              Should be OFF, not on.

LIGHT

LABEL  MEANING         COMMENTS

�����  ��������������  ������������������������������������������������

SD     Send            Should flash when modem  receives  your commands

RD     Read            Should flash when modem replies to your commands

2.  If you have an internal modem, you still have to make the settings

correctly, but you won't see any lights to indicate if your commands

have been accepted.

�������������������������������������������������������������������������������

KEYWORD:  /MODEM/ANSWER/CONNECTING/SPEED/9600/V32

PROBLEM:  Modem answers incoming calls okay at some speeds but not at others.

1.  In configuring your modem, you must take care to set the X setting (i.e.

the ATX command) properly.  For example, some settings may not show the

baud rate properly.  In general, use the highest X setting supported by

your modem.

2.  If you are having trouble at baud rates of 9600 or more, you will have to

consult your modem manual.  A sample problem:  the other person might be

trying to connect an HST (old-style US Robotics) modem at 9600 to a V32

modem at 9600; the two methods are not compatible.

3.  Some modems, when initialized at one speed, will not jump to higher speeds.

In such case, try initializing the modem at the highest speed it supports.

3A. If you are using a 9600+ baud modem, and you have a machine that runs at 8

Mhz or faster, try initializing it at 9600 baud.  If that works, AND you

have a V32bis modem AND you have a machine that is 16 Mhz and faster, you

can try initializing it at 14400.  Do a search on <slash>HIGH SPEED for some

additional notes on this topic.

3B. If your machine is slower than 8 Mhz, you will probably have to initialize

at 2400 baud.  On some modems, this will make it impossible to receive calls

at 9600 baud because the modem assumes that your initializing baudrate is

the maximum you can attain.  This is a flaw in the design philosophy of the

modem, and there is nothing that Sapphire can do about it.

�������������������������������������������������������������������������������

KEYWORD:  /MODEM/ANSWER/CONNECTING/SPEED/HIGH SPEED/9600/V32/DTE/LOCKING/BAUD

PROBLEM:  People get nonsense characers when connecting to my high-speed modem.

Some "Fax Modems" claim to support 9600, but the 9600 may refer to their fax

function, not its data rate (caveat emptor).  Moreover, not all 9600 bps

modems are compatible.  A "non-dual standard" V-Hayes, Telebit PEP or

CompuComm modem will communicate with a different model at 2400 baud, but can

attain higher speeds when talking to the same kind of modem.  If you're

thinking about getting a high-speed modem, get a V32 model (or one of its

descendants, such as V32bis, V32terbo etc.), as this has become the accepted

standard.  (Nevertheless, not all V32 modems are created equal and sometimes

different brands refuse to communicate at more than 2400 baud.)

If you are having other problems with your high-speed modem, read the

following article...

--------------------------------------------------------------------------

HOW HIGH SPEED MODEMS WORK -- BECOME AN INSTANT MODEM GURU BY READING THIS

--------------------------------------------------------------------------

High speed modems support something called "locked baud rates".  This means that

no matter what speed the caller dials in at, the modem will talk to Sapphire at

a specific speed.  When a modem is advertised as supporting more than 14400

baud, it usually means that it is connected to the remote system at a lower

speed, hooked up to the local computer at the higher speed, and uses data

compression to try to achieve the higher throughput.

Each modem has a limit on the highest physical baudrate its communications

hardware can support; this is always lower than the claimed highest speed. For

example, a V32bis modem may claim it can achieve a throughput of 38400 baud, but

this makes use of compression (typically V42bis compression) to boost a 14400

baud connection as high as 38400.  In normal use, you will rarely get 38400 baud

throughput.

Nevertheless, in such a case the connection between the modem and the computer

is locked at 38400 baud.  When data comes in from the phone line (at, say, 14400

baud) the modem decompresses it and sends it on to the computer at 38400.  In

other words, the session between the modem and the remote end (known as the "DCE

Session") is NOT at the same speed as the session between the modem and the

local computer (known as the "DTE Session")

By the way, DCE stands for "Data Communication Equipment" and DTE stands for

"Data Terminating Equipment".

Let's take another example, using a V32 modem (which has slightly less

throughput than the more advanced V32bis modem).  In such case, the baud rates

are a bit lower, but once again, they are not the same on either side of the

modem.  A typical V32 session looks like this:

REMOTE USER   --- 9600baud --->  MODEM   --- 19200baud --->  SAPPHIRE

So far, so good.  But let's see what happens when data goes the other way, as in

this example:

REMOTE USER  <--- 9600baud ---   MODEM  <--- 19200baud ---   SAPPHIRE

If Sapphire is sending data at 19200 baud, how can it squeeze all that data

through the 9600 baud connection?  The answer is:  it can't.  The modem has to

buffer (i.e. save up) the incoming data and let it out a bit at a time (at 9600

baud).

While the modem is doing this, it needs some way to tell Sapphire to stop

sending new data (in other words, the modem says, "Please wait -- I'm busy!") It

does this by "handshaking" with Sapphire, using the CTS (Clear To Send) hardware

line.  (If you have an external modem connected to your computer by an RS-232

cable, one of the wires in the cable is the CTS line -- assuming your cable is

complete.  If you have an internal modem, the CTS line is built in to the card.)

Handshaking is automatically enabled when you set "Locked DTE Speed?" to Yes in

SYSOP:CONFIG:BASIC:MODEM:SETTINGS.

When setting up your high-speed modem for Sapphire, don't set its initializing

baudrate (SYSOP:CONFIG:BASIC:MODEM:SETTINGS) to a value higher than your

computer can handle.  Here are some general guidelines for maximum baudrates:

MAXIMUM

INITIALIZING

TYPE OF COMPUTER         BAUDRATE

----------------       ------------

4.5 Mhz XT               9600

8.0 Mhz XT              14400

8.0 Mhz AT              19200

33.0 Mhz 386             38400

NOTE:  These figures are based on the

assumption that you are not multi-

tasking or running Sapphire under

Windows, neither of which are

recommended.

38400 is the highest baudrate supported by the current version of Sapphire, so

if you have a more powerful machine than a 33 Mhz 386, you will still initial-

ize to 38400.

�������������������������������������������������������������������������������

KEYWORD:  /MODEM/ANSWER/CONNECTING/SPEED/300

PROBLEM:  Modem doesn't work properly on 300 baud calls

Some 9600 baud modems can't support the higher baud-rates and 300 baud at the

same time.  Under some circumstance, the Telebit T1600 exhibits this behaviour.

There is no way to fix this problem except to experiment with the modem's

internal settings.  Be sure to record your modem's present settings before you

start experimenting.

��������������������������������������������������������������������������������

�                              = DISCONNECTING =                               �

��������������������������������������������������������������������������������

KEYWORD:  /MODEM/HANG UP/ECHO

PROBLEM:  When user hangs up, modem endlessly echoes back to Sapphire.

Some modems don't have time to drop the carrier while characters are being

sent.  For this reason, you have to configure the modem not to echo command

characters.  This can cause an echoing loop when somebody hangs up.

������������������������������������������������������������������������������

| The solution given here applies to most modems, but check your modem manual |

�������������������������������������������������������������������������������

������������������������������������������������������������������������������

|   MODEM HAS   |                                                             |

| DIP SWITCHES? | ACTION TO BE TAKEN                                          |

������������������������������������������������������������������������������

|     YES       | Set switch 4 to NOT echo characters                         |

������������������������������������������������������������������������������

|     NO        | Include E0 - not E1 - in your modem initialization command  |

�������������������������������������������������������������������������������

��������������������������������������������������������������������������������

�                              = MISCELLANEOUS =                               �

��������������������������������������������������������������������������������

KEYWORD:  /MODEM/CARRIER

PROBLEM:  Sapphire says that the modem is presenting a carrier.

You must set your modem so that it does NOT present a carrier -- unless some-

body has dialed in and is connected.

������������������������������������������������������������������������������

| The solution given here applies to most modems, but check your modem manual |

�������������������������������������������������������������������������������

������������������������������������������������������������������������������

|   MODEM HAS   |                                                             |

| DIP SWITCHES? | ACTION TO BE TAKEN                                          |

������������������������������������������������������������������������������

|     YES       | Set switch 6 to use CD line                                 |

������������������������������������������������������������������������������

|     NO        | Try including &C1 in your modem initialization command      |

�������������������������������������������������������������������������������

�������������������������������������������������������������������������������

KEYWORD:  /MODEM/NOT READY/READY

PROBLEM:  Sapphire says that the modem is not ready.

1.  If you have an external modem, are all wires of the modem cable connected

properly?

A.  If you have a "null modem cable" (pins 2 and 3 cross-connected)

Sapphire won't answer.

B.  Using some wire, a flashlight battery, and a flashlight bulb as a

simple continuity tester, check that the important pins are connected

end-to-end:

Pin 1 (or Pin 7)   Pin 2   Pin 3   Pin 8   Pin 20

The pin numbers are usually written in tiny letters on one of the

connectors, or pin 1 is marked with a little arrow.

�������������������������������������������������������������������������������

2.  Make sure that the modem is configured to use the Terminal Ready line.

������������������������������������������������������������������������������

| The solution given here applies to most modems, but check your modem manual |

�������������������������������������������������������������������������������

������������������������������������������������������������������������������

|   MODEM HAS   |                                                             |

| DIP SWITCHES? | ACTION TO BE TAKEN                                          |

������������������������������������������������������������������������������

|     YES       | Set switch 1 to use DTR line                                |

������������������������������������������������������������������������������

|     NO        | Try including &D1 or &D2 in modem initialization command    |

�������������������������������������������������������������������������������

�������������������������������������������������������������������������������

3.  Make sure the modem's DSR behaviour is configured properly.

Some modems allow the selection of the behaviour of the Dataset Ready (DSR)

signal.  The proper setting is "always on", but some modems let DSR indicate

whether or not there is a carrier.

On modems without DIP switches, you can usually set DSR behaviour with the &S

setting.  For example, on most Hayes modems, &S0 sets the corrects behaviour.

�������������������������������������������������������������������������������

KEYWORD:  /MODEM/OK/AT COMMANDS

PROBLEM:  The modem seldom or never responds "OK" to the commands I send it.

1.  Try configuring the modem at a lower baud-rate -- it may be missing some

of the responses.  The initialization baud-rate is set via SYSOP:CONFIG:

BASIC:MODEM:SETTINGS.

2.  If the problem happens only after you've used another modem-oriented

program, turn the modem off and on before starting Sapphire.  The method

by which some modems figure out their command baudrate may make it

impossible for Sapphire to reset the modem.

Technical explanation:  Modems determine their command baudrate by looking

for an "AT" after a reset.  Some modems reset on a DTR transition; some

don't.  In such case, the easiest way to do a reset is to turn the modem

off and on.

3.  Your modem may be responding slowly.  Sapphire waits 1.5 seconds for a

modem response to begin, but for some commands (especially the ATZ reset

command), it may take your modem longer than that.  You might be able to

solve the problem by modifying the Command Pacing value in SYSOP:CONFIG:

BASIC:MODEM:SETTINGS.

If that does not solve your problem, watch the process carefully to deter-

mine if your modem is, in fact, accepting the command -- but that for some

reason Sapphire is simply unable to obtain the response.  If everything

else works fine (i.e. users can call in), then you may decide to simply

put up with the fact that Sapphire will attempt each command three times.

�������������������������������������������������������������������������������

KEYWORD:  /MODEM/DIP SWITCHES

PROBLEM:  Modem has DIP switches.

Older modems have tiny DIP (dual-inline-package) switches (sometimes you have

to open the modem to find the bank of switches).  In most cases, they are set

as follows:

������������������������������������������������������������������������������

| The solution given here applies to most modems, but check your modem manual |

�������������������������������������������������������������������������������

Switch 1 UP:   Computer should use the Data Terminal Ready (DTR) line

Switch 2 UP:   Result codes sent as words

Switch 3 DOWN: Result codes are sent to the computer

Switch 4 UP:   Echo incoming characters

Switch 5 DOWN: Don't answer incoming calls

Switch 6 UP:   Computer should use the Carrier Detect (CD) line

These modems may also have switches 7 and 8; check your manual to see how

they should be set.

��������������������������������������������������������������������������������

�                                                                              �

�                                 == NOISE ==                                  �

�                                                                              �

��������������������������������������������������������������������������������

KEYWORD:  /NOISE/RANDOM/CHARACTERS/GARBAGE/CONNECTING

PROBLEM:  Users see some random characters when they connect.

1.  If only one user has this problem, ask him to try on a different modem,

or to try from another location.

2.  If everybody has this problem, disconnect ALL phones at your house (even

those on other lines) except the modem.  If this cures the problem, it

may indicate the problem.  Some conference phones, for example, cause

leakage between lines, which causes line noise.

3.  If you have this problem during early testing, make sure that the other

person has called other BBS's before, at the baud rate being attempted,

and that the baud rate being attempted is within the capabilities of both

modems.

4.  See also the CONNECTING items in the MODEM section.

��������������������������������������������������������������������������������

�                                                                              �

�                            == PROGRAM FAILURE ==                             �

�                                                                              �

��������������������������������������������������������������������������������

KEYWORD:  /PROGRAM/FAILURE/ERROR CODE/RUNTIME ERROR/IO ERROR/BUG

PROBLEM:  Program fails with an Run-time error code.

The following table may help you determine the cause of the error:

������������������������������������������������������������������������������

| ERROR # || COMMENTS                                                          |

������������������������������������������������������������������������������

| 2 and 3 || File/Path not found.  Are all file paths in SYSOP:CONFIG valid?   |

������������������������������������������������������������������������������

|    4    || Too many open files.  Increase your FILES= value in CONFIG.SYS.   |

|         || See your DOS manual for an explanation of the CONFIG.SYS file.    |

������������������������������������������������������������������������������

|    5    || File access denied. For some reason, Sapphire is trying to write  |

|         || to a read-only file or a directory. Check SYSOP:CONFIG carefully. |

������������������������������������������������������������������������������

|   100   || For some reason, a file is shorter than Sapphire thought it would |

|         || be.  Check your Sapphire directory for any zero-length files.  If |

|         || you find one, and it is a file that Sapphire creates (i.e. it is  |

|         || obviously a Sapphire data file but did not come in the original   |

|         || Sapphire package), back up your Sapphire directory, delete the    |

|         || suspect file, and run Sapphire again to see if it creates the     |

|         || file correctly.                                                   |

������������������������������������������������������������������������������

|   101   || Your disk is full.  Clear some space before restarting Sapphire.  |

������������������������������������������������������������������������������

|   150   || Your disk is write-protected, so Sapphire can not write to it.    |

������������������������������������������������������������������������������

|   152   || Disk drive not ready -- may be a hardware malfunction.            |

������������������������������������������������������������������������������

|154 - 162|| Probably a hardware malfunction.  Have your computer checked for  |

|         || proper operation by qualified service personnel.                  |

��������������������������������������������������������������������������������

If the table above does not help you solve your problem, dial in to our BBS at

514-345-8654 and have a look around; we may have a copy of SAPPHTS.DOC that

covers your problem or a new version of SRUN.EXE that solves it.  If that

doesn't help, call our voice support line at 514-345-9578.

If you think you've spotted a bug in Sapphire, the most helpful thing you can do

is learn how to make it happen on demand.  That is to say, before you report a

problem, try to understand it well enough to make it happen on purpose.  Here's

a checklist to help you assess the problem:

1.   What do you think causes the problem?

2.   How often does it happen?

3.   Does it happen to everybody or just certain people?

4.   Did it start just recently or has it always happened?

5.   If it just started, what changed just before it started?

6.   What have you done to try to solve the problem?