This program is a scaled down version of spice that will

simulate and find the frequency response

of any circuit.  It has no time domain simulation

capabilities.  sspice can accept passive and active

circuits.   Voltage controlled current sources and

voltage controlled voltage sources are available.

Node Names

The input format of sspice is a "net list" containing

all component connections and values.  Most components

are described as to be connected between two nodes.

The node names can be a 10 or less ascii character

string.  The node names "Gnd" and "gnd" are reserved

for the common node of ground.  Each circuit is assumed

to have this common node of ground.

Control Commands

FS <val> [modif] ;			The starting frequency in Hz.

FE <val> [modif] ;			The ending frequency in Hz.

FP <val> [modif] ;			The number of partitions the

frequency range is to divided

into.

LOG ;					The output will be calculated in

dB (attenuation).  Only applies

to magnitude response.

RADS ;					The starting, ending, and output

printed frequencies are accepted

and printed in radians instead of Hz.

PHASE ;					Phase response is calculated and

printed in degrees.

PRTCMP ;				Print out a list of the components

used and their node assignments.

OUT <node> ;				Identifies the output node.

VS <node> ;				Identifies the input node.

Passive Component Entries

Rxxxx <node> <node> <value> [modif] ;	A resistor with label "xxxx" connected

between the two nodes with resistance

"value" ohms.

Cxxxx <node> <node> <value> [modif] ;	A capacitor with label "xxxx" connected

between the two nodes with capacitance

"value" farads.

Lxxxx <node> <node> <value> [modif] ;	An inductor with label "xxxx" connected

between the two nodes with inductance

"value" henries.

Active component Entries

VCV <node1> <node2> <node3> <value> [modif] ;

A voltage controlled voltage source.

A voltage of value*(Vnode1-Vnode2)

is applied to node3.  "value" is the

voltage gain constant (for typical

op-amps, make this 100,000).

VCI <node1> <node2> <node3> <node4> <value> [modif] ;

A voltage controlled current source.

A current of value*(Vnode1-Vnode2)

is injected between node3 and node4.

"value" is considered to be the

transconductance of the controlled

source in units of Mhos.

Note: At present, sources cannot be assigned unique names; however,

each entry is considered a unique source.

Modifiers

All input data can be modified by a metric modifier or an exponential power.

The modifiers are p,n,u,m,K,M,G, for ten to the -12,-9,-6,-3,+3,+6,+9

powers, respectively.

Output

The output will be two columns of numbers: the first being the

frequency in hertz, the second being the amplitude response

in absolute units or attenuation in dB, phase response in

degrees, or the step response.  In addition to the x-y

coordinates, further information is supplied to the

output for use by the "plot" program.