NAME

srec - convert DSP56000 load file records to Motorola

S-record format

SYNOPSIS

srec [-1 | -3 | -r] <input file ... >

DESCRIPTION

Srec takes as input a DSP56000 absolute load file and produces

byte-wide Motorola S-record files suitable for PROM burning.

The DSP56000 START and END records are mapped into S0 and

S9 records respectively.  All other DSP56000 record types

are mapped into S1-type records (SYMBOL and COMMENT records

are currently ignored).  Since the DSP56000 uses a 24-bit

word size, the words must be split into bytes and stored in

an appropriate format.  In the default mode of operation the

program writes the low, middle, and high bytes of each word

consecutively to the current S1 record being written.  For

example, given the DSP56000 DATA record below:

address field

|

_DATA P 0000

0008F8 300000 340000 094E3E

|      |      |      |

|      |      |      fourth word

|      |      third word

|      second word

first word

srec would create the following S1 record:

byte count field

| address  field              checksum field

| |                           |

S10D0000F808000000300000343E4E09F9

|     |     |     |

|     |     |     fourth word

|     |     third word

|     second word

first word

Output records are written to a file named according to

the following convention:

<basename>.M

where <basename> is the filename of the input load file

without extension and M is the memory space specifier

(X, Y, L, or P) for this set of data words.  Note that a

separate file is created for each memory space encountered

in the input file; thus the maximum number of output files

in the default mode is 4.

When the -3 option is specified, srec splits each 24-bit

word into bytes and stores the bytes in three parallel S1

records.  For example, the following DSP56000 DATA record:

address field

|

_DATA P 0000

0008F8 300000 340000 094E3E

|      |      |      |

|      |      |      fourth word

|      |      third word

|      second word

first word

would be converted by srec into the three S1 records below:

byte count field

| address  field

| |

S1070000F800003EC2 -- low  byte

S10700000800004EA2 -- mid  byte

S1070000003034098B -- high byte

| | | | |

| | | | checksum field

| | | fourth word

| | third word

| second word

first word

The three records corresponding to the high, middle, and

low bytes of each data word are written to separate files.

The files are named according to the following convention:

<basename>.<M><#>

where <basename> is the filename of the input load file

without extension, <M> is the memory space specifier

(X, Y, L, or P) for this set of data words, and # is

one of the digits 0, 1, or 2 corresponding to low, middle,

and high bytes, respectively.

Note that a separate set of byte-wide files is created for

each memory space encountered in the input file.  Thus the

number of output files generated is (number of memory spaces

in input * 3).  The maximum number of output files generated

with the -3 option is 12.

The -1 option writes all information to a single file,

storing the memory space information in the address field

of the S0 header record.  The values stored in the address

field and their correspondence to the DSP56000 memory

spaces are as follows:

Value           DSP56000 Memory Space

-----           ---------------------

1                     X

2                     Y

3                     L

4                     P

When the memory space changes in the DATA or BLOCKDATA

record, a new S0 header record is generated.  The resulting

output file is named <basename>.s, where <basename> is

the filename of the input load file without extension.

The -1 and -3 options are mutually exclusive.

The -r option causes srec to produce the same type of output

as the default mode, except that the bytes are written high

to low, rather than low to high.

Address fields in the DSP56000 records are two bytes and

are copied as is to the appropriate S1 record (except for

incrementing as necessary in very long DATA records).  Only

the module id in the START record is passed as header data

for the S0 record; the version number, revision number, and

comment are ignored.

NOTES

One (and only one) of the operating system DEFINEs below

must be activated to obtain appropriate conditional

compilation.  This can be done either directly in the source

by changing the 0 DEFINE for the desired operating system

to a 1, or via the compiler command line (ex. -DATT=1).

Note that the UNIX flag is simply a logical ORing of the

more specific Unix OS flags (ATT and BSD) and thus should

not be specified explicitly.

OPTIONS

-1      - write data to a single file, putting memory

space information into the address field of the

S0 header record. Bytes may be reversed with

the -r option.  The -1 and -3 options are

mutually exclusive.

-3      - split each 24-bit word into bytes and store

the bytes in three parallel S1 records.   The

-1 and -3 options are mutually exclusive.

-r      - produce the same type of output as default mode,

but write bytes high to low, rather than low to

high.  to low, rather than low to high.

DIAGNOSTICS

The program does some checking for invalid input record

formats, memory spaces, and hex values.  It obviously

cannot check for bad data values if the load file has

been tampered with.  Both START and END records must be

found in the input file.

HISTORY

1.0     The beginning.

1.1     Added code to support new default mode and -r

option (-3 was old default mode).

1.2     Added support for ATT Unix System V.

1.3     Fix for bug in default mode where S-record address

was not being computed correctly (e.g. not a byte

multiple of the 56000 word size).

1.4     Added support for Macintosh II.

1.5     Added separate include file, getopt () support,

-1 (single output file) option.

1.6     Fix for bug in default mode when handling BLOCKDATA

records; repeated call to reverse bytes was reversing

bytes in place!