Generic MPU-401 Device Driver

Copyright (C) 1990-1991, Delta Music Systems

Installation Instructions

Introduction

The generic MPU-401 Device Driver is for MIDI interfaces in AT compatible

systems with an ISA bus.  It was written under OS/2 1.21, and has been

tested under 1.21, 1.3, and 2.0.

The device driver supports the complete set of MPU commands.  Extensions

specific to OS/2 have been added, and are available through the DMS MPU

Interface Library.  If you are interested in purchasing the library, please

use the order form (see the file ORDER.FRM).

This software is not in the public domain; however, it may be distributed

freely so long as it is not bundled with any commercial product, or any fee

beyond duplication and shipping is charged for distribution.  If you would

like information on distributing this software with another product, please

contact us at the address below.

Delta Music Systems

2615 Ginghamsburg-Frederick Rd.

Tipp City, OH  45371

If you would like to be notified of future products from Delta Music Systems,

please fill out and return the registration form (see the file REGIST.FRM).

Installation

To install the device driver, copy the MPU401.SYS file to your \OS2 directory.

Edit your CONFIG.SYS file, adding the line

DEVICE=C:\OS2\MPU401.SYS

If you find a line DEVICE=C:\OS2\COM01.SYS in your CONFIG.SYS, add the

MPU

statement before the COM line.  Otherwise, location is of no relevance.

If you are using the default port and interrupts (Port 330 and Interrupt 9),

you are finished.  Save the file and reboot your computer to complete the

installation.

If your interface is not configured for the default port or interrupt, add

one or both of the following parameters

Port

/Pppp  -  where ppp is the port location.

e.g.  for port 310:

DEVICE=C:\OS2\MPU401.SYS /P310

Interrupt

/Inn   -  where nn is the interrupt used.

e.g.  for interrupt 5:

DEVICE=C:\OS2\MPU401.SYS /I5

Both

e.g.  /P310 /I5

DEVICE=C:\OS2\MPU401.SYS /P310 /I5

After making the necessary changes, save the file and reboot your computer.

When the system is rebooted, you will see (albeit briefly) a message from

the device driver giving copyright and version information, as well as the

port and interrupt values.  If an error occurs, the booting process will

pause.  Check the error message, and hit F1 to continue the boot.  The

MPU device driver will not install if any errors are detected.  Correct the

errors and reboot.

Installation Check list

=======================

___  Install the interface according to the manufacturer's instructions.

___  Install OS/2

___  Copy the MPU401.SYS file from the installation disk to the C:\OS2

directory.

___  When OS/2 is installed and running, edit the CONFIG.SYS file (You can

use the system editor in the MAIN program group), and add the

DEVICE = C:\OS2\MPU401.SYS line as shown above.  Don't forget to save

the file.

___  Shutdown OS/2 from the Desktop Manager Group.  When the system is

completely shut down, reboot the computer.

___  Watch the screen during boot-up to verify the correct installation

of the device driver.  When the driver is loaded, it will display a

message indicating the port and interrupt being used.