MIDIMENU for the Roland D-5/10/20.

MIDIMENU lets you control the Timbre and Patch settings for a Roland D-5, D-10,

or D-20 synthesizer using an IBM PC computer.  The advantage is that you can

see all the Timbre parameters of all 8 parts at once, rather than finding them

by pressing a lot of buttons and looking at the 32-character LCD display.

Parameters can be changed by moving the menu selection bar to the desired

item, then pressing +, -, or return.  When selecting a new tone, a popup

window lets you choose from a list of all available tone names.  Mouse input

is supported -- volume and pan sliders can be adjusted with simple point-and-

shoot.  Works with all display types.

MIDIMENU version 1.0 is distributed as shareware.  If you like it, please

send $25 to Tony Laundrie, 1005 Tompkins Drive, Madison, WI 53716.  MIDIMENU

provides timbre and patch adjustment and file operations only.  With enough

user support, I will add graphical tone editing in a future version.  If

interest warrants, other types of synthesizers will be accomodated as well.

(In particular, a Roland U-110/220 version is under construction.)

****

Lacking any feedback after several months, I have decided to post the

source code.  Please modify it and let me know your improvements.  Thanks.

****

The program is invoked on the command line with "midimenu".  It works with

MPU-401 compatible devices, and assumes I/O port 0x330.  To change that,

you can provide a command line option: "-p port", where port is in

hexadecimal.  For D5 owners, provide the "-d5" option to prevent MIDIMENU

from attempting Reverb operations.  I haven't thoroughly tested this--please

send me feedback if you own a D5!

Here is all I'm providing now for other documentation (just experiment!):

- Use INITIALIZE after you push any buttons on your synthesizer while MIDIMENU

is in operation.

- Note that the menu items in the lower right can be called by just pressing

the first character.

- TIMBS SAVE only saves the temporary timbre and tone information to disk.

- PATCH SAVE only saves the temporary upper/lower and tone information.

- ALL SAVE saves everything possible, including rhythm setups, etc.

- LOAD FILE just sends a sysex file to the synthesizer.

- The format of the .syx files is plain binary system exclusive messages.  They

can be used with Cakewalk or Getit.

- ^TIMBS WRITE (hotkey = ^T) is used to write one or more of the 8 temporary

timbres to one of the 128 permanent timbre banks on the synth.

- ^PATCH WRITE (hotkey = ^P) is used to write the temporary patch to one of

the 128 permanent patch banks on the synth.  *** bug: ^P is a printer hotkey

- Before calling the ^PATCH WRITE feature, you might want to change the name

of the temporary patch first.  This is accomplished by hitting a period "."

when the Patch Name is highlighted.

- The volumes have only 26 positions.  Use the panel buttons for finer

precision.

- If you have two or more patches with the same name, don't be surprised if

MIDIMENU gets the bank numbers confused.

On old D-20s (ROM level < 1.03), if you create a sysex file with the Data

Transfer All Send panel operation, and later you send that file back to the

D-20 without putting the D-20 in the Data Transfer All Receive mode, the D-20

will crash, and the only way to reset the D-20 is to remove or temporarily

short out the internal battery.  I narrowed the problem down to sysex messages

for rhythm patterns and rhythm tracks and two "mystery" locations, 0B 10 00

and 0B 12 00, which are not documented in my manual.

Therefore, by default, the ALL SAVE feature of MIDIMENU will not

save rhythm patterns or the rhythm tracks, and the LOAD FILE feature will

not attempt to transfer those messages.  If you dare to circumvent this

precaution, then invoke MIDIMENU with the "-all" option.  The latest ROM

version for the D-20 does not suffer this problem.

Like I said, the program lacks tone editing.

Tony Laundrie

ttl@cs.wisc.edu

June 23, 1991

*** September, 1991