CHAPTER 11   MACROS AND CONDITIONAL ASSEMBLY              11-1

Macro Facility

----- --------

A86 contains an easy-to-use, but very powerful macro facility.

The facility subsumes the capabilities of most assemblers,

including operand concatenation, indefinite repeat (often called

IRP), and indefinite-repeat character (IRPC).  Unlike other

assemblers, A86 integrates these functions into the main macro

facility; so they can be invoked without clumsy syntax, or

strange characters in the macro-call operands.

Simple Macro Syntax

All macros must be defined before they are used.  A macro

definition consists of the name of the macro, followed by the

word MACRO, followed by the text of the macro, followed by #EM,

which marks the end of the macro.

Many assembly languages require a list of dummy operand-names to

follow the word MACRO.  A86 does not: the operands are denoted in

the text with the fixed names #1, #2, #3, ... up to a limit of

#9, for each operand in order.  If there is anything following

the word MACRO, it is considered part of the macro text.

Examples:

; CLEAR sets the register-operand to zero.

CLEAR  MACRO  SUB #1,#1 #EM

CLEAR AX        ; generates a SUB AX,AX instruction

CLEAR BX        ; generates a SUB BX,BX instruction

; MOVM moves the second operand to the first operand.  Both operands can be

;   memory-variables.

MOVM  MACRO

MOV AL,#2

MOV #1,AL

#EM

VAR1  DB  ?

VAR2  DB  ?

MOVM VAR1,VAR2    ; generates MOV AL,VAR2 followed by MOV VAR1,AL

11-2

Formatting in macro definitions and calls

The format of a macro definition is flexible.  If the macro text

consists of a single instruction, the definition can be given in

a single line, as in the CLEAR macro given above.  There is no

particular advantage to doing this, however: the assembler prunes

all unnecessary spaces, blank lines, and comments from the macro

text before entering the text into the symbol table.  I recommend

the more spread-out format of the MOVM macro, for program

readability.

All special macro-operators within a macro definition begin with

a hash-sign # (a hex 23 byte).  The letters following the hash-

sign can be given in either upper-case or lower-case.  Hash-sign

operators are recognized even within quoted strings.  If you wish

the hash-sign to be treated literally, and not as the start of a

special macro-operator, you must give 2 consecutive hash signs:

##.  For example:

FOO MACRO

DB '##1'

DB '#1'

#em

FOO abc      ; produces  DB '#1'  followed by  DB 'abc'

The format of the macro call line is also flexible.  A macro call

consists of the name of the macro, followed by the operands to be

plugged into the macro.  The assembler prunes leading and

trailing blanks from the operands of a macro call.  The operands

to a macro call are always separated by commas.  Also, as in all

assembler source lines, a semi-colon occurring outside of a

quoted-string is the start of a comment, ignored by the

assembler.  If you want to include commas, blanks, or semi-colons

in your operands, you must enclose your operand in single-quotes.

Macro operand substitution

Some macro assemblers expect the operands to macro calls to

follow the same syntax as the operands to instructions.  In those

assemblers, the operands are parsed, and reduced to numeric

values before being plugged into the macro definition text.  This

is called "passing by value".  A86 does not pass by value, it

passes by text.  The only parsing of operands done by the macro

processor is to determine the start and the finish of the operand

text.  That text is substituted, without regard for its contents,

for the "#n" that appears in the macro definition.  The text is

interpreted by the assembler only after a complete line is

expanded and as it is assembled.

11-3

If the first non-blank character after the macro name is a comma,

then the first operand is null: any occurrences of #1 in the

macro text will be deleted, and replaced with nothing.  Likewise,

any two consecutive commas with no non-blanks between them will

result in the corresponding null operand.  Also, out-of-range

operands are null; for example, #3 is a null operand if only two

operands are provided in the call.

Null operands to macros are not in themselves illegal.  They will

produce errors only if the resulting macro expansion is illegal.

The method of passing by text allows operand-text to be plugged

anywhere into a macro, even within symbol names.  For example:

; KF_ENTRY creates an entry in the KFUNCS table, consisting of a

;   pointer to a KF_-action-routine.  It also declares the

;   corresponding CF_-symbol, which is the index within the table

;   for that entry.

KF_ENTRY  MACRO

CF_#1  EQU  ($-KFUNCS)/2+080

DW  KF_#1

#EM

KFUNCS:

KF_ENTRY  UP

KF_ENTRY  DOWN

; The above code is equivalent to:

;

;  KFUNCS:

;    DW KF_UP

;    DW KF_DOWN

;

;  CF_UP    EQU  080

;  CF_DOWN  EQU  081

Quoted-string operands

As mentioned before, if you want to include blanks, commas, or

semicolons in your operands, you enclose the operand in single-

quotes.  In the vast majority of cases in which these special

characters need to be part of operands, the user wants them to be

quoted in the final, assembled line also.  Therefore, the quotes

are passed in the operand.  To override this, and strip the

quotes from the string, you precede the quoted string with a

hash-sign.  Examples:

11-4

DBW  MACRO

DB  #1

DW  #2

#EM

DBW  'E', E_POINTER

DBW  'W', W_POINTER

; note that if quotes were not passed, the above lines would have

;   to be   DBW  '''E''', E_POINTER;    DBW  '''W''', W_POINTER

GENERAL_PUSH  MACRO

PUSH#1

#EM

GENERAL_PUSH  F        ; generates a PUSHF instruction

GENERAL_PUSH  #' AX'   ; generates a PUSH AX instruction

The fact that I could not come up with a more useful example than

GENERAL_PUSH is strong evidence that it is much better to pass

the quotes as the default action.

Looping by operands in macros

This macro facility contains two kinds of loops: you can loop

once for each operand in a range of operands; or you can loop

once for each character within an operand.  The first kind of

loop, the R-loop, is discussed in this section; the second kind,

the C-loop, is discussed later.

An R-loop is a stretch of macro-definition code that is repeated

when the macro is expanded.  In addition to the fixed operands #1

through #9, you can specify a variable operand, whose number

changes each time through the loop.  You give the variable

operand one of the 4 names #W, #X, #Y, or #Z.

An R-loop begins with #R, followed immediately by the letter

W,X,Y, or Z naming the variable, followed by the number of the

first operand to be used, followed by the number of the last

operand to be used.  After the #Rxnn is the text to be repeated.

The R-loop ends with #ER.  For example:

STORE3 MACRO

MOV AX,#1

#RY24               ; "repeat for Y running from 2 through 4"

MOV #Y,AX

#ER

#EM

STORE3  VAR1,VAR2,VAR3,VAR4

; the above call produces the 4 instructions MOV AX,VAR1; MOV VAR2,AX;

;   MOV VAR3,AX; MOV VAR4,AX.

11-5

The #L last operator and indefinite repeats

The macro facility recognizes the special operator #L, which is

the last operand in a macro call.  #L can appear anywhere in

macro text; but its big power occurs in conjunction with R-loops,

to yield an indefinite-repeat facility.

A common example is as follows: you can take any macro that is

designed for one operand, and easily convert it into a macro that

accepts any number of operands.  You do this by placing the

command #RX1L, "repeat for X running from 1 through L", at the

start of the macro, and the command #ER at the end just before

the #EM.  Finally, you replace all instances of #1 in the macro

with #X.  We see how this works with the CLEAR macro:

CLEAR MACRO #RX1L

SUB #X,#X

#ER

#EM

CLEAR  AX,BX   ; generates both SUB AX,AX and SUB BX,BX in one

macro!

It is possible for R-loops to iterate zero times.  In this case,

the loop-text is skipped completely.  For example, CLEAR without

any operands would produce no expanded text.

Character-loops

We have seen the R-loop; now we discuss the other kind of loop in

macros, the character-loop, or C-loop.  In the C-loop, the

variable W,X,Y, or Z does not represent an entire operand; it

represents a character within an operand.

You start a C-loop with #C, followed by one of the 4 letters

W,X,Y, or Z, followed by a single operand-specifier.  Following

the #Cxn is the text of the C-loop.  The C-loop ends with #EC.

The macro will loop once for every character in the operand.

That single character will be substituted for each instance of

the indicated variable-operand.  For example:

PUSHC  MACRO  #CW1

PUSH #WX

#EC#EM

PUSHC ABC ; generates the 3 instructions PUSH AX; PUSH BX; PUSH CX

If the C-operand is quoted in the macro call, the quotes ARE

removed from the operand before passing characters to the loop.

It is not necessary to precede the quoted string with a hash-

sign in this case.  If you do, the hash-sign will be passed as

the first character.

If the C-operand is a null operand (no characters in it), the

loop-text is skipped completely.

11-6

The "B"-before and "A"-after operators

So far, we have seen that you can specify operands in your macro

in fourteen different ways: 1,2,3,4,5,6,7,8,9,W,X,Y,Z,L.  We now

multiply these 14 possibilities, by introducing the "A" and "B"

operators.  You can precede any of the 14 specifiers with "A" or

"B", to get the adjacent operand after or before the specified

operand.  For example, BL means the operand just before the last

operand; in other words, the second-to-the-last operand.  AZ

means the operand just after the Z operand.  You can even repeat,

up to a limit of 4 "B"s or 3 "A"s:  BBL is the third-to-last

operand; #AAA9 can be used where you would want to (but cannot)

use #12.

In the case of the variable operand to a C-loop, the "A" and "B"

specifiers denote the characters before or after the current

looping-character.  An example of this is given in the next

section.

Multiple-increments within loops

We have seen that you end an R-loop with a #ER, and you end a C-

loop with a #EC.  We now present another way to end these loops;

a way that lets you specify a larger increment to the macro's

loop-counter.  You can end your loops with one of the 4

additional commands #E1, #E2, #E3, or #E4.

For R-loops terminated by #ER, the variable-operand advances to

the next operand when the loop is made.  If you end your R-loop

with #E2, the variable-operand advances 2 operands, not just one.

For #E3, it advances 3 operands; for #E4, 4 operands.  The #E1

command is the same as #ER.

The most common usage of this feature is as follows:  You will

recall that we generalized the CLEAR macro with an R-loop, so

that it would take an indefinite number of operands.  Suppose we

want to do the same thing with the DBW macro.  We would like DBW

to take any number of operands, and alternate DBs and DWs

indefinitely on the operands.  This is made possible by creating

an R-loop terminated by #E2:

DBW  MACRO  #RX1L

DB  #X

DW  #AX

#E2

#EM

DBW  'E',E_POINTER,  'W',W_POINTER   ; two pairs on same line!

The #E2 terminator means that we are looping on a pair of

operands.  Note the crucial usage of the "A"-after operator to

specify the second operand of the operand-pair.

11-7

A special note applies to the DBW macro above: the assembler just

happens to accept a DW directive with no operands (it generates

no object code, and issues no error).  This means that DBW will

accept an odd number of operands with no error, and do the

expected thing (it alternates bytes and words, ending with a

byte).

You could likewise generalize a macro with 3 or 4 operands, to an

indefinite number of triples or quadruples; by ending the R-loop

with #E3 or #E4.  The operands in each group would be specified

by #X, #AX, #AAX, and, for #E4, #AAAX.

For C-loops terminated by #E1 through #E4, the character-pointer

is advanced the specified number of characters.  You use this in

much the same way as for R-loops, to create loops on pairs,

triplets, and quadruplets of characters.  For example:

PUSHC2  MACRO  #CZ1

PUSH #Z#AZ

#E2

#EM

PUSHC2  AXBXSIDI    ; generates PUSH AX; PUSH BX; PUSH SI; PUSH DI

Negative R-loops

We now introduce another form of R-loop, called the Q-loop-- the

negative repeat-loop.  This loop is the same as the R-loop,

except that the operand number decrements instead of increments;

and the loop exits when the number falls below the finish-number,

not above it.  The Q-loop is specified by #Qxnn instead of #Rxnn,

and #EQ instead of #ER.  You can also use the multiple-decrement

forms #E1 #E2 #E3 or #E4 to terminate an Q-loop.

Example:

MOVN MACRO #QXL2   ; "negative-repeat X from L down to 2"

MOV #BX,#X

#EQ#EM

MOVN AX,BX,CX,DX    ; generates the three instructions:

;    MOV CX,DX

;    MOV BX,CX

;    MOV AX,BX

Note: the above functionality is already built into the MOV

instruction of the assembler.  The macro shows how you would

implement it if you did not already have this facility.

Nesting of loops in macros

This macro facility allows nesting of loops within each other.

Since we provide the 4 identifiers W,X,Y,Z for the loop-operands,

you can nest to a level of 4 without restriction-- just use a

different letter for each nesting level.  You can nest even

deeper, subject to the restriction that a letter W,X,Y,Z refers

to the innermost containing loop that defines it.

11-8

Implied closing of loops

If you have a loop or loops ending when the macro ends, and if

the iteration count for those loops is 1, you may omit the #ER,

#EC, or #EQ.  The assembler closes all open loops when it sees

#EM, with no error.

For example, if you omit the #ER for the loop-version of the

CLEAR macro, it would make no difference-- the assembler

automatically places an #ER code into the macro definition for

you.

Local labels in macros

Some assemblers have a LOCAL pseudo-op that is used in

conjunction with macros.  Symbols declared LOCAL to a macro have

unique (and bizarre) symbol-names substituted for them each time

the macro is called.  This solves the problem of duplicate label

definitions when a macro is called more than once.

In A86, the problem is solved more elegantly, by having a class

of generic local labels throughout assembly, not just in macros.

Recall that symbols consisting of a single letter, followed by

one or more decimal digits, can be redefined.  You can use such

labels in your macro definitions.

I have recommended that local labels outside of macros be

designated L1 through L9.  Within macro definitions, I suggest

that you use labels M1 through M9.  If you used an Ln-label

within a macro, you would have to make sure that you never call

the macro within the range of definition of another Ln-label with

the same name.  By using Mn-labels, you avoid such potential

conflicts.

The following example of a local label within a macro is taken

from the source of the macro-processor itself:

11-9

; "JHASH label" checks to see if AL is a hash sign.  If it is,

;    it processes the hash-sign term, and jumps to label.

;    Otherwise, it drops through to the following code.

JHASH MACRO

CMP AL,'##'         ; is the scanned character a hash-sign?

JNE >M1             ; skip if not

CALL MDEF_HASH      ; process the hash sign

JMP #1              ; jump to the label provided

M1:

#EM

...

L3:                   ; loop here to eat empty lines, leading blanks

CALL SKIP_BLANKS    ; skip over the leading blanks of a line

INC SI              ; advance source ptr beyond the next non-blank

JHASH L3            ; if hash-sign then process, and eat more blanks

CMP AL,0A           ; were the blanks terminated by a linefeed?

JE L3               ; loop if yes, nothing on this line

L5:                   ; loop here after a line is seen to have contents

CMP AL,';'          ; have we reached the start of a comment?

JE L1               ; jump if yes, to consume the comment

JHASH >L6           ; if hash-sign then process it; get next char

...

L6:

LODSB               ; fetch the next definition-char from the source

CMP AL,' '          ; is it blank?

JA L5               ; loop if not, to process it

...

Debugging macro expansions

There is a tool called EXMAC which will help you troubleshoot

program lines that call macros.  If you are not sure about what

code is being generated by your macro calls, EXMAC will tell you.

See Chapter 13 for details.

11-10

Conditional Assembly

----------- --------

A86 has a conditional assembly feature, that allows you to

specify that blocks of source code will or will not be assembled,

according to the values of equated user symbols.  The controlling

symbols can be declared in the program (and can thus be the

result of assembly-time expressions), or they can be declared in

the assembler invocation.

You should keep in mind the difference between conditional

assembly, invoked by #IF, and the structured-programming feature,

invoked by IF without the hash-sign.  #IF tests a condition at

assembly-time, and can cause code to not be assembled and thus

not appear in the program.  IF causes code to be assembled that

tests a condition at run-time, possibly jumping over code.  The

skipped code will always appear in the program.

All conditional assembly lines are identified by a hash-sign #

as the first non-blank character of a line.  The hash-sign is

followed by one of the four keywords IF, ELSEIF, ELSE or ENDIF.

#IF starts a conditional-assembly block.  On the same line,

following the #IF, you provide a name.  If the name is undefined,

or if it has been equated to zero, then the following lines of

code are skipped, up to the next matching #ELSEIF, #ELSE, or

#ENDIF.  If the name is non-zero, then the following lines of

code are assembled normally.  If a subsequent matching #ELSEIF or

#ELSE is encountered, then code is skipped up to the matching

#ENDIF.

#ELSEIF provides a multiple-choice facility for #IF-blocks.  You

can give any number of #ELSEIFs between an #IF and its matching

#ENDIF.  Each #ELSEIF has a name following it on the same line.

If the name following the #IF has zero value, then the assembler

looks for the first non-zero name following an #ELSEIF, and

assembles that block of code.  If there are no non-zero #ELSEIFs,

then the #ELSE-block (if there is one) is assembled.

It is legal to provide an undefined name after #IF or #ELSEIF.

The name is interpreted as being false (zero), with no error.

You may precede the name in an #IF or #ELSEIF line with an

exclamation point "!", which acts as a NOT-operator: code will be

skipped if the name is non-zero instead of zero.

#ELSE marks the beginning of code to be assembled if all the

previous blocks of an #IF have been skipped over.  There is no

operand after the #ELSE.  There can be at most one #ELSE in an

#IF-block, and it must appear after any #ELSEIFs.

#ENDIF marks the end of an #IF-block.  There is no operand after

#ENDIF.

It is legal to have nested #IF-blocks; that is, #IF-blocks that

are contained within other #IF-blocks.  #ELSEIF, #ELSE, and

#ENDIF always refer to the innermost nested #IF-block.

11-11

As an example of conditional assembly, suppose that you have a

program that comes in three versions: one for Texas, one for

Oklahoma, and one for the rest of the nation.  The three programs

differ in a limited number of places.  Instead of keeping three

different versions of the source code, you can keep one version,

and use conditional assembly on the boolean variables TEXAS and

OKLAHOMA to control the assembler output.  A sample block would

be:

#if TEXAS

DB 0,1,2,3

#elseif OKLAHOMA

DB 4,5,6,7

#else

DB 8,9,10,11

#endif

If a block of code is to be assembled only if TEXAS is false,

then you would use the exclamation-point operator:

#if !TEXAS

DB 0FF

#endif

Conditional Assembly and Macros

You may have conditional-assembly blocks either in macro-

definitions or in macro expansions.  The only limitation is that

if you have an #IF-block in a macro expansion, the entire block

(i.e., the matching #ENDIF) must appear in the same macro

expansion.  You cannot, for example, define a macro that is a

synonym for #IF.

To have your conditional-assembly block apply to the macro

definition, you provide the block normally within the definition.

For example:

X1 EQU 0

BAZ MACRO

#if X1

DB 010

#else

DB 011

#endif

#EM

BAZ

X1 EQU 1

BAZ

In the above sequence of code, the conditional-assembly block is

acted upon when the macro BAZ is defined.  The macro therefore

consists of the single line DB 011, with all the conditional-

assembly lines removed from the definition.  Thus, both

expansions of BAZ produce the object-code byte of 011, even

though the local label X1 has turned non-zero for the second

invocation.

11-12

To have your conditional-assembly block appear in the macro

expansion, you must literalize the hash-sign on each

conditional-assembly line by giving two hash-signs:

X1 EQU 0

BAZ MACRO

##if X1

DB 010

##else

DB 011

##endif

#EM

BAZ

X1 EQU 1

BAZ

Now the entire conditional-assembly block is stored in the macro

definition, and acted upon each time the macro is expanded.

Thus, the two invocations of BAZ will produce the different

object bytes 011 and 010, since X1 has become non-zero for the

second expansion.

You will usually want your conditional-assembly blocks to be

acted upon at macro-definition time, to save symbol-table space.

You will thus use the first form, with the single hash-signs.

Conditional Assembly and the XREF Program

In most cases, the XREF program will recognize conditional-

assembly blocks, and ignore skipped-code in its XREF compilation.

The last macro example above, however, is an example in which

XREF will not skip the same blocks that the assembler will;

because it falls under the following

WARNING: The XREF program will use the value of all symbols as it

existed at the end of assembly.  XREF does not parse

statements that change the value of local variables!  Thus, if

you have conditional assembly based on a variable whose value

changes during assembly, XREF will compile different source

than the assembler assembled.

The above warning does not apply to invocation-variables,

described below. If you wish to change the value of a

conditional-control variable during assembly, and if you wish

XREF to give accurate results, you should change the variable

between file-names in the invocation, as described below.

11-13

Declaring Variables in the Assembler Invocation

To facilitate the effective use of conditional assembly, this

assembler allows you to declare boolean (true-false) symbols in

the command-line that invokes the assembler.  The declarations

can appear anywhere in the list of source file names.  They are

distinguished from the file names by a leading equals-sign =.  To

declare a symbol TRUE (value = 1), give the name after the

equals-sign.  DO NOT put any spaces between the equals-sign and

the name! To declare a symbol FALSE (value = 0), you can give an

equals-sign, an exclamation-point, then the name.  Again, DO NOT

embed any blanks!  Example: if your source files are src1.8,

src2.8, and src3.8, then you can assemble with TEXAS true by

invoking the assembler as follows:

a86 =TEXAS src1.8 src2.8 src3.8

You can assemble with TEXAS explicitly set to FALSE as follows:

a86 =!TEXAS src1.8 src2.8 src3.8

Note that if TEXAS is used only as a conditional-assembly

control, then you do not need to include the =!TEXAS in the

invocation, because an undefined TEXAS will automatically be

interpreted as false.

Null Invocation Variable Names

The assembler will ignore an equals-sign by itself in the

invocation line, without error.  This allows you to generate

assembler-invocation lines using parameters that could be either

boolean-variable-names, or null strings. For example, in the

previously-mentioned TEXAS-OKLAHOMA-nation example, the program

could be invoked via a .BAT file called "AMAKE.BAT", coded as

follows:

A86 =%1 *.8

You invoke the assembler by typing one of the following:

amake texas

amake oklahoma

amake

The third line will produce the assembler-invocation  A86 = *.8;

causing no invocation-variables to be declared.  Thus both TEXAS

and OKLAHOMA will be false, which is exactly what you want for

the rest-of-the-nation version of the program.

11-14

Changing Values of Invocation Variables

The usual prohibition against changing the value of a symbol that

is not a local-label does not apply to invocation-variables.  For

example, suppose you have a conditional-control variable DEBUG,

which will generate diagnostic code for debugging when it is

true.  Suppose further that you have already debugged source

files src1.8 and src3.8; but you are still working on src2.8. You

may invoke the assembler as follows:

A86 src1.8 =DEBUG src2.8 =!DEBUG src3.8

The variable DEBUG will be TRUE only during assembly of src2.8,

just as you want.