CDBENCH version 2.4 (C) 1992 Profit Press

CDBENCH reports CD-ROM interface performance information. It will only

operate on the Profit Press MegaDEMO CD-ROM. By operating on only this

CD-ROM (pressed by a single company) more reliable results can be

obtained. The run time of this program can be up to 5 minutes and is

neccessary to produce results that can be measured accurately by the

system timer.

Information on benchmarks

1/3 stroke seek:        The standard for determining the "average" seek

time of a drive. This test performs 60 seeks back

and forth across the disc.

Random seek:            Performs 60 seeks to a "random" areas of the CD-ROM

under the 350 megabyte mark. The seeks are always

the same every time CDBENCH is run. (allows for

relatively short seek comparisons between drives)

1/3 stroke seek & read: Performs 60 staggered seeks (to defeat caching)

over 1/3 of the CD-ROM. After each seek a single

2k sector is read. This test will defeat some

CD-ROM drives and caching programs that ignore

seek commands.

Sustained long read:    Reads 2 megabytes of consecutive sectors off

the CD-ROM in 32k blocks. This represents the

maximum data rate. Multimedia compatible drives

should read more than 150,000 bytes per second.

Sustained short read:   Reads 512k of consecutive sectors in single

sector (2k) blocks. This will show how well

the buffer or cache in the CD-ROM drive handles

small but frequent accesses.

MegaSearch benchmark:   A simulation of common file handling operations

used by MegaSearch Retrieval Software and MegaDOOR.

This should provide a good indication of relative

DOS random access speeds to the CD-ROM.

A database of CD-ROM results will be maintained on the Profit Press BBS.

The phone number is (602)770-0008 and we support 2400-14400 with v.32bis.

If you'd like an ASCII output of your results type CDBENCH <filename>

(preferably a filename that doesn't output to the CD-ROM!). Blanks will

be left in your output file to add information about your CD-ROM and

system configuration. Please upload your results to our BBS!

The table below reflects benchmark results attained from the CDBENCH utility

included on the Profit Press MegaDEMO CD-ROM disc. If a drive is repeated

more than once it's because we received more than one response for it, and

ASCII files are tiny - so we included it.

Results as of 1/26/93 include:

Drive                 1/3 seek  Rseek  Seek/read  Longread   Shortread  MS

------------------------------------------------------------------------------

0 Toshiba 3201B (8/89)   N/A      N/A    407.5    153903.9   153903.9   32.6

1 Panasonic CR-501-C   630.95   473.44   501.83   154526.9    89177.9   31.65

2 Genstar 2000         475.3    310.4    761.0     69020.2     7466.4   N/A

3 NEC CDR-80             N/A      N/A    865.38    54139.24    5059.41  82.80

4 Amdek Laserdrive-1     N/A      N/A   1202.38    53909.84    4985.39 121.43

5 Texel DM3021/5021      N/A      N/A    392.9    154527.0   156426.9   41.8

6 DAK/BSR $199 ext.    802.2    614.5   1046.7     68279.4    14199.5  115.3

7 Chinon ?             280.0    204.0    427.0    153903.0   144576.0   N/A

8 DAK/BSR $399 ext.    257.3    183.2    399.3     95420.4    14157.3   37.5

9 DAK/BSR $199 ext.    793.04   620.88  1076.01    66034.89   14011.81 111.98

10 Sony CDU-531         298.53   206.96   438.64    92193.64   13289.75  39.18

11 Mitsumi              789.38   616.30   940.48   153903.90  144576.39  76.98

12 Toshiba 3201B        379.12   337.00   398.35   154526.99  156426.91  33.46

13 Magnavox CDD461 ext.   N/A      N/A    933.15    30485.76    7261.83  80.82

14 Hitachi CDR3500 ext.   N/A      N/A    418.5     94710.1    25929.5   56.6

15 Texel 3024/5024      301.28   297.62   347.99   310310.30  183500.80  21.81

16 Texel 5024             N/A      N/A    315.93   310310.30  221907.94  26.21

17 MITSUMI #CRMC-LU     758.24   571.43  1011.90    94945.69   56797.87  94.34

18 Texel 3024             N/A      N/A    342.49   310310.30  272629.76  22.64

19 NEC CDR 84             N/A      N/A    390.11   310310.30  156426.91  32.75

20 Sony CDU-6101-11     351.65   267.40   531.14     7655.07   14094.60  54.40

21 Magnavox CDD461        N/A      N/A    935.90    25859.19    7340.03  71.81

22 DAK/BSR $199 ext.    783.8    660.26   916.67    94945.69   26359.23  91.04

N/A = NOT AVAILABLE

Information on drives:

0 Toshiba 3201B (8/89)

- 486/33 system with a WD-7000FAST 16 bit SCSI controller

1 Panasonic CR-501-C

- 486/50 system with a CSM 8 bit SCSI controller

2 Genstar 2000

- 386/20 system with proprietary 8 bit controller

3 NEC CDR-80

- 386/40 system with Trantor 8 bit SCSI

4 Amdek Laserdrive-1

- 386/40 system with proprietary 8 bit controller

5 Texel DM3021

- 386/25 system with Always IN-2000 16 bit SCSI

6 DAK/BSR $199 ext.

- 386/20 system with proprietary 8 bit controller

7 Chinon

- Unknown model number in CompuAd computer

8 DAK/BSR $399 ext.

- 386/40 system with proprietary 8 bit controller

This is an older OEM Sony CD-ROM drive.

9 DAK/BSR $199 ext.

- 386SX/20 system with proprietary 8 bit controller

10 Sony CDU-531

- i486/33 with Sony CDB-241 interface

11 Mitsumi (internal)

- 386DX/40 with Mitsumi interface card

12 Toshiba 3201B

- 386DX/33 with Adaptec 16bit SCSI controller

13 Magnavox CDD 461 External

- 386DX/25 with Philips CDD163/00 controller

14 Hitachi CDR-3500 external

- system and interface unknown

15 Texel 3024 internal doublespeed SCSI

- 486/50 system with a CSM 8 bit controller

16 Texel 5024 external doublespeed SCSI

- 386/33 Gateway with Trantor 130b 8-bit controller

17 MITSUMI #CRMC-LU (StorQuest STQ500CD)

- 386/25 DX with proprietary 8 bit interface

18 Texel 3024 internal doublespeed SCSI

- 486/33 with Trantor 130

19 NEC Intersect Model CDR 84

- 386/33 with MediaVision ProAudio16 SCSI

20 Sony CDU-6101-11

- 386DX/25 with Sony CDB-231 Nov, 1988

21 Magnavox CDD 461

- 486/33

22 DAK-$199-EXT

- 386DX/40 using 8-bit proprietary interface