PERSONALIZED RING FOR RBBS

--------------------------

In many locations, the phone companies offer a service that is known as

'Personalized Ringing', Distinctive Ringing', and other various names.  This

service allows one phone line to have 2 or 3 phone numbers associated with the

one line. Depending upon which number is called, a different ringing pattern

will be heard on a phone connected to this line.  For example, one line may be

assigned 111-1111 and 111-2222. When the first number is dialed by someone,

the ring heard on this line will be a regular sounding ring.  But when the

second number, 111-2222, is dialed by someone, the ring heard consists of 2

short rings in the amount of time that 1 regular ring normally occurs.

Imagine that the dashes in the following diagram represent ringing on this

line and you will be able to see the difference:

NORMAL RING -

------------------------- PAUSE -------------------------

SHORT PERSONALIZED RING -

------ SHORT PAUSE ------ PAUSE ------ SHORT PAUSE ------

This service allows a person to have 2 different phone numbers but only

have 1 phone line!  What's really great is that this service is usually VERY

inexpensive (somewhere around $5 to $8 installation and $2 per month for each

additional number past the first (up to 3 numbers total on a line).  (This

service may not be available on what the phone company calls 'hunting lines'.

A hunting line is a line on which a call can 'roll over' to or from another

line.  This is the type of line that most businesses have when they have more

than 1 line.  Personal Ring service is usually ONLY available on single,

dedicated lines, however, check with your phone company, as this may vary from

phone company to phone company).

This is ideal for someone who wants to share 1 line with 2 or 3 different

devices.  For example, 1 line can be shared with a FAX machine and with a BBS!

The disadvantage to this arrangement is that most BBS's do not have the

capability of detecting the different ring patterns, thus an additional piece

of equipment is needed to decode these different rings. This is still better

than paying monthly for 2 or 3 lines, but it would be great if a BBS could

detect these differences without any additional hardware.

This is where this merge for RBBS comes into play!  It allows this very

thing.  At the present time, it will allow RBBS to answer the phone on either

the regular ring or on a 2-short ring pattern as shown above. This works quite

well if RBBS is set up to answer the line that rings with a normal ring

pattern and the other device (FAX or answering machine) is set to answer on 4

or more rings.  In this case, RBBS will answer after the second normal ring

(just in case the first ring is abnormally short), but if the incoming ring is

the 2-short pattern, RBBS will ignore it and the FAX or answering machine can

then answer on the 4th (or more) ring.

USING THE PERSONAL RING FEATURE

Using the Personal Ring feature is also very easy.  To tell RBBS which ring

to answer on, select parameter 225 in the RBBS CONFIG program.  Then, include

in item 2 (INITIALIZE THE MODEM) S0 = 252 to tell RBBS to answer on the

2-short ring pattern or include S0=253 to tell RBBS to answer on the regular

ring pattern.  When RBBS is running, it will decipher this MODEM INITIALIZATION

STRING and determine which number S0 is set equal to.

This merge was developed for our own use, in order to provide a

customer support BBS sharing the same line with our FAX, but using

different phone numbers.  This works MUCH better than ANY of the typical

FAX SWITCHES that are available.

The copyright to this source code is held by it's author. It can be

freely distributed by non-profit organizations and on non-profit bulletin

boards.  No one may distribute this source code for a profit, nor can it

be included with any commercial product without our consent. Individuals

may freely pass copies of this code to other individuals.  Any copies

that are distributed MUST include this documentation file (DISRNG74.DOC).

We hope that others can benefit from this modification to RBBS.

John Rayfield, Jr.

RAYFIELD COMMUNICATIONS, INC.

2201 W. Battlefield Rd. #2

Springfield, MO  65807

417/887-4663

417/887-1434 (FAX)