Desqview 2.6x & High Speed Communications

-----------------------------------------

Table of Contents

Topic						Section

-----						-------

Preamble .......................................  1.0

Target Audience ................................  1.1

Doucment History ...............................  1.2

Desqview and High Speed Communications .........  2.0

Desqview Beyond COM1 & COM2 ....................  2.1

Desqview Factory Default Settings ..............  2.2

Desqview Related Key DOS Level Settings ........  2.3

QEMM Exception Errors #6,#13 etc ...............  2.4

Desqview Settings ..............................  2.5

Desqview & Shared Interrupts ...................  2.6

Desqview & Port Addressess .....................  2.7

Desqview Windows using Serial Ports ............  2.8

FIFO Transmit and Receive Buffers ..............  3.0

16450 vs 16550A UARTS ..........................  4.0

Hayes ESP Card .................................  5.0

Desqview & Hayes ESP Card ......................  5.1

DTE Locked Baud Rates ..........................  6.0

Modem Profiles NRAM ............................  7.0

Wildcat Multi-Line Setup .......................  8.0

Wildcat Multi-Line Setup Batch Files ...........  8.1

Wildcat .MDM File Settings .....................  8.2

Wildcat Upload and Download Failures............  9.0

Poor Quality Phone Lines........................  9.1

File Transfer Failure Hardware Factors..........  9.2

File Transfer Failure Software Factors..........  9.3

Summary ........................................ 10.0

Contacting the Author .......................... 11.0

1.0 - Preamble

--------------

This paper focuses on Desqview, Qemm, BBS Software, Hayes V.FAST Modems and

the ESP technolgy

It should be noted from the outset that Quarterdeck Office Systems, the OEM

of Desqview, have produced some very helpful .TEC documents which can be

found on CIS, Quarterdecks BBS or my Host BBS.

I have no formal association with the OEM mentioned below and enphasize

that the material that follows reflects my own personal view and

experience.

1.1 - Target Audience

---------------------

Both BBS operators and terminal emulation users will benefit from the

information in this document.

Desqview is used as the multi-tasker for the following BBS Packages:

GAP               REMOTEACCESS      TINYHOST          WWIV

MEGAHOST          ROBOBOARD/FX      TRIBBS

OPUS-CBCS	      SEARCHLIGHT       ULTRABBS

PCBOARD           SPITFIRE          VIRTUAL BBS

POWERBOARD        SYNCRONET         WILDCAT

1.2 - Document History

----------------------

Jan 1995 - v1.0		- First Public Release

v1.1		- Added Contents

- Added Multi-Line Help

- Added QEMM Exception Info

Feb 1995 - v1.2         - Minor changes to document

March 1995 - v1.3       - Began adding Help on File Transfer

Failures

- File renamed from DV&WCV10.TXT to

DV&WCHLP.ZIP

2.0 - Can Desqview Handle High Speed Serial Communications?

-----------------------------------------------------------

Yes!.  Despite the fact that the Desqview manual only mentions 2400 to 9600

bps and implies that 9600 bps is HIGH SPEED, Desqview is able to handle the

new V.FAST Class modems at full throughput.

2.1 - Can Desqview Address more than COM1 and COM2?

---------------------------------------------------

Yes!.  The Desqview documentation mentions COM1 & COM2 but omits to mentions

COM3 onwards.  Nevertheless any Desqview window can use the full range of

COM1 to COM8.  Desqview also indirectly supports FOSSIL drivers.

2.2 - Are the Desqview 'Factory Default' settings suitable for High Speed Links?

--------------------------------------------------------------------------------

No!.  A lot of people have literally given up on Desqview because they do

not have the patience to configure their DOS/Desqview environment to work

with programs that use Serial Ports.

2.3 - What are the Key Settings  -  DOS Level

---------------------------------------------

Firstly, I stronly recommend you use QEMM and not the DOS HIMEM.SYS and

EMM386.EXE drivers.

A typical DOS/QEMM CONFIG.SYS file might look like this in a STANDARD UART

environment with 1 to 4 Modem Lines.

CONFIG.SYS

----------

DEVICE    = C:\QEMM\DOSDATA.SYS

(1) DEVICE    = C:\QEMM\QEMM386.SYS R:2 RAM

(2) DEVICE    = C:\QEMM\DOS-UP.SYS @C:\QEMM\DOS-UP.DAT

DOS       = HIGH

FILES     = 80

(3) BUFFERS   = 30

(4) STACKS    = 0,0

COUNTRY   = 061,437 C:\DOS\COUNTRY.SYS

LASTDRIVE = H

(5) SHELL     = C:\QEMM\LOADHI.COM /R:2 \DOS\COMMAND.COM C:\DOS\ /E:2048 /p

A typical DOS/QEMM AUTOEXEC.BAT file might look like this in a STANDARD UART

environment with 1 to 4 Modem Lines.

AUTOEXEC.BAT

------------

@ECHO OFF

PROMPT=$P$G

(6) PATH=D:\;C:\WC4;\DV;C:\;\DOS;\NU;\QEMM;

\NU\IMAGE C:

CLS

(7) SHARE /F:5120 /L:80

(8) \ESP\ESPCA P280:1 R280:+ P288:1 R288:+ F

(9) \NU\NCACHE2 C: /EXP=4120 /BLOCK=8192 /QUICK=OFF /-I /WRITE=0

Notes:

(1)  The QEMM386.SYS parameters are explained clearly in the QEMM Guide so

I will not touch on them here.

(2)  A handy HINT is to backup all of the QEMM semi-dynamic data files to

avoid lenghty crash recover delays.

(4)  A lot of programs insert a STACKS 9,256 statement in your CONFIG.SYS

file this is of NO value to Desqview which automatically uallocates stack

space when requested by programs.

(5)  A number of Communications programs use DOS environment space for SET

commands such as  SET IRQID=  if insufficent environment space is

available you will loose these commands from the DOS environment table

which may stop your programs from working.  So make sure set aside

extra environment space if using lots of DOS variables

(6)  Program peformance is can be significantly improved if you optimise

your PATH= statement.  Frequently use programs should reside on

directory's closest to the = sign.

(7)  DO NOT load SHARE.EXE High or in your CONFIG.SYS file as an INSTALL

= command.  SHARE will not function correctly in HIGH memory.

(8)  If you using the HAYES ESP card, then configure the card on a single

line, for some reason invoking the program twice on some PC's causes

lock-ups.  (See the ESP Section for More Info)

(9)  Many users have reported problems with file transfers when using the

SMARTDRV program.  If you ensist on using SMARTDRV then use

SMARTDRV /X

to TURN OFF write behind caching.  I have found the Nortons NCACHE2

program to be a better DISK cache for use in the COMMS environment.

You may be able to get away with usingd delayed writes using NCACHE2.

If so the command line would be something like this:

\NU\NCACHE2 C: /EXP=4120 /BLOCK=8192 /MULTI=ON /WRITE=24

The main thing to remember about cache programs is that they can be

guilty of grabing software interrupts at just the WRONG time.

A typical DOS/QEMM CONFIG.SYS file might look like this in a DIGIBOARD or

FOSSIL driver environment with 1 to 8 Modem Lines.

DEVICE     = C:\QEMM\DOSDATA.SYS

(10) DEVICE     = C:\QEMM\QEMM386.SYS X=D000-DFFF RAM ST:M R:2

DEVICE     = C:\QEMM\DOS-UP.SYS @C:\QEMM\DOS-UP.DAT

DOS        = HIGH

FILES      = 200

BUFFERS    = 8

STACKS     = 0,0

COUNTRY    = 061,437 C:\DOS\COUNTRY.SYS

SHELL      = C:\QEMM\LOADHI.COM /R:3 /RES=4800 /SQT=D000-DAFF C:\COMMAND.COM /E:2048 /P

LASTDRIVE  = Z

(11) DEVICE     = C:\DIGI\DOS\XIDOS5.SYS

Notes:

(10) In the DigiBoard environment its is important to EXCLUDE the memory being

used by the Digiboards XIDOS5.SYS driver.

(11) Take Note that the Digiboard driver is Loaded LOW not high.  You will

experience problems if you load the driver HIGH.

2.4 - QEMM - Exception Errors

-----------------------------

One of the first problems that you may encounter when setting up your

system at the DOS Level is QEMM Exception Errors, typically #13.

The following .TEC Document Text from Quarterdeck may help!

QUARTERDECK QUARTERDECK QUARTERDECK QUARTERDECK QUARTERDECK QUARTERDECK

-----------------------------------------------------------------------

ID:X3 Troubleshooting Exception #6, #12, and #13 Errors

Quarterdeck Technical Note #232

Subject: Detailed explanation and troubleshooting procedures for

Exception #6, #12, and #13 error messages and system lockups.

OVERVIEW

--------

This document addresses Exception #6, #12 and #13 error messages.  These

three Exceptions are so similar in cause, nature, and solution that they

will all be covered by the information below.  Any reference to an

Exception error in this document applies to an Exception #6, Exception

#12, or Exception #13 error.

Please note that sometimes QEMM cannot report the error to the screen, so

a blank screen or a lockup occurs.  For troubleshooting purposes, treat a

system lockup or a blank screen (that accepts no input) to be the same as

an Exception error.

This troubleshooting procedure will isolate and resolve the conflict most

effectively if you are able to reproduce the conflict at will (i.e., you

know of a specific action or series of actions that will cause the

conflict to occur).  The reason for this is that this procedure follows a

logical set of tests to determine when a conflict is occuring and when it

is not.  A conflict that randomly occurs is difficult to troubleshoot

because you do not know for sure when the configuration being used is

actually resolving the conflict.

Q:  What is an Exception error?

A:  An Exception #13 is the most common exception error reported by QEMM.

It is an error generated by the PC's processor notifying you that an

invalid instruction has been issued.  The invalid instruction may

have come directly from software running on your machine, the result

of a conflict between software and hardware, or a conflict between

two hardware devices.

An Exception #12 is caused by the stack boundary being crossed.  This

Exception can often be generated by an Exception #13.

An Exception #6 is an invalid opcode resulting from the CPU's

instruction pointer being pointed at a value that is not a valid

operation or instruction for the CPU.

An Exception error is not generated by QEMM, but by your PC's

processor.  QEMM has the ability to detect these errors and display

them to the screen.  The error message informs you what has happened

so that you may take the necessary steps to resolve the conflict.

Without QEMM's warning, your system would have simply crashed or hung

without an error message, or would have become unstable.  For more

detailed information about the error messages themselves, please

refer to Technical Note #142, "QEMM-386:  Exception 13 Explained"

(EXCEPT13.TEC).

Q:  How do I resolve my Exception error?

A:  Follow these steps:

DETERMINING YOUR QEMM VERSION

-----------------------------

Before beginning this troubleshooting guide, you must first ensure that

you have QEMM 7 installed.  Previous QEMM versions do not apply to this

flowchart.  To determine the version of QEMM that is installed on your

computer, type QEMMREG from the \QEMM directory.

???  If the version of QEMM is not 7, you should upgrade to ensure

complete compatibility with all of the current hardware and software

on the market.  Since new hardware and software is released

regularly by other vendors, QEMM is constantly being updated to

perform optimally and without conflict with the latest hardware and

software products on the market.  QEMM is also updated to work

around flaws in other products to ensure system stability.  The

APPENDIX at the end of this technical note contains details about

updating QEMM.

???  If the QEMM version is 7.01, you should obtain an update to the

latest version of QEMM to avoid any potential conflicts.  Even if

your QEMM version is later than 7.01, updating to the latest QEMM

version is advised.  The APPENDIX at the end of this technical note

contains details about updating QEMM.

With version 7.02 or later, please continue.

STEP 1

TAMING DOWN QEMM

----------------

*STEALTH*

Run the QSETUP program from the \QEMM directory, select P, "Review or

change QEMM parameters," and examine your DEVICE=QEMM386.SYS line.  If

you do NOT see an ST:M or ST:F anywhere on that line, proceed to the next

section, *TROUBLESHOOTING MODE*.

If you DO see an ST:M or ST:F, select S, "Stealth ROMs," then O to turn

Stealth off.  Pressing <Enter> will bring you back to the "Review or

change QEMM parameters" screen, where you should press "A" to accept the

changes, followed by "S" to save the changes.  At this point you should

reboot the machine and try to recreate the Exception error.

???  IF THE EXCEPTION ERROR DOES NOT OCCUR:  QEMM's Stealth ROM feature

is involved with the Exception error on your system.  You should

follow the STEALTH.TEC technote in the \QEMM\TECHNOTE directory to

configure Stealth to avoid the conflict.  You are finished with this

technote.

???  IF THE EXCEPTION ERROR STILL OCCURS:  Continue with the next section,

*TROUBLESHOOTING MODE*.

*TROUBLESHOOTING MODE*

Run the QSETUP program from the \QEMM directory, select P, "Review or change

QEMM parameters," press <Page Down> twice, and select "Set up QEMM for

troubleshooting."  This will add several troubleshooting parameters to the

QEMM386.SYS device line for you.  Pressing <Enter> will bring you back to

the "Review or change QEMM parameters" screen, where you should press "A"

to accept the changes, followed by "S" to save the changes.

Reboot the machine and try to reproduce the Exception error.

???  IF THE EXCEPTION ERROR DOES NOT OCCUR:  Congratulations, you're

done!  At this time, you may want to remove any unnecessary

troubleshooting parameters from the QEMM386.SYS device line in your

CONFIG.SYS file.  Leaving the parameters on the line isn't harmful,

but may be inefficient.  To do this, edit the CONFIG.SYS (QEMM's

Manifest utility in the \QEMM directory allows you to easily do

this) and remove the parameters one at a time.  Reboot the machine

and try to reproduce the conflict to isolate the parameter that

resolves the conflict.  The following parameters are the

troubleshooting parameters added by QSETUP:

DB:2, RH:N, SH:N, TM:N, XBDA:N, TR:N, CF:N, FILL:N, MR:N

Once you have identified the parameter that resolves the conflict,

you may remove the remaining troubleshooting parameters; you are then

finished with this technote.

???  IF THE EXCEPTION ERROR STILL OCCURS:  Continue with the next

section, *DISABLE DOS-UP*.

*DISABLE DOS-UP*

QEMM's DOS-UP feature may be involved with your Exception error.  From

the \QEMM directory, run the QSETUP program, press the letter "U" to

select "Enable or disable DOS-Up."  Press the letter "N" to disable the

DOS-Up feature followed by <Enter> to go back to the QSETUP OPTIONS menu.

Press the letter "S" to "Save configuration and Quit."  You will be asked

by QSETUP if you would like to run Optimize.  Press <Enter> to begin the

Optimize process.  When Optimize has completed, try to recreate the Exception

error.

???  IF THE EXCEPTION ERROR DOES NOT OCCUR:  DOS-Up is adding to the

conflict.  By partially enabling the DOS-Up feature you can still

receive the benefits of DOS-Up while avoiding the Exception error.

From the QSETUP main menu, press the "U" key to "Enable or disable

DOS-Up," and then the "P" key for a "Partial" DOS-Up configuration.

This brings you to the DOS-Up Options screen, which allows you to

turn on or off the loading high of the 4 parts of DOS.

1 = DOS Resources No

2 = COMMAND.COM   No

3 = DOS Data      No

H = DOS=HIGH      No

Test the system by enabling only one of these options at a time,

saving the configuration, rebooting, and trying to reproduce the

Exception error.  Once the option that is causing the

conflict is isolated, setting the option to "No" in QSETUP will

ensure that QEMM's Optimize program will not enable this part of

DOS-Up in the future. You have resolved the conflict and are

finished with this technote.

???  IF THE EXCEPTION ERROR STILL OCCURS:  Go to STEP 2.

STEP 2

CLEAN BOOT

----------

In order to ensure that another driver isn't adding to the conflict, you

should temporarily disable those drivers and TSRs not required to boot

the machine or to reproduce the conflict.

Some conflicts require other drivers or TSRs to be loaded in order to

attempt to reproduce the conflict.  For example, if the conflict is

related to a CD-ROM, the drivers for the CD-ROM must be loaded. Also note

that if your hard drive is compressed or requires a driver to be loaded

then these drivers should NOT be REMarked out.

Using your favorite editor, edit your CONFIG.SYS file.  On every line in

this file (EXCEPT the QEMM386.SYS line and those required to boot the

machine or reproduce the conflict) place the word REM at the beginning of

that line.  For example:

DEVICE=C:\QEMM\QEMM386.SYS RAM R:1

REM DOS=HIGH

REM FILES=30

REM BUFFERS=30

REM DEVICE=C:\DOS\ANSI.SYS

Your CONFIG.SYS will have different lines, of course, with different

values on them.

Save this file.  Perform the same operation on your AUTOEXEC.BAT file,

adding REM to all lines that are not necessary to recreate the

Exception error (but you MAY leave statements that begin with PATH or

PROMPT).  Reboot your system and try to reproduce the Exception error.

???  IF THE EXCEPTION ERROR DOES NOT OCCUR:  Remove each of your REMark

statements one at a time from your AUTOEXEC.BAT file, saving and

rebooting after each, to try and recreate the problem.  Should the

Exception error occur, the last line you unREMarked is the source of

the problem.

If you have removed the REMs from all the lines in the AUTOEXEC.BAT

file without encountering the error, the problem is in the

CONFIG.SYS file.  In that case, repeat the instructions in the

previous paragraph, but perform the steps for the CONFIG.SYS file

this time.  Similarly, the last line you unREMarked prior to the

error reappearing is the line responsible for the problem.  Go to

the section *RESOLVING CONFLICTS WITH OTHER DRIVERS*.

???  IF THE EXCEPTION ERROR STILL OCCURS: Go to STEP 3.

*RESOLVING CONFLICTS WITH OTHER DRIVERS*

You have isolated the conflict and determined which driver or TSR is

causing it.  Contacting the manufacturer of the program is often the

best way to resolve the conflict; however, Quarterdeck Technical

Support has found that the following suggestions resolve many other

programs' incompatibilities:

o  Configure the program to NOT use expanded memory.  Some programs

misuse expanded memory, and by configuring them to use extended

or conventional memory the conflict with your system may be

avoided.  Consult the program's documentation for configuration

options.

o  Try loading the program low.  Some programs are written with the

assumption that they will be loaded in conventional memory and

fail to function properly when loaded into upper memory.

Programs that load themselves into upper memory should be

tested with this feature disabled.

o  Contact the manufacturer of the program to acquire the latest

version of the program.  Newer versions may contain

compatibility fixes for known conflicts.

You are finished with this technote.

STEP 3

ELIMINATE HIGH RAM CONFLICTS

----------------------------

To ensure that there is no conflict in upper memory, you should eliminate

all High RAM and test the system.  Run QSETUP from the \QEMM directory,

select P, "Review or change QEMM parameters" followed by R, "Fill upper

memory with RAM."  You should select "No" to ensure that QEMM will not

create High RAM.  Pressing "A" to accept the changes followed by "S" to

save the configuration will allow you to exit QSETUP.  Reboot the system

and attempt to reproduce the conflict (ignore any messages about programs

not loading into upper memory, as you have removed all High RAM for this

test).

???  IF THE EXCEPTION ERROR DOES NOT OCCUR:  There is a conflict in upper

memory.  If you are using a bus-mastering SCSI hard disk controller

(consult the SCSI card's documentation if you are unsure), the

BUS-MAST.TEC technote in the \QEMM\TECHNOTE directory will help you

resolve the conflict.  Otherwise, the EXCLUDE.TEC technote in the

\QEMM\TECHNOTE directory will help you resolve the upper memory

conflict.  You are finished with this technote.

???  IF THE EXCEPTION ERROR STILL OCCURS:  There is no upper memory

conflict.  You should proceed to STEP 4.

STEP 4

TEST THE CONFIGURATION WITH DOS'S MEMORY MANAGERS

-------------------------------------------------

In this step, you will try to recreate the conflict without QEMM

installed.  If the conflict occurs without QEMM on the system then QEMM

is not involved in the conflict.

Edit your CONFIG.SYS with your favorite text editor, locate the

QEMM386.SYS line, and at the beginning of that line, add the word REM, as

in the example below:

REM DEVICE=C:\QEMM\QEMM386.SYS <additional parameters>

Directly below this line, add the following two lines:

DEVICE=C:\DOS\HIMEM.SYS

DEVICE=C:\DOS\EMM386.EXE ON RAM 2048

These two lines assume that HIMEM.SYS and EMM386.EXE are in the directory

\DOS.  If they are in some other directory, replace the word DOS in the

above two lines with the correct directory name.  If there are any

eXcludes or Includes on your QEMM386.SYS line that are necessary to boot

the machine, add them to the EMM386.EXE line as well.  Save your changes

and reboot the machine.  After rebooting, QEMM will NOT be loaded.

Attempt to reproduce the conflict.

???  IF A CONFLICT DOES NOT OCCUR:  If you're loading DOS's memory

managers and you have followed every step of this technical note,

then further troubleshooting is required.  Contacting our technical

support department is the quickest and easiest way to resolve the

conflict.  Please mention that you have followed this technical note

and were instructed by STEP 4 to contact Quarterdeck for additional

troubleshooting steps.

???  IF A CONFLICT STILL OCCURS:  QEMM is NOT involved with the conflict;

you are finished with this technote.  Since Exception errors will

generally only display with QEMM enabled, the symptoms of a

remaining problem with the DOS memory managers will most likely be a

machine hang.  If a conflict still occurs without QEMM loading,

further QEMM troubleshooting will not affect the conflict.  You

should consult the documentation that comes with the software that

generated the Exception error with QEMM or causes a hang with HIMEM

and EMM386, or contact the manufacturer of the program.

Our technical support department is easily reached via electronic

channels such as CompuServe (GO QUARTERDECK), Internet (mail

SUPPORT@QDECK.COM or the comp.os.msdos.desqview Usenet newsgroup),

the Quarterdeck BBS (310-314-3227), or fax (310-314-3217).  When

contacting Quarterdeck, be sure to fully explain the symptoms of the

conflict and the results of the tests performed while following this

technical note.

You can also call our Technical Support line at 310-392-9701 for

further assistance.  When you call, please be at the machine that is

experiencing the conflict.

APPENDIX

OBTAINING THE LATEST QEMM VERSION

---------------------------------

If you currently have a version of QEMM earlier than QEMM 7, you should

upgrade to ensure complete compatibility with all of the latest software

and hardware.  Contact the Quarterdeck Upgrade Department at 800-354-3222

or 310-314-3222 for special upgrade pricing.  Please note that we give

you a special upgrade price ONLY after you have registered your product!

If you are using QEMM 7 and would like to update your QEMM version to the

latest maintenance release, free update patches are available on

Compuserve (GO QUARTERDECK, LIB 10), the Quarterdeck BBS (310-314-3227),

anonymous FTP (qdeck.com), or BIX (join desqview).  The updated version

can also be ordered from Quarterdeck's Customer Service dept

(800-354-3222) for the cost of shipping & handling.

*          Copyright (C) 1994 by Quarterdeck Office Systems            *

2.5 - Desqview Settings

-----------------------

The most important DESQVIEW SETUP window is "P" for PERFORMANCE.

-------------------------------------------

|  Task Processing Time (in Clock Ticks)  |

|                                         |

|          Foreground: 2                  |

|          Background: 2                  |

|                                         |

|   Memory Useage (in K)                  |

|                                         |

|          Common Memory:      20         |

|          DOS Buffer for EMS:  5         |

|                                         |

|   Optimise Communications:   (Y/N)  Y   |

|   Manage printer contention: (Y/N)  N   |

|                                         |

-------------------------------------------

Processing:

It is important for communication programs to get access to

CPU on a frequent basis, therefore the default DV setting of

Foreground ticks:9 and Background ticks:3 is inappropriate.  I would

suggest at a 2 to 2 ratio is a good starting point.  The table below may

help to determine what is best for you:?

CPU	Foreground	Background

----------------------------------

80386       3	            3

80486       2               2

Pentium     1               1

Clock tick settings are NOT set in stone.  I know for a fact that 18:18

will work on a Pentium without dropping carriers.

Optimize Communications:

This question asks for a YES or NO ANSWER, but in reality will accept a numeric

answer.  Optimize Commnications deals with IRQ priority's and has nothing to

do with COM port numbers.

If you answer "Y" to this question then DV assigns a prioriy sequence of

IRQ 2 3 4 5 6 7 8 9 10 11 12 13 14 15 0 1

unless a mouse is detected at IRQ 2 then the prority would be

IRQ 3 4 5 6 7 8 9 10 11 12 13 14 15 0 1 2

If you are only using COM port "1" then you should answer this question with

a "4" this will yield the following IRQ priority list:

IRQ 4 5 6 7 8 9 10 11 12 13 14 15 0 1 2 3

The main thing to note here is that if you were to then start using COM2 or

decide to only use COM2 then you would be in big trouble, because COM2 will

now be down the bottom of the priority list.

For BBS operators using COM1 and COM2 at standard IRQ's I suggest you set

Optimize Comminications (Y/M): to "3", ensuring that COM1 and COM2 both get

High priority service.

Non-Standard IRQ's:

You can optimize Non-standard COM port IRQ's in the exact same way as the

above example.  If you have move COM port 1 to IRQ 5 then enter 5 at the

(Y/N): prompt.

If you are using Non-Standard IRQ's you may find the table below helpful

IRQ	Typical Owners

--------------------------------------------------

0	Timer

1     Keyboard

2     Mouse

3     COM2       Numerous Add-On Boards

4     COM1

5     LPT2       Network Cards

6     Floppy Diskette Drive

7     LPT1

8     Realtime clock

9     Reserved

10     Reserved    CD-ROM Cards    Network Cards

11     Reserved

12     Reserved    AUX Pointing Device

13     Coprocessor

14     Fixed Disk Drive

15     Reserved

Suggested Non-Standard IRQ's:

I would suggest using 5 9 11 or 12

2.6 - Can you Share Interrupts Under Desqview?

----------------------------------------------

The answer is a Strong 'NO' for Serial Communications.

A number of multiport cards come configure as per the table below:

COM Port   IRQ	   Address

--------------------------

1       4        3F8

2	    3        2F8

3       4        3E8

4       3        2E8

From the table you will notice that COM1 & 3 and COM2 & 4 share interrupts,

this will NOT work under Desqview.  If you try to run all four ports sharing

IRQ's at least two of the Modems will respond at startup with:

"Modem Not Responding"  or "Did Not Receive OK"

or a similar message indicating that your modem isn't responding.

Each COM port needs a unique IRQ, which you can most likely be derived from

the earlier typical IRQ table.

2.7 - Desqview and Port Addressess

----------------------------------

Port addresses are of relevance to most communication programs, if you are

forced to use Non-Standard addresses then this table may help:

COM1	COM2	COM3	COM4

----------------------------

2E0	2E8	2F0	2F8

2C0	2C8	2D0	2D8

280	288	290	298

200	208	210	218

120	128	130	138

100	108	110	118

2.8 - Configuring a Desqview Window to use Serial Ports

-------------------------------------------------------

When you ADD a program to Desqview you are asked to complete STANDARD

information and are given the opportunity to customize ADVANCED settings.

Typical setup screens are as follows:

------------------------------------------------------------------------------

Standard Options

Program Name............: Serial Communications

Keys to Use on Open Menu: C1              Memory Size (in K): 460

Program...: COMMS1.BAT

Parameters:

Directory.: C:\COMMS

Options:

Writes text directly to screen.......: [Y]

Displays graphics information........: [N]  (1)

Virtualize text/graphics (Y,N,T).....: [T]  (1)

Uses serial ports (Y,N,1,2)..........: [1]  (2)

Requires floppy diskette.............: [N]

------------------------------------------------------------------------------

Notes:

(1) Both Items marked with a (1) can be important in the area of

memory management.  If you having problems with a program running

under Desqview that does not use graphics, then leaving graphics set OFF

and Virtualize set to T for TEXT can be a good fault finding technique.

(2) Uses Serial Ports is a VERY IMPORTANT question. See the table below

for the best settings:

Port	Set to

--------------

COM1      1

COM2	  2

COM3	  Y

COM4	  Y

-----------------------------------------------------------------------------

Advanced Options

System Memory (in K).......:   0     Maximum Program Memory Size (in K):

Script Buffer Size.......:  1000     Maximum EMS/XMS/VCPI/DPMI (in K):

Text Pages: 4               Graphics Pages: 0         Initial Video Mode:

Window Position:                                                            �

Maximum Height:  25       Starting Height:  25      Starting Row...:   0

Maximum Width.:  80       Starting Width.:  80      Starting Column:   0

Shared Program

Pathname..:

Data......:

*Close on exit (Y,N,blank)......: [N]  Uses its own colors.............: [Y]

Allow Close Window command.....: [Y] *Runs in background (Y,N,blank)..: [Y]

+Uses math coprocessor..........: [N] =Keyboard conflict (0-F).........: [0]

Share CPU when foreground......: [Y]  Share EGA when foreground/zoomed: [Y]

*Can be swapped out (Y,N,blank).: [N]  Protection level (0-3)..........: [0]

-----------------------------------------------------------------------------

* Close on exit:  NO  - If set to yes when running a BBS the corresponding

node will be terminated

* Can be swapped out: NO  - if set to yes a BBS node could ne swapped to disk

and therefore terminated

* Runs in Background:  NO - Multiple BBS Nodes run in Background sharing the

CPU with the Foreground

+ Its ok! to set uses Math coprocesser to ON.  Some Quick Basic and Turbo

Pascal programs require it.

= Explaining Keyboard conflict does beyond the scope of this document,

however it should be noted that a setting of 8 is know to help fix

ill behaved 3270 emulation sessions

3.0 - FIFO Transmit and Receive Trigger Levels

----------------------------------------------

This is relevant for WILDCAT! 4.x and above

If your communications program allows you to customize FIFO transmit

and receive trigger level settings, then you should take advantage of these

facilities:

For Desqview the optimum settings are:

FIFO Receive Trigger Level:  4

FIFO Transmit Trigger Level: 1

These settings are designed to minimise processor activity which will in

turn improve Desqview performance.  If you experience difficulties

with file transfers at 28800 bps you may need to set the Receive Trigger

Level to 1.

4.0 - 16450 vs 16550A UARTS

---------------------------

Most users are now aware of the issues surronding the use of the older 16450

UART's.  The superior buffering of the 16550A goes a long way towards solving

most COM Port overruns.  If your computer does not support replacement UART's

i.e. the UARTS are permantely soldered in place, then perhaps a new serial

interface card may be the best option.

5.0 - Hayes ESP Card

--------------------

I believe the Hayes Communications Accelerator card is a sound product.

However, I have lost count of the number of messages I have seen on the

CIS GO Hayforum threatening to do unspeakable things to the card.  In

my opinion most of the problems are a result of users being far too

impatient.

5.1 - Can you use the ESP Card with Desqview?

---------------------------------------------

Desqview will quite happily support two dual-port ESP cards configured

as COM1-COM4.

My suggest card configuration settings are:

Card 1		Card 2

----------------------

COM1	280

COM2	288

COM3		180

COM4		188

On the diskette supplied with the ESP card is a program called ESPCA

which is the DOS configuration program.  The following ESPCA.EXE command

will configure all ports with: 	Port Multiplier: x1,  RTS/CTS flow control

and all ports FIFO enabled:

ESPCA P280:1 P288:1 P180:1 P188:1 R280:+ R288:+ R180:+ R188:+ F

You can ofcourse experiment with various port multipliers.

6.0 - DTE Locked Baud Rates

---------------------------

If you have a 28800 bps modem you should lock the DTE speed at least

38400 bps.  Don't lock the speed at 19200.

7.0 - Modem Profile

-------------------

Accessible via the AT&V command in TERMINAL Mode

The active modem profile is Important.  A typical Hayes Optima 28.8 profile

might look like this:

ACTIVE PROFILE: B0 B76 E0 L2 M1 N1 P Q0 V1 W0 X4 Y0 &A0 &C1 &D2 &G0 &K3 &Q9 &R0

&S0 &T4 &U0 &X0 &Y0     S00:000 S02:043 S03:013 S04:010 S05:008 S06:004 S07:050

S08:004 S09:006 S10:014 S11:090 S12:050 S25:005 S26:001 S36:007 S37:000 S38:020

S46:002 S48:007 S49:064 S50:250 S108:001 S109:4094 S110:002

The Key Settings are:

E0	- Don't ECHO echo characters from the keyboard in command

state

Q0	- Return RESULT CODES

V1	- Display RESULT CODES in words

&C1	- Track presence of CARRIER DETECT signal

&D2	- Monitor DTR

&K3	- Enable RTS/CTS Flow control

&Q9	- Communicate in V.42bis/MNP2-4 error control

S0	- Select RING to auto answer ON

8.0 - Wildcat! Multi-Line Setup?

--------------------------------

The Wildcat! Sysop Guide covers MULTI-LINE operation in some detail so I

don't want to waste too much time on this subject.

There are two key issues:

1. One Node = One DV! Window = One Modem = One Telephone Line

2. Node 0 is the SYSOP Node in Version 4.x not NODE 1

8.1 - Multi-Line Setup Batch Files

----------------------------------

Each DV! window should be linked to a unique CATx.BAT File. i.e. CAT0.BAT,

CAT1.BAT, CAT2.BAT and so on......

Sample Batch Files

CAT0.BAT	CAT1.BAT	CAT2.BAT	CAT3.BAT

@ECHO OFF	@ECHO OFF	@ECHO OFF	@ECHO OFF

:RELOAD		:RELOAD		:RELOAD		:RELOAD

CD\WC4		CD\WC4		CD\WC4		CD\WC4

SET WCNODEID=0	SET WCNODEID=1	SET WCNODEID=2	SET WCNODEID=3

SET WCPORTID=0	SET WCPORTID=1	SET WCPORTID=2	SET WCPORTID=3

WILDCAT		WILDCAT		SET WCMDM=HAYES	SET WCIRQ=11

GOTO RELOAD	GOTO RELOAD	WILDCAT		SET WCMDM=USROB1

GOTO RELOAD	WILDCAT

GOTO RELOAD

CAT0.BAT

--------

Note:	WCPORTID=0 - Tells Wildcat! to leave the Modem port alone

WCNODEID=0 - Tells Wildcat! that its the SYSOP Node

CAT1.BAT

--------

Note:	WCPORTID=1 - Tells Wildcat! to use Serial or Digiport 1 (COM1)

WCNODEID=1 - Tells Wildcat! to set the Node Number to 1

CAT2.BAT

--------

Note:	WCPORTID=2 - Tells Wildcat! to use Serial or Digiport 2 (COM2)

WCNODEID=2 - Tells Wildcat! to set the Node Number to 2

WCMDM=HAYES - Tells Wildcat! to look in the \MODEM subdir for

a .MDM File with the name HAYES which has been customize for

the modem being used.

CAT3.BAT

--------

Note:	WCPORTID=3 - Tells Wildcat! to use Serial or Digiport 3 (COM3)

WCNODEID=3 - Tells Wildcat! to set the Node Number to 3

WCMDM=USROB1 - Tells Wildcat! to look in the \MODEM subdir for

a .MDM File with the name USROB1 which has been customize for

the modem being used.

SET WCIRQ=11 - Tells Wildcat! that the default share interupt 4

is NOT to be used (remember we cannot share Interupts) instead

we are using IRQ 11 in this example.

Use the example above to configure your system to your needs.  Try Not to

use non stardard BASE I/O Addressess

8.2 - Wildcat .MDM Files

------------------------

You can modify the .MDM modem profile files that come with Wildcat!

using the MSI supplied WCMODEM program.

A typical .MDM File Looks like this:

������������������������������������������������������������������������������Ŀ

�    wcMODEM Copyright (c) 1991,94 Mustang Software, Inc.  Version 4.01  MP    �

����������������������������� View/Edit MDM Files ��������������������������ͻ��

��  Modem name                              : Hayes Optima 28.8              #��

��  Type of serial port                     : Serial port                    ���

��  Communication port number               : 1                              ���

��  Communication port IRQ                  : 4                              ���

��  Communication port base address         : $03F8                          ���

��  Determine ringing using                 : Ring Detect                    ���

��  Determine baud rate using               : Result Code 1                  ���

��  Initialize port at what baud rate       : 38400                          ���

��  Lock DTE at initialization baud rate    : Y                              ���

��  Use CTS/RTS flow control                : Y                              ���

��  Take modem off hook on exit (busy line) : N                              ���

��  Number of seconds to wait for carrier   : 60                             ���

��  Milliseconds before answering           : 1000                           ���

��  Milliseconds to lower DTR for hangup    : 2000                           ���

��  Milliseconds to delay before prelog     : 1000                           ���

��  Milliseconds to wait for result code    : 3800                           #��

����������������������������������������������������������������������������Ķ��

��   F2 Pick             F3 Load             F4 Save              F10 Done   ���

����������������������������������������������������������������������������ͼ��

��������������������������������������������������������������������������������

��������������������������������������������������������������������������������

Port is CLOSED � Port #: 1  � Base Address: 03F8 � Port Irq: 4  � Speed: 9600

������������������������������������������������������������������������������Ŀ

�    wcMODEM Copyright (c) 1991,94 Mustang Software, Inc.  Version 4.01  MP    �

����������������������������� View/Edit MDM Files ��������������������������ͻ��

��  Milliseconds between command strings    : 2000                           #��

��  Trigger level for receive FIFO          : 4                              ���

��  Trigger level for transmit FIFO         : 1                              ���

��  Verbal code for RING                    : RING                           ���

��  Modem reset string                      : ATZ                            ���

��  Modem error correction strings          : V.42|MNP2-4|REL|ARQ            ���

��  Modem on hook string (ready for calls)  : ATH0                           ���

��  Modem off hook string (busy)            : ATH1                           ���

��  Modem initialization string             : ATE0V1&C1&D2S95=3S0=0S7=60     ���

��  Modem answer string                     : ATA                            ���

��  Modem caller ID string                  :                                ���

����������������������������  Modem Nram settings  �������������������������İ��

��  Modem reset command                     :                                ���

��  Modem setup string # 1                  :                                ���

��  Modem setup string # 2                  :                                ���

��  Modem setup string # 3                  :                                #��

����������������������������������������������������������������������������Ķ��

��   F2 Pick             F3 Load             F4 Save              F10 Done   ���

����������������������������������������������������������������������������ͼ��

��������������������������������������������������������������������������������

��������������������������������������������������������������������������������

Port is CLOSED � Port #: 1  � Base Address: 03F8 � Port Irq: 4  � Speed: 38400

*** PAGE #1 - IMPORTANT SETTINGS ***

Type of serial port                     : Serial port

Communication port number               : 1

Communication port IRQ                  : 4

Communication port base address         : $03F8

Determine ringing using                 : Ring Detect

Determine baud rate using               : Result Code 1

Initialize port at what baud rate       : 38400

Lock DTE at initialization baud rate    : Y

Use CTS/RTS flow control                : Y

******************************

port IRQ  - Should be matched on the physical port or Multi-port I/O Board

base add  - Should be matched on the physical port or Multi-port I/O Board

Determine Ringing Using:  Ring Detect:  Signal From Modem RI Circuit

Ring Result:  Uses the Modem Result Code

Auto Answer:  Modem does all the work via ATS0=0

Determine Baud Rate Using: Result Code 1:  Monitor CD while waiting for Connect String

Result Code 2:  Don't Monitor while waiting

CR:  Use the old style PRESS CR a few times method

DTE  Assume calls is at Locked DTE Rate

*** PAGE #2 - IMPORTANT SETTINGS ***

Trigger level for receive FIFO          : 4

Trigger level for transmit FIFO         : 1

Verbal code for RING                    : RING

Modem reset string                      : ATZ

Modem error correction strings          : V.42|MNP2-4|REL|ARQ

Modem on hook string (ready for calls)  : ATH0

Modem off hook string (busy)            : ATH1

Modem initialization string             : ATE0V1&C1&D2S95=3S0=0S7=60

****************************

FIFO Trigger Levels:            See Section 3.0

Modem Error Correction Strings: Match these with your modem manual

On & OFF Strings:		These are correct for the Hayes 28.8.

v4.00 of Wildcat! stated them incurrectly

Modem Init String:		See Section 7.0

9.0 - Wildcat! Upload and DownLoad Failures

-------------------------------------------

The are alot of users still asking about file transfer failures which manifest

themselves in two main ways:

1.  Excessive CRC errors

2.  Link seems to run ok! for around 120K or so then dies dropping back

and eventually losing carrier.

I have experience both!

9.1 - Poor Quality Phone Lines

------------------------------

There is NO doubt that a poor quality phone line can cause the two problems

nentioned above to occur.  Ask you local telephone company to check the lines

for you.

Assuming that your problems are being caused by line noise or interference you

might like to try the following:

1.  Get hold of a Noise testing program and run it on your BBS and see

what degree of noise the program picks up,

2.  Make sure your BBS lines do NOT have more than three Ring Equivilent

devices attached to them.

3.  Disconnect any membrain style handsets from your modem line and retest

them.

Try and determine where your street distribution box is for the telephone lines

that run into your home or office.  If your lines are pole based look for a

pole with a bunch of wires all descending together towards the ground.  If

you have underground lines look for a lockable waist height pilar and note

its location relative to your home.

If your street distribution point is a dozen or so houses or average sized

blocks of land away, then chances are your in for trouble with little

hope of fault resolution.

9.2 - File Transfer Failure Hardware Factors

--------------------------------------------

Always check the obvious:

1.  Check your installation by using a different brand of modem

2.  Check all phsyical connections.

3.  Check all environmental facts, such as heat, cold or dirty power

supply.

9.3 - File Transfer Failure Software Factors

--------------------------------------------

If you are certain that your Telephone Lines are Fine and that your modem is

fuctioning correctly then the port of last resort is software.

Here are some things to try:

1.  Carefully review your modems NRAM settings for data compression, if

in doubt use AT&F to restore your modem to its factory ready state.

2.  Carefully review your modems NRAM settings for error correction, if

in doubt use AT&F to restore your modem to its factory ready state.

3.  Don't be frightended to play with items 1 or 2.

4.  Remove any vsheild style programs from your boot up files

5.  In MAKEWILD under Memory and Hardware Information set the Following:

Size of Scollback Buffer (in K):       5

Where Should Overlay be Held:          Disk

Extra Memory in K for overlay Buffer:  0

Where should swapped data be held:     EMS

6.  If your PC uses SHADOW memory of any type disable it in CMOS some

users have claimed it can cause problems.

7.  Discontinue the use of Desqview, Qemm, DOS Share, EMM386, Smartdrv

and run Wildcat as a SINGLE LINE DOS System.

Use STARTUP batch files similar to these:

AUTOEXEC.BAT

------------

@ECHO OFF

PROMPT=$P$G

PATH=C:\;\DOS;\WILDCAT;

SET WCPORTID=1

SET WCNODEID=1

CD\WILDCAT

WILDCAT

CONFIG.SYS

----------

DEVICE    = C:\DOS\HIMEM.SYS /TESTMEM:OFF

DOS       = HIGH

FILES     = 80

BUFFERS   = 30

NUMLOCK   = OFF

SHELL     = \DOS\COMMAND.COM C:\DOS\ /E:2048 /p

10.0 - Summary

--------------

This is the first PUBLIC release of this file.  It is not yet complete and

will be added to on an ongoing basis.  WATCH for updates.

11.0 - Contacting Me

--------------------

I am happy to answer any questions that you may have. I carry MSI SupportNet

so a message left in any of the [E] conferences will reach me.

Michael Purdy

(Sysop) The Speed Limit BBS

Melbourne, Australia +61 3 331 2543

Internet:100026,1177@Compuserve.com