QEMM - Exception Errors

-----------------------------

One of the first problems that you may encounter when setting up your

system at the DOS Level is QEMM Exception Errors, typically #13.

The following .TEC Document Text from Quarterdeck may help!

QUARTERDECK QUARTERDECK QUARTERDECK QUARTERDECK QUARTERDECK QUARTERDECK

-----------------------------------------------------------------------

ID:X3 Troubleshooting Exception #6, #12, and #13 Errors

Quarterdeck Technical Note #232

Subject: Detailed explanation and troubleshooting procedures for

Exception #6, #12, and #13 error messages and system lockups.

OVERVIEW

--------

This document addresses Exception #6, #12 and #13 error messages.  These

three Exceptions are so similar in cause, nature, and solution that they

will all be covered by the information below.  Any reference to an

Exception error in this document applies to an Exception #6, Exception

#12, or Exception #13 error.

Please note that sometimes QEMM cannot report the error to the screen, so

a blank screen or a lockup occurs.  For troubleshooting purposes, treat a

system lockup or a blank screen (that accepts no input) to be the same as

an Exception error.

This troubleshooting procedure will isolate and resolve the conflict most

effectively if you are able to reproduce the conflict at will (i.e., you

know of a specific action or series of actions that will cause the

conflict to occur).  The reason for this is that this procedure follows a

logical set of tests to determine when a conflict is occuring and when it

is not.  A conflict that randomly occurs is difficult to troubleshoot

because you do not know for sure when the configuration being used is

actually resolving the conflict.

Q:  What is an Exception error?

A:  An Exception #13 is the most common exception error reported by QEMM.

It is an error generated by the PC's processor notifying you that an

invalid instruction has been issued.  The invalid instruction may

have come directly from software running on your machine, the result

of a conflict between software and hardware, or a conflict between

two hardware devices.

An Exception #12 is caused by the stack boundary being crossed.  This

Exception can often be generated by an Exception #13.

An Exception #6 is an invalid opcode resulting from the CPU's

instruction pointer being pointed at a value that is not a valid

operation or instruction for the CPU.

An Exception error is not generated by QEMM, but by your PC's

processor.  QEMM has the ability to detect these errors and display

them to the screen.  The error message informs you what has happened

so that you may take the necessary steps to resolve the conflict.

Without QEMM's warning, your system would have simply crashed or hung

without an error message, or would have become unstable.  For more

detailed information about the error messages themselves, please

refer to Technical Note #142, "QEMM-386:  Exception 13 Explained"

(EXCEPT13.TEC).

Q:  How do I resolve my Exception error?

A:  Follow these steps:

DETERMINING YOUR QEMM VERSION

-----------------------------

Before beginning this troubleshooting guide, you must first ensure that

you have QEMM 7 installed.  Previous QEMM versions do not apply to this

flowchart.  To determine the version of QEMM that is installed on your

computer, type QEMMREG from the \QEMM directory.

???  If the version of QEMM is not 7, you should upgrade to ensure

complete compatibility with all of the current hardware and software

on the market.  Since new hardware and software is released

regularly by other vendors, QEMM is constantly being updated to

perform optimally and without conflict with the latest hardware and

software products on the market.  QEMM is also updated to work

around flaws in other products to ensure system stability.  The

APPENDIX at the end of this technical note contains details about

updating QEMM.

???  If the QEMM version is 7.01, you should obtain an update to the

latest version of QEMM to avoid any potential conflicts.  Even if

your QEMM version is later than 7.01, updating to the latest QEMM

version is advised.  The APPENDIX at the end of this technical note

contains details about updating QEMM.

With version 7.02 or later, please continue.

STEP 1

TAMING DOWN QEMM

----------------

*STEALTH*

Run the QSETUP program from the \QEMM directory, select P, "Review or

change QEMM parameters," and examine your DEVICE=QEMM386.SYS line.  If

you do NOT see an ST:M or ST:F anywhere on that line, proceed to the next

section, *TROUBLESHOOTING MODE*.

If you DO see an ST:M or ST:F, select S, "Stealth ROMs," then O to turn

Stealth off.  Pressing <Enter> will bring you back to the "Review or

change QEMM parameters" screen, where you should press "A" to accept the

changes, followed by "S" to save the changes.  At this point you should

reboot the machine and try to recreate the Exception error.

???  IF THE EXCEPTION ERROR DOES NOT OCCUR:  QEMM's Stealth ROM feature

is involved with the Exception error on your system.  You should

follow the STEALTH.TEC technote in the \QEMM\TECHNOTE directory to

configure Stealth to avoid the conflict.  You are finished with this

technote.

???  IF THE EXCEPTION ERROR STILL OCCURS:  Continue with the next section,

*TROUBLESHOOTING MODE*.

*TROUBLESHOOTING MODE*

Run the QSETUP program from the \QEMM directory, select P, "Review or change

QEMM parameters," press <Page Down> twice, and select "Set up QEMM for

troubleshooting."  This will add several troubleshooting parameters to the

QEMM386.SYS device line for you.  Pressing <Enter> will bring you back to

the "Review or change QEMM parameters" screen, where you should press "A"

to accept the changes, followed by "S" to save the changes.

Reboot the machine and try to reproduce the Exception error.

???  IF THE EXCEPTION ERROR DOES NOT OCCUR:  Congratulations, you're

done!  At this time, you may want to remove any unnecessary

troubleshooting parameters from the QEMM386.SYS device line in your

CONFIG.SYS file.  Leaving the parameters on the line isn't harmful,

but may be inefficient.  To do this, edit the CONFIG.SYS (QEMM's

Manifest utility in the \QEMM directory allows you to easily do

this) and remove the parameters one at a time.  Reboot the machine

and try to reproduce the conflict to isolate the parameter that

resolves the conflict.  The following parameters are the

troubleshooting parameters added by QSETUP:

DB:2, RH:N, SH:N, TM:N, XBDA:N, TR:N, CF:N, FILL:N, MR:N

Once you have identified the parameter that resolves the conflict,

you may remove the remaining troubleshooting parameters; you are then

finished with this technote.

???  IF THE EXCEPTION ERROR STILL OCCURS:  Continue with the next

section, *DISABLE DOS-UP*.

*DISABLE DOS-UP*

QEMM's DOS-UP feature may be involved with your Exception error.  From

the \QEMM directory, run the QSETUP program, press the letter "U" to

select "Enable or disable DOS-Up."  Press the letter "N" to disable the

DOS-Up feature followed by <Enter> to go back to the QSETUP OPTIONS menu.

Press the letter "S" to "Save configuration and Quit."  You will be asked

by QSETUP if you would like to run Optimize.  Press <Enter> to begin the

Optimize process.  When Optimize has completed, try to recreate the Exception

error.

???  IF THE EXCEPTION ERROR DOES NOT OCCUR:  DOS-Up is adding to the

conflict.  By partially enabling the DOS-Up feature you can still

receive the benefits of DOS-Up while avoiding the Exception error.

From the QSETUP main menu, press the "U" key to "Enable or disable

DOS-Up," and then the "P" key for a "Partial" DOS-Up configuration.

This brings you to the DOS-Up Options screen, which allows you to

turn on or off the loading high of the 4 parts of DOS.

1 = DOS Resources No

2 = COMMAND.COM   No

3 = DOS Data      No

H = DOS=HIGH      No

Test the system by enabling only one of these options at a time,

saving the configuration, rebooting, and trying to reproduce the

Exception error.  Once the option that is causing the

conflict is isolated, setting the option to "No" in QSETUP will

ensure that QEMM's Optimize program will not enable this part of

DOS-Up in the future. You have resolved the conflict and are

finished with this technote.

???  IF THE EXCEPTION ERROR STILL OCCURS:  Go to STEP 2.

STEP 2

CLEAN BOOT

----------

In order to ensure that another driver isn't adding to the conflict, you

should temporarily disable those drivers and TSRs not required to boot

the machine or to reproduce the conflict.

Some conflicts require other drivers or TSRs to be loaded in order to

attempt to reproduce the conflict.  For example, if the conflict is

related to a CD-ROM, the drivers for the CD-ROM must be loaded. Also note

that if your hard drive is compressed or requires a driver to be loaded

then these drivers should NOT be REMarked out.

Using your favorite editor, edit your CONFIG.SYS file.  On every line in

this file (EXCEPT the QEMM386.SYS line and those required to boot the

machine or reproduce the conflict) place the word REM at the beginning of

that line.  For example:

DEVICE=C:\QEMM\QEMM386.SYS RAM R:1

REM DOS=HIGH

REM FILES=30

REM BUFFERS=30

REM DEVICE=C:\DOS\ANSI.SYS

Your CONFIG.SYS will have different lines, of course, with different

values on them.

Save this file.  Perform the same operation on your AUTOEXEC.BAT file,

adding REM to all lines that are not necessary to recreate the

Exception error (but you MAY leave statements that begin with PATH or

PROMPT).  Reboot your system and try to reproduce the Exception error.

???  IF THE EXCEPTION ERROR DOES NOT OCCUR:  Remove each of your REMark

statements one at a time from your AUTOEXEC.BAT file, saving and

rebooting after each, to try and recreate the problem.  Should the

Exception error occur, the last line you unREMarked is the source of

the problem.

If you have removed the REMs from all the lines in the AUTOEXEC.BAT

file without encountering the error, the problem is in the

CONFIG.SYS file.  In that case, repeat the instructions in the

previous paragraph, but perform the steps for the CONFIG.SYS file

this time.  Similarly, the last line you unREMarked prior to the

error reappearing is the line responsible for the problem.  Go to

the section *RESOLVING CONFLICTS WITH OTHER DRIVERS*.

???  IF THE EXCEPTION ERROR STILL OCCURS: Go to STEP 3.

*RESOLVING CONFLICTS WITH OTHER DRIVERS*

You have isolated the conflict and determined which driver or TSR is

causing it.  Contacting the manufacturer of the program is often the

best way to resolve the conflict; however, Quarterdeck Technical

Support has found that the following suggestions resolve many other

programs' incompatibilities:

o  Configure the program to NOT use expanded memory.  Some programs

misuse expanded memory, and by configuring them to use extended

or conventional memory the conflict with your system may be

avoided.  Consult the program's documentation for configuration

options.

o  Try loading the program low.  Some programs are written with the

assumption that they will be loaded in conventional memory and

fail to function properly when loaded into upper memory.

Programs that load themselves into upper memory should be

tested with this feature disabled.

o  Contact the manufacturer of the program to acquire the latest

version of the program.  Newer versions may contain

compatibility fixes for known conflicts.

You are finished with this technote.

STEP 3

ELIMINATE HIGH RAM CONFLICTS

----------------------------

To ensure that there is no conflict in upper memory, you should eliminate

all High RAM and test the system.  Run QSETUP from the \QEMM directory,

select P, "Review or change QEMM parameters" followed by R, "Fill upper

memory with RAM."  You should select "No" to ensure that QEMM will not

create High RAM.  Pressing "A" to accept the changes followed by "S" to

save the configuration will allow you to exit QSETUP.  Reboot the system

and attempt to reproduce the conflict (ignore any messages about programs

not loading into upper memory, as you have removed all High RAM for this

test).

???  IF THE EXCEPTION ERROR DOES NOT OCCUR:  There is a conflict in upper

memory.  If you are using a bus-mastering SCSI hard disk controller

(consult the SCSI card's documentation if you are unsure), the

BUS-MAST.TEC technote in the \QEMM\TECHNOTE directory will help you

resolve the conflict.  Otherwise, the EXCLUDE.TEC technote in the

\QEMM\TECHNOTE directory will help you resolve the upper memory

conflict.  You are finished with this technote.

???  IF THE EXCEPTION ERROR STILL OCCURS:  There is no upper memory

conflict.  You should proceed to STEP 4.

STEP 4

TEST THE CONFIGURATION WITH DOS'S MEMORY MANAGERS

-------------------------------------------------

In this step, you will try to recreate the conflict without QEMM

installed.  If the conflict occurs without QEMM on the system then QEMM

is not involved in the conflict.

Edit your CONFIG.SYS with your favorite text editor, locate the

QEMM386.SYS line, and at the beginning of that line, add the word REM, as

in the example below:

REM DEVICE=C:\QEMM\QEMM386.SYS <additional parameters>

Directly below this line, add the following two lines:

DEVICE=C:\DOS\HIMEM.SYS

DEVICE=C:\DOS\EMM386.EXE ON RAM 2048

These two lines assume that HIMEM.SYS and EMM386.EXE are in the directory

\DOS.  If they are in some other directory, replace the word DOS in the

above two lines with the correct directory name.  If there are any

eXcludes or Includes on your QEMM386.SYS line that are necessary to boot

the machine, add them to the EMM386.EXE line as well.  Save your changes

and reboot the machine.  After rebooting, QEMM will NOT be loaded.

Attempt to reproduce the conflict.

???  IF A CONFLICT DOES NOT OCCUR:  If you're loading DOS's memory

managers and you have followed every step of this technical note,

then further troubleshooting is required.  Contacting our technical

support department is the quickest and easiest way to resolve the

conflict.  Please mention that you have followed this technical note

and were instructed by STEP 4 to contact Quarterdeck for additional

troubleshooting steps.

???  IF A CONFLICT STILL OCCURS:  QEMM is NOT involved with the conflict;

you are finished with this technote.  Since Exception errors will

generally only display with QEMM enabled, the symptoms of a

remaining problem with the DOS memory managers will most likely be a

machine hang.  If a conflict still occurs without QEMM loading,

further QEMM troubleshooting will not affect the conflict.  You

should consult the documentation that comes with the software that

generated the Exception error with QEMM or causes a hang with HIMEM

and EMM386, or contact the manufacturer of the program.

Our technical support department is easily reached via electronic

channels such as CompuServe (GO QUARTERDECK), Internet (mail

SUPPORT@QDECK.COM or the comp.os.msdos.desqview Usenet newsgroup),

the Quarterdeck BBS (310-314-3227), or fax (310-314-3217).  When

contacting Quarterdeck, be sure to fully explain the symptoms of the

conflict and the results of the tests performed while following this

technical note.

You can also call our Technical Support line at 310-392-9701 for

further assistance.  When you call, please be at the machine that is

experiencing the conflict.

APPENDIX

OBTAINING THE LATEST QEMM VERSION

---------------------------------

If you currently have a version of QEMM earlier than QEMM 7, you should

upgrade to ensure complete compatibility with all of the latest software

and hardware.  Contact the Quarterdeck Upgrade Department at 800-354-3222

or 310-314-3222 for special upgrade pricing.  Please note that we give

you a special upgrade price ONLY after you have registered your product!

If you are using QEMM 7 and would like to update your QEMM version to the

latest maintenance release, free update patches are available on

Compuserve (GO QUARTERDECK, LIB 10), the Quarterdeck BBS (310-314-3227),

anonymous FTP (qdeck.com), or BIX (join desqview).  The updated version

can also be ordered from Quarterdeck's Customer Service dept

(800-354-3222) for the cost of shipping & handling.

*          Copyright (C) 1994 by Quarterdeck Office Systems            *