ORIGINAL DATE:  11-17-92

REVISION DATE:  11-23-94

## INTERGRAPH CORPORATION

SOFTWARE TECHNICAL NOTE

STN#:		G94-021

DISCIPLINE:		GRAPHICS

PRODUCT:		MICROSTATION

PLATFORM:		ALL SUPPORTED

VERSION:		4.0 - 5.0

LEVEL/TYPE:		Beginner/Information

TITLE:		Working in Both English and Metric Units

The purpose is the subject and what the procedure is meant to accomplish.

ABSTRACT:		Changing the working units in a design file after placing  elements is ordinarily

discouraged because doing so haphazardly scales the elements in the design.

However, it is possible to set up and change working unit settings without

scaling the elements by initially setting up and then maintaining working units

that specify a known relationship between a Metric unit and an English unit.

This produces the benefit of allowing users to alternate between English and

Metric systems for displaying measurements and dimensions.

PREPARED BY:	Sandra K. Harrison-Popp

CSSS, MicroStation Support

COPYRIGHT:		Copyright 1994, Intergraph Corporation.

All rights reserved.

Including software, file formats, and audio-visual displays; may be used pursuant to applicable 	software license agreement; contains confidential and proprietary information of Intergraph and/or third 	parties which is protected by copyright and trade secret law and may not be provided or otherwise 	made available without proper authorization.

RESTRICTED RIGHTS LEGEND.

Use, duplication, or disclosure by the government is subject to restrictions as set forth in 	subparagraph (c) (1) (ii) of the Rights in Technical Data and Computer Software clause of DFARS 	252.227-7013 or subparagraphs (c) (1) and (2) of Commercial Computer Software -- Restricted 	Rights at 48 CFR 52.227-19, as applicable.

Unpublished -- rights reserved under the copyright laws of the United States.

Intergraph Corporation, Huntsville, Alabama  35894-001

# Technical Data: Technical Data:

Setting up the d

None of the English or Metric seed files supplied with MicroStation have working unit settings that establish a relationship between a unit in each system.  Settings establish the relationship between a unit in each of these systems, so it is recommended that users create a custom seed file.  To accomplish this:

1.	Open one of the supplied seed files and use the Working Units Settings box to specify English or Metric settings.  Assuming the file is set up with English units, the following English working units settings are based on the relationship between inches and millimeters (1” = 25.4 mm).  (The corresponding working areas are shown for information purposes only.)

Master 	Sub Units 	Sub-units	Positional Units	 Working Area In

Units		       per Master unit	   Per Sub-unit		   Master Units

‘ (ft)	     “(in)		      12		           254		        1409110

‘ (ft)	     “(in)		      12		         2540		          140911

‘ (ft)	     “(in)		      12		       25400		            14091

‘ (ft)	     “(in)		      12		     254000		              1409

2.	Save the settings by choosing Save Settings from the File menu.

3.	Save the seed file with a different name, then create the design file bycopying the new seed file.  It is now ready to be used.

To set up the file with Metric settings, see “Changing between English and Metric working units”  in this document for corresponding Metric field values.

The following table shows English settings based on the relationship 1” = 25.4 mm, and also displays the corresponding Metric settings with master units of meters (m) and sub-units of millimeters (mm).

English				      Metric

Sub Units 	 Positional Units	    Sub Units		Positional Units

Per Master Unit	   Per Sub Unit		Per Master Unit	   Per Sub Unit

12		           254		        1000			    10

12		         2540		        1000			  100

12		       25400		        1000			1000

12		     254000		        1000		           10000

For example, suppose the design file is set up (before you began to work) with the following English working units:

·   master units of feet (‘)

·   sub-units of inches (“)

·   12 sub-units per master unit (because 1‘ = 12“)

·   254 positional units per sub-unit (based on the relationship 1” = 25.4 mm)

The corresponding Metric settings are as follows (see the first row in the above table):

·   master units of meters (m)

·   sub-units of millimeters (mm)

·   1000 sub-units per master unit (because 1 m = 1000 mm)

·   10 positional units per sub-unit (because 254 pu/10 = 25.4)

User command

You can create a user command to switch between English and Metric measuring units.  The user command (UCM) code is listed at the end of this note.

To create the user command:

1.	Using a text editor, type the UCM code into a text file named “eng_met.ucm.”

2.	(Optional) Customize the UCM to reflect the working units in your design by

editing the SET statements before the START label. These statements define

corresponding English and Metric settings.  The SET statements in the code

shown below are for working units based on the relationship 1” = 25.4 mm  and

a working area of 1,409,110’ (429,496 m) (as in the examples above).

3.	Copy the file into MicroStation’s “ucm” directory.

To run the UCM:

1.	Load MicroStation and open the design file.

2.	Key in UC=eng_met

(The system prompts you to select English or Metric working units.)

3.	To change to English working units, key in E,

or

To change to Metric unit display, key in M

The UCM code follows:







	

		

		

		

		

		

	

		



































































